3GPP TSG-RAN WG3 Meeting #125	R3-244381

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	17.3

Source:	Nokia

Title:	Open Issues in support of On-Demand SIB1

Document for:	Discussion

# 1	Introduction

According to the Rel-19 NES WID (RP-234065), support of on-demand SIB1 for UEs in idle/inactive mode shall be studied by RAN1, RAN2 and RAN3 with a check-point for normative work at RAN#105 (September 2024). In this context RAN1 also evaluates the NES gain achievable by this technique.

In [1], the list of open issues and initial feedback from companies was collected in RAN3#123-bis.

In this paper we provide our view on these issues, their potential RAN3 impacts and challenges, based on the current discussion in RAN1.

# 2	Discussion

It was agreed in RAN3#124 to focus on Case 2 where the UE obtains the UL WUS configuration from Cell A and transmits UL WUS to NES Cell.

We focus on these open issues assuming the architecture of case 2. Fig.1 is the call flow that shows the RAN3 signalling of On Demand SIB1 activation procedure for the Case 2.

Open issues that are related to the same topic are shown together for better readability.

Issues related to UL WUS configuration provisioning:

Issue 1: The WUS configuration signalling to support NES Cell to request WUS configuration provision in Cell A and to support NES cell to receive WUS configuration provision status feedback from Cell A?

Issue 2: The WUS configuration signalling between NES gNB-DU and NES gNB-CU?

Issue 3: Which network node entity decides the content of the WUS configuration?

Issue 4: The UL WUS configuration signalling provision and update from NES cell to Cell A?

Table 3 in [3] was agreed at RAN1#117 to be used as the starting point to discuss the required parameters/contents inside the UL WUS configuration.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| To which cell does the config applies | NES-CellId | PhysCellId |  |
|  |  | ARFCN-ValueNR |  |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResource | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |
|  |  | rsrp-ThresholdSSB |  |
|  |  | prach-RootSequenceIndex |  |
|  |  | msg1-SubcarrierSpacing |  |
|  |  | restrictedSetConfig |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  | frequencyInfoUL | frequencyBandList |  |
|  |  | absoluteFrequencyPointA |  |
|  |  | offsetToCarrier |  |
|  |  | p-Max |  |
|  |  | frequencyShift7p5khz |  |
| SIB1 reception | pdcch-ConfigSIB1 | ssb-SubcarrierOffset |  |
|  |  | controlResourceSetZero |  |
|  |  | searchSpaceZero |  |
| RAR Reception | pdcch-ConfigOD-SIB1-RAR | controlResourceSet |  |
|  |  | monitoringSlotPeriodicityAndOffset |  |
|  |  | duration |  |
|  |  | monitoringSymbolsWithinSlot |  |
|  |  | aggregationLevels |  |

As can be seen, most of those parameters are cell specific (eg: RACH configuration). Based on the cell specific nature of the UL-WUS configuration, it is most appropriate to be configured and managed at the gNB-DU, which provides answer to issue 3.

Observation-1: The WUS configuration consists of cell specific parameters and hence, could be part of the served cell configuration.

Proposal-1: The gNB-DU of the NES cell decides the content of the UL WUS configuration.

The other issues relate to signalling of the UL WUS configuration. In order to illustrate our view, we provide in Fig. 1 a possible call flow. Steps 1-3 capture the WUS configuration signalling from the gNB-DU in the gNB hosting NES cell to gNB-CU hosting the Cell A.

It could be then sent to the gNB-CU over F1 e.g. as part of Served Cell Information.

The gNB-CU (NES) then sends the same informationto the gNB-CU controlling the Cell A over Xn. Note that the gNB-CU (NES) could consolidate this information across multiple gNB-DUs. The gNB-CU (Cell A) could store this information in a list with the respective Cell ID (Step 2) and then responds (Step 3) with the acknowledgement that the WUS configuration has been saved. This acknowledgement ensures that when the gNB (NES) goes to OD-SIB1 active state later, it can be sure that its WUS Configuration is available with the gNB (Cell A) and could be broadcast to the UE.

Observation-2: gNB-CU (NES) shares the WUS Configuration with the gNB-CU (Cell A) which saves the same and confirms with an acknowledgement. Any subsequent change in the WUS Configuration shall be shared over F1 and Xn. The acknowledgement is required for gNB (NES) to enable OD-SIB1 mode.

Proposal-2: The gNB-DU (NES) shares the WUS configuration with the gNB-CU (NES) over F1 and the gNB-CU then shares the same with the gNB-CU (Cell A) over Xn. The configuration messages are acknowledged over F1 and Xn. Any subsequent changes in the WUS configuration shall be shared over F1 and Xn.

Fig.1: Possible call flow for for Case 2: On-Demand SIB1 Activation

The following issues (5, 8, 9 & 10) relate to activation of the OD-SIB1 mode and associated signalling.

Issue 5: The SIB1 broadcast status indication or On-demand SIB1 operation activation/deactivation indication?

Issue 8: Which network node entity decides operating a cell in periodic or on-demand SIB1 transmission mode?

Issue 9: Signal the SIB1 broadcast status indication or On-demand SIB1 operation activation/deactivation indication from the NES cell to the Cell A over Xn?

Issue 10: On-demand transmission activation/deactivation, include the split architecture?

The proposed signalling for the OD-SIB1 activation is given in steps 4-11 in Fig.1

The first aspect is which node deciding activation and deactivation of the OD SIB1 mode. There are many possible considerations that would be required to decide if a given cell is to operate in OD-SIB1 or normal, periodic SIB1 mode. These could include measurements like the following

- The existing load conditions in the network (this includes NES cell, Cell A and neighbouring cells)

- For an NES cell already in OD-SIB1 mode, the number of UEs requesting OD-SIB1 or number of slots that transmit OD-SIB1 in a configurable time period could be used to disable OD-SIB1 mode.

As can be observed, these measurements are not limited only to DU level ones, but could include CU level measurements and even measurements from neighbouring gNBs.

Observation-3: The measurements required for taking the decision could be at the DU-level/CU-level and network level too (involving neighbouring gNBs). Such measurements are available at the gNB-CU and hence, the decision also is more appropriate at the gNB-CU of the NES cell.

Additionally, the gNB-CU is in charge of both connected mode and idle mode load balancing, and would need to take into account that a UE not supporting OD-SIB1 mechanism (e.g. a pre-Rel-19 UE) can not camp on a cell in OD-SIB1 mode. This means that the decision to enable/disable OD-SIB1 mode is better done at the gNB-CU of the NES cell rather than at the gNB-DU.

Proposal-3: The gNB-CU of the NES Cell decides to enable OD-SIB1 mode in one or more of its NES cells.

For what concerns the associated signalling, in the example provided in Figure 1, the gNB-CU (NES) sends the activation indication along with the cell IDs to the gNB-CUs of the associated Cell As over Xn. This triggers those gNB-CUs to transmit the corresponding UL-WUS configuration (that was shared earlier in step 2) over the air. The gNB-CUs then confirms this action to the gNB-CU of NES Cells in Step 8. Note that the UEs are expected to store the WUS configuration in order to be used when they required SIB1 in the NES cell later.

In Step 9, the gNB-CU (NES Cell) then requests the corresponding gNB-DU over F1 to take the following actions

- Update MIB to indicate that the SIB1 broadcast is stopped

- Stop SIB1 broadcast

It is important to note that the stopping of OD-SIB1 broadcast shall be done in the NES cell only after the Cell A confirms that WUS configuration is being transmitted. Otherwise, there could be a period where UEs have no way of accessing the NES cell at all (ie SIB1 stopped and no WUS configuration available). The acknowledgement messages over Xn and F1 ensure that this situation is prevented.

Observation-4: F1 and Xn signalling are required to ensure that the OD-SIB1 activation is done in a split architecture in a safe manner and UEs have a way of accessing the NES cell at all times.

Proposal-4: The gNB-CU of the NES cell communicates the activation and deactivation of the OD-SIB1 mode with the gNB-CU of Cell A over Xn and its own gNB-DUs over F1.

As can be seen in the earlier discussion, there are multiple scenarios which could trigger the enabling or disabling of OD-SIB1 mode and these triggers could be extended in the future, both in terms of the reason and the handling. This calls for a separate activation/deactivation procedure rather than just SIB1 status indication for making the solution future-proof.

Observation-5: Separate activation/deactivation signalling over Xn/F1 is more future proof to support the different, related scenarios.

Proposal-5: The Xn/F1 signalling could be based on activation/deactivation indication.

Issues related to NES Cell/Cell A Mapping:

Issue 6: There might be multiple NES cells under a single Cell A?

Issue 7: There might be multiple Cell A for a single NES cell?

Given that the Cell A is likely to be a coverage cell that is always available and the NES cell a capacity cell that could be available or not in a dynamic fashion, it is quite possible that a single Cell A could be much larger in area as compared to the NES cell. This could lead to one Cell A supporting multiple NES cells without any impact to its own performance and capacity. So, this option is possible and should be supported.

Observation-6: The Cell A is likely to be a coverage cell and NES cell – a capacity cell – this could lead to the prospect of one Cell A supporting multiple NES cells and hence, this deployment option should be supported.

Proposal-6: A given Cell A could support multiple NES cells.

Mapping multiple Cell As with a single NES cell could enable the resilience of the OD-SIB1 solution. In the case where the Cell A fails or is removed, then the NES cell will be left with no supporting cell and hence, will not be reachable at all by a UE. In such a scenario, having another Cell A mapped to the same NES cell could enable a UE to access the NES cell. Such a mapping could also be enabled after the 1st Cell A fails (so that the NES cell could mark a 2nd cell as its Cell A). So, such a deployment option should be provided to improve the overall solution robustness.

Observation-7: Mapping multiple Cell A’s to a single NES cell shall improve the overall robustness of the OD-SIB1 solution and should be supported.

Proposal-7: A given NES cell could be mapped to multiple Cell As.

Note : RAN2 has already agreed that the above 2  cardinality options should be supported in RAN2#126 [4]

Issue 11: Cell A needs to be aware of the SIB transmission mode (current ES state) of all the NES cells under its coverage. Accordingly, it can decide which WUS configurations are to be broadcast?

The Cell A is a supporting role in the OD-SIB1 solution. As per the reasons given in Observation-5, it is the gNB-CU of the NES cell that has all the information to decide on enabling/disabling OD-SIB1 mode. The Cell A just broadcasts the WUS configuration of those cells that are marked by the NES Cell to operate in OD-SIB1 mode. In a case where the Cell A is mapped to multiple NES cells, it is not mandatory for it to know the status of all those NES cells. It just broadcasts based on the latest NES cell status that it has.

Observation-8: The cell A being a supporting role, and just broadcasts the WUS configuration of all those cells that are marked as OD-SIB1 in the latest update by the NES cell. It is not mandatory that the Cell A has the OD-SIB1 status of all its associated NES cells.

Proposal-8: Cell A need not be aware of the OD-SIB1 status of all the NES cells under its coverage at all times.

# 3	Conclusions

Observation-1: The WUS configuration consists of cell specific parameters and hence, could be part of the served cell configuration.

Proposal-1: The gNB-DU of the NES cell decides the content of the WUS configuration.

Observation-2: gNB-CU (NES) shares the WUS Configuration with the gNB-CU (Cell A) which saves the same and confirms with an acknowledgement. Any subsequent change in the WUS Configuration shall be shared over F1 and Xn. The acknowledgement is required for gNB (NES) to enable OD-SIB1 mode.

Proposal-2: The gNB-DU (NES) shares the WUS configuration with the gNB-CU (NES) over F1 and the gNB-CU then shares the same with the gNB-CU (Cell A) over Xn. The configuration messages are acknowledged over F1 and Xn. Any subsequent changes in the WUS configuration shall be shared over F1 and Xn.

Observation-3: The measurements required for taking the decision could be at the DU-level/CU-level and network level too (involving neighbouring gNBs). Such measurements are available at the gNB-CU and hence, the decision also is more appropriate at the gNB-CU of the NES cell.

Proposal-3: The gNB-CU of the NES Cell decides to enable OD-SIB1 mode in one or more of its NES cells.

Observation-4: F1 and Xn signalling are required to ensure that the OD-SIB1 activation is done in a split architecture in a safe manner and UEs have a way of accessing the NES cell at all times.

Proposal-4: The gNB-CU of the NES cell communicates the activation and deactivation of the OD-SIB1 mode with the gNB-CU of Cell A over Xn and its own gNB-DUs over F1.

Observation-5: Separate activation/deactivation signalling over Xn/F1 is more future proof to support the different, related scenarios.

Proposal-5: The Xn/F1 signalling could be based on activation/deactivation indication.

Observation-6: The Cell A is likely to be a coverage cell and NES cell – a capacity cell – this could lead to the prospect of one Cell A supporting multiple NES cells and hence, this deployment option should be supported.

Proposal-6: A given Cell A could support multiple NES cells.

Observation-7: Mapping multiple Cell A’s to a single NES cell shall improve the overall robustness of the OD-SIB1 solution and should be supported.

Proposal-7: A given NES cell could be mapped to multiple Cell As.

Observation-8: The cell A being a supporting role, and just broadcasts the WUS configuration of all those cells that are marked as OD-SIB1 in the latest update by the NES cell. It is not mandatory that the Cell A has the OD-SIB1 status of all its associated NES cells.

Proposal-8: Cell A need not be aware of the OD-SIB1 status of all the NES cells under its coverage at all times.

# References

[1] R3-243868	Summary of offline discussions: Support on-demand SIB1 for UEs

[2] R1-2400181, On-demand SIB1 for Idle/Inactive mode UEs, Nokia, Nokia Shanghai Bell

[3] R1-2405106, Study of on-demand SIB1 for UEs in idle inactive mode for NES

[4] R2-2405701 Report from session on V2X/SL, R19 NES and MOB