3GPP RAN WG3 Meeting #125	R3-244511

# Maastricht, Netherlands, 19th – 23rd August 2024

Source:	CATT

Title:	(TP for TR 38.743) Enhancement of UE performance metrics

Agenda Item:	11.4

Document for:	Discussion and decision

- Introduction

Last meeting RAN3 decided that the UE performance metrics introduced in the Rel-18 AI/ML WI are kept as per-UE ones (and clarified as average among all DRBs), whereas finer granularity are to be discussed in the Rel-19 AI/ML SI.

- Discussion

  - Cases that per-UE average may not work well

The UE performance value in current RAN3 specs is a per-UE average among all DRBs as agreed last meeting. We consider this value prone to disruptions caused by adding or removing DRBs, and may not be sufficient enough to indicate the service status for flows with relatively stricter QoS requirements.

For adding or removing DRBs, assuming that there are two DRBs at the source side, one with strict delay requirement and one with loose delay requirement. After handover, a new DRB is established for the UE with loose delay requirement, and thus the per-UE average delay will become longer even if the performance for the old DRBs are kept the same. Or on the other side, the DRB with loose delay requirement is released, and thus the per-UE average delay will become significantly shorter even if the performance for the kept DRB does not change. If such per-UE average delay is provided back to the source node, the source will clearly misevaluate the decision it made.

For the sufficiency, assume that there are two flows for a UE, one requires strict packet delay / packet loss whereas the other requires loose one.

- The load in the source node is low so the packet delay / packet loss for both flows are quite low, well within the requirement of the “strict” flow.

- The load in the target node, on the other side, is relatively high, but still capable to serve the “strict” flow well. The “loose” flow, on the other side, suffers a relatively larger packet delay / packet loss, but still well within the QoS requirement of the “loose” flow itself. As a result, the average packet loss / packet delay after handover increases significantly.

However, such increase in packet loss / packet delay is not essential. What is important is the packet delay / packet loss of the “strict” flow, not the “loose” one, but the source node cannot know how the packet delay / packet loss of the “strict” flow changes by receiving the per-UE average packet delay / packet loss from the target node.

Observation 1: The source node may misunderstand the performance if DRBs are added or removed after handover, and cannot get virtually any information for flows with strict QoS requirement if there are also flows with loose QoS requirement for the considered UE.

  - Options for finer granularity

Last meeting there were some options raised for finer granularity of UE performance, including per-QoS-flow-group option and per-slice option.

The rationale to consider per-QoS-flow-group granularity instead of per-QoS-flow granularity is that all of the measurements are measured per DRB which is mapped from a group of QoS flows. So, it would always be the case that a group of QoS flows share the same measurement results. As to why DRB level granularity is not proposed, it is because the DRB ID for each configured DRB may change during handover procedure and DRB ID configured at the target node may be not interpretable by the source node. Obviously, Per-QoS-flow-group granularity is measureable and simultaneously effective in avoiding duplicated information transfer (comparing to per-QoS-flow granularity) on Xn interface.

Observation 2: Per-QoS-flow-group-level performance feedback is measurable and could also avoid duplicated information transfer on Xn interface.

For per-slicing option, since flows with low QoS requirements and flows for high QoS requirements may still coexist within in one slice, only making use of this option e.g. derived slicing level measurement via averaging the measurement result of all DRBs belong to this slice, still could not provide very accurate/valuable performance feedback

Observation 3: Adding only per-slice UE performance cannot completely solve the problem, as measurement results may still be mixed among QoS flows.

Nevertheless, we still consider per-slice measurements may be useful for some cases, e.g. for the slice optimisation use case, therefore we consider it acceptable to add per-slice metrics along with per-QoS-flow-group metrics.

Proposal: To introduce per-QoS-flow-group UE performance metrics and may also consider per-slice UE performances, depending on the discussion of the slice optimisation use case.

- Conclusion

Observation 1: The source node may misunderstand the performance if DRBs are added or removed after handover, and cannot get virtually any information for flows with strict QoS requirement if there are also flows with loose QoS requirement for the considered UE.

Observation 2: Per-QoS-flow-group-level performance feedback is measurable and could also avoid duplicated information transfer on Xn interface.

Observation 3: Adding only per-slice UE performance cannot completely solve the problem, as measurement results may still be mixed among QoS flows.

Proposal: To introduce per-QoS-flow-group UE performance metrics and may also consider per-slice UE performances, depending on the discussion of the slice optimisation use case.

The impact on the TR is shown in the annex.

- Annex: Impact on TS 38.743

## 5.x	Finer granularity of UE performance

Editor Note: Capture the description and its potential standard impacts.

### 5.x.1 Use case description

In Rel-18, UE performance measurement results are used as feedback for AI/ML training and monitoring for all the three use cases, and it is supported to be exchanged over XnAP. However, for each type of UE performance only one value per UE can be exchanged over XnAP, which is prone to pollution caused by adding or removing DRBs, and is often useless for services with strict QoS requirements. Therefore introducing finer granularity of UE performance is beneficial.

### 5.x.2 Potential standard impacts

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18:

Xn interface:

- Per-QoS-flow-group measured UE performance (each QoS-flow-group corresponds to one DRB at the node which performs the measurement).

- Per Slicing measured UE performance (depending on the discussion in Slicing topic)