3GPP TSG-RAN WG3 Meeting #125	R3-244574

Maastricht, Netherlands, Aug 19th – 23th, 2024

Agenda Item:	11.4

Source:	CMCC

Title:	(TP to 38.743) Discussion on AI/ML based Mobility Optimization for NR-DC

Document for:	Discussion and Decision

# 1 Introduction

In RAN3#124 meeting, there was discussion on AI/ML based Mobility optimization for NR-DC and the agreement was achieved as below:

NR-DC mobility optimization:

Enhance the Dual Connectivity procedure (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance.

In this contribution, we would further discuss on the scenarios and solutions for NR-DC mobility optimization and provide some proposals.

# 2 Discussion

For NR-DC, there are many sub-scenarios supported in 3GPP, for example, SN Addition, SN Change (MN/SN initiated), inter-Master Node handover with/without SN change, etc. We think the scenarios of Mobility Optimization for NR-DC is one of the key issues at the beginning of the discussion. Considering the potential benefits and complexity, we think SN Addition and MN-initiated SN change should be considered as the baseline in Rel-19 Mobility Optimization for NR-DC. And if time allows, inter-MN handover scenario with/without SN change could be also considered in Rel-19 to enable AI/ML based Mobility optimization in more NR-DC scenarios.

Proposal 1: RAN3 considers the following scenarios in Rel-19 Mobility Optimization for NR-DC.

- SN Addition

- MN-initiated SN change

- Inter-MN handover scenario with/without SN change (if time allows)

In Rel-18, the UE trajectory prediction and measured UE trajectory collection for first-hop target NG-RAN node was supported. The complete mechanism was defined, including the information included in predicted UE Trajectory and measured UE Trajectory, the signaling procedures of UE Trajectory prediction and measured UE Trajectory collection, and the Configuration of UE Trajectory Collection, etc. From our point of view, we could consider the normative work of Mobility Optimization for NR-DC from the above aspects.

Prediction Information of Mobility Optimization for NR-DC

In the case that AI/ML model deployed in NG-RAN node has the capability to support the mobility prediction in NR-DC and the enough data collection for AI/ML model training is available, we think the mobility prediction (e.g., Cell-based Predicted UE Trajectory) in NR-DC can be used as the assistance information by MN for the target SN decision and by target SN for resource reservation. And we think the following prediction information could be useful for NR-DC Mobility Optimization:

- The predicted information of PCell and PSCell, which could include the list of PCells w/o PSCells, that means each of PCell is indicated together with the expected PSCell(s) only in the case of dual-connectivity, e.g., one or multiple pairs of PCell ID and PSCell ID;

- The time duration that UE stay in dual-connectivity or the time duration of connectivity with the PSCell in each pair of PCell and PSCell;

- The predicted UE traffic, e.g., the predicted data volume to be transmitted by SN(s) for SN Addition and MN-initiated SN change or the predicted total data volume;

- The related information of SCG activation and/or deactivation, e.g., the time duration of SCG activation for each pair of PCell and PSCell.

In the above information, the predicted PCells and PSCells is helpful for MN to select the target SN; the time duration of connectivity with the PSCell and SCG activation for each pair of PCell and PSCell could help MN to decide whether to set up the dual-connectivity for UE; and the predicted UE traffic could help the target SN to decide the proper resource allocation for UE.

Observation 1: The predicted PCells and PSCells is helpful for MN to select the target SN.

Observation 2: The predicted UE traffic could help the target SN to decide the proper resource allocation for UE.

Observation 3: The related information of SCG activation/deactivation or Dual Connectivity could help MN get to know the predicted utilization of Dual Connectivity and further decide whether to set up the dual-connectivity for UE.

Therefore, we have the proposals as below.

Proposal 2: RAN3 considers to support the predicted PCells and PSCells as the baseline of NR-DC Mobility Optimization in Rel-19.

Proposal 3: The time duration that UE stay in dual-connectivity or the time duration of connectivity with the PSCell in each pair of PCell and PSCell;

Proposal 4: The predicted UE traffic could be considered as the prediction information for NR-DC Mobility Optimization.

Proposal 5: The related information of SCG activation/deactivation (e.g., the time duration of SCG activation) and the time duration of connectivity with each PSCell could be considered as the prediction information for NR-DC Mobility Optimization.

Regarding to AI/ML model input, we think the current or historical UE Trajectory (e.g., the combination of PCells and PSCells), UHI/MHI and the resource status information of MN and the candidate target SN(s) could be considered as the inputs of AI/ML model for model training or inference.

Proposal 6: The current or historical UE Trajectory (e.g., the combination of PCells and PSCells), UHI/MHI and the resource status information of MN and the candidate target SN(s) could be considered as AI/ML model input used for model training or inference.

The collection of measured information of Mobility Optimization for NR-DC and the configuration

As per the agreement achieved in RAN3#124 meeting, the collection of measured UE performance shall be supported as the feedback in Rel-19. Besides that, we think the actual combination of PCells and PSCells is also necessary for AI/ML model performance testing and model retraining. Therefore, the following information could be considered in the collection of measured information for NR-DC Mobility Optimization:

- The measured UE Trajectory information, e.g., the actual combination of PCells and PSCells;

- The actual data volume provided by the target SN(s)/PSCell(s);

- The information of actual utilization of Dual Connectivity, e.g., the time duration of SCG activation and the time duration of connectivity with each PSCell;

Proposal 7: RAN3 considers to enhance the Dual Connectivity procedure to trigger the collection of the following information for AI/ML model performance testing and model retraining:

- The actual combination of PCells and PSCells;

- The time duration of connectivity with each PSCell;

- The actual data volume provided by the target SN(s)/PSCell(s);

- The time duration of SCG activation.

In Rel-18, there are four conditions introduced to terminate the collection of measured UE Trajectory:

- The time since UE was successfully handed over to the target node is equal to the value of the Collection Time Duration;

- The configured number of hand-overs within the same target node is reached;

- UE moves to RRC_INACTIVE or RRC_IDLE state;

- UE is handed over to a cell belonging to an NG-RAN node different from the target node.

But from our point of view, some of the existing condition triggers and Configuration of UE Trajectory Collection in Rel-18 maybe not applicable to NR-DC scenarios. For example, the measured UE Trajectory collection starts from successful Handover Execution, and the target node could terminate the collection when UE is handed over to another node. But there is no handover execution in SN Addition scenario or SN change scenario with the same MN for NR-DC. We think the new trigger conditions should be considered to start or terminate the collection of measurement for NR-DC.

Observation 4: Some of the existing condition triggers and Configuration of UE Trajectory Collection in Rel-18 maybe not applicable to NR-DC scenarios.

Proposal 8: The following existing condition triggers and Configuration of UE Trajectory Collection in Rel-18 could be considered to reused for NR-DC scenarios:

- UE moves to RRC_INACTIVE or RRC_IDLE state;

- UE is handed over to a cell belonging to an NG-RAN node different from the target node;

- The configured number of hand-overs within the same target node is reached;

Proposal 9: The new trigger conditions should be considered to start or terminate the collection of measurement for NR-DC.

As mentioned above, there is no handover for SN Addition and MN-initiated SN change, so the collection of measured UE performance and measured UE Trajectory could start from the successful establishment of NR-DC with the target PSCell/SN. For Inter-MN handover scenario with/without SN change, the collection of measured UE performance and measured UE Trajectory could start from the successful Inter-MN Handover Execution, that means UE handover to target MN from source MN successfully. Besides that, similar with SN Addition and MN-initiated SN change, the successful establishment of NR-DC with the target PSCell/SN could also be considered as the trigger condition to start the collection of measured UE performance and measured UE Trajectory for Inter-MN handover scenario.

Proposal 10: RAN3 considered the trigger conditions to start the collection of measurement for NR-DC, for example:

- Successful establishment of NR-DC with the target PSCell/SN (for SN Addition, MN-initiated SN change and Inter-MN handover scenario)

- Successful Inter-MN Handover Execution (for Inter-MN handover scenario)

Proposal 11: The following trigger conditions could be considered to terminate the collection of measurement for NR-DC:

- The time since the successful establishment of NR-DC with the target PSCell/SN is equal to the value of the Collection Time Duration.

- The execution of Secondary Node Release procedure;

- The configured number of PSCells change within the same SN is reached;

- UE moves to SCG deactivation or SCG failure;

Signaling procedures

In light of the agreement achieved in RAN3#123bis meeting [1], we have “inference at the MN” as the study assumption for NR-DC.

Agree to take as baseline for the Mobility Optimization for NR-DC that the use case is studied assuming inference at the MN.

In the case that AI/ML model is deployed in MN, the MN could generate the prediction information of NR-DC in advance. So, we think on one hand, the prediction information generated by the MN could be used by MN itself as the assistance information for target PSCell/SN selection for SN Addition and MN-initiated SN Change. And the prediction information also could be used in the target MN for target PSCell/SN selection for Inter-MN handover scenario. On the other hand, the MN could send the prediction information to the target SN for resource reservation.

Proposal 12: After MN generate UE Trajectory prediction information of NR-DC,

- The MN could use the prediction information as the assistance information for target PSCell selection for SN Addition and MN-initiated SN Change.

- The MN could send the prediction information to the target SN for resource reservation.

- The target MN could use the prediction information as the assistance information for target PSCell selection for Inter-MN handover scenario.

For the collection of measured information of Mobility Optimization for NR-DC, we think the target SN(s) could report the measured information to MN for SN Addition and MN-initiated SN Change. For Inter-MN handover scenario, the target MN should report the measured information to the source MN that generate the prediction information of NR-DC.

Proposal 13: For the collection of measured information for NR-DC,

- The target SN(s) report the measured information to MN for SN Addition and MN-initiated SN Change;

- The target MN reports the measured information to the source MN that generate the prediction information of NR-DC, for Inter-MN handover scenario.

Proposal 14: Agree the TP to TS 38.743 in the annex related to Mobility optimization for NR-DC.

# 3 Conclusion

In this paper, we discuss on the scenarios and solutions for NR-DC mobility optimization, we have the following observations and proposals:

Proposal 1: RAN3 considers the following scenarios in Rel-19 Mobility Optimization for NR-DC.

- SN Addition

- MN-initiated SN change

- Inter-MN handover scenario with/without SN change (if time allows)

Prediction Information of Mobility Optimization for NR-DC

Observation 1: The predicted PCells and PSCells is helpful for MN to select the target SN.

Observation 2: The predicted UE traffic could help the target SN to decide the proper resource allocation for UE.

Observation 3: The related information of SCG activation/deactivation or Dual Connectivity could help MN get to know the predicted utilization of Dual Connectivity and further decide whether to set up the dual-connectivity for UE.

Proposal 2: RAN3 considers to support the predicted PCells and PSCells as the baseline of NR-DC Mobility Optimization in Rel-19.

Proposal 3: The time duration that UE stay in dual-connectivity or the time duration of connectivity with the PSCell in each pair of PCell and PSCell;

Proposal 4: The predicted UE traffic could be considered as the prediction information for NR-DC Mobility Optimization.

Proposal 5: The related information of SCG activation/deactivation (e.g., the time duration of SCG activation) and the time duration of connectivity with each PSCell could be considered as the prediction information for NR-DC Mobility Optimization.

Proposal 6: The current or historical UE Trajectory (e.g., the combination of PCells and PSCells), UHI/MHI and the resource status information of MN and the candidate target SN(s) could be considered as AI/ML model input used for model training or inference.

The collection of measured information of Mobility Optimization for NR-DC and the configuration

Proposal 7: RAN3 considers to enhance the Dual Connectivity procedure to trigger the collection of the following information for AI/ML model performance testing and model retraining:

- The actual combination of PCells and PSCells;

- The time duration of connectivity with each PSCell;

- The actual data volume provided by the target SN(s)/PSCell(s);

- The time duration of SCG activation.

Observation 4: Some of the existing condition triggers and Configuration of UE Trajectory Collection in Rel-18 maybe not applicable to NR-DC scenarios.

Proposal 8: The following existing condition triggers and Configuration of UE Trajectory Collection in Rel-18 could be considered to reused for NR-DC scenarios:

- UE moves to RRC_INACTIVE or RRC_IDLE state;

- UE is handed over to a cell belonging to an NG-RAN node different from the target node;

- The configured number of hand-overs within the same target node is reached;

Proposal 9: The new trigger conditions should be considered to start or terminate the collection of measurement for NR-DC.

Proposal 10: RAN3 considered the trigger conditions to start the collection of measurement for NR-DC, for example:

- Successful establishment of NR-DC with the target PSCell/SN (for SN Addition, MN-initiated SN change and Inter-MN handover scenario)

- Successful Inter-MN Handover Execution (for Inter-MN handover scenario)

Proposal 11: The following trigger conditions could be considered to terminate the collection of measurement for NR-DC:

- The time since the successful establishment of NR-DC with the target PSCell/SN is equal to the value of the Collection Time Duration.

- The execution of Secondary Node Release procedure;

- The configured number of PSCells change within the same SN is reached;

- UE moves to SCG deactivation or SCG failure;

Signaling procedures

Proposal 12: After MN generate UE Trajectory prediction information of NR-DC,

- The MN could use the prediction information as the assistance information for target PSCell selection for SN Addition and MN-initiated SN Change.

- The MN could send the prediction information to the target SN for resource reservation.

- The target MN could use the prediction information as the assistance information for target PSCell selection for Inter-MN handover scenario.

Proposal 13: For the collection of measured information for NR-DC,

- The target SN(s) report the measured information to MN for SN Addition and MN-initiated SN Change;

- The target MN reports the measured information to the source MN that generate the prediction information of NR-DC, for Inter-MN handover scenario.

Proposal 14: Agree the TP to TS 38.743 in the annex related to Mobility optimization for NR-DC.

# Reference

- RAN3_123bis_agenda_20240419_EOM, April 2024.

- RAN3_124_agenda_20240524_eom, May 2024.

Annex: TP to TS 38.743

5.1	Mobility optimization for NR-DC

Editor Note: Capture the description and its potential standard impacts.

5.1.1	Use case description

Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only (e.g., no conditional Dual Connectivity procedures are in scope).

<<<<<< Start of CHANGES >>>>>>

The following scenarios of NR-DC are possible:

  - SN Addition

  - MN-initiated SN change

  - Inter-MN handover scenario with/without SN change

5.1.2	Solutions and Potential Standard impacts

The following information can be considered as output data for AI-based Mobility optimization for NR-DC:

  - Predicted PCells and PSCells list

  - Time duration that UE stay in dual-connectivity or the time duration of connectivity with the PSCell in each pair of PCell and PSCell

  - Predicted UE traffic

  - Related information of SCG activation and/or deactivation

The following information can be considered as input data:

  - Current or historical UE Trajectory (e.g., the combination of PCells and PSCells)

  - UHI/MHI

  - Resource status information of MN and the candidate target SN(s)

The following information can be considered as feedback from target node for AI/ML model performance testing and model retraining:

  - Measured UE performance

  - Actual combination of PCells and PSCells

  - The time duration of connectivity with each PSCell,

  - Actual data volume provided by the target SN(s)/PSCell(s)

  - Time duration of SCG activation

The new trigger conditions can be considered to start or terminate the collection of measurement for NR-DC:

  - Trigger conditions to start the collection, e.g., Successful establishment of NR-DC, Successful Inter-MN Handover Execution.

  - Trigger conditions to terminate the collection, e.g., the time since the successful establishment of NR-DC with the target PSCell/SN is equal to the value of the Collection Time Duration, SN release, UE moves to SCG deactivation or SCG failure.

<<<<<< End of CHANGES >>>>>>