3GPP TSG-RAN WG3 Meeting #R3-125		R3-244034

Maastricht, NL, 19 - 23 August 2024

Agenda Item:	11.3

Source: 	Jio

Title:  	Further Enhancements on AI/ML CCO as SON use case

Document for:	Discussion and decision

# Introduction

This discussion is mirrored to Jio document R3- 244035 “AI/ML CCO as SON use case” [1].

According to prior discussions in Rakuten R3-243767 [2], regarding the Rel-19 SI “RP-234054” focusing on enhancements for AI/ML in NG-RAN [3], the integration of Artificial Intelligence (AI) and Machine Learning (ML) plays a critical role in Rel. 18 & Rel.19 5G RAN. The study of two objectives was proposed, i.e. Network Slicing and CCO, with existing NG-RAN interfaces including non-split architecture and split architecture, and the further discussions on the Rel-18 leftovers as candidates for normative work, based on the Rel-18 principles.

This contribution will discuss the SON CCO use case with the focus on identifying and rectifying the CCO issues using autonomous AIML methods shared across the serving node and the neighbor nodes via Xn signaling.

# Discussion

At the heart of SON lies the Coverage and Capacity Optimization (CCO) function, which plays a critical role in autonomously managing and optimizing network performance. The CCO function's primary objective is to identify, address, and mitigate issues related to coverage and capacity within the RAN, ensuring robust and reliable communication services. The significance of the CCO function in SON is presented by detailed documentation in technical standards such as TS 37.320 [3]. This standard provides a comprehensive framework for understanding the various CCO challenges and the methodologies employed to overcome them. By leveraging advanced algorithms and real-time data analysis, the CCO function enables self-healing and self-optimization capabilities within the network, thus reducing the need for manual intervention and enhancing overall network performance.

The NG-RAN node may determine CCO issues based on several observations including UE radio measurements, Radio Link Failure, Mobility performance, Radio Connection Establishment Failure (i.e. RACH accessibility) and observed performance (e.g. throughput, packet loss) of the attached UEs.

On a next reactive step, when a CCO issue is detected, the NG-RAN node may perform dynamic cell-level and beam-level coverage configuration adjustments and changes, including SSB and traffic beam power parameters, beam-width adjustments, beam steering adjustments and potential dynamic beam switching actions.

Οne NG-RAN node shall execute the AI/ML training and the corresponding inference in the local node and then inform the neighboring NG-RAN nodes about the change and the corresponding CCO cause, for e.g. “coverage” or “cell edge capacity” or “cell edge interference” or “insufficient SINR performance for Accessibility”. A node receiving coverage configuration change information from its neighbors may similarly adjust its own CCO configuration, taking into account the CCO cause.

In the split gNB architecture, the CCO function resides in the gNB-CU.

For a NG-RAN node to make informed decisions about the CCO configuration changes to alleviate the detected CCO issue, it relies on various parameters like beam management information, Power control parameters (i.e. SSB beam boosting, UL PUSCH/PUCCH PC), UE measurement reports, and conditionally neighboring NG-RAN nodes’ coverage information.

Furthermore, it relies on the AIML based prediction of the load conditions of the cells under consideration.

Proposal 1: When a CCO issue is detected in a specific NG-RAN node, it shall autonomously adjust its CCO configurations dynamically and in coordination with the neighbor nodes.

Proposal 2: When an NG-RAN node receives the CCO configuration status and causes update from another NG-RAN node via Xn signaling, it may adjust its own CCO configurations based on that received update.

Proposal 3: To support CCO operations, the NG-RAN node relies on cell-level and UE-level parameters from the serving and the neighboring NG-RAN nodes.

Following the analysis in document [1] the Functional framework for AI/ML in Air interface is properly presented as in the figure 4.4-1 from 3GPP TR 38.843 [5], with the proper framework analysis including Data Collection, Model Training, Management, Inference and Model Storage.

Furthermore, 3GPP TR 28.908 [6] outlines the management capabilities required for AI/ML across different phases: training, validation, testing, deployment, and inference. This includes aspects such as ML entity loading, inference control, and performance evaluation. The workflow in figure 4.3.1-1[6] involves 3 main phases; the training, deployment and inference phase, including the main operational tasks for each phase.

Figure 4.3.1-1: AI/ML operational workflow

During last RAN3 meeting it was agreed that for AI/ML based CCO [7] that the AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the NG-RAN node (gNB-CU), or as an alternative the AI/ML Model Training and AI/ML Model Inference may be both located in the NG-RAN node (gNB-CU). To support AI/ML in NG-RAN, inputs from local serving node and the neighbours NG-RAN are required:

- Inputs from local node:

- Measured and/or Predicted (i.e. AI/ML) Radio Resource Status,

- Predicted Coverage status, including both SSB beam-width adjustments and SSb beam azimuth/elevation steering corrections.

- Measured/Predicted UE Traffic (e.g., data volume)

- Inputs from neighboring node:

- Measured and/or Predicted (i.e. AI/ML) Radio Resource Status

- Predicted Coverage status, including both SSB beam-width adjustments and SSb beam azimuth/elevation steering corrections.

- Measured/Predicted UE Traffic (e.g., data volume)

- Measured/Predicted UE trajectory

The SON AIML entity proving input to the NG-RAN nodes can reside at OAM and/or the NG-RAN nodes. It is recommended that the training data from the NG-RAN nodes needs to be made available to the AIML entity at either of these locations, depending on the deployment scenario.

Proposal 4: For the CCO use case, AIML model training takes place either at the OAM or at gNB-CU. The trained models are used for inferencing before evaluating the optimal CCO configurations.

Proposal 5: For the CCO use case, AIML inference takes place at the gNB-CU. The trained models in the gNB-CU are used for inferencing before evaluating the optimal CCO configurations.

Following the discussions in [1], Coverage (CCO) and the corresponding SINR can be adjusted based on a joint approach of dynamically shaping the multi SSB beams beam-width along with the proper SSB beam boosting and beam steering angles in azimuth/elevation. These corrections would improve the SINR and keeping the corresponding coverage on a certain desired level.

Proposal 6: For AI/ML based CCO, as for the SON functionality and for the existing SSB beam sweeping functionality on serving node, different SSB beam-width configurations and SSB beam steering in azimuth/elevation  are dynamically and autonomously selected per node to optimize coverage and SINR.

CCO predictions and fault detection information among the corresponding nodes can be shared via Xn signaling.

Proposal 7: The user device is unaffected and follows the specifications as per 3GPP and GSMA.

The autonomous SON AI/ML proposed algorithm, in order to support AI/ML optimization functions in NG-RAN, gets inputs from connected UEs in the serving cell, the serving cell itself and the neighbour cells, building a cost function to predict the best SSB beam-width and steering angles per optimization period. This CCO optimization is not forwarded to the neighbour cells, thus minimizing the Xn signaling load.

Proposal 8: For AI/ML based CCO, the inference input may be collected from UE and the nodes (serving, neighbors) including:

- Inputs from connected UEs as following:

- connected UE measurement report (e.g., L1 SS-RSRP, SS-RSRQ, SS-SINR measurement, CQI, etc), including cell level and beam level UE measurements

- connected UE performance (HARQ NACK KPI retransmissions (BLER>10%)),

- connected UE location information (e.g., coordinates, serving cell ID)

- idle mode UE’s Random Access performance (RACH success rate, number of RACH attempts) to convert to SINR from existing graph of number of PRACH attempts vs SINR).

- connected UE RLF report

- Inputs from serving node:

- CCO failures detection

- Measured/Predicted radio resource status

-  Measure performance status (i.e. RLF, CEF, RA, cell/user throughput etc)[8]

-  Estimate energy cost function [9]

-  Current CCO State including all necessary parameters (i.e. SSB beamwidth, Gain, etc[8]

- Inputs from neighbor node:

- CCO failures detection

- Measured/Predicted radio resource status

-  Measure performance status (i.e. RLF, CEF, RA, cell/user throughput etc)[8]

-  Estimate energy cost function [9]

-  Current CCO State including all necessary parameters (i.e. SSB beamwidth, Gain, etc[8]

Proposal 9: The future coverage status may be Cell Coverage Modification(s) (i.e. power parameters, SSB boosting, etc) or SSB Coverage Modification(s) (i.e. SSB beam width modifications, beam steering), inter-exchanged via Xn interface to the neighbors.

DL performance is investigated based AI/ML CCO, for SON SSB power boosting, SSB beam-width shaping and steering for different SSB beam gains on the P1 beam-sweeping procedure, building a proper cost function.

Proposal 10: For AI/ML based SON CCO, the future coverage status may include the following NG-RAN gNB or gNB-CU/gNB-DU algorithm steps:

- Preliminary step: receive the Xn-signaling CCO issues from neighbour cells

- Step 1: Start SSB beam sweeping on the cell level with the default (operator configurable) SSB beam-width shape, for all SSB sweeping beams in a 5ms cycle.

- Step 2: Estimate the DL SINR per SSB sweeping beam in a jointly cost function consisting of:

- number of UL PRACH counter attempts (build a graph of PRACH attempts vs SINR) of idle UEs,

- connected UEs UL CQI reports

- HARQ KPI retransmissions (BLER>10%)

- L1-SINR and L1-RSRP reporting

- Step 3: Select the next SSB beam-width shape (in a configurable way) and repeat step 1 and step 2, until all SSB beam-width shapes are exhaustively tested and completed.

- Step 4: Estimate SSB beam coverage footprint as a percentage (%) of default SSB beam shape.

- Step 5: Select the beam with the argmax DL UL_SINR performance and the coverage footprint > configurable threshold, from step 3.

- Step 6: For the selected SSB beam-width shape, adjust the SSB power boosting parameters to improve SINR. Estimate DL SINR based on the selected beam-width shapes.

- Step 7: exchange that info to the neighbour cells via Xn signaling

Proposal 11: Encourage collaboration between different network layers (e.g., physical, MAC, network) to optimize CCO configurations holistically, leveraging AI/ML for cross-layer analysis and decision-making.

Proposal 12: Implement adaptive beamforming techniques that dynamically adjust based on real-time network conditions and AI/ML predictions to improve coverage and capacity.

# Signalling Chart for CCO

Following [10], the AIML model training function could be located at the OAM or the gNB-CU.

In the first case, the training data is continuously sent from the NG-RAN nodes to the AIML entity hosted at the OAM.

In the second case, the training data is available where the AIML training entity resides at the NG-RAN nodes.

UE Measurements

At the NG-RAN nodes, the attached UEs are configured with measurement objects.

The UE measurement reports to the serving NG-RAN nodes include neighboring cells’ RSRP/Q, SINR/CQI measurements, UE location, mobility data and corresponding KPI Performance management info (SSSR, RACH Failure, Handover Failures etc). These reports are a useful indicator of an existing CCO issue.

Data Collection and Model Training

In the context of optimizing Cellular Coverage and Optimization (CCO) within the Next Generation Radio Access Network (NG-RAN), the data collection process is intricate and multifaceted. The training data encompasses a broad spectrum of cell-level parameters, which are crucial for developing robust and accurate AIML models. These parameters include energy cost metrics that reflect the power consumption patterns within the cell, and energy efficiency indicators that provide insights into the performance per unit of energy used. Additionally, CCO status metrics help identify existing coverage and capacity optimization issues, while load metrics offer a snapshot of the cell's traffic conditions and resource utilization.

A significant portion of the training data is derived from UE (User Equipment) measurement reports. These reports are pivotal as they contain detailed information about the network's performance from the user's perspective. Key metrics include the Reference Signal Received Power (RSRP) and Reference Signal Received Quality (RSRQ) of neighboring cells, which help in understanding the signal strength and quality received by the UE. Signal-to-Interference-plus-Noise Ratio (SINR) and Channel Quality Indicator (CQI) measurements provide further granularity on the quality of the radio link. UE location and mobility data are essential for tracking user movement patterns and identifying potential mobility issues that could impact network performance, such as handover failures and ping-pong effects. Additionally, performance management information such as Successful Service Setup Ratio (SSSR), Random Access Channel (RACH) failures, and handover failures are included to provide a comprehensive view of the network's operational efficiency.

The data collection process extends to beam management information, which is crucial in modern NG-RAN architectures that utilize advanced beamforming techniques. Parameters such as SSB (Synchronization Signal Block) beam-width, SSB beam boosting, and SSB beam steering are collected to understand how beams are being managed and optimized. Cell-edge throughput data provides insights into the performance experienced by users at the periphery of the cell, which is often a critical area for CCO improvements. Interference data, which includes information on the levels and sources of interference, is also gathered to help in identifying and mitigating interference issues.

Once collected, this extensive dataset is used to train AIML models. The training process involves using sophisticated algorithms to learn from the historical data and identify patterns and correlations that can predict CCO issues and other network performance problems. The models are continuously updated with new data to ensure they remain accurate and effective. This process of model training is iterative and dynamic, ensuring that the models adapt to changes in network conditions and usage patterns.

The trained AIML models are then deployed at the NG-RAN nodes, where they are used for real-time inference. These models leverage the current cell and UE-level data to provide actionable insights and recommendations for CCO corrective actions. This seamless integration of data collection, model training, and inference enables the NG-RAN nodes to proactively manage and optimize network performance, ensuring optimal coverage and capacity for users. The deployment of these models at the NG-RAN nodes ensures that decisions are made close to where the data is generated, allowing for faster response times and more effective network management.

Inference and Corrective Actions

Once the AIML models are trained and deployed at the NG-RAN nodes, the focus shifts to real-time inference and the implementation of corrective actions. The AIML models, which have been developed using extensive historical data and continuously updated with new data, are instrumental in identifying and addressing CCO (Cellular Coverage Optimization) issues. These models analyze the current cell and UE (User Equipment)-level data, providing a detailed and dynamic understanding of the network's performance. This real-time analysis is crucial as it allows the NG-RAN nodes to make informed decisions swiftly and accurately.

The inference process involves leveraging the trained AIML models to assess the ongoing network conditions. This includes evaluating a multitude of factors such as signal strength, signal quality, user mobility patterns, interference levels, and traffic load. By analyzing these parameters, the models can predict potential CCO issues before they become critical, such as coverage holes, capacity bottlenecks, and areas with high interference. This predictive capability enables proactive management of the network, ensuring that users experience consistent and reliable service.

Upon identifying a CCO issue, the NG-RAN node uses the insights provided by the AIML models to determine the most effective corrective actions. These actions might include adjusting the power levels of cells, changing the antenna tilt, optimizing the beamforming parameters, or redistributing the traffic load across neighboring cells. For example, if a coverage gap is detected, the NG-RAN node may increase the power output or adjust the beam direction of certain cells to improve coverage in the affected area. Similarly, if high interference is identified, the node may alter the frequency allocations or implement advanced interference management techniques to mitigate the problem.

The decision-making process at the NG-RAN nodes is designed to be both autonomous and adaptive. Once a corrective action is determined, the NG-RAN node implements the necessary configuration adjustments immediately. These adjustments are not limited to the affected cell but also consider the impact on neighboring cells, ensuring a holistic approach to network optimization. The NG-RAN node communicates these changes and the underlying reasons to adjacent nodes, maintaining a coordinated effort across the network. This inter-node communication is essential for maintaining overall network stability and performance, especially in densely deployed environments where cells are closely spaced.

Furthermore, the NG-RAN nodes continuously monitor the outcomes of the implemented corrective actions. This monitoring involves collecting data on key performance indicators (KPIs) such as handover success rates, dropped call rates, user throughput, and overall network efficiency. By comparing the pre- and post-adjustment performance metrics, the NG-RAN nodes can validate the effectiveness of the corrective actions. This feedback loop is critical for refining the AIML models, as it provides real-world data that can be used to enhance the accuracy and reliability of future inferences.

In summary, the inference and corrective actions process at the NG-RAN nodes is a sophisticated and dynamic operation. It leverages advanced AIML models to predict and resolve CCO issues proactively, ensuring optimal network performance. By continuously monitoring the network and adapting to changing conditions, the NG-RAN nodes maintain a high quality of service for users, contributing to a seamless and efficient mobile communication experience.

Monitoring and Feedback

Monitoring and feedback are critical components in the ongoing management and optimization of the NG-RAN network, ensuring that CCO (Cellular Coverage Optimization) issues are addressed promptly and effectively. Continuous monitoring enables the NG-RAN nodes to track network performance in real-time, providing the necessary data to validate the effectiveness of the implemented corrective actions and to detect emerging issues before they impact user experience.

The monitoring process involves the collection and analysis of a wide range of performance metrics. The CCO issues may be predicted in advance with the help of AIML methods and/or determined based on current data available at the NG-RAN nodes. The corrective actions can be taken at the NG-RAN nodes in form of CCO configuration updates. Parameters like UE measurement reports, cell-edge performance data, RLF data and HO metrics could be used to monitor and validate the CCO issue. This data is also used for feedback to the AIML training entity for reinforced learning.

This continuous flow of performance data is essential for the feedback loop, which is a vital part of the AIML (Artificial Intelligence and Machine Learning) training process. The feedback loop involves sending the collected performance data back to the AIML training entity, whether it resides at the OAM (Operations, Administration, and Maintenance) or the NG-RAN nodes. This data is used to refine and update the AIML models, ensuring they remain accurate and effective in predicting and mitigating CCO issues. The feedback loop supports reinforced learning, where the models continuously learn from the latest data, improving their ability to anticipate network problems and recommend optimal corrective actions.

Effective monitoring and feedback also facilitate the prediction of CCO issues before they manifest. AIML methods can analyze historical and current data to identify patterns and trends that may indicate an impending coverage or capacity problem. By leveraging these predictive capabilities, NG-RAN nodes can proactively implement configuration updates to prevent issues from affecting the network's performance.

The role of feedback extends beyond model refinement; it also includes validating the impact of corrective actions. After a CCO correction is implemented, the NG-RAN nodes monitor the network to assess the changes' effectiveness. This involves comparing the pre- and post-correction performance metrics to ensure that the adjustments have resolved the identified issues without introducing new problems. If the desired improvements are not observed, the feedback data helps in identifying the reasons and guiding further adjustments.

In conclusion, monitoring and feedback are indispensable for maintaining and enhancing the performance of NG-RAN networks. By continuously collecting and analyzing performance data, NG-RAN nodes can ensure that AIML models are accurately trained and updated, enabling proactive and effective management of CCO issues. This dynamic and iterative process not only improves the quality of service for users but also enhances the overall efficiency and reliability of the network.

# Conclusion

We have the following proposals:

Proposal 1: When a CCO issue is detected in a specific NG-RAN node, it shall autonomously adjust its CCO configurations dynamically and in coordination with the neighbor nodes.

Proposal 2: When a NG-RAN node receives the CCO configuration status and causes update from another NG-RAN node via Xn signaling, it may adjust its own CCO configurations based on that received update.

Proposal 3: To support CCO operations, the NG-RAN node relies on cell-level and UE-level parameters from the serving and the neighboring NG-RAN nodes.

Proposal 4: For the CCO use case, AIML model training takes place either at the OAM or at gNB-CU. The trained models are used for inferencing before evaluating the optimal CCO configurations.

Proposal 5: For the CCO use case, AIML inference takes place at the gNB-CU. The trained models in the gNB-CU are used for inferencing before evaluating the optimal CCO configurations.

Proposal 6: For AI/ML based CCO, as for the SON functionality and for the existing SSB beam sweeping functionality on serving node, different SSB beam-width configurations and SSB beam steering in azimuth/elevation  are dynamically and autonomously selected per node to optimize coverage and SINR.

CCO predictions and fault detection information among the corresponding nodes can be shared via Xn signaling.

Proposal 7: The user device is unaffected and follows the specifications as per 3GPP and GSMA.

Proposal 8: For AI/ML based CCO, the inference input may be collected from UE and the nodes (serving, neighbors) including:

- Inputs from connected UEs as following:

- connected UE measurement report (e.g., L1 SS-RSRP, SS-RSRQ, SS-SINR measurement, CQI, etc), including cell level and beam level UE measurements

- connected UE performance (HARQ NACK KPI retransmissions (BLER>10%)),

- connected UE location information (e.g., coordinates, serving cell ID)

- idle mode UE’s Random Access performance (RACH success rate, number of RACH attempts) to convert to SINR from existing graph of number of PRACH attempts vs SINR).

- connected UE RLF report

- Inputs from serving node:

- CCO failures detection

- Measured/Predicted radio resource status

-  Measure performance status (i.e. RLF, CEF, RA, cell/user throughput etc)[8]

-  Estimate energy cost function [9]

-  Current CCO State including all necessary parameters (i.e. SSB beamwidth, Gain, etc[8]

- Inputs from neighbor node:

- CCO failures detection

- Measured/Predicted radio resource status

-  Measure performance status (i.e. RLF, CEF, RA, cell/user throughput etc)[8]

-  Estimate energy cost function [9]

-  Current CCO State including all necessary parameters (i.e. SSB beamwidth, Gain, etc[8]

Proposal 9: The future coverage status may be Cell Coverage Modification(s) (i.e. power parameters, SSB boosting, etc) or SSB Coverage Modification(s) (i.e. SSB beam width modifications, beam steering), inter-exchanged via Xn interface to the neighbors.

Proposal 10: For AI/ML based SON CCO, the future coverage status may include the following NG-RAN gNB or gNB-CU/gNB-DU algorithm steps:

- Preliminary step: receive the Xn-signaling CCO issues from neighbour cells

- Step 1: Start SSB beam sweeping on the cell level with the default (operator configurable) SSB beam-width shape, for all SSB sweeping beams in a 5ms cycle.

- Step 2: Estimate the DL SINR per SSB sweeping beam in a jointly cost function consisting of:

- number of UL PRACH counter attempts (build a graph of PRACH attempts vs SINR) of idle UEs,

- connected UEs UL CQI reports

- HARQ KPI retransmissions (BLER>10%)

- L1-SINR and L1-RSRP reporting

- Step 3: Select the next SSB beam-width shape (in a configurable way) and repeat step 1 and step 2, until all SSB beam-width shapes are exhaustively tested and completed.

- Step 4: Estimate SSB beam coverage footprint as a percentage (%) of default SSB beam shape.

- Step 5: Select the beam with the argmax DL UL_SINR performance and the coverage footprint > configurable threshold, from step 3.

- Step 6: For the selected SSB beam-width shape, adjust the SSB power boosting parameters to improve SINR. Estimate DL SINR based on the selected beam-width shapes.

- Step 7: exchange that info to the neighbour cells via Xn signalling

Proposal 11: Encourage collaboration between different network layers (e.g., physical, MAC, network) to optimize CCO configurations holistically, leveraging AI/ML for cross-layer analysis and decision-making.

Proposal 12:Implement adaptive beamforming techniques that dynamically adjust based on real-time network conditions and AI/ML predictions to improve coverage and capacity.

# Reference

[1] R3- 244035 “AI/ML CCO as SON use case”

[2] R3-243767, “Identification and Rectification of CCO Issues Using AIML”

[3] RP-234054, "New SID: Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN".

[4] 3GPP TS 37.320, "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[5] 3GPP TR 38.843 “Study on AI ML for NR Air Interface”

[6] 3GPP TR 28.905 “Study on Artificial Intelligence/Machine Learning (AI/ML)”

[7] R3-242231, (TP to TR 38.743) “AI/ML enabled CCO”, Samsung, Ericsson, Huawei, Telecom Italia, ZTE, Nokia, CATT, Deutsche Telekom, LG Electronics, Telecom Italia

[8] Jio’s White paper

[9] Jio Patent P1314

[10] R3-243767 “Identification and Rectification of CCO Issues Using AIML”

[11] Jio patent SYSTEM AND METHOD FOR SELECTIVE MULTI-SSB CONFIGURATIONS AND BEAM SHAPING