3GPP TSG-RAN WG3 Meeting #R3-125		R3-244035

Maastricht, NL, 19 - 23 August 2024

Agenda Item:	11.3

Source: 	Jio

Title:  	AI/ML CCO as SON use case

Document for:	Discussion and decision

# Introduction

Following Rakuten R3-243767 [1], a new Rel-19 SI “RP-234054” focusing on enhancements for AI/ML in NG-RAN [2] was approved in the RAN#102 meeting.

It is well accepted and approved that the integration of Artificial Intelligence (AI) and Machine Learning (ML) plays a critical role in addressing the intricate challenges associated with 5G-Advanced system design. In Release 18, the normative work of NR_AIML_NGRAN-Core has been completed.

In this meeting it was proposed for two objectives, the study of two new AI/ML based use cases, i.e., Network Slicing and CCO, with existing NG-RAN interfaces including non-split architecture and split architecture, and the further discussions on the Rel-18 leftovers as candidates for normative work, based on the Rel-18 principles.

In addition, there are a few Rel-18 topics not finalized within the Rel-18 time frame. Such topics are listed here for further selection/down selection for normative work.

Following [2] the Rel-18 leftovers included:

- the Mobility optimization for NR-DC,

- the Split architecture support for Rel-18 use cases based on the conclusions from Rel-18 WI,

- topics on Energy Saving enhancements (i.e. Energy Cost Prediction),

- Continuous MDT collection targeting the same UE across RRC states

- Multi-hop UE trajectory across gNBs

This contribution will discuss the SON CCO use case with the focus on identifying and rectifying/improve the CCO issues using autonomous AIML methods running solely on the serving node, minimizing the need for Xn signaling.

# Discussion

The concept of Self-Organizing Networks (SON) has emerged as a pivotal innovation aimed at enhancing the efficiency and performance of radio access networks (RAN). At the heart of SON lies the Coverage and Capacity Optimization (CCO) function, which plays a critical role in autonomously managing and optimizing network performance. The CCO function's primary objective is to identify, address, and mitigate issues related to coverage and capacity within the RAN, ensuring robust and reliable communication services.

The challenges that the CCO function seeks to resolve are diverse and complex. These include coverage holes, where certain areas lack sufficient network coverage; weak coverage areas, which experience degraded signal strength; pilot pollution, where overlapping signals interfere with network performance; overshooting coverage, where signals extend beyond their intended range causing interference; and inadequate uplink (UL) coverage, which affects the transmission quality from user devices to the network. These issues, if left unaddressed, can significantly impact user experience and network efficiency.

The significance of the CCO function in SON is underscored by its detailed documentation in technical standards such as TS 37.320 [3]. This standard provides a comprehensive framework for understanding the various CCO challenges and the methodologies employed to overcome them. By leveraging advanced algorithms and real-time data analysis, the CCO function enables self-healing and self-optimization capabilities within the network, thus reducing the need for manual intervention and enhancing overall network performance.

The NG-RAN node may determine CCO issues based on several observations including UE radio measurements, Radio Link Failure, Mobility performance, Radio Connection Establishment Failure (i.e. RACH accessibility) and observed performance (e.g. throughput, packet loss) of the attached UEs.

On a next reactive step, when a CCO issue is detected, the NG-RAN node may perform dynamic cell-level and beam-level coverage configuration adjustments and changes, including SSB and traffic beam power parameters, beam-width adjustments, beam steering adjustments and potential dynamic beam switching actions.

Οne NG-RAN node shall execute the AI/ML CCO training and the corresponding inference autonomously in the local node without informing the neighbor cells in the cluster about the changes. The neighbor nodes will run on their own autonomously the AI/ML CCO and the overall cluster will auto-adjust dynamically in predefined optimization periods the CCO parameters without any further signaling over Xn.

In the split gNB architecture, the CCO function resides in the gNB-CU.

For a NG-RAN node to make informed decisions about the CCO configuration changes to alleviate the detected CCO issue, it relies on various parameters like beam management information, Power control parameters (i.e. SSB beam boosting, UL PUSCH/PUCCH PC), UE measurement reports,

Proposal 1: When a CCO issue is detected, each NG-RAN node shall autonomously adjust its CCO configurations dynamically without any further Xn signaling.

Proposal 2: All other NG-RAN neighbor nodes of a cluster, when a CCO issue is detected on their own cell performance, shall autonomously adjust its CCO configurations dynamically without any further Xn signaling to the neighbors.

Proposal 3: To support CCO operations, the NG-RAN node relies on cell-level and UE-level parameters from the serving node.

3GPP TR 38.843 [4] focuses on enhancing performance through AI/ML models, including potential use cases such as channel state information (CSI) feedback enhancement, beam management, and positioning accuracy improvements. In this standard the functional framework for AI/ML for NR air interface is illustrated in Figure 4.4-1. The aim of this framework is to cover a general functional architecture addressing both model-ID-based LCM and functionality-based LCM, introduced in clause 4.2 [4].

As seen in Figure 4.4-1, the general framework consists of the following:

- Data Collection is a function that provides input data to the Model Training, Management, and Inference functions.

- Training Data: Data needed as input for the AI/ML Model Training function.

- Monitoring Data: Data needed as input for the Management of AI/ML models or AI/ML functionalities.

- Inference Data: Data needed as input for the AI/ML Inference function.

- Model Training is a function that performs AI/ML model training, validation, and testing which may generate model performance metrics which can be used as part of the model testing procedure. The Model Training function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Training Data delivered by a Data Collection function, if required.

- Trained/Updated Model: In case of having a Model Storage function, this is used to deliver trained, validated, and tested AI/ML models to the Model Storage function, or to deliver an updated version of a model to the Model Storage function.

- Management is a function that oversees the operation (e.g., selection/(de)activation/switching/fallback) and monitoring (e.g., performance) of AI/ML models or AI/ML functionalities. This function is also responsible for making decisions to ensure the proper inference operation based on data received from the Data Collection function and the Inference function.

- Management Instruction: Information needed as input to manage the Inference function. Concerning information may include selection/(de)activation/switching of AI/ML models or AI/ML-based functionalities, fallback to non-AI/ML operation (i.e., not relying on inference process), etc…

- Model Transfer/Delivery Request: Used to request model(s) to the Model Storage function.

- Performance Feedback / Retraining Request: Information needed as input for the Model Training function, e.g., for model (re)training or updating purposes.

- Inference is a function that provides outputs from the process of applying AI/ML models or AI/ML functionalities, using the data that is provided by the Data Collection function (i.e., Inference Data) as an input. The Inference function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Inference Data delivered by a Data Collection function, if required.

- Inference Output: Data used by the Management function to monitor the performance of AI/ML models or AI/ML functionalities.

- Model Storage is a function responsible for storing trained/updated models that can be used to perform the Inference function.

- Note: The Model Storage function in Figure 4.4-1 is only intended as a reference point (if any) when applicable for protocol terminations, model transfer/delivery, and related processes. It should be stressed that its purpose does not encompass restricting the actual storage locations of models. Therefore, the specification impact of all data/information/instruction flows (i.e., the arrows in Figure 4.4-1) to/from this function should be studied case by case.

- Model Transfer/Delivery: Used to deliver an AI/ML model to the Inference function.

AI/ML techniques are widely used in 5GS (including 5GC, NG-RAN and management system), and the generic workflow of the operational steps in the lifecycle of an ML model or entity, is depicted in the figure 4.3.1-1. 3GPP TR 28.908 [5] outlines the management capabilities required for AI/ML across different phases: training, validation, testing, deployment, and inference. This includes aspects such as ML entity loading, inference control, and performance evaluation.

Figure 4.3.1-1: AI/ML operational workflow

The workflow in figure 4.3.1-1 involves 3 main phases; the training, deployment and inference phase, including the main operational tasks for each phase. These are briefly described below:

Training phase:

- ML Training: Learning by the Machine from the training data to generate the (new or updated) ML entity (see 3GPP TS 28.105 [6]) that could be used for inference. The ML Training may also include the validation of the generated ML entity to evaluate the performance variance of the ML entity when performing on the training data and validation data. If the validation result does not meet the expectation (e.g. the variance is not acceptable), the ML entity needs to be re-trained. This is the initial step of the workflow. The ML Training MnS is specified in 3GPP TS 28.105 [6].

- ML Testing: Testing of the validated ML entity with testing data to evaluate the performance of the trained ML entity for selection for inference. When the performance of the trained ML entity meets the expectations on both training data and validation data, the ML entity is finally tested to evaluate the performance on testing data. If the testing result meets the expectation, the ML entity may be counted as a candidate for use towards the intended use case or task, otherwise the ML entity may need to be further (re)trained. In some cases, the ML entity may need to be verified which is the special case of testing to check whether it works in the AI/ML inference function or the target node. In other cases, the verification step may be skipped, for instance in case the input and output data, data types and formats, have been unchanged from the last ML entity.

Deployment phase:

- ML Deployment: Deployment of the trained and tested ML entity to the target inference function which will use the subject ML entity for inference.

NOTE:	The deployment phase may not be needed in some cases, for example when the training function and inference function are in the same entity.

Inference phase:

- AI/ML Inference: Performing inference using the ML entity by the inference function.

In telco-grade environments, it is worth noting that the selected learning method (i.e. examples of learning methods captured in table 4.1-1 in 3GPP TS 28.105 [6]) can influence on how AI/ML operational workflow executes. In some cases (e.g. when using supervised learning methods), the inference phase cannot start until training phase gets ended. In other cases (e.g. when using reinforcement learning methods), the inference phase can start while training phase is still in progress.

During last RAN3 meeting it was agreed that for AI/ML based CCO [7]:

- AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the NG-RAN node (gNB-CU).

- AI/ML Model Training and AI/ML Model Inference may be both located in the NG-RAN node (gNB-CU).

Currently, to support AI/ML in NG-RAN, inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results) are required. For AI/ML based CCO, for AI/ML Model Training or AI/ML Model inference, the inference input may be collected from UE or neighbour NG-RAN node (in case of non-split architecture) or neighbour gNB-CU/gNB-DU (in case of split architecture):

- Inputs from UE:

- UE measurement report (e.g., UE SS-RSRP, SS-RSRQ, SS-SINR measurement, etc), including cell level and beam level UE measurements

- UE location information (e.g., coordinates, serving cell ID)

- Random Access performance and RLF report from the UE

- Inputs from local node:

- Measured and/or Predicted (i.e. AI/ML) Radio Resource Status,

- Predicted Coverage status, including both SSB beam-width adjustments and SSb beam azimuth/elevation steering corrections.

- Measured/Predicted UE Traffic (e.g., data volume)

- Inputs from neighboring node:

- Measured and/or Predicted (i.e. AI/ML) Radio Resource Status

- Predicted Coverage status, including both SSB beam-width adjustments and SSb beam azimuth/elevation steering corrections.

- Measured/Predicted UE Traffic (e.g., data volume)

- Measured/Predicted UE trajectory

The SON AIML entity proving input to the NG-RAN nodes can reside at OAM and/or the NG-RAN nodes. It is recommended that the training data from the NG-RAN nodes needs to be made available to the AIML entity at either of these locations, depending on the deployment scenario. The trained models are used at the NG-RAN node for inferencing before evaluating the optimal CCO configurations taking into consideration the real-time measurement and other information elements, as mentioned previously.

Proposal 4: For the SON CCO use case, AIML model training takes place either at the OAM or at gNB-CU. The model inference takes place either at the gNB-CU or gNB-DU

Proposal 5: For the SON CCO use case, AIML model training takes place solely at the gNB-CU. The model inference takes place either at the gNB-CU or gNB-DU.

Coverage and the corresponding SINR can be adjusted based on a joint approach of dynamically shaping the multi SSB beams beam-width along with the proper SSB beam boosting. These corrections would improve the SINR and keeping the corresponding coverage on a certain desired level.

This Discussion document proposes CCO corrective actions as part of a set of different SSB beam shapes with different beam-width configurations and SSB beam power (SSB power boosting), to be dynamically selected and adjusted by the AI/ML optimization algorithm per sector. As per the SON functionality an AI/ML algorithm will dynamically select, among the available beam-width beams and the possible steering azimuth/elevation angle, the proper beam shape on the proper orientation and the most suitable SSB power per individual SSB beam of the beam-sweeping set per sector, under the constraint of desired coverage and SINR.

Proposal 6: For AI/ML based CCO, as for the SON functionality and for the existing SSB beam sweeping functionality on serving node, different SSB beam-width configurations and SSB beam steering in azimuth/elevation  are dynamically and autonomously selected to optimize coverage and SINR.

The autonomous SON AI/ML algorithm will optimize the node CCO performance with the minimum signaling overhead and overload in the Xn interface. The ultimate goal is to use the P1 beam management procedures to adjust the SSb beams and minimize the intra-cell and inter-cell beam overlapping, preserve the sector coverage and maximize the expected SINR for different inter-site distance and geography. The algorithm will update the parameter settings periodically based on a predefined periodicity. Hence ultimately the Multi-SSB beam-width beams will improve the DL coverage and throughput without affecting the UE capability.

Proposal 7: For SON AI/ML based CCO, the proposed algorithm refers to P1 beam management algorithm, It will be an autonomous algorithm selection per node, without the need of any Xn specific signaling, periodically updated to dynamically adjust to changing radio environment conditions.

Proposal 8: The user device is unaffected and follows the specifications as per 3GPP and GSMA.

The autonomous SON AI/ML proposed algorithm, in order to support AI/ML optimization functions in NG-RAN, gets inputs only from connected UEs in the serving cell and the serving cell itself, building a cost function to predict the best SSB beam-width and steering angles per optimization period. This CCO optimization is not forwarded to the neighbour cells, thus minimizing the Xn signaling load.

Proposal 9: For AI/ML based CCO, the inference input may be collected from UE and local node. including:

- Inputs from connected UEs as following:

- connected UE measurement report (e.g., L1 SS-RSRP, SS-RSRQ, SS-SINR measurement, CQI, etc), including cell level and beam level UE measurements

- connected UE performance (HARQ NACK KPI retransmissions (BLER>10%)),

- connected UE location information (e.g., coordinates, serving cell ID)

- idle mode UE’s Random Access performance (RACH success rate, number of RACH attempts) to convert to SINR from existing graph of number of PRACH attempts vs SINR).

- connected UE RLF report

The future coverage status may be Cell Coverage Modification(s) (i.e. power parameters, SSB boosting, etc) or SSB Coverage Modification(s) (i.e. SSB beam width modifications, beam steering). Hence for AI/ML based CCO, the future coverage status may include:

- Cell Coverage Modification including SSB beam power boosting for the Cell Coverage State which is used to solve the predicted CCO issue of the certain predicted cell;

- SSB Coverage Modification including SSB beam width Coverage State which is used to solve the predicted CCO issue of the certain predicted SSB.

DL performance is investigated based AI/ML CCO, for SON SSB power boosting, SSB beam-width shaping and steering for different SSB beam gains on the P1 beam-sweeping procedure, building a proper cost function.

Proposal 10: For AI/ML based SON CCO, the future coverage status may include the following NG-RAN gNB or gNB-CU/gNB-DU algorithm steps:

- Step 1: Start SSB beam sweeping on the cell level with the default (operator configurable) SSB beam-width shape, for all SSB sweeping beams in a 5ms cycle.

- Step 2: Estimate the DL SINR per SSB sweeping beam in a jointly cost function consisting of:

- number of UL PRACH counter attempts (build a graph of PRACH attempts vs SINR) of idle UEs,

- connected UEs UL CQI reports

- HARQ KPI retransmissions (BLER>10%)

- L1-SINR and L1-RSRP reporting

- Step 3: Select the next SSB beam-width shape (in a configurable way) and repeat step 1 and step 2, until all SSB beam-width shapes are exhaustively tested and completed.

- Step 4: Estimate SSB beam coverage footprint as a percentage (%) of default SSB beam shape.

- Step 5: Select the beam with the argmax DL UL_SINR performance and the coverage footprint > configurable threshold, from step 3.

- Step 6: For the selected SSB beam-width shape, adjust the SSB power boosting parameters to improve SINR. Estimate DL SINR based on the selected beam-width shapes

# Conclusion

Our Discussion document proposes the following proposals:

Proposal 1: When a CCO issue is detected, each NG-RAN node shall autonomously adjust its CCO configurations dynamically without any further Xn signaling.

Proposal 2: All other NG-RAN neighbor nodes of a cluster, when a CCO issue is detected on their own cell performance, shall autonomously adjust its CCO configurations dynamically without any further Xn signaling to the neighbors.

Proposal 3: To support CCO operations, the NG-RAN node relies on cell-level and UE-level parameters from the serving node.

Proposal 4: For the SON CCO use case, AIML model training takes place either at the OAM or at gNB-CU. The model inference takes place either at the gNB-CU or gNB-DU

Proposal 5: For the SON CCO use case, AIML model training takes place solely at the gNB-CU. The model inference takes place either at the gNB-CU or gNB-DU.

Proposal 6: For AI/ML based CCO, as for the SON functionality and for the existing SSB beam sweeping functionality on serving node, different SSB beam-width configurations and SSB beam steering in azimuth/elevation  are dynamically and autonomously selected to optimize coverage and SINR.

Proposal 7: For SON AI/ML based CCO, the proposed algorithm refers to P1 beam management algorithm, It will be an autonomous algorithm selection per node, without the need of any Xn specific signaling, periodically updated to dynamically adjust to changing radio environment conditions.

Proposal 8: The user device is unaffected and follows the specifications as per 3GPP and GSMA.

Proposal 9: For AI/ML based CCO, the inference input may be collected from UE and local node. including:

- Inputs from connected UEs as following:

- connected UE measurement report (e.g., L1 SS-RSRP, SS-RSRQ, SS-SINR measurement, CQI, etc), including cell level and beam level UE measurements

- connected UE performance (HARQ NACK KPI retransmissions (BLER>10%)),

- connected UE location information (e.g., coordinates, serving cell ID)

- idle mode UE’s Random Access performance (RACH success rate, number of RACH attempts) to convert to SINR from existing graph of number of PRACH attempts vs SINR).

- connected UE RLF report

The future coverage status may be Cell Coverage Modification(s) (i.e. power parameters, SSB boosting, etc) or SSB Coverage Modification(s) (i.e. SSB beam width modifications, beam steering). Hence for AI/ML based CCO, the future coverage status may include:

- Cell Coverage Modification including SSB beam power boosting for the Cell Coverage State which is used to solve the predicted CCO issue of the certain predicted cell;

- SSB Coverage Modification including SSB beam width Coverage State which is used to solve the predicted CCO issue of the certain predicted SSB.

Proposal 10: For AI/ML based SON CCO, the future coverage status may include the following NG-RAN gNB or gNB-CU/gNB-DU algorithm steps:

- Step 1: Start SSB beam sweeping on the cell level with the default (operator configurable) SSB beam-width shape, for all SSB sweeping beams in a 5ms cycle.

- Step 2: Estimate the DL SINR per SSB sweeping beam in a jointly cost function consisting of:

- number of UL PRACH counter attempts (build a graph of PRACH attempts vs SINR) of idle UEs,

- connected UEs UL CQI reports

- HARQ KPI retransmissions (BLER>10%)

- L1-SINR and L1-RSRP reporting

- Step 3: Select the next SSB beam-width shape (in a configurable way) and repeat step 1 and step 2, until all SSB beam-width shapes are exhaustively tested and completed.

- Step 4: Estimate SSB beam coverage footprint as a percentage (%) of default SSB beam shape.

- Step 5: Select the beam with the argmax DL UL_SINR performance and the coverage footprint > configurable threshold, from step 3.

# Reference

[1] R3-243767, “Identification and Rectification of CCO Issues Using AIML”

[2] RP-234054, "New SID: Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN".

[3] 3GPP TS 37.320, "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[4] 3GPP TR 38.843 “Study on AI ML for NR Air Interface”

[5] 3GPP TR 28.905 “Study on Artificial Intelligence/Machine Learning (AI/ML)”

[6] 3GPP TS 28.105 “Artificial Intelligence / Machine Learning (AI/ML) management”

[7] R3-242231, (TP to TR 38.743) “AI/ML enabled CCO”, Samsung, Ericsson, Huawei, Telecom Italia, ZTE, Nokia, CATT, Deutsche Telekom, LG Electronics, Telecom Italia