3GPP TSG-RAN WG3 Meeting #R3-125	R3-244036

Maastricht, NL, 19 - 23 August 2024

Agenda item:	 11.3

Source:		Jio

Title:		Discussion on Enhancements for AI/ML based CCO

Document for:		Discussion

# 1	Introduction

In this contribution, we would further propose and discuss details of AI/ML based CCO.

# 2	General Discussion

3GPP Release 18 and subsequent Release 19 introduced advanced AI/ML techniques for CCO, which are poised to revolutionize network management and optimization. A critical aspect of this optimization involves the management of SSB beamwidth, which plays a pivotal role in network coverage and capacity.

As mobile networks become increasingly complex with diverse traffic patterns and user behaviors, traditional optimization methods fall short in addressing dynamic and real-time requirements. This is where AI/ML comes into play, offering a data-driven approach to predict and manage network conditions effectively. By setting specifications and guidelines, 3GPP ensures that the implementation of AI/ML models is consistent, interoperable, and secure across different network components and vendors. The 3GPP specifications outline how AI/ML models should be trained, validated, and deployed for various network functions, including CCO.

The integration of AI/ML in CCO aims to enable self-organizing networks (SON) that can autonomously manage and optimize network parameters. This includes tasks such as load balancing, interference management, and dynamic configuration of network resources.

The relevant 3GPP documents for AI/ML integration in CCO are:

- Release 18:

  - TS 38.300: NR; Overall description; Stage-2

  - TR 38.913: Study on scenarios and requirements for next generation access technologies

  - TR 38.816: Study on enhanced network automation

- Release 19:

  - TS 38.331: NR; Radio Resource Control (RRC); Protocol specification

  - TR 38.820: Study on AI/ML for NR

SSB beamwidth refers to the angular width of the beam used to transmit synchronization signals. Proper management of SSB beamwidth is essential for ensuring optimal coverage and capacity. Narrower beams can provide higher gain and better coverage in specific directions, while wider beams can cover larger areas with lower gain. The trade-off between beamwidth and coverage area is crucial for network performance. Potential Improvements with SSB Beamwidth Management are:

- Enhanced Coverage - By dynamically adjusting the SSB beamwidth based on real-time network conditions, AI/ML algorithms can ensure that coverage is maximized in areas with high user density or poor signal quality. This adaptive beamwidth management allows for more efficient utilization of network resources, reducing coverage gaps and enhancing overall user experience.

- Improved Capacity - AI/ML algorithms can analyze traffic patterns and predict areas where capacity enhancements are needed. By narrowing the SSB beamwidth in these areas, the network can provide higher signal strength and better data rates to users, effectively increasing network capacity. This is particularly beneficial in urban environments with high user density.

- Interference Management -Interference is a significant challenge in dense network deployments. By intelligently managing the SSB beamwidth, AI/ML algorithms can minimize interference between neighboring cells. Narrower beams can be used to reduce overlap and interference, while wider beams can be employed in less congested areas to maintain coverage without causing excessive interference.

- Energy Efficiency - Adjusting SSB beamwidth can also contribute to energy efficiency. By focusing energy only where it is needed, the network can reduce unnecessary power consumption. AI/ML algorithms can optimize beamwidth settings to balance coverage and capacity needs with energy efficiency goals.

  - 2.1 AI/ML Training and Inference algorithm model for CCO

3GPP TR 37.817 in clause 5.1, sub-clause 5.1.1 refers to AI/Ml and the Efficient energy consumption can also be achieved by other means such as reduction of load, coverage modification, or other RAN configuration adjustments. The optimal energy saving decision depends on many factors including the load situation at different RAN nodes, RAN nodes capabilities, KPI/QoS requirements, number of active Ues and UE mobility, cell utilization, etc

Training AI/ML models for CCO involves using vast amounts of network data, including user mobility patterns, signal strength measurements, traffic loads, and environmental factors. This data is used to train algorithms to recognize patterns and predict network performance under varying conditions.

The training process includes several stages:

- Data Collection and Preprocessing: Collecting data from various network elements and preprocessing it to ensure quality and relevance.

- Feature Selection: Identifying key features that influence network coverage and capacity.

- Model Training: Using supervised or unsupervised learning techniques to train the model. Techniques such as neural networks, decision trees, or support vector machines can be used depending on the complexity of the task.

- Validation and Testing: Validating the model with a separate dataset to ensure accuracy and generalizability. This step includes testing the model under different scenarios to evaluate its performance.

Typically once trained, the AI/ML model is deployed in the network to perform real-time inference. The model continuously analyzes incoming data to make predictions and recommendations for optimizing coverage and capacity.

Key aspects of the inference process include:

- Real-Time Data Processing: Continuously collecting and processing data from network elements to feed into the model.

- Predictive Analysis: Using the trained model to predict potential coverage gaps, congestion hotspots, and other network issues.

- Decision Making: Making real-time decisions on parameter adjustments, such as antenna tilt, power levels, and handover thresholds, to optimize network performance.

- Feedback Loop: Implementing a feedback mechanism to update the model based on the effectiveness of the optimization actions taken.

3GPP TR 38.843 focuses on enhancing performance through AI/ML models, including potential use cases such as channel state information (CSI) feedback enhancement, beam management, and positioning accuracy improvements.

3GPP TR 28.908 outlines the management capabilities required for AI/ML across different phases: training, validation, testing, deployment, and inference. This includes aspects such as ML entity loading, inference control, and performance evaluation.

During last RAN3 meeting it was agreed that for AI/ML based CCO [1]:

-	AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the NG-RAN node (gNB-CU).

-	AI/ML Model Training and AI/ML Model Inference may be both located in the NG-RAN node (gNB-CU).

Currently, to support AI/ML in NG-RAN, inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results) are required. For AI/ML based CCO, for AI/ML Model Training or AI/ML Model inference, the inference input may be collected from UE or neighbour NG-RAN node (in case of non-split architecture) or neighbour gNB-CU/gNB-DU (in case of split architecture):

- Inputs from UE:

-	UE measurement report (e.g., UE SS-RSRP, SS-RSRQ, SS-SINR measurement, etc), including cell level and beam level UE measurements

-	UE location information (e.g., coordinates, serving cell ID)

-	Random Access performance and RLF report from the UE

- Inputs from local node:

-	Measured/Predicted Radio Resource Status

-	Measured/Predicted UE Traffic (e.g., data volume)

- Inputs from neighboring node:

-	Measured/Predicted Radio Resource Status

-	Measured/Predicted UE Traffic (e.g., data volume)

-	Measured/Predicted UE trajectory

In most of nowadays mobile networks, the gNB densification and the multi-layer deployment leads into pilot pollution with downgraded SINR. Moreover the SSB beam gains (i.e. typical values are 13-14 dBi or 16 dBi) are lacking somehow 4-5 dBs against the traffic beams (i.e. typical values are 18.5 – 21 dBi) leading into a coverage mismatch between idle and connected mode Ues performance. It is thus important to improve the coverage and the corresponding SINR based on a joint approach of dynamically shaping the multi SSB beams beam-width along with the proper SSB beam boosting, thus improving the SINR and keeping the corresponding coverage on a certain desired level.

As for nowadays, by performing SSB beam sweeping the cell is covered by higher antenna gains for the entire sector coverage area compared to the previous sectorized antenna with single antenna SSB beam with wider beamwisth coverage. The major disadvantage (also problem) of this method is that in general, multi-SSB beam shapes with specific beam-width configurations are fixed across all the sites based on the antenna equipment capability. This leads to unnecessary transmissions to undesired directions, overlapping sector areas and inter-cell interference across neighbor dense sectors.

Our proposal is for a set of different SSB beam shapes with different beam-width configurations and SSB beam power (SSB power boosting), to be dynamically selected and adjusted by the AI/ML optimization algorithm per sector. An AI/ML algorithm will dynamically select, among the available beam-width beams, the proper beam shape and the most suitable SSB power per individual SSB beam of the beam-sweeping set per sector, under the constraint of desired coverage and SINR.

Proposal 1: For AI/ML based CCO, for the existing SSB beam sweeping functionality on serving node, different SSB beam-width configurations and SSB beams are dynamically selected to optimize coverage and SINR.

The ultimate goal is to minimize the intra-cell and inter-cell beam overlapping, preserve the sector coverage and maximize the expected SINR for different inter-site distance and geography. Hence ultimately the Multi-SSB beam-width beams will improve the DL coverage and throughput without affecting the UE capability.

Proposal 2: For AI/ML based CCO, the proposed algorithm refers to P1 beam management algorithm, hence the user device is unaffected and follows the specific 3GPP and GSMA.

The proposed algorithm, to support AI/ML in NG-RAN, gets inputs only from connected UEs in the serving cell, building a cost function to predict the SSB coverage per individual SSB beam width out of the existing SSB beam sweeping beams.

Proposal 3: For AI/ML based CCO, the inference input may be collected from UE or local node or neighbour node, e.g. including:

- Inputs from connected UEs as following:

- connected UE measurement report (e.g., L1 SS-RSRP, SS-RSRQ, SS-SINR measurement, CQI, etc), including cell level and beam level UE measurements

- connected UE performance (HARQ NACK KPI retransmissions (BLER>10%)),

- connected UE location information (e.g., coordinates, serving cell ID)

- idle mode UE’s Random Access performance (RACH success rate, number of RACH attempts) to convert to SINR from existing graph of number of PRACH attempts vs SINR).

- connected UE RLF report

The future coverage status may be Cell Coverage Modification(s) (i.e.power parameters, SSB boosting etc) or SSB Coverage Modification(s) (i.e. SSB beam width modifications).

Proposal 4: For AI/ML based CCO, the future coverage status may include:

-	Cell Coverage Modification including SSB beam power boosting for the Cell Coverage State which is used to solve the predicted CCO issue of the certain predicted cell;

-	SSB Coverage Modification including SSB beam width Coverage State which is used to solve the predicted CCO issue of the certain predicted SSB.

DL performance is investigated based on SSB power boosting and SSB beam-width shaping for different SSB beam gains on the beam-sweeping procedure, building a proper cost function.

Ultimate goal is to improve (increase) DL SINR and average DL cell throughput

Proposal 5: For AI/ML based CCO, the future coverage status may include the following NG-RAN gNB or gNB-CU/gNB-DU algorithm steps:

- Step 1: Start SSB beam sweeping on the cell level with the default (operator configurable) SSB beam-width shape, for all SSB sweeping beams in a 5ms cycle.

- Step 2: Estimate the DL SINR per SSB sweeping beam in a jointly cost function consisting of:

- number of UL PRACH counter attempts (build a graph of PRACH attempts vs SINR) of idle UEs,

- connected UEs UL CQI reports

- HARQ KPI retransmissions (BLER>10%)

- L1-SINR and L1-RSRP reporting

- Step 3: Select the next SSB beam-width shape (in a configurable way) and repeat step 1 and step 2, until all SSB beam-width shapes are exhaustively tested and completed.

- Step 4: Estimate SSB beam coverage footprint as a percentage (%) of default SSB beam shape.

- Step 5: Select the beam with the argmax DL UL_SINR performance and the coverage footprint > configurable threshold, from step 3.

- Step 6: For the selected SSB beam-width shape, adjust the SSB power boosting parameters to improve SINR. Estimate DL SINR based on the selected beam-width shapes

# 3	Conclusion

In this contribution, AI/ML based CCO are discussed. We have the following proposals:

Proposal 1: For AI/ML based CCO, for the existing SSB beam sweeping functionality on serving node, different SSB beam-width configurations and SSB beam are dynamically selected to optimize coverage and SINR.

Proposal 2: For AI/ML based CCO, the proposed algorithm refers to P1 beam management algorithm, hence the user device is unaffected and follows the specific 3GPP and GSMA.

Proposal 3: For AI/ML based CCO, the inference input may be collected from UE or local node or neighbour node, e.g. including:

- Inputs from connected UEs as following:

- connected UE measurement report (e.g., L1 SS-RSRP, SS-RSRQ, SS-SINR measurement, CQI, etc), including cell level and beam level UE measurements

- connected UE performance (HARQ NACK KPI retransmissions (BLER>10%)),

- connected UE location information (e.g., coordinates, serving cell ID)

- idle mode UE’s Random Access performance (RACH success rate, number of RACH attempts) to convert to SINR from existing graph of number of PRACH attempts vs SINR).

- connected UE RLF report

Proposal 4: For AI/ML based CCO, the future coverage status may include:

-	Cell Coverage Modification including SSB beam power boosting for the Cell Coverage State which is used to solve the predicted CCO issue of the certain predicted cell;

-	SSB Coverage Modification including SSB beam width Coverage State which is used to solve the predicted CCO issue of the certain predicted SSB.

Proposal 5: For AI/ML based CCO, the future coverage status may include the following NG-RAN gNB or gNB-CU/gNB-DU algorithm steps:

- Step 1: Start SSB beam sweeping on the cell level with the default (operator configurable) SSB beam-width shape, for all SSB sweeping beams in a 5ms cycle.

- Step 2: Estimate the DL SINR per SSB sweeping beam in a jointly cost function consisting of:

- number of UL PRACH counter attempts (build a graph of PRACH attempts vs SINR) of idle UEs,

- connected UEs UL CQI reports

- HARQ KPI retransmissions (BLER>10%)

- L1-SINR and L1-RSRP reporting

- Step 3: Select the next SSB beam-width shape (in a configurable way) and repeat step 1 and step 2, until all SSB beam-width shapes are exhaustively tested and completed.

- Step 4: Estimate SSB beam coverage footprint as a percentage (%) of default SSB beam shape.

- Step 5: Select the beam with the argmax DL UL_SINR performance and the coverage footprint > configurable threshold, from step 3.

- Step 6: For the selected SSB beam-width shape, adjust the SSB power boosting parameters to improve SINR. Estimate DL SINR based on the selected beam-width shapes

# References

[1] R3-242231, (TP to TR 38.743) AI/ML enabled CCO, Samsung, Ericsson, Huawei, Telecom Italia, ZTE, Nokia, CATT, Deutsche Telekom, LG Electronics, Telecom Italia.

[2] 3GPP TS 38.300, "NR; Overall description; Stage 2".

[3] 3GPP TR 38.820: Study on AI/ML for NR

[4] 3GPP TR 38.913: Study on scenarios and requirements for next generation access technologies

[5] 3GPP TR 38.816: Study on enhanced network automation

[6] White paper ……..

- [7] R3-213391, CCO over F1: functional split and signaling between gNB-CU and gNB-DU (NEC)

- [8] R3-213410, (TP for SON BLCR for 38.473) Coverage and Capacity Optimization (Huawei)

- [9] R3-213660, Coverage and Capacity Optimization (Qualcomm Incorporated)

- [10] R3-213687, Coverage aspect of NR CCO (Nokia, Nokia Shanghai Bell)

- [11] R3-213820, Inter-gNB CCO (Ericsson)

- [12] R3-214006, Support of CCO (Samsung)

- [13] R3-214035, Further Discussion on Coverage and Capacity Optimization in NR (ZTE, China Telecom, China Unicom)

- [14] R3-221249 TP for BL CR 38.423 on Coverage and Capacity Optimization

- [15] R3-216189 LS on CCO related assistance information from OAM to NG-RAN  RAN3

- [16] R3-220887 (TP for SON BL CR 38.300) Coverage and Capacity Optimization  ZTE, China Telecom, China Unicom

- [17] R3-220888 (TP for SON BL CR 38.423) Coverage and Capacity Optimization  ZTE, China Telecom, China Unicom

- [18] R3-220889 (TP for SON BL CR 38.473) Coverage and Capacity Optimization  ZTE, China Telecom, China Unicom

- [19] R3-243385 Discussion on AIML based CCO 2024

- [20] R3-243255_CCO AI ML 2023

- [21] R3-243458 - Further discussion on AIML support for CCO

- [22] R3-243467 AI ML CCO