3GPP TSG-RAN3 Meeting #125	R3-244037

Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	11.3

Source: 	Jio

Title: 	(TP for TR 38.743) Enhancements for AI ML NG-RAN CCO

Document for:	Discussion/Decision

# Introduction

This paper contains the TP to capture the agreement for the AI/ML assisted CCO, based on R3-242243 [2] and following the discussion in R3-241148 [3]

# Text proposal to TR 38.743

------------------------------------------- Start of changes -------------------------------------------

## 4.2	AI/ML based Coverage and Capacity Optimization

### 4.2.1	Use case description

Editor Note: Capture the description of use case

Following R3-242243 3GPP TR 38.743 [2] proposal, the objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. An NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included.

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

AI/ML has been extensively discussed in 3GPP TR 37.817 [1]. With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

With an AI/ML based CCO, a proactive approach is used to optimize an existing CCO state based on some threshold configuration settings for:

- Energy cost scenario, including an Energy Cost prediction function [3] for the energy saving as per 3GPP TR 37.817 [1]and 3GPP TR 38.864 [6]

- Energy efficiency scenario, including all necessary proactive functions to optimize it (i.e. Digital Pre-Distortion PA actions as per TR 38.864)[5,6]

- Interference Management scenario, including SSB beam shaping and SSB beam steering [4]

- Traffic profile (throughput) scenario, [4]

- User distribution geolocation scenario [4]

### 4.2.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.2.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based CCO:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

The following solutions can be considered for supporting AI/ML-based CCO:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the ng-eNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the ng-eNB.

In case of ng-eNB CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the ng-eNB-CU.

- AI/ML Model Training and Model Inference are both located in the ng-eNB-CU.

#### 4.2.2.2		Input data of AI/ML based CCO

For a proactive prediction and resolution of a CCO issue, a gNB/ng-eNB may need the following information as input data for AI/ML-based CCO:

From local node:

-	Measured/Predicted radio resource status

-     Measure performance status (i.e. RLF, CEF, RA, cell/user throughput etc)[4]

-	Estimate energy cost function [3]

-     Current CCO State including all necessary parameters (i.e. SSB beamwidth, Gain, etc[4]

From neighbouring gNBs

-	Measured/Predicted radio resource status.

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

SON Reports (e.g., RLF, CEF, RA)

#### 4.2.2.3	Output data of AI/ML based CCO:

AI/ML-based CCO model in the node can generate following information as output:

-	Predicted CCO issue

-     Proposed CCO optimization (radio parameters, SSB beam steering, SSB beam shaping, SSB beam boosting adjustments etc)[4]

-    Predicted CCO performance [4]

-   Predicted energy efficiency based on energy cost function [3] and  proactively Digital Pre-Distortion corrective actions [5]

4.2.2.4	Feedback of AI/ML based CCO:

To optimize the performance of AI/ML-based CCO model, following feedback can be considered to be collected from gNBs/ng-eNBs:

- Measured radio resource status

- Legacy UE performance feedback (i.e. Accessibility, throughput etc)

- Legacy UE performance feedback for those UEs handed over from the source gNB

- SON Reports (e.g., RLF, CEF, RA, energy cost function)

4.2.2.5	Potential standard impacts:

Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

- Future CCO State

- Future Energy Cost function status

F1 interface:

- Predicted CCO Issue (including predicted cells/beams)

- Proposed CCO optimization (radio parameters, SSB beam steering, SSB beam shaping, etc)

- Predicted CCO performance

- Proposed Energy Efficiency corrective actions (i.e. Digital Pre-distortion)

# References

[1] 3GPP TR 37.817

[2] R3-242243 for 3GPP TR 38.743

[3] R3-241148

[4] Jio’s White paper

[5] Jio Patent P1314

[6] 3GPP TR 38.864