3GPP TSG-RAN WG3 Meeting #R3-125	R3-244038

Maastricht, NL, 19 - 23 August 2024

Agenda item:	 11.3

Source:		Jio

Title:		Discussion on AI/ML based CCO

Document for:		Discussion

# 1	Introduction

In this contribution, we would further propose and discuss details of AI/ML based CCO, based on 3GPP TR 28.908 [19] and the RAN3 #114bis-e R3-221329 [20] and 3GPP TR 37.817 [21]

This is mirrored to the RAN#125 Jio R3-244036 document

# 2	General Discussion

The advent of 5G and the rapid expansion of mobile networks have necessitated more sophisticated approaches to network management. One such approach is the use of artificial intelligence (AI) and machine learning (ML) for Coverage and Capacity Optimization (CCO). 3GPP plays a pivotal role in such approaches, being at the forefront of integrating AI/ML into mobile networks to enhance performance and efficiency.

Since 3GPP Release 17 Coverage and Capacity Optimization is a critical function in mobile networks, aimed at ensuring that users experience seamless connectivity and optimal data rates. As mobile networks become increasingly complex with diverse traffic patterns and user behaviors, traditional optimization methods fall short in addressing dynamic and real-time requirements. This is where AI/ML comes into play, offering a data-driven approach to predict and manage network conditions effectively. By setting specifications and guidelines, 3GPP ensures that the implementation of AI/ML models is consistent, interoperable, and secure across different network components and vendors. The 3GPP specifications outline how AI/ML models should be trained, validated, and deployed for various network functions, including CCO.

  - 2.1 AI/ML Training and Inference algorithm model for CCO

Training AI/ML models for CCO involves using vast amounts of network data, including user mobility patterns, signal strength measurements, traffic loads, and environmental factors. This data is used to train algorithms to recognize patterns and predict network performance under varying conditions.

The training process includes several stages:

- Data Collection and Preprocessing: Collecting data from various network elements and preprocessing it to ensure quality and relevance.

- Feature Selection: Identifying key features that influence network coverage and capacity.

- Model Training: Using supervised or unsupervised learning techniques to train the model. Techniques such as neural networks, decision trees, or support vector machines can be used depending on the complexity of the task.

- Validation and Testing: Validating the model with a separate dataset to ensure accuracy and generalizability. This step includes testing the model under different scenarios to evaluate its performance.

Typically once trained, the AI/ML model is deployed in the network to perform real-time inference. The model continuously analyzes incoming data to make predictions and recommendations for optimizing coverage and capacity.

Key aspects of the inference process include:

- Real-Time Data Processing: Continuously collecting and processing data from network elements to feed into the model.

- Predictive Analysis: Using the trained model to predict potential coverage gaps, congestion hotspots, and other network issues.

- Decision Making: Making real-time decisions on parameter adjustments, such as antenna tilt, power levels, and handover thresholds, to optimize network performance.

- Feedback Loop: Implementing a feedback mechanism to update the model based on the effectiveness of the optimization actions taken.

3GPP TR 38.843 focuses on enhancing performance through AI/ML models, including potential use cases such as channel state information (CSI) feedback enhancement, beam management, and positioning accuracy improvements.

3GPP TR 28.908 outlines the management capabilities required for AI/ML across different phases: training, validation, testing, deployment, and inference. This includes aspects such as ML entity loading, inference control, and performance evaluation.

Furthermore following the RAN3 #114bis-e R3-221329 [20] and 3GPP TR 37.817[21], NG-RAN makes decisions using AI/ML model trained from OAM, as per figure 5.1.2.1-1:

Following 3GPP TR 37.817 2022 version, see following figure 4.2-1, the following solutions can be considered for supporting AI/ML-based network energy saving:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

However in figure 5.1.2.2-1 [20] NG-RAN and 3GPP 37.817[21] will be responsible for overall model training and generates energy saving decisions.

In latest RAN3 meetings in order to realized CCO, a typical optimization solution for the use of AI/ML was firmly proposed to prevent the rise of a CCO issue with the consequent degradation of network (and UE) performance. Hence during RAN3#123-bis the following had been agreed for AI/ML based CCO, there has been two proposals.

For proposal case 1, the solution for the use AI/ML was to directly predict corrections on cell coverage configuration information and to apply the predicted cell coverage configuration information into the RF parameter adjustment without any update towards the neighbour cells:

- For the non-split architecture:

Step 0: gNB predicts the CCO issue.

Step 1: gNB generates the future coverage status based on the predicted CCO issue and other information.

Step 2: gNB sends the future coverage status to neighbour gNBs.

- For the split-architecture:

Step 0: gNB-CU predicts the CCO issue

Step 1: gNB-CU sends the predicted CCO issue to gNB-DU.

Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether only local information is used can be further discussed.

Step 3: gNB-DU sends the future coverage status to gNB-CU.

Step 4: gNB-CU sends the future coverage status to neighbour gNBs.

Furthermore, another solution for the use AI/ML was to directly predict corrections on cell coverage configuration information, apply the predicted cell coverage configuration information into the RF parameter adjustment and inform (update) the neighbour cells. That was the proposal case 2, as another potential implementation option:

- For non-split architecture

Step 1: gNB predicts the cell coverage configuration based on the input information from local, UE and neighbouring nodes.

Step 2: gNB sends the predicted cell coverage configuration to neighbour gNBs.

- For split-architecture

Step 1: gNB-CU predicts the cell coverage configuration based on the input information from local, UE, neighbouring nodes and gNB-DU.

Step 2: gNB-CU sends the predicted cell coverage configuration to neighbour gNBs.

Both aforementioned AI/ML based CCO proposals have similar Xn impact, i.e. gNB sends the predicted/further cell coverage configuration to neighbour gNBs. The main difference between the two aforementioned case proposals is the type of output, predict CCO issue or predicts the cell coverage configuration, from the model inference.

Furthermore during last RAN3 meeting it was agreed that for AI/ML based CCO [1]:

-	AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the NG-RAN node (gNB-CU).

-	AI/ML Model Training and AI/ML Model Inference may be both located in the NG-RAN node (gNB-CU).

Currently, to support AI/ML in NG-RAN, inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results) are required. For AI/ML based CCO, for AI/ML Model Training or AI/ML Model inference, the inference input may be collected from UE or neighbour NG-RAN node (in case of non-split architecture) or neighbour gNB-CU/gNB-DU (in case of split architecture):

- Inputs from UE:

-	UE measurement report (e.g., UE SS-RSRP, SS-RSRQ, SS-SINR measurement, etc), including cell level and beam level UE measurements

-	UE location information (e.g., coordinates, serving cell ID)

-	Random Access performance and RLF report from the UE

- Inputs from local node:

-	Measured/Predicted Radio Resource Status

-	Measured/Predicted UE Traffic (e.g., data volume)

- Inputs from neighboring node:

-	Measured/Predicted Radio Resource Status

-	Measured/Predicted UE Traffic (e.g., data volume)

-	Measured/Predicted UE trajectory

Our discussion proposal is to:

- Select the scenario where the AI/ML Model Training and AI/ML Model Inference may be both located in the NG-RAN node (or gNB-CU).

- We are further proposing to isolate the coverage prediction mostly to the serving node (NG-RAN gNB or gNB-CU/gNB-DU) in order to get the neighbor node involved as less as possible, eliminate the Xn signaling load and act proactively on the best coverage.

Proposal 1: The inventive step is on network/server (NG-RAN gNB or gNB-CU), by using specific AI/ML sw algorithms to enhance the network performance and implement the changes on gNB-DU. Furthermore the coverage prediction to be isolated mostly to the serving node (NG-RAN gNB or gNB-CU/gNB-DU) in order to get the neighbor node the less possible involved, decrease the Xn signaling load and act proactively on the best coverage

In non-split architecture:

-	Step 0: gNB predicts the CCO coverage and/or SINR issue.

-	Step 1: gNB generates the future coverage status based on the predicted CCO issue and other information.

-	Step 2: gNB executes the corrections (SSB beam width and/or SSB beam boosting) to resolve or improve the CCO issue.

And in split-architecture, it has agreed that:

-	Step 0: gNB-CU predicts the CCO coverage and/or SINR issue.

-	Step 1: gNB-CU sends the predicted CCO issue to gNB-DU.

-	Step 2: gNB-DU generates the necessary actions (SSB beam width and/or SSB beam boosting) to resolve or improve the CCO issue.

-	Step 3: gNB-DU executes the resolve or improve the CCO issue.

-	Step 4: gNB-DU informs gNB-CU about the current new coverage (SSB beam width and/or SSB beam boosting) status.

-  Step 5: gNB-CU sends over Xn, if becessary, the future coverage status to neighbour gNBs

Proposal 2: The predicted CCO issue should be derived with AI/ML for both split-architecture and non-split architecture.

Refer to earlier legacy CCO mechanism, the predicted CCO issue may be a coverage issue (coverage hole) or a cell edge capacity issue (SINR or coverage overlapping) of a certain predicted SSB beam out of the available SSB beams of a certain cell.

Proposal 3: For AI/ML based CCO, the predicted CCO issue may include:

-	type of the predicted CCO issue(s), e.g., coverage issue SINR or cell edge capacity issue SINR and/or hole;

-	id of affected cell(s) in which there is the predicted CCO issue(s);

-	SSB index(s) of affected beam(s) in which there is the predicted CCO issue(s).

# 3	Conclusion

In this contribution, AI/ML based CCO are discussed. We have the following proposals:

Proposal 1: The inventive step is on network/server (NG-RAN gNB or gNB-CU), by using specific AI/ML sw algorithms to enhance the network performance and implement the changes on gNB-DU. Furthermore the coverage prediction to be isolated mostly to the serving node (NG-RAN gNB or gNB-CU/gNB-DU) in order to get the neighbor node the less possible involved, decrease the Xn signaling load and act proactively on the best coverage.

Proposal 2: The predicted CCO issue should be derived with AI/ML for both split-architecture and non-split architecture.

Proposal 3: For AI/ML based CCO, the predicted CCO issue may include:

-	type of the predicted CCO issue(s), e.g., coverage issue SINR or cell edge capacity issue SINR and/or hole;

-	id of affected cell(s) in which there is the predicted CCO issue(s);

-	SSB index(s) of affected beam(s) in which there is the predicted CCO issue(s).

# References

[1] R3-242231, (TP to TR 38.743) AI/ML enabled CCO, Samsung, Ericsson, Huawei, Telecom Italia, ZTE, Nokia, CATT, Deutsche Telekom, LG Electronics, Telecom Italia.

[2] 3GPP TS 38.300, "NR; Overall description; Stage 2".

- [3] R3-213391, CCO over F1: functional split and signaling between gNB-CU and gNB-DU (NEC)

- [4] R3-213410, (TP for SON BLCR for 38.473) Coverage and Capacity Optimization (Huawei)

- [5] R3-213660, Coverage and Capacity Optimization (Qualcomm Incorporated)

- [6] R3-213687, Coverage aspect of NR CCO (Nokia, Nokia Shanghai Bell)

- [7] R3-213820, Inter-gNB CCO (Ericsson)

- [8] R3-214006, Support of CCO (Samsung)

- [9] R3-214035, Further Discussion on Coverage and Capacity Optimization in NR (ZTE, China Telecom, China Unicom)

- [10] R3-221249 TP for BL CR 38.423 on Coverage and Capacity Optimization

- [11] R3-216189 LS on CCO related assistance information from OAM to NG-RAN  RAN3

- [12] R3-220887 (TP for SON BL CR 38.300) Coverage and Capacity Optimization  ZTE, China Telecom, China Unicom

- [13] R3-220888 (TP for SON BL CR 38.423) Coverage and Capacity Optimization  ZTE, China Telecom, China Unicom

- [14] R3-220889 (TP for SON BL CR 38.473) Coverage and Capacity Optimization  ZTE, China Telecom, China Unicom

- [15] R3-243385 Discussion on AIML based CCO 2024

- [16] R3-243255_CCO AI ML 2023

- [17] R3-243458 - Further discussion on AIML support for CCO

- [18] R3-243467 AI ML CCO

- [19] 3GPP TR 28.908, Study on Artificial Intelligence/Machine Learning (AI/ML) management (Rel.18)

- [20] R3-221329 Intel Corporation, TP to 37.817 on AI/ML based network energy saving

- [21] 3GPP TR 37.817 Study on enhancement for Data Collection for NR and EN-DC (Release 17)