3GPP TSG-RAN3 Meeting #125                                                      R3-244039

Maastricht, Netherlands, AUG 20-24, 2024

Agenda Item:	11.2

Source: 	Tejas Networks

Title: 	Discussion on NG-RAN AI/ML for Network Slicing

Document for:	Discussion/Decision

# Background

In RAN3#124 meeting, there were a few proposals and it resulted in identification of some immediate next steps.

RAN3#124 CN-RAN information exchange: - Per slice load level information from CN to RAN node - Predicted AMF capacity from CN to RAN node - RAN predicted Slice capacity from RAN to CN CATT: in TS23.288, NWDAF can provide per cell list or per TA list slice level load information E///, Nok: Input information does not include predicted information ZTE: As described in R18 use case, local predicted resource status information can be used as input information as well CATT: Cannot agree with E///, predicted information as input has already been supported in previous use cases HW: Question on using other domain information Lenovo: Open to discuss NEC: how to use such information from CN in RAN side CMCC: Open to discuss the information from CN ZTE: Supporting the transfer of predicted information between CN and RAN is not a task that disrupts the existing architecture, as it is already supported, e.g., expected UE behavior. Interdigital: Using existing data is preferred UE traffic prediction/measurement: Defined as data volume in current spec Per Slice per UE? Or per UE? CMCC, CATT: Support this CATT: During HO request, the source node can send the predicted UE traffic to the target node, and after HO, the target node can transfer the measured UE traffic to the source node as part of UE performance feedback E///: How can the source predict this? Lenovo: gNB can do the prediction. Can further discuss the usage of exchanging this information between gNBs SLA fulfilment information: Scenario and benefits? Signalling measured/predicted per slice requirement fulfilment level between gNBs? Summary of offline disc R3-243801 noted In Rel-19, predicted information from CN to NG-RAN is not considered. Agree to improve the granularity of UE performance feedback in Rel-19 for slicing. FFS whether to include measured/predicted UE throughput per slice as input from local node. FFS whether to introduce delivery of slice level UE throughput prediction/measurement during handover procedure. FFS on SLA requirements fulfilment information exchange. Nok: SLA requirements fulfilment information is not interested by majority company

# Introduction

## In this contribution we investigate the following FFS mentioned in the above chairman’s report

- Whether to include measured/predicted UE throughput per slice as input from local node.

- Whether to introduce delivery of slice level UE throughput prediction/measurement during handover procedure

- SLA requirements fulfilment information exchange.

and outline the observations and proposals as below

# Discussion

3.1

Observation 1:

- Whether to include measured/predicted UE throughput per slice as input from local node

- Whether to introduce delivery of slice level UE throughput prediction/measurement during handover procedure

Proposal1: Yes, measured/predicted UE throughput per slice shall be included as input from local node, and specifically this has to be included during the handover procedure.

Proposal2:

UEs are moving in various random trajectories across a terrain, resulting in multiple handovers across a network of gNBs. Each gNB and the OAM(SMO) is involved in various phases of Network slicing.

For proper operational management of the network slice each gNB and its OAM needs information regarding the UE traffic in its vicinity for calculating the slices at various future time instants (T1,T2,T3,…Tn). Also, since the UEs are moving in various random trajectories across the terrain it results in multiple handovers across for each gNBs. In this scenario, sitting in the instance of time Tn, it becomes extremely difficult for each gNB and its associated OAM in the Network slice operation phase to do resource capacity planning and modification in needed of the Network slices for Tn+1

At the time instance Tn, the aggregated UE traffic measurement(B) obtained from various k gNBs, B1n ,B2n,B3n ,..BKn,  can be send by the gNBs to the RAN OAM(SMO). Using this current aggregated UE traffic measured and also aggregated UE traffic measured from the various past Tn-1 time periods a multivariate (Multiple parallel series) and multistep time-series forecasting model in the RAN OAM(SMO) can predict the aggregated UE traffic for k gNBs for the Tn+1 time instance, B1n+1 ,B2n+1,B3n+1 ,..BKn+1(for uni-step prediction) and multiple time instances in advance, Tn+1,Tn+2,Tn+3,…Tn+z (for multistep prediction).

There are two approaches proposed below. The choice of which approach to use may be left to the deployment.

Approach-1: Basic model Model training, inference and feedback

- Training and Inference – In the basic approach the Model is trained in the SMO with the data obtained from the gNBs and trained model is deployed at each gNBs which predicts the aggregated UE traffic Bk, multiple time instances in advance, Tn+1, Tn+2, Tn+3,…,Tn+z (for multistep prediction)

for the gNBs to do the resource capacity planning and modification needed for the Network slices. Each time-step Tn can be typically in the order of 5-10 seconds given the handover criteria is in order of 500ms~1 sec and aggregated over a certain time. The value k indicates the number of gNBs in an Area and dependent on deployment.

- Feedback - When the future time-instance n+1 is reached, each gNB can measure the difference between the actual and predicted UE traffic d1n+1 , d2n+1,d3n+1 ,..dKn+1 which can calculate the performance accuracy score of the predictions send this information to the SMO, the SMO can re-train the model with the data, so that performant predictions are obtained in the future time-steps.

Fig.1: Basic learning model

Approach-2: Federated Learning Model training inference and feedback

- Training and Inference – In the Federated Leaning approach, the Federated learning server is located in the SMO and the Federated learning clients are present in the gNBs. The Federated learning process can be as below

- The Federated learning server(SMO) sends a “MLModel training subscribe/info” to the clients(gNBs) to participate in the local model training with the model parameters determined by the server(Initial ML model, model metric, wait time etc)

- The Federated learning client(Model hosted in the gNBs) collects the local data(aggregated UE traffic measurement(Bk))

- During Federated Learning training procedure, each FL Client(Model hosted in the gNBs) further trains the ML Model provided by the FL Server(SMO)based on its local data and reports the interim local ML Model information to the FL Server(SMO)in MLModelTraining_Notify or MLModelTrainingInfo_Request response. The MLModelTraining_Notify or MLModelTrainingInfo_Request response may also include the Status report of FL training that includes local ML Model metric value (and optionally the used metric) computed by the FL Client and Training Input Data Information, and also if needed the Global model metric value. The local ML Model, which is sent from the FL Clients to the FL Server during the FL training process, is the information needed by the FL Server to build the aggregated model.

- The FL Server(SMO) aggregates all the local ML Model information retrieved at step 3, to update the global ML Model. The FL Server may also compute the global ML Model metric value, e.g. based on the local ML Model metric value(s) provided by the FL Clients or by applying the global model on the validation dataset (if available). The FL Server may update the global ML Model each time a FL Client provides updated local ML Model information, or the FL Server may decide to wait for local ML Model information from all FL Client before updating the global ML Model.

- If the FL procedure continues, FL Server sends MLModelTraining_Subscribe or MLModelTrainingInfo_Request that includes the aggregated ML Model information to selected FL Client for next round of Federated Training. The request may also include the ML Model Accuracy Check Flag, that indicates the FL Clients to use the local training data as the testing dataset to calculate the Model Accuracy of the global ML Model provided by the FL Server.

- Each FL Client updates its own ML Model based on the aggregated ML Model information distributed by the FL Server at step 5.

- Each trained FL Client(model present in the gNB), now predicts the aggregated UE traffic Bk, multiple time instances in advance, Tn+1,Tn+2,Tn+3,…Tn+z (for multistep prediction).  for the gNBs to do the resource capacity planning and modification needed for the Network slices. Each time-step Tn can be typically in the order of 5-10 seconds given the handover criteria is in order of 500ms~1 sec and aggregated over a certain time. The value k indicates the number of gNBs in an Area and dependent on deployment.

- Feedback -  When the future time-instance n+1 is reached, each FL client(gNB) can measure the difference between the actual and predicted UE traffic d1n+1 ,d2n+1,d3n+1 ,..dKn+1 which can calculate the performance accuracy score of the predictions locally and update the local metric and also global metric and send to the FL server(SMO), the FL server(SMO) can re-train itself with this data so that performant predictions are obtained in the future time-steps.

Fig. 2: Federated learning model

## 3.2 Consideration on existing FFS: RAN-CN information exchange

This observation come from proposals and actions in RAN3#124 meeting as outlined in section1 and below and specifically focusses on point3.

CN-RAN information exchange

- Per slice load level information from CN to RAN node

- Predicted AMF capacity from CN to RAN node

- RAN predicted Slice capacity from RAN to CN

- Observation 2: We believe that the aggregated UE traffic prediction information and subsequent Network slice resource utilization prediction may be send to Core Networks as an input for different functionalities like charging (CHF) as part of NGAP or other mechanisms.

- Proposal 2: Based on this observation we request the RAN3 team to enable this exchange between RAN and core. The details may be discussed.

## 3.3Consideration on existing FFS: Exchange SLA fulfilment information between NG-RAN nodes

Service Level Agreements (SLAs) in network slicing define the specific performance and quality parameters that each network slice must meet to satisfy the requirements of different services and applications. SLAs ensure that each network slice delivers the expected level of service, tailored to the needs of various use cases.

The approaches described above may be used for continuous monitoring, prediction of resource requirements and dynamic allocation of resources to fulfil the SLA. A typical use case would be to ensure reliable and low-latency communication(URLLC) for autonomous vehicles (AVs) by exchanging SLA fulfilment information between NG-RAN nodes. Metrics may include latency (below 1ms), packet loss (near zero), and jitter (minimal). SLA metrics are aggregated at each NG-RAN node.

  - This results in Improved SLA compliance for URLLC network slices, enhanced reliability and reduced latency for AV communication, proactive identification and mitigation of potential SLA breaches.

  - AI/ML models can be deployed to analyse the data to identify patterns, predict potential SLA breaches, and recommend proactive measures. Based on the analysis, the AI/ML engine generates recommendations for network optimization. Recommendations may include adjustments in resource allocation, load balancing, handover strategies, and network configuration to meet URLLC SLA requirements.

Observation 3: Data privacy/ security requirements are not well understood if SLA information is shared between NG-RAN nodes.

Proposal 3: Privacy/ security aspects of the information exchange may be further discussed.

# Conclusions

- Observation 1:

  - Whether to include measured/predicted UE throughput per slice as input from local node

  - Whether to introduce delivery of slice level UE throughput prediction/measurement during handover procedure

- Proposal1: Yes, measured/predicted UE throughput per slice shall be included as input from local node, and specifically this has to be included during the handover procedure

- Proposal 2: At the time instance Tn, the aggregated UE traffic measurement(B) obtained from various k gNBs, B1n , B2n,B3n ,..BKn,  can be send by the gNBs to the RAN OAM(SMO). Using this current aggregated UE traffic measured and also aggregated UE traffic measured from the various past Tn-1 time periods a multivariate (Multiple parallel  series) and multistep time-series forecasting model in the RAN OAM(SMO) can predict the aggregated UE traffic for k gNBs for the Tn+1 time instance, B1n+1 ,B2n+1,B3n+1 ,..BKn+1(for uni-step prediction) and multiple time instances in advance, Tn+1,Tn+2,Tn+3,…Tn+z (for multistep prediction). This can be done using two approaches

  - Basic model Model training, inference and feedback – Using model training at SMO and inference at gNBs

  - Federated learning approach – Using SMO as FL server and gNBs as FL clients and inference at gNBs

- Observation 2: We believe that the aggregated UE traffic prediction information and subsequent Network slice resource utilization prediction may be send to Core Networks as an input for different functionalities like charging (CHF) as part of NGAP or other mechanisms.

- Proposal 2: Based on this observation we request the RAN3 team to enable this exchange between RAN and core. The details may be discussed

- Observation 3: Data privacy/ security requirements are not well understood if SLA information is shared between NG-RAN nodes.

- Proposal 3: Privacy/ security aspects of the information exchange may be further discussed.