3GPP TSG-RAN WG3 #124	R3-244040

Fukuoka, Japan, 20th–24th May 2024

Agenda Item:	11.4

Source:	Jio

Title: 	Enhancements on AI/ML Network Energy Saving

Document for:	Discussion

# Background

During Rel-19, new potential enhancements were proposed on document R3-243449 by Ericsson for the AI/ML-based Network Energy Saving use case. It was further agreed among others to: have discussions on AI/ML based energy saving at the next meeting, where enhancements will be based on new solutions not discussed in Rel18….

In this Discussion document, following the 3GPP TR 38.743 [4], the RAN3 #114bis-e R3-221329 [5] and 3GPP TR 37.817[6], we further elaborate the subject matter of Energy Cost Predictions in the scope of energy saving and propose some extra ideas. Moreover this document is also related to two Jio patents [7,8]

This Document is mirrored to the R3-244037 (TP for TR 38.743) Enhancements for AI/ML Energy Saving.

# Prediction of Energy Cost

## Energy Cost function considerations in Release 19

During Rel-18, RAN3 came to an understandingbut did not agree to itthat an Energy Cost (EC) prediction function or formula or content, which is to be exchanged between NG-RAN nodes, should be derived with respect to a tentative and potential offloading action between the nodes in order to be interpretable or actionable by the receiving node.

Moreover to this understanding, we further enhance this idea by several different way.

Adding Coverage and Capacity Optimization (CCO) considerations to an Energy Cost (EC) prediction function for a single gNB (gNodeB) in a 5G Radio Access Network (RAN) offers several critical enhancements for energy saving. The integration of CCO involves optimizing the network's coverage and capacity, which directly impacts energy consumption

Proposal 1:  An Energy Cost prediction should be combined with CCO actions.

One of the primary aspects of CCO is the dynamic adjustment of transmission power to meet coverage and capacity needs without wasting energy. In a static system, gNBs might operate at a fixed power level, potentially over-providing coverage and capacity, especially during periods of low traffic.

By integrating CCO, the transmission power of a gNB can be adjusted in real-time based on actual demand and network conditions. This means during off-peak hours or in less populated areas, the transmission power can be reduced, thus conserving energy. Conversely, power can be increased only when necessary, such as during high traffic loads or to support edge users. This targeted use of energy prevents wastage and optimizes power consumption.

Proposal 2:  An Energy Cost prediction should be combined with CCO actions including Dynamic Adjustements on Transmitted Power.

CCO strategies often include the implementation of sleep modes or low-power states for gNBs during periods of low activity. This involves selectively powering down certain components of the gNB, such as antennas or transmitters, while maintaining minimal functionality for essential operations.

The ability to switch gNBs or their components into low-power modes during low traffic periods significantly reduces the energy consumption. The integration of CCO in the EC prediction function allows the network to dynamically determine when to activate these modes based on real-time data, such as traffic volume and user distribution. The overall reduction in active operational time for gNBs directly translates to lower energy usage.

Proposal 3:  An Energy Cost prediction should be combined with CCO actions including Low-Power mode.

Beam management including dynamic SSB/Traffic beam shaping or beam steering is another critical technology in 5G that allows gNBs to direct their signals specifically towards user equipment (UE) rather than broadcasting them indiscriminately. CCO includes optimizing beamforming parameters to ensure that coverage and capacity are provided where needed, without excess.

Optimized beamforming reduces the power needed to cover a given area, especially when fewer UEs are present or in scenarios where precise targeting can minimize interference and energy loss. By managing UE connections more efficiently (e.g., connecting them to the optimal gNB or beam), the network can reduce the power required for maintaining quality of service. This precise control and targeting reduce the overall energy consumption of the gNBs.

Proposal 4:  An Energy Cost prediction should be combined with CCO actions including beamforming parameter optimization actions.

CCO includes advanced traffic prediction capabilities, leveraging AI/ML algorithms to forecast user demand and network conditions. This foresight allows the network to prepare and allocate resources efficiently.

By predicting traffic patterns, the network can proactively adjust gNB settings in anticipation of changes in demand. For example, during predicted low-traffic periods, gNBs can be preemptively transitioned into low-power states, and transmission power can be lowered. Conversely, during expected peak times, resources can be efficiently ramped up without overshooting, thus preventing unnecessary energy expenditure.

Proposal 5:  An Energy Cost prediction should be combined with CCO actions including dynamic traffic predictions.

Follows an algorithm which outlines a systematic approach to integrating CCO considerations into an EC prediction function for a single gNB in 5G RAN Release 19. By leveraging AI/ML-based predictions, dynamic adjustments, and continuous optimization, the algorithm aims to minimize energy consumption while maintaining or improving network performance. The adaptive nature of the algorithm ensures that it can respond effectively to real-time network conditions and evolving traffic demands, thereby achieving enhanced energy savings.

Proposal 6:  An Energy Cost prediction algorithm, including CCO parameters should be implemented with AI/ML assisted optimization actions.

The following algorithm integrates Coverage and Capacity Optimization (CCO) considerations into the Energy Cost (EC) prediction function for a single gNB (gNodeB) in a 5G RAN, with the ultimate goal of enhancing energy savings. The algorithm comprises initialization, real-time monitoring, prediction, dynamic adjustment, and continuous optimization steps.

1. Initialization

- Input Parameters:

  - Pmax​: Maximum transmission power.

  - Pmin​: Minimum transmission power.

  - Psleep​: Power consumption in sleep mode.

  - Lth​: Traffic load threshold for activating sleep mode.

  - Bmin​: Minimum SSB beam shaping angle width.

  - Cmax​: Maximum capacity threshold.

  - Historical data for training AI/ML models.

- Output Parameters:

  - ECbase​: Baseline Energy Cost.

  - ECopt​: Optimized Energy Cost.

2. Real-Time Data Collection

- Collect Real-Time Metrics:

  - Network performance metrics: SS-SINR, SS-RSRP, SS-RSRQ.

  - Traffic metrics: Current traffic load (#RRC Connected UEs), user equipment (UE) distribution.

  - Channel prediction: Channel modeling predictions based on physical obstructions.

3. Baseline Energy Cost Calculation

- Calculate:

ECbase=t=0TPcurrentt∙Δt

based on current power consumption without CCO optimizations, where Pcurrentt is the power consumption at time t during the time interval of measurements and Δt.

4. AI/ML-Based Prediction

- Traffic and Load Forecasting:

  - Use machine learning models to predict future traffic patterns and load distribution based on historical and real-time data.

- Coverage Demand Analysis:

  - Predict coverage demand and user mobility patterns.

5. Dynamic Adjustment of Network Parameters

- Adaptive Transmission Power Control:

  - Calculate optimal transmission power as:

  - Popt=minPmax,maxPmin,fLpredt

based on predicted traffic load and coverage requirements, where Lpredt is the predicted traffic load at time t.

- Beamforming Optimization:

  - Adjust beamforming angles and widths based on user density and demand:

  - Boptt=maxBmin,gUEpredt

where UEpredt is the predicted UE distribution.

- Sleep Mode Activation:

  - If Lcurrent<Lth activate, transition of gNB to sleep mode:

​Pcurrentt=Psleep

6. Real-Time Energy Cost Calculation and Optimization

- Calculate Optimized EC:

ECopt=t=0TPoptt∙Δt=t=0TminPmax,maxPmin,fLpredt∙Δt

- Comparison and Adjustment:

  - Compare ECopt, ECbase​.

  - Adjust optimization parameters if ECopt​ is not sufficiently lower than ECbase​.

7. Continuous Learning and Feedback Loop

- Performance Evaluation:

  - Evaluate the performance of optimizations in terms of energy savings and network quality.

- Model Refinement:

  - Update AI/ML models with new data for improved prediction accuracy.

- Optimization Refinement:

  - Refine CCO strategies and parameters based on performance evaluations and feedback.

## Energy Cost function with combined CCO and traffic off-loading considerations in Release 19

Adding Coverage and Capacity Optimization (CCO) considerations with the potential off-loading of traffic to neighboring cells to an Energy Cost (EC) prediction function for a single gNB (gNodeB) in a 5G Radio Access Network (RAN) offers several critical enhancements for energy saving. The integration of CCO involves optimizing the network's coverage and capacity, which directly impacts energy consumption.

By distributing the traffic load more evenly across gNBs, the network can prevent situations where some gNBs are overburdened while others are underutilized. Underutilized gNBs can be placed into low-power modes or even turned off, reducing the overall energy consumption. Efficient load balancing ensures that no single gNB is pushed to operate at full capacity unnecessarily, thus conserving energy across the network.

Proposal 7:  An Energy Cost prediction should be combined with CCO actions including potential offloading action to neighbor cells.

Hence combining Coverage and Capacity Optimization (CCO) with potential neighbor cell off-loading (mobility management) into a single Energy Cost (EC) prediction function can significantly enhance energy savings in 5G Radio Access Networks (RAN) for several reasons. This integrated approach allows for a more holistic and dynamic management of network resources, leading to more efficient energy usage while maintaining the desired service quality.

Follows some key reasons and the corresponding proposals for this combined CCO and off-loading functionality:

- Dynamic Load Balancing and Traffic Distribution:

  - Context: In a 5G network, traffic loads can vary significantly across different cells and times of the day. Some cells may become congested, while others remain underutilized.

  - Benefit: By incorporating neighbor cell off-loading into the EC function, the network can dynamically distribute traffic from congested cells to underutilized neighboring cells. This redistribution helps prevent excessive power usage in heavily loaded cells and allows for potential power reductions or sleep mode activation in less utilized cells, thus optimizing overall energy consumption.

Proposal 8:  An Energy Cost prediction should include an enhanced traffic distribution off-loading action to improve energy efficiency.

- Adaptive Coverage and Capacity Management:

  - Context: Coverage and capacity requirements fluctuate based on user density and demand patterns. For instance, during special events or peak hours, higher capacity and coverage might be needed, whereas off-peak hours might allow for reductions. Moreover Mobility management, including handovers and load balancing, is crucial in a mobile network to maintain service quality. However, inefficient handling can lead to unnecessary power consumption, especially if users frequently move between cells with differing loads

  - Benefit: The combined consideration of CCO and mobility in the EC function enables adaptive adjustments in both coverage (by controlling transmission power and antenna tilts) and capacity (by adjusting resource allocation and scheduling). This means that the network can minimize energy usage by reducing unnecessary coverage areas and reallocating resources more efficiently based on real-time demand.

Proposal 9:  An Energy Cost prediction should include both CCO and enhanced traffic distribution off-loading action to improve energy efficiency.

- Proactive Energy Saving Strategies:

  - Context: Traditional networks often react to traffic changes after they occur. However, predictive analytics and proactive measures can significantly enhance energy efficiency.

  - Benefit: The EC prediction function, when informed by CCO and mobility data, can forecast potential traffic patterns and energy consumption trends. This predictive capability enables the network to pre-emptively adjust parameters, such as reducing power in areas with anticipated low demand or preparing cells for higher traffic. This proactive approach helps in smoothing out energy consumption peaks and valleys, resulting in more consistent and lower overall energy usage.

  - Proposal 10:  An Energy Cost prediction with proactive energy saving strategy would enhance energy efficiency

A proper algorithm effectively integrates CCO with the potential off-loading of traffic to neighboring cells into an EC prediction framework for a single gNB. By dynamically adjusting transmission power, optimizing beamforming, leveraging neighboring cells' capacity, and employing sleep modes, the algorithm significantly reduces energy consumption. The continuous learning and feedback loop ensure that the system adapts and optimizes performance, achieving the ultimate goal of enhanced energy savings in 5G RAN Release 19.

1. Initialization

- Input Parameters:

  - Pmax​: Maximum transmission power.

  - Pmin​: Minimum transmission power.

  - Psleep​: Power consumption in sleep mode.

  - Lth​: Traffic load threshold for activating sleep mode.

  - Bmin​: Minimum SSB beam shaping angle width.

  - Cmax​: Maximum capacity threshold.

  - Neighbor Cells Data: Capabilities, current load, and potential for handling off-loaded traffic.

  - Historical data for training AI/ML models.

- Output Parameters:

  - ECbase​: Baseline Energy Cost.

  - ECopt​: Optimized Energy Cost.

2. Real-Time Data Collection

- Collect Real-Time Metrics:

  - Network performance metrics: SS-SINR, SS-RSRP, SS-RSRQ.

  - Traffic metrics: Current traffic load (#RRC Connected UEs), user equipment (UE) distribution.

  - Channel prediction: Channel modeling predictions based on physical obstructions.

  - Neighboring cells' load and capacity

3. Baseline Energy Cost Calculation

- Calculate:

ECbase=t=0TPcurrentt∙Δt

based on current power consumption without CCO optimizations, where Pcurrentt is the power consumption at time t during the time interval of measurements and Δt.

4. AI/ML-Based Prediction

- Traffic and Load Forecasting:

  - Use machine learning models to predict future traffic patterns and load distribution based on historical and real-time data.

- Coverage Demand Analysis:

  - Predict coverage demand and user mobility patterns and user distribution (density).

- Neighbor Cell Assessment:

- Evaluate neighboring cells' current and projected load capacity for potential off-loading.

5. Dynamic Adjustment of Network Parameters

A. Adaptive Transmission Power and Beamforming Control

- Adaptive Transmission Power Control:

  - Calculate optimal transmission power as:

Popt=minPmax,maxPmin,fLpredt

based on predicted traffic load and coverage requirements, where Lpredt is the predicted traffic load at time t.

- Beamforming Optimization:

  - Adjust beamforming angles and widths based on user density and demand to focus coverage and minimize interference:

Boptt=maxBmin,gUEpredt

where UEpredt is the predicted UE distribution.

B. Traffic off-loading to neighbor cells

- Off-Loading Criteria:

  - Trigger off-loading when Lcurrent>Lth or during predicted high-demand periods.​

- Selection of Target cells:

  - Select neighboring cells based on their current and predicted load, coverage overlap, and capacity:

  - Target Cell=argminjLcurrentj+Lpredj

- Off-loading execution:

  - Handover UEs to selected neighboring cells to balance load and reduce energy consumption:

Lnew=Lcurrent-Loffloaded

​ C. Sleep mode activation

- Criteria for Sleep Mode:

- If Lcurrent is significantly reduced post off-loading, consider transitioning gNB to sleep mode:

Pcurrentt=Psleep

6. Real-Time Energy Cost Calculation and Optimization

- Calculate Optimized EC:

ECopt=t=0TPoptt∙Δt=t=0TminPmax,maxPmin,fLpredt∙Δt

- Comparison and Adjustment:

  - Continuously compare ECopt, ECbase​ to assess energy savings

  - Adjust off-loading thresholds, power levels and beamforming optimization parameters if ECopt​ is not sufficiently lower than ECbase​.

7. Continuous Learning and Feedback Loop

- Performance Evaluation:

  - Evaluate the performance of optimizations in terms of energy savings and network quality.

- Model Refinement:

  - Update AI/ML models with new data for improved prediction accuracy.

- Optimization Refinement:

  - Refine CCO and off-loading strategies and parameters based on performance evaluations and feedback.

## 2.3 Energy Cost function with mutually exclusive CCO and off-loading metrics in Release 19

In the realm of 5G networks, efficient management of resources and energy consumption is paramount. The intricacies of these tasks often involve two primary strategies: offloading and Coverage and Capacity Optimization (CCO). While traditionally these actions have been considered in tandem, recent research suggests the potential benefits of treating them as mutually exclusive, independent actions.

Offloading involves redistributing network traffic from one cell to another, typically to balance the load and alleviate congestion in high-demand areas. This action can also be driven by energy efficiency considerations, where traffic is offloaded to cells with lower energy costs, thereby reducing overall energy consumption. CCO actions, on the other hand, focus on optimizing the coverage and capacity of a network by adjusting parameters like antenna tilt, transmission power, and beamforming characteristics.

Traditionally, these two strategies are interlinked; an increase in traffic in a particular area often triggers both CCO actions and offloading to manage the load and ensure network stability. For instance, when a cell experiences high traffic, CCO actions might include expanding coverage through beam steering, while offloading could simultaneously redistribute some of the traffic to neighboring cells.

During an Energy Cost (EC) prediction action, a potential offloading action could be avoided due to further improved energy cost reduction and energy savings by a beam characteristics change (i.e. SSB beam-shaping, SSB beam steering or high-resolution beam prediction) in Coverage and Capacity Optimization (CCO) action.

Energy Efficiency through CCO and Advanced Beamforming: By employing sophisticated beam prediction techniques, such as high-resolution beamforming, the network can precisely target UEs with optimal beams, enhancing signal quality and reducing energy wastage. This precision can improve energy efficiency to a point where offloading becomes unnecessary, as the existing cell can handle the load without exceeding its energy budget. CCO actions involving SSB/traffic beam-shaping and steering can dynamically adjust the coverage area and direct energy more efficiently towards high-demand regions. By fine-tuning these parameters, the network can optimize coverage without the need to offload traffic, thereby maintaining energy efficiency and service quality.

Hence treating offloading and CCO as independent actions reduces the complexity of network management. Instead of simultaneously adjusting multiple parameters and managing interdependencies, network operators can focus on optimizing one aspect without being constrained by the other. This separation simplifies decision-making processes and allows for more targeted interventions.

Concluding by decoupling offloading from CCO actions, the network can better predict energy costs. CCO actions, specifically designed to optimize energy usage through advanced beam control, can lead to more accurate EC predictions. Offloading, often a reactive measure, introduces variability that complicates energy cost assessments. Separating these actions allows for a more consistent and reliable estimation of energy savings. An EC prediction related to a potential offloading action could be reconsidered under the view of a beam characteristics change (i.e. SSB beam-shaping, SSB beam steering or high-resolution beam prediction) in CCO.

Proposal 11:  An Energy Cost prediction should be derived with a mutual exclusive consideration of a potential offloading action in coordination with CCO beam improvements. Thus a holistic approach of EC function would enhance the energy saving actions between NG-RAN nodes in order to be interpretable or actionable by the receiving node.

The intricate interplay between offloading and CCO beam improvements can significantly enhance energy efficiency in NG-RAN networks. By strategically offloading UEs with high EC profiles and optimizing beam configurations, network operators can achieve substantial energy savings. The decoupling of high energy demand nodes from those capable of efficient energy management creates a more balanced and efficient network. This approach not only reduces overall energy costs but also improves the sustainability and reliability of the network, aligning with the broader goals of modern telecommunications to support growing data demands while minimizing environmental impact.

Consequently a predicted high EC might be a consequence of the NG-RAN node serving several UEs with low, bursty, but frequent traffic that prevents it from enabling energy saving features due to offloading but it might be assisted by the energy saving actions due to CCO beam improvements. Hence, the neighboring NG-RAN node may be able to serve the new, offloaded UEs with a marginal increment in its EC; at the same time, the offloading might enable the source NG-RAN node to reduce its own EC significantly and to optimize its energy saving by proper CCO optimization actions.

Proposal 12:  An Energy Cost prediction on one node should be tolerated with respect to a potential offloading action between neighbor NG-RAN nodes with potential CCO beam optimization action capabilities.

Under this paradigm, the source node of the offloading action should signal to the target node a description of the “additional load” for the target node; it would then receive from the target node a prediction of the Energy Cost at the target node, assuming the offloading action was successfully executed.

Follows a proposed algorithm to integrate Coverage and Capacity Optimization (CCO) with off-loading actions into an Energy Cost (EC) prediction function for a gNB in a 5G RAN in a more flexible mutually exclusive way.. The primary goal is to enhance energy savings by optimizing network performance and resource usage. The algorithm follows specific rules: off-loading conditions are mutually exclusive to CCO actions, off-loading should include CCO considerations, and CCO actions should be prioritized to manage the load before considering off-loading.

1. Initialization

- Input Parameters:

  - Pmax​: Maximum transmission power.

  - Pmin​: Minimum transmission power.

  - Psleep​: Power consumption in sleep mode.

  - Lth_CCO​: Traffic CCO load activation threshold.

  - Lth_offload​: Traffic off-load (mobility) activation threshold

  - Bmin​: Minimum SSB beam shaping angle width.

  - Neighbor Cells Data: current load and capacity info.

  - Historical traffic profile data: data to understand traffic patterns based on AI/ML models.

- Output Parameters:

  - ECbase​: Baseline Energy Cost.

  - ECopt​: Optimized Energy Cost.

2. Real-Time Data Collection

- Collect Real-Time Metrics:

  - Network performance metrics: SS-SINR, SS-RSRP, SS-RSRQ.

  - Traffic metrics: Current traffic load (#RRC Connected UEs), user equipment (UE) distribution.

  - Channel prediction: Channel modeling predictions based on physical obstructions.

  - Neighboring cells' load and capacity

- Baseline Energy Cost Calculation

- Calculate:

ECbase=t=0TPcurrentt∙Δt

based on current power consumption without CCO optimizations, where Pcurrentt is the power consumption at time t during the time interval of measurements and Δt.

3. AI/ML-Based Prediction

- Traffic and Load Forecasting:

  - Use machine learning models to predict future traffic patterns and load distribution based on historical and real-time data.

- Coverage and Capacity Demand Analysis:

  - Predict coverage and capacity demand and user mobility patterns and user distribution (density).

- Neighbor Cell Assessment:

- Evaluate neighboring cells' current and projected load capacity for potential off-loading.

4. Dynamic Adjustment of Network Parameters

A. Check current load against thresholds

- Low Load Conditions:

- if Lcurrent<Lth_CCO no additional action is required, with potential sleep mode if low traffic conditions persist

- Intermediate Load Conditions:

- if Lth_CCO< Lcurrent<Lth_offload additional action is required with CCO activation.

B. CCO actions consideration

- Power control optimization:

- Calculate optimal transmission power as:

Popt=minPmax,maxPmin,fLpredt

based on predicted traffic load and coverage requirements, where Lpredt is the predicted traffic load at time t.

- Beamforming adjustments:

  - Adjust beamforming angles and widths based on user density and demand to focus coverage and minimize interference:

Boptt=maxBmin,gUEpredt

where UEpredt is the predicted UE distribution.

- Evaluate impact on EC function:

  - Calculate the potential reduction in EC due to CCO:

ECopt_CCO=t=0TPoptt∙Δt=t=0TminPmax,maxPmin,fLpredt∙Δt

- Check sufficiency of CCO actions:

  - Assess if CCO actions sufficiently reduce load and EC without needing off-loading

C. Traffic off-loading Consideration to neighbor cells

- Off-Loading Criteria:

  - Consider potential off-loading when Lcurrent>Lth or during predicted high-demand periods.

  - ​

D. Traffic off-loading Actions to neighbor cells

- Selection of Target cells:

  - Select neighboring cells based on their current and predicted load, coverage overlap, and capacity:

Target Cell=argminjLcurrentj+Lpredj

- Off-loading execution:

  - Handover UEs to selected neighboring cells to balance load and reduce energy consumption:

Lnew=Lcurrent-Loffloaded

- Combined CCO and off-loading EC calculation:

  - Calculate the total EC with off-loading and additional CCO:

ECopt_Offload=t=0TPoptt+Pneighbort∙Δt

- Criteria for Sleep Mode:

- If Lcurrent is significantly reduced post off-loading, consider transitioning gNB to sleep mode:

Pcurrentt=Psleep

5. Real-Time Energy Cost Calculation and Optimization

- Comparison and Adjustment:

  - Continuously compare ECopt, ECbase​ to assess energy savings

  - Adjust off-loading thresholds, power levels and beamforming optimization parameters if ECopt​ is not sufficiently lower than ECbase​.

- Performance Evaluation:

  - Evaluate the performance of optimizations in terms of energy savings and network quality.

- Model Refinement:

  - Update AI/ML models with new data for improved prediction accuracy.

- Optimization Refinement:

  - Refine CCO and off-loading strategies and parameters based on performance evaluations and feedback.

## 2.4 Energy Cost function enhancements in Release 19

Despite extensive discussions in Rel-18 3GPP meetings, RAN3 could not agree on how the additional load is described. The accurate estimation of Effective Cell (EC) load in cellular networks is crucial for efficient resource management and quality of service (QoS). A common approach to estimating EC load involves counting the number of User Equipments (UEs) in the network, particularly those in the RRC_Connected state. Hence there were various proposals on EC parameters and variables, including number of UEs (active and in RRC_Connected state). However the simple option like “number of UEs” do not provide enough information to obtain accurate predictions; for example, the same number of UEs but with different types of traffic and radio conditions might result in significantly different EC values. However, relying solely on this metric has significant limitations, as highlighted in a well-regarded research paper.

  - As an example of the considered limitation is the fact that different UEs may generate vastly different types of traffic, such as high-bandwidth video streaming, latency-sensitive gaming, or low-data-rate IoT applications. Even with the same number of UEs, the network load varies significantly depending on the nature of the traffic. For instance, 100 UEs streaming video will impose a much heavier load than 100 UEs sending occasional text messages. Other factor might be the radio conditions where UEs under various radio conditions—affected by factors such as distance from the cell tower, physical obstructions, or interference—require different amounts of network resources. UEs experiencing poor signal quality consume more power and bandwidth for maintaining acceptable communication quality, increasing the EC load disproportionately compared to UEs in optimal conditions.

  - Combining the user traffic behavior, including data consumption patterns and mobility, adds another layer of complexity. Mobile users may frequently switch between cells, causing additional signaling overhead and requiring more sophisticated handover processes, which are not captured by simple UE counts. Finally the network might dynamically allocate resources based on priority levels, QoS requirements, and service agreements. Simple metrics do not account for the variations in resource allocation, where high-priority UEs may consume disproportionate resources compared to others.

Given these aforementioned complexities, it is clear that counting the number of UEs, whether active or connected, offers a limited view of the actual network load.

Proposal 13:  The definition of “potential load,” should be expanded to contain more parameters and metrics/variables than the number of UEs.

For a more accurate and actionable understanding of EC load, a multi-faceted approach is required. This approach should consider:

- Traffic Characteristics: Differentiating between traffic types and their respective bandwidth, latency, and QoS requirements. Hence other factors to contribute to the EC function might be the PRB utilization, UL/DL data volume, etc.

- Radio and Environmental Conditions: Accounting for variations in signal quality (UE RSRP, SINR) and the additional resources (i.e. RSRP/SINR vs. PRB) needed for UEs in challenging conditions.

- Temporal Variations: Recognizing patterns over time, such as peak hours or event-driven traffic spikes. This could include RRM measurements.

- Behavioral Insights: Understanding user mobility patterns and their impact on resource usage based on QoS characteristics.

However selecting the aforementioned metrics would end up with a very complex representation that falls short of expressing important aspects impacting the energy consumption a UE might generate, such as UE capabilities and types of services used.

Proposal 14:  The definition of “potential load,” with many more and detailed metrics and factors involved might generate excessive processing requirements, further resulting into higher complexity with potentially limited future enhancements. The selection of EC and the metrics involved should fulfill the constraints of processing load and complexity.

# Conclusions

In this contribution, we discussed open issues and potential enhancements of the AI/ML based Network Energy Saving use case and made the following observations and proposals:

Proposal 1:  An Energy Cost prediction should be combined with CCO actions.

Proposal 2:  An Energy Cost prediction should be combined with CCO actions including Dynamic Adjustements on Transmitted Power.

Proposal 3:  An Energy Cost prediction should be combined with CCO actions including Low-Power mode.

Proposal 4:  An Energy Cost prediction should be combined with CCO actions including beamforming parameter optimization actions

Proposal 5:  An Energy Cost prediction should be combined with CCO actions including dynamic reaffic predictions.

Proposal 6:  An Energy Cost prediction algorithm, including CCO parameters should be implemented with AI/ML assisted optimization actions.

Proposal 7:  An Energy Cost prediction should be combined with CCO actions including potential offloading action to neighbor cells.

Proposal 8:  An Energy Cost prediction should include an enhanced traffic distribution off-loading action to improve energy efficiency.

Proposal 9:  An Energy Cost prediction should include both CCO and enhanced traffic distribution off-loading action to improve energy efficiency.

  - Proposal 10:  An Energy Cost prediction with proactive energy saving strategy would enhance energy efficiency

Proposal 11:  An Energy Cost prediction should be derived with a mutual exclusive consideration of a potential offloading action in coordination with CCO beam improvements. Thus a holistic approach of EC function would enhance the energy saving actions between NG-RAN nodes in order to be interpretable or actionable by the receiving node.

Proposal 12:  An Energy Cost prediction on one node should be tolerated with respect to  a potential offloading action between neighbor NG-RAN nodes with potential CCO beam optimization action capabilities.

Proposal 13:  The definition of “potential load,” should be expanded to contain more parameters and metrics/variables than the number of UEs.

Proposal 14:  The definition of “potential load,” with many more and detailed metrics and factors involved might generate excessive processing requirements, further resulting into higher complexity with potentially limited future enhancements. The selection of EC and the metrics involved should fulfill the constraints of processing load and complexity.

# Reference

[1]	RP-234054	New SID: Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

[2]	R3-242073	AI/ML Network Energy Saving, Ericsson

[3] Jio white paper

[4] 3GPP TR 38.743

[5] R3-221329 Intel Corporation, TP to 37.817 on AI/ML based network energy saving

[6] 3GPP TR 37.817 Study on enhancement for Data Collection for NR and EN-DC (Release 17)

[7] Provisional patent 1

[8] Provisional patent 2