3GPP TSG-RAN3 Meeting #125	R3-244041

Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	11.4

Source: 	Jio

Title: 	(TP for TR 38.743) Enhancements for AI/ML Energy Saving

Document for:	Discussion/Decision

# Introduction

This paper contains the TP to capture the agreement for the AI/ML assisted energy savings based on 3GPP TR 37.817 and 38.864. It further follows the discussion in R3-243448  & R3-243449

Finally it mirrors the RAN#125 R3-244037 Jio discussions contribution entitled “Enhancements for AI ML NG-RAN CCO” and the R3-244040 “Enhancements on AI-ML Network Energy Saving”

# Discussion

Following 3GPP TR 38.864 [2], Network energy saving is of great importance for among other things to reduce operational cost. As 5G and subsequently forthcoming 6G is becoming pervasive across industries and geographical areas, handling more advanced services and applications requiring very high data rates (e.g. XR), networks are being denser, use more antennas, larger bandwidths and more frequency bands. The environmental impact of 5G needs to stay under control, and novel solutions to improve network energy savings need to be developed.

Energy consumption has become a key part of the operators' OPEX. According to the report from GSMA [7], the energy cost on mobile networks accounts for ~23% of the total operator cost. Most of the energy consumption comes from the radio access network and in particular from the AAU, with data centres and fibre transport accounting for a smaller share.

The power consumption of a radio access can be split into two parts:

- the dynamic part which is only consumed when data transmission/reception is ongoing (i.e. including the Digital Pre-Distortion),

- the static part which is consumed all the time to maintain the necessary operation of the radio access devices, even when the data transmission/reception is not on-going.

It is important to focus on 3GPP TR 38.864 Power domain discussion, specifically the sub-clause 6.1.4, where gNB may implement digital pre-distortion (DPD) to compensate for the non-linear impairments of the transmitter in standard transparent manner. Following the earlier contribution 3GPP R1-2212745: "NW energy savings performance evaluation", it proposes and supports over the air digital pre-distortion at the gNB. In gNB digital pre-distortion over the air, the UEs assist the gNB in reducing nonlinear impairments introduced by the PA, by processing on training signals, and reporting the information needed for gNB digital pre-distortion

This discussion focuses on the proposed enhancements on Energy Saving in coordination with AI/ML algorithms in Rel.19.

# Text proposal to TR 38.743

------------------------------------------- Start of changes -------------------------------------------

5.3	Energy saving enhancements

Editor Note: Capture the description and its potential standard impacts.

According to 3GPP TR 38.864 [2], most of the energy consumption comes from the radio access network and in particular from the AAU, with data centres and fibre transport accounting for a smaller share.

Therefore, there is a need to study and develop a network energy consumption model especially for the base station to illustrate the energy savings as per 3GPP TR 37.817 [1], KPIs, an evaluation methodology and to identify and study AI/ML based network energy savings techniques in targeted deployment scenarios.

A general description, solutions, and standard impacts of the AI/ML based Network Energy Saving use case are presented in TR 37.817 [1]. In that previous study and specifically in the subclause 5.1, to deal with issues listed above, ML techniques could be utilized to optimize the energy saving decisions by leveraging on the data collected in the RAN network. ML algorithms may predict the energy efficiency and load state of the next period, which can be used to make better decisions on cell activation/deactivation for energy saving. Based on the predicted load, the system may dynamically configure the energy-saving strategy (e.g., the cell switch-off timing and granularity, mobility offloading actions as per TR 38.864) to keep a balance between system performance and energy efficiency and to reduce the energy consumption.

Cell activation/deactivation is an energy saving scheme in the spatial domain that exploits traffic offloading in a layered structure to reduce the energy consumption of the whole radio access network (RAN). When the expected traffic volume is lower than a fixed threshold, the cells may be switched off, and the served Ues may be offloaded to a new target cell.

Efficient energy consumption can also be achieved by other means such as reduction of load by offloading mobility actions, coverage modification like SSB beamwidth adjustments and SSB beam steering actions [3], or other RAN configuration adjustments including among others the AI/ML assisted Digital Pre-Distortion fast adjustment actions [4]. The optimal energy saving decision depends on many factors including the load situation at different RAN nodes, RAN nodes capabilities, KPI/QoS requirements, number of active Ues and UE mobility, cell utilization, etc.

The AI/ML model inference, as per [1], is assumed to be at NG-RAN node 1, which is the local node that executes the Network Energy Saving (NES) action according to the model inference output. NG-RAN node 2, which is a neighboring NG-RAN node involved in the NES action, e.g., as a target node of an offloading action, is assumed to have an AI/ML model optionally.

As part of the present study, an energy cost prediction function is discussed, the necessary parameter inputs to the energy cost function, the corrective actions and the corresponding metric to measure and improve the energy savings as being part of the Energy Cost predictions in NG-RAN mode1 and node 2 is developed. This solution uses an AI/ML model at NG-RAN nodes 1,2 for deriving improved Energy Cost predictions.

5.3.1	Locations for AI/ML Model Training and AI/ML Model Inference

According to TR 37.817, the following solutions can be considered for supporting AI/ML-based network energy saving:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

5.3.2 Input of AI/ML-based Network Energy Saving

To predict the optimized network energy saving decisions, NG-RAN may need following information as input data for AI/ML-based network energy saving:

From local node:

-	Current/Predicted Energy efficiency, including an Energy Cost function with among others the Power Amplifier energy efficiency and the crucial functionality of the Digital Pre-Distortion (DPD) optimization [4]

-	Current/Predicted resource status

- 	Current SSB beam gain, beamwidth and coverage/capacity [3]

- 	UE mobility/trajectory prediction

From neighbouring NG-RAN nodes:

-	Current/Predicted Energy efficiency, including including an Energy Cost function with among others the Power Amplifier energy efficiency and the crucial functionality of the Digital Pre-Distortion (DPD) optimization [4]

-	Current/Predicted resource status

-	Current energy state (e.g., active, high, low, inactive)

- 	Current SSB beam gain, beamwidth and coverage/capacity [3]

5.3.3 Output of AI/ML-based Network Energy Saving

AI/ML-based network energy saving model can generate following information as output:

-	Energy saving strategy, such as:

- recommended cell activation/deactivation,

- power adjustments on SSB beam boosting, gain and beam-shaping [3]

- power adjustments on SSB beamwidth and beam steering [3]

- Power adjustments in the RRUs or mMIMO RF chain Pas, including AI/ML assisted Digital Pre-Distortion (DPD) optimization [4]

-	Handover strategy, including recommended candidate cells for taking over the traffic

-	Predicted Energy efficiency,  including the proper cost function

-	Predicted energy state (e.g., active, high, low, inactive)

-

5.3.4	Feedback of AI/ML-based Network Energy Saving

To optimize the performance of AI/ML-based network energy saving model, following feedback can be considered to be collected from NG-RAN nodes:

-	Resource status of neighbouring NG-RAN nodes

-	Energy efficiency of serving and neighbour cells

-	UE performance affected by the energy saving action (e.g., mobility handed-over Ues), including bitrate, packet loss, accessibility, RLF, latency etc).

-	System KPIs (e.g., integrity throughput, delay, RLF of current and neighbouring, retainability,  NG-RAN node)

#### 5.3.5 Standard Impact

Potential Xn interface impact:

-	New signalling procedure or enhanced existing procedure to collect the input data information [5, 6]

-	Predicted energy efficiency between neighbouring NG-RAN nodes and source NG-RAN node

-	Predicted resource status between neighbouring NG-RAN nodes and source NG-RAN node

-	New signalling procedure or enhanced existing procedure to retrieve feedback information

# References

[1] 3GPP TR 37.817

[2] 3GPP TR 38.864

[3] Jio’s White paper

[4] Jio Patent P1314

[5] R3-243448

[6] R3-243449

[7] GSMA, 5G energy efficiencies: Green is the new black, https://data.gsmaintelligence.com/api-web/v2/research-file-download?id=54165956&file=241120-5G-energy.pdf.