3GPP TSG-RAN WG3#125 	R3-244042

Maastricht, Netherlands, 19-23 August 2024

Agenda item:	13.2 Support for inter-CU LTM

Source:	Nokia

Title:	Discussion on Inter-gNB-CU LTM Procedure

Document for:	Discussion and Decision

# 1	Introduction

In regard to L1/L2 Triggered Mobility (LTM), a new Work Item has been approved (RP-234036 NR mobility enhancements Phase 4), including the following related objectives and scope impacting RAN3:

- Specify support for inter-CU Layer 2 Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - As secondary priority, support the case when NR-DC is configured and CU is acting as SN and MCG is unchanged - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released - Note: The case that LTM is configured in both MCG and SCG is excluded - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1] - Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritise intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

This paper discusses the basic required mobility procedures for inter-CU L1/L2 triggered mobility (LTM) and its potential impact to the disaggregated gNB architecture signalling.

# 2	Stage-2 Inter-CU LTM for non-split gNB Architecture

In Rel-18, LTM was specified with limitation that LTM mobility could be configured and carried out only to cells within the same gNB-CU (i.e. intra-gNB LTM). In Rel-19, this feature expands to also cover inter-gNB-CU (i.e. inter-gNB LTM) mobility.

For Inter-CU LTM, we foresee the following as necessary to be considered.

- Rel-18 LTM framework and LTM handover phases for intra-gNB-CU LTM are basis also for inter-gNB-CU LTM. That is, inter-CU LTM also has the following phases:

(a) Preparation phase, (b) Early synchronization phase, (c) Execution phase, and (d) Completion phase.

The following are agreements from the previous RAN3 meeting, which are basis for proposals in this paper.

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. Early data forwarding can be supported for inter-CU LTM. Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution. RAN3#124: Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS. FFS on the LTM modification procedures. FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

## 2.1	Preparation Phase

The following steps are needed during the preparation phase and similar to L3 handover as defined in TS 38.300.

Step 0: The UE context within the source gNB contains information regarding roaming and access restrictions which were provided either at connection establishment or at the last TA update.

Step 1: The source gNB configures the UE measurement procedures and the UE reports according to the measurement configuration.

Step 2: The source gNB decides to configure LTM based on measurement reports.

Candidate cell information exchange

In Rel-18 Intra-CU LTM, RAN3 agreed that the SSB related information concerning possible target cells is available to the gNB-CU via the F1 interface setup and configuration update procedures when cells are setup or updated. Hence, avoiding transferring of static(/semi-static) general cell configuration multiple times over each UE context signalling.

For Rel-19 Inter-CU LTM, a similar approach can be considered as SSB related information can also made available to the Source gNB from the target gNB nodes via Xn interface setup and configuration update procedures.

Proposal 1: SSB related information concerning cells at Candidate gNBs is exchanged via Xn Setup and Configuration Update procedures between Source gNB and Target gNB(s).

Additionally, for Rel-19 Inter-CU LTM, given that TCI states and cell switch for LTM can also be based on CSI-RS, there is need for signalling CSI-RS related information regarding possible target cells to the Source gNB. For this scenario a similar approach as with SSB can be used. That is, signal this information between Source gNB and Target gNB via Xn Interface Setup and Configuration Update procedures. Likewise, between Candidate gNB-DU and Candidate gNB-CU via corresponding F1 Setup and Configuration Update procedures.

Proposal 2: Periodic CSI-RS related information concerning cells at Candidate gNBs is exchanged via Xn Setup and Configuration Update procedures between Source/Target gNBs, and via F1 Setup and Configuration Update procedures between gNB-DU(s) and gNB-CU.

Step 3: The source gNB issues a HANDOVER REQUEST message(s) to the candidate target gNB(s).

Single vs. Parallel messages

In Rel-18, the aspect of whether a single message could configure multiple LTM cells, or whether multiple parallel messages should be used was thoroughly discussed in context of intra-CU LTM, with RAN3 opting for the multiple message approach. For Rel-19, we suggest taking this principle as baseline and have of separate messages per each LTM cell being configured also for inter-gNB LTM.

Proposal 3:  For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

Step 4: Admission control is performed by target gNB(s) according to legacy procedure.

Step 5: The candidate target gNB(s) sends an LTM response via the HANDOVER REQUEST ACKNOWLEDGE message including configurations of LTM candidate cell to the source gNB.

Step 6: The source gNB sends an RRCReconfiguration message to the UE, containing the configuration of LTM candidate cells.

Step 7: The UE sends an RRCReconfigurationComplete message to the source gNB.

## 2.2	Early Synchronization Phase

RAN3 agreed at last meeting to “introduce a new procedure on Xn to transfer the TA information”. However, left inconclusive the details regarding procedure’s Class and whether it is non UE-associated.

The early synchronization can be supported for inter-CU LTM taking Rel-18 approach for intra-CU case as baseline in a straightforward manner. Hence, a new non-UE associated procedure is required over Xn from Target gNB to Source gNB. Likewise, in case of disaggregated architecture, existing DU-CU/CU-DU TA INFORMATION TRANSFER messages can be reused.

Proposal 4: Introduce a new Class-2 non-UE associated XnAP TA INFORMATION TRANSFER procedure from Target gNB to Source gNB to signal Early TA related information for LTM.

Proposal 5: Existing F1AP DU-CU TA INFORMATION TRANSFER and CU-DU TA INFORMATION TRANSFER messages can be reused in case of disaggregated gNB architecture for Inter-CU LTM scenario.

Step 8: Early synchronization to the target candidate cell(s) may be performed as specified in TS 38.300.

Step 9: The Target gNB send a TA INFORMATION TRANSFER message to the UE indicating the acquired TA values.

However, RAN3 should study whether TA values is enough to be sent from target gNB to source gNB or more information is needed in order to determine the TA validity at the source gNB. Note that TA validity was left up to source DU implementation in case of Rel. 18 intra-CU LTM. We believe, in case of Rel. 19 inter-CU LTM, it can not be left up to source DU/gNB implementation.

Proposal 6: RAN3 to study which information is to be sent from target gNB to source gNB along with TA values within XnAP TA INFORMATION TRANSFER and CU-DU/DU-CU TA INFORMATION TRANSFER messages.

## 2.3	LTM Execution Phase

Following steps are performed during execution phase.

Step 10-11: Based on L1 measurement reports, source gNB decides on the target cell and sends Cell Switch Command (CSC) towards the UE.

Step 12: The Source-CU sends a LTM CELL SWITCH NOTIFICATION to the Target gNB indicating the Target Cell ID and TCI State related information.

SN Status Transfer

As with other legacy handovers, the Source gNB should indicate to the Target gNB information related to Sequence Number. In our view, given that the information to be signalled is in line with that of legacy L3 handovers, the existing XnAP SN STATUS TRANSFER message can be reused. The timing for signalling this information is dependent on when data forwarding is triggered by the Source gNB-CU and is left up to implementation based on previous RAN3 agreement.

Proposal 7: The Source gNB indicates the SN Status to the Target gNB via the existing XnAP SN STATUS TRANSFER message.

Step 13: Source gNB-CU sends a SN STATUS TRANSFER message to target gNB and data forwarding is performed.

Step 14: UE marks LTM handover completion according to RAN2 specified indication (e.g., on reception of grant from target gNB).

## 2.4	LTM Completion Phase

During the Completion Phase, the following steps are performed.

Indication of LTM Handover Completion to Source gNB:

The Target gNB can reuse existing XnAP HANDOVER SUCCESS message to indicate to the Source gNB that the LTM Handover has completed.

Proposal 8: The existing XnAP HANDOVER SUCCESS message is reused to indicate to Source gNB that the LTM Handover has completed.

Step 15: RRCReconfigurationComplete message is sent to target gNB.

Step 16: The Target gNB sends the HANDOVER SUCCESS message to Source gNB.

Step 17-19: Path switch procedure is performed.

Step 20: Target gNB may send UE CONTEXT RELEASE message to release the UE context at the Source gNB.

UE Context Release after inter-CU LTM:

In case of LTM, it is assumed that configuration will not be released by UE unless it is explicitly reconfigured by the network. Hence, it should not be mandated that the network perform release procedure in case of inter-CU LTM. Similarly, in case that after inter-CU LTM mobility no LTM candidates remain at the Source gNB-CU, the “new” Source gNB-CU may trigger a UE context release at the old gNB as in legacy mobility.

Proposal 9: It is up to network configuration whether to release the resources for candidate cells after LTM cell switch.

Proposal 10: If after inter-CU LTM mobility the Source gNB-CU contains no LTM candidates, the existing XnAP UE CONTEXT RELEASE message is reused to release the UE context.

## 2.5	Stage-2 Signalling Flow for non-split Inter-CU LTM

Based on the above procedure, the stage-2 signaling flow for Inter-CU LTM is shown below. For subsequent LTM scenario, additional signalling may be required between Source and Target gNBs and to be FFS.

Figure 1: Inter-gNB-CU LTM Signaling with non-split architecture

# 3	Support for Subsequent inter-CU LTM Aspects

Support for Subsequent LTM will require additional information to be provided to Candidate gNB(s) that have LTM candidates. The following items are several aspects to be considered. Similarly, a signalling flows depicting these proposals is shown below.

## 3.1	Preparation Phase

Updating of LTM Candidate gNB(s):

To support Subsequent inter-CU LTM, the Candidate gNB(s) need to be aware of information pertaining all other candidates (e.g., TCI State information), which is consolidated and signaled by the Source gNB-CU. In our view, this process is specific to LTM and thus it is not appropriate to re-use the legacy XnAP HANDOVER REQUEST procedure. Hence, a new XnAP Class 1 procedure is needed for purpose of updating this information at the Candidate gNBs. Likewise, update of Candidate gNB(s) should take place during the preparation phase to speed up the mobility execution.

Proposal 11: For Subsequent LTM, the Source gNB updates the LTM related information toward all the Candidate gNBs during the preparation phase.

Proposal 12: Introduce a new Class-1 UE associated XnAP LTM MODIFICATION REQUEST procedure from Source gNB to Candidate gNBs to update LTM related information.

Figure 1: Preparation Phase Considerations for Subsequent Inter-gNB-CU LTM

Xn Interface between Candidate gNBs:

In case of Subsequent inter-CU LTM, there is need for the new Source gNB to be able to exchange messages with the Candidate gNBs. That is, there is need for an Xn control interface between all Candidate gNBs configured. Therefore, a prerequisite for inter-CU LTM shall be that an Xn control interface link exists between all Candidate gNBs.

Proposal 13: It is a pre-requisite for Inter-CU LTM candidate configuration that all Candidate gNBs have an Xn control interface established between each other.

## 3.3	LTM Execution Phase

Data Forwarding and TNL Information Update:

To support data forwarding in case of inter-CU LTM, the Source gNB needs to transfer information regarding PDU Session Resource Information List of all other Candidate gNBs to the Target gNB. Otherwise, the Target gNB (which becomes Source gNB after LTM mobility) will lack the necessary TNL information to execute data forwarding for subsequent inter-CU LTM. One possible alternative to address this issue is to signal this information after cell switch has been triggered toward the UE and include this information as part of the XnAP LTM CELL SWITCH NOTIFICATION message.

Proposal 14: The Source gNB includes the PDU Session Resource Information List information of all Candidate gNBs in the XnAP LTM CELL SWITCH NOTIFICATION message.

Figure 1: Execution Phase Considerations for Subsequent Inter-gNB-CU LTM

## 3.4	LTM Completion Phase

Handling of XnAP Identifiers:

When inter-CU LTM is firstly configured, a UE-associated connection will be established between the Source gNB and every Candidate gNB (e.g., when XnAP HANDOVER REQUEST and HANDOVER REQUEST ACKNOWLEDGE messages are exchanged). Hence, the XnAP Identifiers corresponding to a specific UE context will always point toward the Source gNB that firstly initiated the inter-CU LTM configuration. In other words, one pair of identifiers is linked by a parameter given by the Source gNB and another given by the Candidate gNB.

However, in case of Subsequent inter-CU LTM there is issue that after mobility from Source gNB to Target gNB, XnAP identifiers for the UE context become outdated, since the Source gNB has changed. Therefore, the UE-association should be updated with parameter that points toward the new Source gNB.

One alternative for carrying out this task is that after inter-CU LTM, the new Source gNB sends a UE-associated message toward all Candidate gNBs. This message will serve the purpose of updating the XnAP identifiers for the UE context.

Proposal 15: For Subsequent LTM, the new Source gNB signals its XnAP identifiers to all Candidate gNBs within the XnAP LTM MODIFICATION REQUEST message to update the information related to a given UE context.

Candidate Triggered gNB release:

At prior meeting, RAN3 agreed that a Candidate gNB can initiate release of LTM candidate cell(s) of its own. However, after inter-CU LTM mobility, there is problem that the new Source gNB-CU is unknown to all Candidate gNB(s) other than the gNB that initiated the mobility (i.e. the old Source gNB). Therefore, for a Candidate gNB to be able to trigger release of its own LTM candidate cells, it always needs to be aware of which gNB is the current Source gNB.

One way to solve this issue is to have the new Source gNB signal to all Candidate gNBs that it has become the new Source gNB. This can take place via the XnAP LTM MODIFICATION REQUEST message proposed above.

Proposal 16: After inter-CU LTM mobility, the new Source gNB signals the change of gNB via the Xn LTM MODIFICATION REQUEST message.

Figure 1: Completion Phase Considerations for Subsequent Inter-gNB-CU LTM

## 3.5	Subsequent Inter-CU LTM Signalling

Taking the prior proposals into account, the resulting signalling flow is the following.

Figure 1: Inter-gNB-CU LTM Signaling considering Subsequent LTM between Source gNB and Candidate gNBs with non-split architecture

# 4	Stage 2 Inter-CU LTM for split gNB Architecture

Similar to Section 2, this section describes the stage-2 signalling flow for inter-CU LTM considering disaggregated gNB architecture.

As starting baseline, existing procedures in Rel-18 in F1AP, E1AP and XnAP should be reused and applicable also for Rel-19 inter-CU LTM. With this assumption, the stage-2 signaling flow for Inter-CU LTM with disaggregated architecture is shown below.

Proposal 17: Reuse the existing Rel-18 F1/E1/Xn procedures for Rel-19 inter-CU LTM scenario as much as possible.

Figure 2: Inter-gNB-CU LTM Signaling with split architecture

Inclusion of source DU and source CU/gNB identity:

In Rel-18, CFRA resources to be used for random access can be shared only among UEs belonging to same DU. This allows the CFRA resource sharing among UEs without additional burden of signalling exchange for coordination among DUs. This is because, to have collision free random access only one CFRA resource (preamble) can be used at a given time.

In our view, this holds true also for inter-CU case and we should allow the possibility of CFRA resource sharing. Hence, both the source gNB-DU ID IE and source gNB ID IE are included in the HANDOVER REQUEST message.

Observation 1: CFRA resource sharing is allowed in intra-CU LTM and is beneficial for dense deployments where CFRA resources are scarce.

Proposal 18: Both source gNB ID (global ID) and source gNB-DU ID (local ID per CU) are provided to candidate target gNBs.

Proposal 19: The candidate target gNB allocates CFRA resources based on the source gNB ID and source gNB-DU ID.

# 5	Conclusions

Preparation Phase

Proposal 1: SSB related information concerning cells at Candidate gNBs is exchanged via Xn Setup and Configuration Update procedures between Source gNB and Target gNB(s).

Proposal 2: Periodic CSI-RS related information concerning cells at Candidate gNBs is exchanged via Xn Setup and Configuration Update procedures between Source/Target gNBs, and via F1 Setup and Configuration Update procedures between gNB-DU(s) and gNB-CU.

Proposal 3:  For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

Early Synchronization Phase

Proposal 4: Introduce a new Class-2 non-UE associated XnAP TA INFORMATION TRANSFER procedure from Target gNB to Source gNB to signal Early TA related information for LTM.

Proposal 5: Existing F1AP DU-CU TA INFORMATION TRANSFER and CU-DU TA INFORMATION TRANSFER messages can be reused in case of disaggregated gNB architecture for Inter-CU LTM scenario.

Proposal 6: RAN3 to study which information is to be sent from target gNB to source gNB along with TA values within XnAP TA INFORMATION TRANSFER and CU-DU/DU-CU TA INFORMATION TRANSFER messages.

LTM Execution Phase

Proposal 7: The Source gNB indicates the SN Status to the Target gNB via the existing XnAP SN STATUS TRANSFER message.

LTM Completion Phase

Proposal 8: The existing XnAP HANDOVER SUCCESS message is reused to indicate to Source gNB that the LTM Handover has completed.

Proposal 9: It is up to network configuration whether to release the resources for candidate cells after LTM cell switch.

Proposal 10: If after inter-CU LTM mobility the Source gNB-CU contains no LTM candidates, the existing XnAP UE CONTEXT RELEASE message is reused to release the UE context.

Subsequent LTM Considerations

Proposal 11: For Subsequent LTM, the Source gNB updates the LTM related information toward all the Candidate gNBs during the preparation phase.

Proposal 12: Introduce a new Class-1 UE associated XnAP LTM MODIFICATION REQUEST procedure from Source gNB to Candidate gNBs to update LTM related information.

Proposal 13: It is a pre-requisite for Inter-CU LTM candidate configuration that all Candidate gNBs have an Xn control interface established between each other.

Proposal 14: The Source gNB includes the PDU Session Resource Information List information of all Candidate gNBs in the XnAP LTM CELL SWITCH NOTIFICATION message.

Proposal 15: For Subsequent LTM, the new Source gNB signals its XnAP identifiers to all Candidate gNBs within the XnAP LTM MODIFICATION REQUEST message to update the information related to a given UE context.

Proposal 16: After inter-CU LTM mobility, the new Source gNB signals the change of gNB via the Xn LTM MODIFICATION REQUEST message.

Disaggregated gNB Considerations

Proposal 17: Reuse the existing Rel-18 F1/E1/Xn procedures for Rel-19 inter-CU LTM scenario as much as possible.

Observation 1: CFRA resource sharing is allowed in intra-CU LTM and is beneficial for dense deployments where CFRA resources are scarce.

Proposal 18: Both source gNB ID (global ID) and source gNB-DU ID (local ID per CU) are provided to candidate target gNBs.

Proposal 19: The candidate target gNB allocates CFRA resources based on the source gNB ID and source gNB-DU ID.

# References

- 3GPP TS 38.401

- RP-234036 NR mobility enhancements Phase 4, Intel (moderator)