3GPP TSG-RAN WG3#125 	R3-244043

Maastricht, Netherlands, 19-23 August 2024

Agenda item:	13.2 Support for inter-CU LTM

Source:	Nokia

Title:	Discussion on RRC Implications to RAN3 Signaling for Inter-gNB-CU LTM

Document for:	Discussion and Decision

# 1	Introduction

In regard to L1/L2 Triggered Mobility (LTM), a new Work Item has been approved (RP-234036 NR mobility enhancements Phase 4), including the following related objectives and scope impacting RAN3:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1] - Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritise intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

The following are agreements from the previous RAN3 meeting, which are basis for proposals in this paper.

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. Early data forwarding can be supported for inter-CU LTM. Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution. RAN3#124: Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS. FFS on the LTM modification procedures. FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

This paper discusses additional considerations to take into account for the Inter-CU LTM procedure and support for NR-DC.

# 2	RRC Signaling implications to RAN3 signaling

In our RAN2 TDoc contribution R2-2407073, it is proposed a simpler and modular RRC signaling structure which is both backward compatible as well as forward compatible. According to the proposed RRC signaling structure, Rel. 18 LTM reuse is maximized by enabling per-CU operations.

Support of multiple reference configurations:

One example of per-CU operation is the reference configuration plus delta configuration which is an LTM feature. In our proposal, reference configurations should be per-CU. That means, Rel. 19 inter-CU LTM should allow the support of multiple reference configurations. In our view, the benefits of such multiple reference configurations are multi-fold:

- It maximizes the use of the already agreed Rel. 18 structure.

- It minimizes signaling overhead associated with sending the RRC configurations and the required inter-CU coordination.

- It mitigates potential inter-operability issues arising in multi-vendor deployments of LTM.

Note that allowing multiple reference configuration does not preclude the possibility of using source CU reference configuration as the only reference configuration.

Observation 1: Supporting multiple reference configuration will minimize the inter-CU coordination.

Proposal 1: RAN3 to specify inter-CU signaling considering the support of reference configuration per CU.

Figure 1: Inter-CU LTM Preparation Phase with reference configuration per CU

Support of per-CU CSI Resource Configurations:

In the last RAN2 meeting, it was agreed that the source CU is responsible for generating LTM-CSI-Resource-configuration that contains of list of LTM-CSI-Resource-config elements based on the SSB configurations received from each candidate CU.  However, CU source determining the LTM-CSI-Resource-configuration that needs to be measured is not in line with the principle of serving cell determining the measurement and report configuration for subsequent cell changes.

Observation 2:  The current serving cell should determine the LTM-CSI-Resources to be measured and reported for LTM. In case of subsequent LTM it should be the candidate CU which controls the candidate CU. This is in line with the principle of candidate determining the Measurement-configuration for L3 based subsequent CPAC.

Moreover, source CU generation of LTM-CSI-Resource config will lead to restrictive configuration for subsequent LTM. For example, the LTM-CSI-Resource-configuration generated by source CU may only have one resource-set for each candidate cell that consists of all SSB of the candidate cell.  In case of CSI-RS based measurements source CU generate one CSI-RS resource set for each candidate cell.

Observation 3: When source CU generate LTM-CSI-Resource-config based on SSB and CSI-RS configurations of candidate cells it may only generate one resource set containing all the RS of candidates as possible resource configuration.

In this configuration, if the candidate CU wants the UE to measure its multiple neighbor cells, it needs to configure 2 LTM Report configurations each pointing to resource sets of each candidate cells. Moreover, if the candidate cell knows that only specific beams of neighboring cells are relevant for measurements it cannot generate resource-configuration limiting the specific beams.

The impact is more significant for CSI-RS resource-configuration where, measuring all the CSI-RS of candidate cells is not necessary. Even though it is possible for source node also to generate different combinations of resource-sets it may need to generate large number of resource-sets which may not be referred by candidate CU in report-configuration. Because the source node is not aware of the actual target cells /beams that each candidate cells wants to measure when it becomes serving cell.

Observation 4: When source CU generates LTM-CSI-Resource-config based on SSB and CSI-RS configurations of candidate cells, it is not an optimal configuration for the candidate CU after mobility when it becomes the Source CU and performs measurements for subsequent LTM.

For event based L1 measurement reporting, the LTM-CSI-Resource configurations to be used as source and target resource configuration needs to be determined by the serving cell. In case of subsequent LTM, it should be the candidate cell that determines the configuration. For instance, if a LTM candidate cell wants to configure reporting event that compares serving cell active beam against a subset of beams of a target cell, these subset of beams of the target cell should be configured as LTM-CSI-Resource configuration for UE to measure and compare. In this case this resource-set should be generated by the candidate CU and included as part of LTM-CSI-Resource-configuration.

Observation 5: For event based reporting the candidate cell should determine the target resource set (whether all beams or a subset of beams).  This is not possible if source CU determines LTM CSI-Resource-configuration only based on SSB or CSI-RS configurations.

Based on the above observations, the resulting signaling sequence for LTM preparation phase would be as follows.

- Exchange of SSB or CSI-RS configurations of candidate cells. This information exchange is carried out as non-UE associated signaling as proposed in our contribution in R3-244042, which allows each CU to be aware of latest SSB and CSI-RS configuration of other candidate cells.

- Source CU triggers LTM Handover preparation procedure with candidate CU. Each candidate CU prepare the LTM CSI Resource Configuration as well as CSI Report Configuration based on their awareness of SSB/CSI-RS of candidate cells and provides them to source CU.

- Source CU combines all the collected CSI-RS resource configurations from candidate CU.

- Source CU delivers the final LTM Configuration to UE.

Proposal 2: RAN3 to specify signaling to support LTM CSI Resource Configuration per CU.

Proposal 3: RAN3 to consider the above message sequence as baseline for L1 measurement configuration in case of inter-CU LTM.

Figure 1: Inter-CU LTM Preparation Phase with CSI Resource Configuration per CU

Configuration of Event-Triggered Reporting per-CU:

For event-triggered reporting, in addition to measurement resource configuration and reporting configuration, events will also be defined. Following the same principles, it would make sense if each CU could configure events to be used in the report configuration of each of its candidate cells.

Proposal 4: RAN3 to specify signaling to support the configuration of events for event-triggered reporting per CU.

# 3	Considerations for NR-DC and LTM Interworking

Scenarios for NR-DC with LTM

The following are NR-DC scenarios applicable for Rel-19 LTM work scope:

- Inter-MN PCell change

  - With SCG released

  - With SCG retained

- Inter-SN PSCell change with unchanged PCell

  - MN initiated

  - SN initiated

Based on WID [1] scope, the scenario of “Inter-SN PSCell change with simultaneous PCell change” is excluded from Rel-19 work.

In our view out of the NR-DC scenarios in scope of the WID, the “Inter-SN PSCell changed with unchanged PCell” scenario should be treated with higher priority.

Proposal 5: RAN3 to prioritize the NR-DC scenario of “Inter-SN PSCell change with unchanged PCell” to be specified under inter-CU LTM work in Rel-19.

Simultaneous MCG LTM and SCG LTM Configuration

There was agreement at last RAN3#123bis meeting that MCG LTM and SCG LTM are not to be configured simultaneously.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Given that LTM is configured by a gNB-CU based on L3 measurements, there is no guarantee that e.g., the SN is aware that LTM has been configured at the MN (MCG), since the SN addition may have occurred prior to LTM being configured. Thus, the SN may independently decide to configure LTM in the SCG unless it is explicitly aware that LTM is already configured at the MN. A similar scenario can occur in the opposite direction. Therefore, RAN3 should study whether and how to avoid LTM being configured at both MCG and SCG.

To avoid simultaneous LTM configuration at the MN and SN, one approach is to have an indication over Xn that informs the other node that LTM is to be configured. With this information, the receiving node can avoid configuring LTM. Likewise, a similar indication would be signalled upon LTM being released.

Proposal 6: In case of NR-DC, MN indicates to SN about MCG LTM configuration/de-configuration.

Proposal 7: In case of NR-DC, SN indicates to MN about SCG LTM configuration/de-configuration.

Inter-CU LTM when NR-DC is Configured

In TS 37.340, different dual connectivity scenarios and their respective signalling for PSCell addition, change, release or retention are described.  These cell changes or reconfigurations can be MN or SN initiated. Likewise, in Rel-18 various issues related to inter-SN PSCell change were discussed and agreements were made under S-CPAC feature that could be considered for LTM work in Rel-19.

In Rel-18 S-CPAC, data forwarding may be realized either as direct data forwarding or as indirect data forwarding. The same principles and framework may be reused for inter-CU SCG LTM scenarios. However, to enable data forwarding for inter-CU SCG LTM with subsequent cell change, since the new serving SN is not aware of configuration of other candidate SNs, the following should be considered:

- Transferring the Admitted PDU session resources configuration of each prepared candidate node to all the prepared candidate inter-CU SCG LTM nodes via Xn signaling to enable data forwarding after each inter-CU SCG LTM PSCell change.

  - Option 1: At LTM preparation, the source node signals the Admitted PDU session resources configuration of all the prepared candidate nodes to all the prepared candidate SNs.

  - Option 2: At the time of inter-CU SCG LTM PSCell change execution, the source node signals the Admitted PDU session resources configuration of all the prepared candidate nodes only to the next serving SN. The new serving SN would carry out the same procedure at the next inter-CU SCG LTM mobility execution.

Observation 6: The data forwarding framework and principles of Rel-18 S-CPAC may be re-used for inter-CU SCG LTM.

Proposal 8: To support direct and indirect data forwarding for inter-CU SCG for subsequent LTM, the configuration of Admitted PDU session resources of the prepared inter-CU SCG LTM candidate SNs is signalled to all the prepared candidate SNs.

Inter-CU SCG LTM preparation

In both MN initiated and SN initiated inter-CU SCG LTM preparation scenarios, the MN triggers the preparation of candidate SN by requesting the candidate SN(s) to allocate resources for the UE by means of the SN Addition procedure. The MN may prepare one or multiple candidate target SNs. Likewise, the candidate SN prepares the LTM candidate configuration including SCG configuration and LTM CSI Resource configuration.

As part of existing S-CPAC functionality, the S-CPAC Request IE was introduced in the SN ADDITION REQUEST message to indicate that the SN change is for S-CPAC. A similar approach can be used to enable inter-CU SCG LTM, where a specific IE within the SN ADDITION REQUEST message indicates to the candidate target SN that the preparation is for inter-CU SCG LTM.

Proposal 9: Introduce a new IE in SN ADDITION REQUEST message to indicate that the SN change is for inter-CU SCG LTM.

# 3	Conclusions

Per-CU Configuration

Observation 1: Supporting multiple reference configuration will minimize the inter-CU coordination.

Proposal 1: RAN3 to specify inter-CU signaling considering the support of reference configuration per CU.

Observation 2:  The current serving cell should determine the LTM-CSI-Resources to be measured and reported for LTM. In case of subsequent LTM it should be the candidate CU which controls the candidate CU. This is in line with the principle of candidate determining the Measurement-configuration for L3 based subsequent CPAC.

Observation 3: When source CU generate LTM-CSI-Resource-config based on SSB and CSI-RS configurations of candidate cells it may only generate one resource set containing all the RS of candidates as possible resource configuration.

Observation 4: When source CU generates LTM-CSI-Resource-config based on SSB and CSI-RS configurations of candidate cells, it is not an optimal configuration for the candidate CU after mobility when it becomes the Source CU and performs measurements for subsequent LTM.

Observation 5: For event based reporting the candidate cell should determine the target resource set (whether all beams or a subset of beams).  This is not possible if source CU determines LTM CSI-Resource-configuration only based on SSB or CSI-RS configurations.

Proposal 2: RAN3 to specify signaling to support LTM CSI Resource Configuration per CU.

Proposal 3: RAN3 to consider the above message sequence as baseline for L1 measurement configuration in case of inter-CU LTM.

Proposal 4: RAN3 to specify signaling to support the configuration of events for event-triggered reporting per CU.

NR-DC and LTM Interworking

Proposal 5: RAN3 to prioritize the NR-DC scenario of “Inter-SN PSCell change with unchanged PCell” to be specified under inter-CU LTM work in Rel-19.

Proposal 6: In case of NR-DC, MN indicates to SN about MCG LTM configuration/de-configuration.

Proposal 7: In case of NR-DC, SN indicates to MN about SCG LTM configuration/de-configuration.

Observation 6: The data forwarding framework and principles of Rel-18 S-CPAC may be re-used for inter-CU SCG LTM.

Proposal 8: To support direct and indirect data forwarding for inter-CU SCG for subsequent LTM, the configuration of Admitted PDU session resources of the prepared inter-CU SCG LTM candidate SNs is signalled to all the prepared candidate SNs.

Proposal 9: Introduce a new IE in SN ADDITION REQUEST message to indicate that the SN change is for inter-CU SCG LTM.

# References

- 3GPP TS 38.401

- RP-234036 NR mobility enhancements Phase 4, Intel (moderator)