3GPP TSG-RAN3 Meeting #125	R3-244046

Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	11.4

Source: 	Jio

Title: 	(TP for TR 38.744) AI/ML NG-RAN mobility

Document for:	Discussion/Decision

# Introduction

This paper contains the TP to capture the agreement for the AI/ML assisted mobility.

Following last’s WG3 meeting Huawei document R3-243469, a new SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” was approved for Rel-19 at RAN #102 meeting [3], then revised in RAN#103 and eventually approved in [4]. One of the objectives of the new SI is to further discuss the Rel-18 leftovers. The agreements and FFS of Rel-18 leftovers of RAN AI/ML in RAN3#123bis meeting are as follows [5]:

Agree to take as baseline for the Mobility Optimization for NR-DC that the use case is studied assuming inference at the MN. The main use case is limited to dual connectivity only (e.g. no conditional procedures are in scope). Agree to have discussions on AI/ML based energy saving at the next meeting, where enhancements will be based on new solutions not discussed in Rel18. It is strongly recommended not to reiterate Rel18 proposals on which consensus could not be achieved. If consensus on a solution approach is not reached at May meeting, the topic is down prioritized. It is agreed to evaluate solutions on multi hop trajectory and check their impact on specifications with the aim of minimizing that. If consensus on a solution approach is not reached at May meeting, the topic is down prioritized. There is no consensus on whether L1 measurements and their L3 translation for CCO and potential inference is at the gNB-DU for Network Slicing and CCO. We need to identify whether inference at the gNB-DU is needed based on the benefits and the identified cases compared with inference at the gNB-CU. We need to formulate the use case and solution for split architecture support, NR-DC

Layer 1 measurements in 5G include various metrics that reflect the radio environment and the performance of the physical layer. Key L1 measurements relevant to coverage and capacity optimization include:

- Reference Signal Received Power (RSRP): Measures the power of the received reference signal.

- Reference Signal Received Quality (RSRQ): Indicates the quality of the received reference signal.

- Signal-to-Interference-plus-Noise Ratio (SINR): Reflects the quality of the signal relative to interference and noise.

- Channel State Information (CSI): Provides detailed information about the state of the radio channel.

- Beam Reference Signal (BRS) Measurements: Used to evaluate the performance of specific beams.

On the other hand SSB Beam Shaping involves adjusting the shape of the beam to optimize coverage and capacity. Beam Steering refers to directing the beam towards specific users or areas based on real-time conditions. These techniques leverage the directional capabilities of massive MIMO (Multiple Input Multiple Output) antennas to enhance network performance.

The Coverage and Capacity Optimization Using SSB beam adjustments for L1 Measurements might include the

1. Beam Selection and Alignment [1]

- RSRP and RSRQ Measurements: By analyzing RSRP and RSRQ, the network can determine the optimal beams for each user. Higher RSRP values indicate stronger signal strength, while higher RSRQ values indicate better signal quality.

- Beam Steering: Once the optimal beam is identified, beam steering can be used to direct the beam towards the user, ensuring maximum signal strength and quality.

2. Dynamic Beam Management [1]

- SINR Analysis: By continuously monitoring SINR, the network can dynamically adjust beams to maintain optimal performance. High SINR values indicate a good quality signal, prompting the network to maintain the current beam direction.

- Beam Shaping: In areas with high interference, beam shaping can be used to adjust the beam pattern, minimizing interference and improving signal quality.

3. Adaptive Beamforming [1]

- CSI Utilization: Channel State Information provides detailed insights into the radio channel conditions. By using CSI, the network can adapt the beamforming strategy in real-time, optimizing the beam direction and shape for the current channel conditions.

- Coverage Optimization: Adaptive beamforming ensures that the beam is shaped to cover the intended area effectively, reducing coverage holes and ensuring consistent signal quality.

4. Interference Management [1]

- BRS Measurements: Beam Reference Signal measurements help in identifying and mitigating interference from neighboring cells. By analyzing BRS, the network can adjust the beams to minimize interference, enhancing both coverage and capacity.

- Digital Pre-Distortion, DPD: Estimate the available channel and apply AI/ML dynamic DPD correction to retain the system capacity and throughput in Interference limited environment [2]

- Coordinated Multi-Point (CoMP) Techniques: L1 measurements enable the implementation of CoMP techniques, where multiple cells coordinate their beamforming strategies to reduce interference and improve overall network performance.

5. User Mobility Management

- Mobility Patterns: By tracking the mobility patterns of users through L1 measurements, the network can predict user movement and proactively adjust beams. This ensures seamless coverage and consistent capacity, even as users move through the network.

- Handover Optimization: L1 measurements assist in optimizing handovers between cells and beams. By ensuring that handovers occur at the optimal time and to the best beam, the network can maintain high signal quality and capacity.

6. Load Balancing and Capacity Management

- Traffic Analysis: L1 measurements provide insights into traffic load and distribution. By analyzing these measurements, the network can steer beams to balance the load, ensuring that no single cell or beam is overloaded.

- Capacity Enhancement: In high-traffic areas, beam shaping and steering can be used to direct additional capacity to where it is needed most, enhancing the overall network capacity.

This contribution addresses the Rel-18 leftovers related to “Mobility optimization for NR-DC” providing our considerations and proposals for discussion.

# Text proposal to TR 38.744

## 4	AI mobility use case

Editor Note: this section intends to capture the study goal, detail description of use cases.

Managing user mobility efficiently has become a critical aspect of ensuring network performance and user satisfaction. Traditional mobility management techniques face challenges in coping with the dynamic and complex nature of modern wireless networks. AI and ML offer promising solutions to these challenges by enabling intelligent and adaptive mobility management.

Mobility management (MM) in NR NG-RAN involves several key functions, including cell reselection, handover, and context transfer. These functions are traditionally based on signal strength measurements. Therefore AI/ML threshold-based decision-making and heuristic algorithms should be included in order to enhance the potential performance. Solutions as CCO SSB beam shaping, SSB beam steering, Interference mitigation and Energy Cost function should be considered.

However there are severe limitations in the MM like

- Inaccurate handover decisions leading to call drops or suboptimal user experience.

- Inefficient resource allocation causing network congestion or underutilization.

- Inability to adapt to rapidly changing network conditions and user behaviors.

## 4.1	RRM measurement prediction

Editor Note: The RRM measurement refers to either L3 cell level measurement and/or L1 beam level measurement, from which L3 level measurement can be predicted.

Integrating AI/ML for predicting L3 cell level measurements from L1 beam level measurements and adjustments related to CCO [1] holds significant promise for enhancing mobility management in NR NG-RAN.

Current RRM techniques involve extensive L3 cell level measurements and L1 beam level measurements to manage handovers, load balancing, and other mobility functions. However, these methods face challenges such as increased signaling overhead and delayed decision-making.

AI/ML CCO SSB beam adjustments [1] and DPD optimization [2] will enhance the performance with minimum signaling. By reducing signaling overhead and enabling more proactive decision-making, AI/ML can lead to more efficient and resilient networks, meaning that by leveraging AI/ML, it is possible to predict L3 measurements from L1 measurements, thereby reducing overhead and improving efficiency.

## 4.2	Measurement Event prediction

Editor Note: The measurement event refers to those measurement events over Uu interface and for mobility purpose defined in clause 5.5.4 in 38.331. RAN2 can pick one or some of them as typical measurement event for study e.g. event A3.

Measurement events in 5G NR, such as Event A3, play a vital role in determining when a UE (User Equipment) should handover from one cell to another. Event A3 is triggered when the signal strength of a neighboring cell becomes better than the serving cell by a certain offset. Accurate prediction of such events, under the dynamic SSB beam shaping and steering [1], can optimize handover processes, reduce latency, and improve user experience.

AI/ML models, particularly supervised learning techniques, can be trained using historical measurement data to predict the occurrence of Event A3.

## 4.3	Failure event prediction

Editor Note: Here the intended events refer to radio link failure and handover failure.

Radio Link Failure (RLF) occurs when the connection between the UE (User Equipment) and the network is lost due to poor radio conditions, while Handover Failure (HOF) happens when a handover attempt fails, often leading to dropped calls or session interruptions. Current approaches to managing these failures are reactive and can benefit significantly from predictive techniques enabled by AI/ML.

To evaluate the performance a simulation setup is proposed to simulate an NR NG-RAN environment that mimics real-world conditions with various mobility patterns and network loads, implementing AI/ML models for predicting RLF and HOF and integrate them into the mobility management framework.

# References

# [1] Jio’s White paper

[2] Jio Provisional Patent P1314

[3] RP-234054, “New SID: Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#102

[4] RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

[5] 3GPP RAN3#123b Chairman’s notes (RAN3_123bis_agenda_20240419_EOM.doc)

[6] RP-234469, “Rel-18 leftovers: AI/ML-based mobility enhancements” Huawei, 3GPP RAN#124