3GPP TSG-RAN WG3 Meeting #125	R3-244051

Maastricht, NL, 19th – 23rd August 2024

Agenda Item:	14.3

Source:	Eutelsat Group

Title:	Regenerative payload considerations

Document for:	Information and Discussion

# Introduction

This contribution aims to provide an input to the RAN3 discussions on regenerative payload and its implications on RAN3 considerations as set out in the Rel-19 WID [1].

The information provided herein is based on public domain sources and is given to facilitate informed discussion.

# Discussion

## Terminology

Satellites in Geosynchronous Orbit (GSO) are approximately 35,786 km above average sea level and their orbital duration matches that of the earth’s rotation. A GSO satellite in inclined orbit will have, as seen from a terminal located on Earth, an apparent ‘figure of 8’ movement in the North-South direction. A Geostationary Equatorial Orbit (GEO) is a special case of GSO which maintains a constant position in the sky directly above the equator, therefore allowing the use of fixed parabolic dish or flat panel array antenna with fixed pointing at boresight. In practice, a GEO satellite maintains a close approximation to this ideal orbit.

Other orbits fall under the broad heading of Non-GSO (NGSO). Low Earth Orbit (LEO) is in the range 200 – 2000 km (more typically communications satellites orbital heights range from 300 – 1500 km as stated in [2]), and Medium Earth Orbit (MEO), as its name suggests sits between LEO and GEO, typically in the range 8,000 – 20,000 km.

So far, 3GPP has considered LEO-600, LEO-1200 and GSO/GEO orbit references from a RAN1/RAN2 and RAN4 work perspective. The orbits of interest in this contribution are illustrated diagrammatically in Figure 1 below.

Figure 1: Diagrammatic representation of GEO with LEO and MEO shown in polar orbits

## Orbital characteristics in the NGSO case

Figure 1 illustrates a plane of LEO satellites in polar orbit, however, non-GSO orbits can be inclined. In that case, satellites do not go over the poles but only to lower latitude regions. A special case of polar orbit is ‘sun-synchronous orbit’ whereby the satellite revisits the same point on earth at the same time of day. This type of orbit is typically used for earth observation and is not so relevant for communications – for the purposes of satellite communication systems, it can be reasonably assumed that a typical LEO satellite will be operating in darkness (on battery power) for a significant portion of its orbit.

Typical LEO satellite orbital velocities are around 7 – 7.5 km/s and orbit durations (one complete circle) are around 90 – 110 minutes depending on the altitude of the satellite. MEO orbital velocities are lower than LEO (circa. 3 km/s) and orbital durations are several hours, 12 hours may be considered a typical figure.

It should be noted that during the satellite orbit the Earth rotates, so at the next immediate orbit the satellite will not pass over the same locations. As the satellite orbit plane remains fixed with respect to stars, the same location may cross again the orbit plane in 12 hours depending on the orbital parameters – so the same satellite will see again the same location on earth after approximately 12 hours, and moving in opposite direction (e.g., North to South instead of South to North).

From a fixed location on earth, approximately all satellites of the constellation will be visible within 12 to 24 hours. However, not all visible satellites can provide service, due to different reasons, e.g.:

- The elevation of the satellite is too low for a proper service (poor performance of flat array antennas at low elevation + bad link budget due to distance);

- There are radio path blockages due to the terminal environment;

- The satellite is too close to a GEO arc that needs to be prioritized, and therefore service must be turned off to avoid any interference to GEO satellites or terminals;

- The satellite is too close to other satellites (of the same or different operator);

- There is already another satellite of the same operator serving this location.

Observation 1: A satellite in Low Earth Orbit (LEO) typically completes one orbit of the earth every 90 – 110 minutes. Thus, typically, any given LEO satellite will ‘fly over’ approximately the same position on earth (i.e., provide coverage to, rather than pass exactly over) twice per day (once travelling North to South and once travelling South to North).

It is important to note that ‘fly over’ visibility durations are not the same on each pass; visibility depends upon the geometry (i.e., the relative positions of the satellite and terrestrial station) and minimum ‘look angles’ of the satellite and terrestrial station antennas. An example is provided below, from [3] illustrates the visibility duration of a single satellite at 600 km for a terrestrial station antenna minimum ‘look angle’ of 30 degrees.

Figure 2: Flyovers and duration of visibility (≥ 30 degrees elevation) 
for a single LEO-600 km satellite

Observation 2: The LEO satellite case is the ‘critical case’ for Earth to Space connectivity management (GEO satellites stay permanently over the same geographic area/ stay connected to the same ground station, MEO satellites coverage durations are significantly longer than for LEO satellites).

Observation 3: NGSO LEO satellite visibility times (to users and ground stations) vary from a very short duration (“glancing” contact) to several minutes (zenith pass).

## Satellite implementation

At present, it is expected that GSO/GEO satellite implementations will rely on transparent payloads for the foreseeable future. However, the trend for satellites in LEO and MEO is towards regenerative payloads for:

- more efficient support of ISLs;

- improved link budget (which improves data rates and coverage);

- extended coverage (or further reduction of ground stations) thanks to the possible use of Inter-Satellite Links (ISLs);

- reduced signalling latency (e.g. for HARQ process).

A satellite with a transparent payload is functionally a repeater, typically with frequency translation between the feeder link and service link. This was the baseline assumption for Rel-17. The advantage of this approach is two-fold: flexibility (gNB hardware on the ground can be upgraded) and low complexity.  However, this approach brings the technical disadvantages associated with repeaters including impaired RF performance: e.g., transmitter and receiver noise figure (the satellite transceiver subsystem performance cascades with the transceiver on the ground). The additional delay over the feeder link will reduce the effectiveness of HARQ - either requiring more HARQ processes or rendering HARQ of no/little practical value.

In contrast, a satellite with a regenerative payload has at least part of the gNB onboard (in the Rel-19 WID [1] the assumed architecture is full gNB onboard).

Hosting Layer 1 onboard enables better RF performance since, in reception, the signal from the UE is processed onboard and digitized before being sent over the feeder link and, similarly, in transmission analogue imperfections are not 'amplified'. Beamforming can also be performed onboard as the beamformer weights/ active spot beams can be signalled digitally. Feeder link compensation is also simplified as frequency/ time errors on the feeder link no longer directly affect the service link. Layer 1 onboard thus simplifies the feeder link implementation.

Furthermore, including Layer 2/3 processing onboard provides additional potential performance benefits including reduced HARQ ACK/NACK response time and reduced signalling latency (e.g., typically halving signalling round trip delay thus enabling more responsive slot-wise scheduling).

For the reasons set out above, it was decided that a full gNB onboard would be the working assumption for Rel-19.

Observation 4: Hosting a regenerative payload onboard a LEO/ NGSO satellite (full gNB in the context of Rel-19) brings several advantages over Rel-17/18 assumptions of transparent payload.

Observation 5: It is worth noting the potential dissymmetry of preferred NTN implementation choices between LEO (or NGSO in general) and GEO based services; the latter will remain associated with transparent satellite architecture whilst LEO will adopt a regenerative payload.

## Constrained resources on-board a regenerative satellite

Satellite payload design presents some unique challenges and a different set of constraints compared with TN.

- Limited power – limited by solar cell capability, battery and thermal design considerations

  - This limits RF power and processing power available

  - Need to charge up battery when in sunlight (apart from satellites in sun-synchronous orbits (SSO), most LEO satellites will experience periods of no sunlight – typically about half of each orbit)

  - Need to conserve energy (including shutting down) when running on onboard battery when out of sunlight, or for periods of time during which less/no onboard processing activity is required.

    - When multiple satellites cover an area of low capacity it is desirable to minimise energy consumption to the maximum extent possible,

    - Power saving may also imply shutting off some beams, reducing the processed bandwidth, etc.

- Limited thermal dissipation capabilities

- Limited onboard memory – onboard memory is both expensive and vulnerable to corruption (requires redundancy)

- Limited size and mass – launch costs depend upon the size and mass of the satellite, hence the need to keep onboard electronic systems to a realistic minimum.

- Limited in-service upgradability

  - Software upgrades only possible within limitations of the launch onboard processing power and memory sizes and configuration,

  - No hardware upgrades possible in the foreseeable future.

- Limited feeder link (‘backhaul’) capacity to any one satellite

Observation 6: The impact of constrained resources onboard a LEO/ NGSO satellite should be taken into account.

## Frequent change of association between LEO satellites and ground stations

The association between any given LEO satellite and any given ground station is transitory. A change of association between ground stations and LEO satellites may occur at time intervals as low as 5 to 15 minutes depending upon the geometry of the satellite’s path relative to the ground station.

Observation 7: The association between a LEO satellite and a ground station is transitory with a typical duration of a few minutes.

Proposal: RAN3 is requested to take the observations made in this contribution into account in their work on ‘Support of regenerative payload’.

# Conclusions

This contribution has outlined the aspects of NGSO constellation and satellite payload design that we consider to be most relevant to the RAN3 work on Regenerative Payload support in Rel-19. The most important differences between service from a GEO/GSO satellite (or a fleet of GEO/GSO satellites) and a NGSO constellation have also been explained.

Observation 1: A satellite in Low Earth Orbit (LEO) typically completes one orbit of the earth every 90 – 110 minutes. Thus, typically, any given LEO satellite will ‘fly over’ approximately the same position on earth (i.e., provide coverage to, rather than pass exactly over) twice per day (once travelling North to South and once travelling South to North).

Observation 2: The LEO satellite case is the ‘critical case’ for Earth to Space connectivity management (GEO satellites stay permanently over the same geographic area/ stay connected to the same ground station, MEO satellites coverage durations are significantly longer than for LEO satellites).

Observation 3: NGSO LEO satellite visibility times (to users and ground stations) vary from a very short duration (“glancing” contact) to several minutes (zenith pass).

Observation 4: Hosting a regenerative payload onboard a LEO/ NGSO satellite (full gNB in the context of Rel-19) brings several advantages over Rel-17/18 assumptions of transparent payload.

Observation 5: It is worth noting the potential dissymmetry of preferred NTN implementation choices between LEO (or NGSO in general) and GEO based services; the latter will remain associated with transparent satellite architecture whilst LEO will adopt a regenerative payload.

Observation 6: The impact of constrained resources onboard a LEO/ NGSO satellite should be taken into account.

Observation 7: The association between a LEO satellite and a ground station is transitory with a typical duration of a few minutes.

Proposal: RAN3 is requested to take the observations made in this contribution into account in their work on ‘Support of regenerative payload’.

# References

[1] RP-241667 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3 (Rel-19) - RAN#104 - Shanghai

[2] 3GPP TS 38.300 NR; NR and NG-RAN Overall description; Stage-2, v.18.2.0

[3] R1-2106776 On LEO satellite flyover timing and discontinuous coverage, Eutelsat, RAN WG1 Meeting #106-e