3GPP TSG-RAN WG3 Meeting #125	R3-244052

Maastricht, NL, 19th – 23rd Aug 2024

Title: 	Discussion on emergency call support for (e)RedCap and 2RX XR UEs in barred cells

Source: 	ZTE Corporation, China Telecom, China Unicom

Agenda item:	8.3

Document for:	Discussion and Decision

- Introduction

In the RAN2 #125bis meeting, RAN2 has sent an LS (R3-243012/ R2-2403997: LS on emergency call support for (e)RedCap in barred cells) to RAN3, details as below.

1. Overall Description: In RAN2#125bis RAN2 have endorsed the attached CRs for (e)RedCap UEs with the description of the CRs as below: If a (e)RedCap UE is barred in a cell where (e)RedCap is enabled since (e)RedCap UEs with 1Rx branch or 2Rx branches or both are barred in the cell, network may allow those (e)RedCap UEs to consider the cell as acceptable cell for emergency calls if cell selection criteria is fulfilled and, if the (e)RedCap UE supports only half duplex FDD operation, HD-FDD operation is allowed in the cell. The agreed RAN2 CRs also include the means for the cell to allow or not, the above operation via a SIB1 field. The CRs (R2-2402902, R2-2402903) when agreed, are considered to be implementable by Rel-17 UEs without any interoperability issue. 2. Actions: To RAN3 ACTION: RAN2 respectfully asks RAN3 to take the above information into account and update RAN3 specifications if needed.

Moreover, in the RAN2 #126 meeting and post-email discussion, RAN2 re-discussed this issue and decided to design a common solution for (e)RedCap and 2RX XR UEs.

Agreements

1	NES (i.e. ingoring MIB barring) will not be considered in our common solution discussion.  FFS if anything specific for NES will need to be done.  If anything needs to be done, it would not be part of the common solution.

2	For Rel-18, we introduce 1 bit that enables EM call for RedCap, eRedCap, and 2Rx XR. One RRC Rel-18 with magic sentence that it is early implementable in Rel-17.  A CR for 38.304 doesn’t need to have the magic sentence.

3	This replaces the previous agreement and we will notify RAN3 verbally via delegates

Based on the above agreement, RAN2 agreed the following CRs.

R2-2405956	Introduction of barring exemption for RedCap UEs for emergency calls [RedCap_EM_Call]	Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson, Nokia	CR	Rel-18	38.304	18.1.0	0380	5	B	TEI18

=> Agreed

R2-2405957	Introduction of barring exemption for RedCap UEs for emergency calls [RedCap_EM_Call]	Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson, Nokia	CR	Rel-18	38.331	18.1.0	4570	4	B	TEI18

=> Agreed

=> FFS check in RAN2-127 if further clarifications are required to ensure that once the cell is considered acceptable according to new conditions, the UE doesn’t follow the procedures in 38.304 associated with cell barred case.

R2-2405958	Introduction of barring exemption for (e)RedCap and 2RX XR UEs for emergency calls [EM_Call_Exemption]	Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson. Nokia	CR	Rel-18	38.304	18.1.0	0381	6	B	TEI18

=> Agreed

This paper addresses to enhance RAN3 specifications to align with RAN2 solution on emergency call support for (e)RedCap and 2RX XR UEs in barred cells.

- Discussion

  - RAN2 solution

In TS38.331 CR, a new IE” barringExemptEmergencyCall-r18” is added in the SIB1 message, to allow (e)RedCap and 2RX XR UEs to have access to the cell to make an emergency call or receive emergency information broadcast .

SIB1 message <Skip irrelated part> SIB1-v1800-IEs ::= SEQUENCE { ncr-Support-r18 ENUMERATED {true} OPTIONAL, -- Need S mt-SDT-ConfigCommonSIB-r18 MT-SDT-ConfigCommonSIB-r18 OPTIONAL, -- Need R musim-CapRestrictionAllowed-r18 ENUMERATED {true} OPTIONAL, -- Need R featurePriorities-v1800 SEQUENCE { msg1-Repetitions-Priority-r18 FeaturePriority-r17 OPTIONAL, -- Need R eRedCapPriority-r18 FeaturePriority-r17 OPTIONAL -- Need R } OPTIONAL, -- Need R si-SchedulingInfo-v1800 SI-SchedulingInfo-v1800 OPTIONAL, -- Need R cellBarredATG-r18 ENUMERATED {barred, notBarred} OPTIONAL, -- Need S cellBarredNES-r18 ENUMERATED {notBarred} OPTIONAL, -- Need R mobileIAB-Cell-r18 ENUMERATED {true} OPTIONAL, -- Need R eDRX-AllowedInactive-r18 ENUMERATED {true} OPTIONAL, -- Cond EDRX-RC intraFreqReselection-eRedCap-r18 ENUMERATED {allowed, notAllowed} OPTIONAL, -- Need S nonServingCellMII-r18 ENUMERATED {true} OPTIONAL, -- Need R sdt-BeamFailureRecoveryProhibitTimer-r18 ENUMERATED {ms50, ms100, ms200, ms500, ms1000, ms1500, ms2000, ms3000} OPTIONAL, -- Need R eRedCap-ConfigCommon-r18 ERedCap-ConfigCommonSIB-r18 OPTIONAL, -- Need R cellBarredFixedVSAT-r18 ENUMERATED {barred, notBarred} OPTIONAL, -- Cond NTN cellBarredMobileVSAT-r18 ENUMERATED {barred, notBarred} OPTIONAL, -- Cond NTN reselectionMeasurementsNR-r18 ENUMERATED{true} OPTIONAL, -- Need R cellBarred2RxXR-r18 ENUMERATED {true} OPTIONAL, -- Need R intraFreqReselection2RxXR-r18 ENUMERATED {allowed, notAllowed} OPTIONAL, -- Need R barringExemptEmergencyCall-r18 ENUMERATED {true} OPTIONAL, -- Cond EM-Barring nonCriticalExtension SEQUENCE {} OPTIONAL } UAC-AccessCategory1-SelectionAssistanceInfo ::= ENUMERATED {a, b, c} UAC-AC1-SelectAssistInfo-r16 ::= ENUMERATED {a, b, c, notConfigured} SDT-ConfigCommonSIB-r17 ::= SEQUENCE { sdt-RSRP-Threshold-r17 RSRP-Range OPTIONAL, -- Need R sdt-LogicalChannelSR-DelayTimer-r17 ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1} OPTIONAL, -- Need R sdt-DataVolumeThreshold-r17 ENUMERATED {byte32, byte100, byte200, byte400, byte600, byte800, byte1000, byte2000, byte4000, byte8000, byte9000, byte10000, byte12000, byte24000, byte48000, byte96000}, t319a-r17 ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms2000, ms3000, ms4000, spare7, spare6, spare5, spare4, spare3, spare2, spare1} } RedCap-ConfigCommonSIB-r17 ::= SEQUENCE { halfDuplexRedCapAllowed-r17 ENUMERATED {true} OPTIONAL, -- Need R cellBarredRedCap-r17 SEQUENCE { cellBarredRedCap1Rx-r17 ENUMERATED {barred, notBarred}, cellBarredRedCap2Rx-r17 ENUMERATED {barred, notBarred} } OPTIONAL, -- Need R ... } ERedCap-ConfigCommonSIB-r18 ::= SEQUENCE { cellBarredeRedCap-r18 SEQUENCE { cellBarredeRedCap1Rx-r18 ENUMERATED {barred, notBarred}, cellBarredeRedCap2Rx-r18 ENUMERATED {barred, notBarred} } } FeaturePriority-r17 ::= INTEGER (0..7) MT-SDT-ConfigCommonSIB-r18 ::= SEQUENCE { mt-SDT-RSRP-Threshold-r18 RSRP-Range OPTIONAL, -- Need S sdt-LogicalChannelSR-DelayTimer-r18 ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1} OPTIONAL, -- Cond MT-SDT1 t319a-r18 ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms2000, ms3000, ms4000, spare7, spare6, spare5, spare4, spare3, spare2, spare1} OPTIONAL -- Cond MT-SDT2 } -- TAG-SIB1-STOP -- ASN1STOP SIB1 field descriptions. barringExemptEmergencyCall Indicates whether the cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [20].. cellBarred2RxXR Indicates whether the cell is barred for 2Rx XR UEs. This field is ignored by all UEs that are not 2Rx XR UEs. This field may be configured only if the cell operates in a frequency band where 4Rx antenna ports are mandated as specified in TS 38.101-1 [15]. If this field is absent on a cell operating in a frequency band where 4RX antenna ports are mandated, a 2RX XR UE shall treat the cell as not barred, as specified in TS 38.304 [20].. cellBarredATG Value barred means that the cell is barred for connectivity to ATG, as defined in TS 38.304 [20]. Value notBarred means that the cell is allowed for connectivity to ATG. If not present, the UE considers the cell is not allowed for connectivity to ATG, as defined in TS 38.304 [20]. This field is only applicable to ATG-capable UEs.. cellBarred-eRedCap1Rx Value barred means that the cell is barred for an eRedCap UE with 1 Rx branch, as defined in TS 38.304 [20]. This field is ignored by non-eRedCap UEs.. cellBarred-eRedCap2Rx Value barred means that the cell is barred for an eRedCap UE with 2 Rx branches, as defined in TS 38.304 [20]. This field is ignored by non-eRedCap UEs.; cellBarredFixedVSAT Value barred means that the cell is barred for fixed VSAT UEs, as defined in TS 38.304 [20]. If not present, the cell is not allowed for fixed VSAT UEs. This field is ignored by non-VSAT UEs.. cellBarredFixedVSAT Value barred means that the cell is barred for fixed VSAT UEs, as defined in TS 38.304 [20]. If not present, the cell is not allowed for fixed VSAT UEs. This field is ignored by non-VSAT UEs.; cellBarredMobileVSAT Value barred means that the cell is barred for mobile VSAT UEs, as defined in TS 38.304 [20]. If not present, the cell is not allowed for mobile VSAT UEs. This field is ignored by non-VSAT UEs.. cellBarredMobileVSAT Value barred means that the cell is barred for mobile VSAT UEs, as defined in TS 38.304 [20]. If not present, the cell is not allowed for mobile VSAT UEs. This field is ignored by non-VSAT UEs.; cellBarredNES This field indicates the cell barring status for UEs supporting nes-CellDTX-DRX as described in 5.2.2.4.2.. cellBarredNES This field indicates the cell barring status for UEs supporting nes-CellDTX-DRX as described in 5.2.2.4.2.; cellBarredNTN Value barred means that the cell is barred for connectivity to NTN, as defined in TS 38.304 [20]. Value notBarred means that the cell is allowed for connectivity to NTN. If not present, the UE considers the cell is not allowed for connectivity to NTN, as defined in TS 38.304 [20]. This field is only applicable to NTN-capable UEs.. cellBarredNTN Value barred means that the cell is barred for connectivity to NTN, as defined in TS 38.304 [20]. Value notBarred means that the cell is allowed for connectivity to NTN. If not present, the UE considers the cell is not allowed for connectivity to NTN, as defined in TS 38.304 [20]. This field is only applicable to NTN-capable UEs.; cellBarredRedCap1Rx Value barred means that the cell is barred for a RedCap UE with 1 Rx branch, as defined in TS 38.304 [20]. This field is ignored by non-RedCap UEs.. cellBarredRedCap1Rx Value barred means that the cell is barred for a RedCap UE with 1 Rx branch, as defined in TS 38.304 [20]. This field is ignored by non-RedCap UEs.; cellBarredRedCap2Rx Value barred means that the cell is barred for a RedCap UE with 2 Rx branches, as defined in TS 38.304 [20]. This field is ignored by non-RedCap UEs.. cellBarredRedCap2Rx Value barred means that the cell is barred for a RedCap UE with 2 Rx branches, as defined in TS 38.304 [20]. This field is ignored by non-RedCap UEs..

Observation 1: In RAN2 solution, a new IE “barringExemptEmergencyCall” is added in the SIB1 message for (e)RedCap and 2RX XR UEs to have access to the cell to make an emergency call or receive emergency information broadcast.

  - RAN3 specification enhancement

In current XnAP and F1AP specifications, the “(e)RedCap Broadcast Information” IE has indicated intraFreqReselection-eRedCap, halfDuplexRedCapAllowed, cellBarredRedCap1Rx and cellBarredRedCap2Rx, which are defined in SIB1.

Since the new IE“barringExemptEmergencyCall” is added in the SIB1 message, used for (e)RedCap and 2RX XR UEs, it is also suggested to add this new IE in RAN3 specs accordingly.

Proposal 1: In both TS38.423 and TS38.473, a new IE (e.g., Barring Exemption) in added in the IE (Served Cell Information NR) to align with RAN2 agreement.

Proposal 2: Agree the TS38.423 CR and TS38.473CR, and send LS to RAN2.

- Conclusion

After the above analysis, we provide the following proposals.

Observation 1: In RAN2 solution, a new IE “barringExemptEmergencyCall” is added in the SIB1 message for (e)RedCap and 2RX XR UEs to have access to the cell to make an emergency call or receive emergency information broadcast.

Proposal 1: In both TS38.423 and TS38.473, a new IE (e.g., Barring Exemption) in added in the IE (Served Cell Information NR) to align with RAN2 agreement.

Proposal 2: Agree the TS38.423 CR and TS38.473CR, and send LS to RAN2.

- References

- R3-243012 LS on emergency call support for (e)RedCap in barred cells LSin

- R3-244053 Introduction of barring exemption for (e)RedCap and 2RX XR UEs, Rel-18, TS 38.423

- R3-244078 Introduction of barring exemption for (e)RedCap and 2RX XR UEs, Rel-18, TS 38.473

- R3-244054 [Draft] Reply LS on emergency call support for (e)RedCap and 2RX XR UEs