3GPP TSG-RAN WG3 Meeting #125	R3-244055

Maastricht, NL, 19th – 23rd Aug 2024

Agenda Item:	13.2

Source:	ZTE Corporation

Title:	Discussion on inter-CU LTM

Document for:	Discussions & Approval

- Introduction

We made the following progress at the RAN3#123-bis meeting.

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. Early data forwarding can be supported for inter-CU LTM. Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

We made the following progress at the RAN3#124 meeting.

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS. FFS on the LTM modification procedures. FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Meanwhile, RAN2 #126 meeting also made the following progress

Agreements on inter-CU LTM:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this paper, we would like to further discuss how to support inter-CU LTM.

- Discussion

For Rel-18 intra-CU LTM, the overall procedure has been specified in the TS 38.300 [3]. The overall procedure includes the following four steps: LTM preparation, early sync, LTM cell switch execution, and LTM cell switch completion.

  - LTM preparation

    - One or more candidate cells within each LTM configuration

We have agreed to enhance current XnAP: Handover preparation procedure for inter-CU LTM. However, we have not achieved consensus either one candidate cell or multiple candidate cell within the handover request message.

For the L3 (Conditional) Handover preparation procedure, there is only one candidate cell within the HO request message. The configuration procedure is triggered per cell.

So, we propose to reuse the same way, to avoid unnecessary complexity and enhancement, i.e., one handover request message includes one candidate cell for inter-CU LTM, which is aligned with the cell specific information provided in the handover messages.

Proposal 1: Parallel procedures are used to prepare LTM candidate configurations, i.e., one handover preparation procedure is for one LTM candidate cell.

    - Maximum Number of LTM Preparations

In current CHO preparation, the maximum numbers can be included in the HO ACK message. Detail is as below

If the Maximum Number of CHO Preparations IE is included in the Conditional Handover Information Acknowledge IE contained in the HANDOVER REQUEST ACKNOWLEDGE message, then the source NG-RAN node should not prepare more candidate target cells for a CHO for the same UE towards the target NG-RAN node than the number indicated in the IE.

For the similar reason, we also propose to introduce maximum number of LTM preparation.

Proposal 2: Introduce a new IE (i.e., Maximum Number of LTM preparations) in Handover request ACK message.

    - One or more reference configurations

In Rel-18, a single reference configuration is used for intra-CU LTM. For inter-CU LTM, each candidate CU can have different capability and different configuration, so RAN2 is now discussing whether one or more reference configurations.

From RAN3 perspective, the reference configuration design shall also impact the Xn interaction signalling design. If only one reference configuration is introduced, the source CU needs to decide which CU (e.g. the source CU, or any one of candidate CUs) to generate the reference configuration. If a candidate CU is selected to generate the reference configuration, the source CU sends a reference configuration request indication to the candidate DU, e.g. via HO request message. The candidate CU responds with the generated reference configuration to the source CU, e.g. via HO request ACK message. And then the source CU needs to inform the received reference configuration to other candidate CUs. However, if multiple reference configurations are used, e.g. each reference per CU, the candidate CU can decide whether to use the reference configuration and generate the reference configuration by itself. Then the candidate CU can send the generated reference configuration to the source CU. The source CU does not need to inform the reference to other candidate CUs. Therefore, the inter-node interaction for multiple reference solution is simpler than the single reference solution.

Observation 1: From the perspective of inter-node interaction design, multiple reference solution is simpler than the single reference solution.

Proposal 3: Wait for RAN2 progress on whether one or more reference configurations for inter-CU LTM.

    - LTM Modification

In R16 CHO, we decided to reuse the HANDOVER REQUEST message to replace a prepared CHO candidate configuration by including the CHO trigger IE set to "CHO-replace". For LTM, the source node may decide to modify an already prepared LTM candidate cell in the candidate gNB, e.g., the source gNB may update the LTM reference configuration and provide the updates to the candidate gNB.

More, we shall also use LTM modification procedure to signal Common CSI resource configuration.

RAN2 agreement

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Based on the RAN2 agreement as above, regarding how to share the candidate cells information across multiple gNB-CUs/gNB-DUs, the similar coordination procedure as inter-DU LTM can be reused for inter-CU LTM by extending the interaction between source gNB-CU and candidate gNB-CU via Xn messages.

In current TS38.423, the code point “CHO-replace” in HANDOVER REQUEST message, is used to modify the prepared Handover produce.  However, in order for easy extension, we prefer to introduce a new Class1 procedure.

Proposal 4: Introduce a new UE associated Class 1 message, e.g., Handover modification procedure, to perform e.g., LTM modification including notification of common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs.

    - Data forwarding

At the RAN3#123bis meeting, we have agreed the following:

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Early Data Forwarding is initiated before the UE executes the handover. For L3 Handover procedure, it can be initiated after either source gNB receives HO request ACK message from candidate gNB or source gNB receives RRCReconfigurationComplete message from UE.

Proposal 5: Early data forwarding is initiated during inter-CU LTM preparation phase. It’s up to source gNB’s implementation when to trigger the early data forwarding during inter-CU LTM preparation phase.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

Late Data Forwarding is initiated after the source NG-RAN node knows that the UE has successfully accessed a target NG-RAN node.

Proposal 6: Late data forwarding is initiated	after source gNB receives Handover Success message from target gNB.

In [R3-243395], it provides several options to support subsequent inter-CU LTM.

Take the UE moves from the source gNB to the candidate gNB1 and then to the candidate gNB2 for example. There are three options for the new source gNB to know the data forwarding information associated with other candidate gNBs. - Option 1: the source gNB collects the data forwarding information associated with all the candidate gNBs from each candidate gNB, and sends it to all the candidate gNBs during the LTM preparation phase. For example, the source gNB sends the data forwarding information to the gNB1 and gNB2 during the LTM preparation phase. This option may introduce new XnAP signalling between the source gNB and the candidate gNBs. - Option 2: the source gNB collects the data forwarding information associated with all the candidate gNBs from each candidate gNB, and sends it to the selected target gNB, e.g., via Cell Switch Notification message. For the subsequent inter-CU LTM, the new source gNB sends the data forwarding information associated with all the candidate gNBs to the new selected target gNB. For example, the source gNB sends the data forwarding information to the gNB1 during the first-time cell switch, and then gNB1 sends the data forwarding information to the gNB2 during the second-time cell switch. - Option 3: the source gNB collects the data forwarding information associated with all the candidate gNBs from each candidate gNB, and sends it to the selected target gNB, no matter which time the inter-CU LTM is performed. For example, the source gNB sends the data forwarding information to the gNB1 for the first-time cell switch, and the source gNB sends the data forwarding information to the gNB2 for the second-time cell switch.

From our view, all of above options are workable, but we think option 2 is straightforward. The reason is that, data forwarding is an optional feature, in legacy, it is the source node to request data forwarding if needed, and then the target node reply the data forwarding address to the source node if agreeable.

In legacy handover procedure, The source NG-RAN node may suggest downlink data forwarding per QoS flow established for a PDU session and may provide information how it maps QoS flows to DRBs. The target NG-RAN node decides data forwarding per QoS flow established for a PDU Session. In other words, which QoS flow will be suggested depends on the new source gNB, and which QoS flow will be accepted is determined by new candidate gNB.

Further, the DRB mapping in source gNB and target gNB also has impact on the need of data forwarding. In LTM, the QoS to DRB mapping in the new source gNB may be different to that in the old source gNB. Thus, per DRB data forwarding configuration need to be negotiated between the new source gNB and candidate gNB(s).

So, it is reasonable to negotiate new data forwarding TNL information after a LTM procedure is completed and before the subsequent LTM.

Proposal 7: After LTM completion, for subsequent LTM, the new serving gNB can request other candidate node(s) to perform data forwarding per PDU session/DRB. New serving gNB and other candidate gNB negotiate data forwarding after LTM completion and before next subsequent LTM.

    - LTM cancel

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

On the other hand, similar to conditional HO cancel procedure, candidate gNB is also allowed to cancel the existing LTM.

Proposal 8: The Conditional Handover Cancel message is reused by the target gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

  - Early sync

Introduce a new procedure on Xn to transfer the TA information.

The UE may perform UL synchronization (e.g. UE-based TA measurement, PDCCH order triggered early RACH) with the candidate cell(s) before receiving the cell switch command, if indicated by the NW. Taking the PDCCH order triggered early RACH as an example, if the UL synchronization is triggered by a PDCCH order from the source cell, the UE sends preamble towards the indicated candidate cell. The candidate gNB calculates the TA value of the candidate cell. Then the candidate gNB sends the TA value of the candidate cell and the associated CFRA resource information, to the source gNB. This procedure needs to extend the existing TA information transfer procedure via F1 interface to Xn interface, e.g. via the TA information transfer message.

Proposal 9: Introduce a new non-UE associated Class 2 message, e.g., TA information transfer message, to transfer the TA information.

  - LTM cell switch execution

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

The UE sends the L1 measurement report(s) to the source gNB-DU, and on their basis the source gNB-DU decides to execute LTM cell switch to the target cell by sending an LTM cell switch command to the UE. Then, the source gNB-CU sends a notification message to the target gNB-CU to indicate the triggering of LTM. The message may include the target cell ID and/or the selected TCI-state ID(s) of the target cell. In Rel-18 LTM, the DU-CU/CU-DU CELL SWITCH NOTIFICATION procedure is designed for the notification of LTM cell switch and selected TCI state. Following the same principle, new LTM cell switch notification procedure needs to be introduced in XnAP.

  - LTM cell switch completion

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

After receiving the ACCESS SUCCESS message from the target gNB-DU, the target gNB-CU initiates the path switch procedure to CN via the existing path switch message and CN can start to forward the DL data to the target gNB-CU accordingly.

Meanwhile, the target CU sends existing XnAP: Handover Success message to the source CU to indicate that UE has accessed to the target Cell.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

After completion of the LTM, the L3 handover similar procedure at the NW side after completion of the handover can be reused for inter-CU LTM. Then, if the source gNB is not configured as a candidate gNB for the subsequent LTM, the target gNB shall send the UE CONTEXT RELEASE to inform the source gNB about the success of the handover and trigger the release of radio and C-plane related resources associated to the UE context in the source gNB.

Proposal 10: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

After completion of the LTM, new UE associated signalling connection should be established between the new serving gNB and candidate gNB(s) in order to perform subsequent LTM. To establish the new signaling connections, the new serving gNB needs to know the information of other candidate gNB(s), e.g. the gNB ID and corresponding Xn AP ID allocated for the UE. With these information, the new serving gNB can initiate procedure to setup signaling connection with other candidate gNB(s).

The old serving gNB has such information of all candidate gNB(s).The old serving gNB can transfer such information to the new serving gNB in one of following procedures:

- Handover modification procedure after path switch.

- Cell switch notification

Proposal 11: Old serving gNB transfers the gNB ID of other candidate gNBs and Xn AP ID used by the candidate gNB to new serving gNB. FFS which procedure is used.

During path switch procedure, the AMF may update configuration for the UE, e.g. new per PDU session UL TNL address, released PDU session(s), and changed QoS parameters for QoS flows. The updated configuration needs to be updated to all candidate gNB(s) before next subsequent LTM. So after receiving PATH SWITCH REQUEST ACKNOWLEDGE message from AMF, the new serving gNB needs to initiate Xn procedure to update the configuration with all candidate gNBs.

In addition, the Xn procedure should also accomplish the setup of UE associated signaling connection between new serving gNB and candidate gNB(s), and update of data forwarding TNL information based on updated configuration.

If UE side configuration is involved, candidate gNB needs to update the prepared LTM configuration and include the update LTM configuration in the message sent to new serving gNB.

So the handover modification procedure can be used to accomplish at least following functions:

- setup Xn signaling for the UE between new serving gNB and candidate gNB(s)

- update LTM configuration

- update data forwarding TNL information

Proposal 12: After path switch procedure, handover modification procedure can be used to setup signaling connection between other candidate gNB(s), update LTM configuration, and update data forwarding TNL information.

Another issue is whether we support to release the UE associated Xn signaling connections between the old serving gNB and other candidate gNB(s). In our understanding, it is unnecessary always maintain the old Xn signaling connection. Release the old signaling connections should be allowed by specification. At least release it is needed in the case the old serving gNB is not a candidate gNB after LTM completion.

There are following options to release the old UE associated Xn signaling connections:

Option 1: During handover modification procedure after path switch, the new serving gNB indicates candidate gNB(s) explicitly or implicitly to release the UE associated signalling connection between the old serving gNB.

Option 2: The new serving gNB indicates to old serving gNB that signaling connection between old serving gNB and candidate gNB can be released explicitly or implicitly after handover success or path switch completed. The old serving gNB initiates a class 1 Xn procedure to candidate gNBs to release UE associated Xn signaling connection between candidate gNB(s).

Note that to release UE associated signaling connection only release the resources related to the UE associated signalling connection between old serving gNB and candidate gNB(s). The prepared LTM configuration in the candidate gNB is not released in this procedure.

Proposal 13: RAN3 to discuss release of UE associated signaling connections between old serving gNB and candidate gNB.

  - Inter-CU LTM without PDCP changed

In the last RAN3 #124 meeting, this solution in [R3-243522] has been simply discussed. Meanwhile, it in [R2-2405469] has been also discussed in RAN2 #126 meeting.

In RAN2, it is not sure whether this solution has RAN2 impact, but RAN2 decides to postpone this discussion until SA3 responds the security result on subsequent inter-CU LTM.

We agree that this solution has many benefits, e.g., avoiding security issue, avoiding NG Path switch, avoiding NG data forwarding.

However, in RAN3, we think this solution will introduce more latency than the inter-CU LTM with PDCP changed. The reason is that SRB shall be transferred via Xn-C interface.

Meanwhile, we also think this solution will increase additional Xn-U load, because DRB data has to be forwarded from target gNB to source gNB.

In order to align with RAN2 discussion, we also propose to postpone this discussion in RAN3.

Proposal 14: RAN3 to postpone the discussion on inter-CU LTM with PDCP unchanged.

- Conclusion

After the above analysis, we provide the following proposals.

Proposal 1: Parallel procedures are used to prepare LTM candidate configurations, i.e., one handover preparation procedure is for one LTM candidate cell.

Proposal 2: Introduce a new IE (i.e., Maximum Number of LTM preparations) in Handover request ACK message.

Observation 1: From the perspective of inter-node interaction design, multiple reference solution is simpler than the single reference solution.

Proposal 3: Wait for RAN2 progress on whether one or more reference configurations for inter-CU LTM.

Proposal 4: Introduce a new UE associated Class 1 message, e.g., Handover modification procedure, to perform e.g., LTM modification including notification of common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs.

Proposal 5: Early data forwarding is initiated during inter-CU LTM preparation phase. It’s up to source gNB’s implementation when to trigger the early data forwarding during inter-CU LTM preparation phase.

Proposal 6: Late data forwarding is initiated	after source gNB receives Handover Success message from target gNB.

Proposal 7: After LTM completion, for subsequent LTM, the new serving gNB can request other candidate node(s) to perform data forwarding per PDU session/DRB. New serving gNB and other candidate gNB negotiate data forwarding after LTM completion and before next subsequent LTM.

Proposal 8: The Conditional Handover Cancel message is reused by the target gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Proposal 9: Introduce a new non-UE associated Class 2 message, e.g., TA information transfer message, to transfer the TA information.

Proposal 10: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Proposal 11: Old serving gNB transfers the gNB ID of other candidate gNBs and Xn AP ID used by the candidate gNB to new serving gNB. FFS which procedure is used.

Proposal 12: After path switch procedure, handover modification procedure can be used to setup signaling connection between other candidate gNB(s), update LTM configuration, and update data forwarding TNL information.

Proposal 13: RAN3 to discuss release of UE associated signaling connections between old serving gNB and candidate gNB.

Proposal 14: RAN3 to postpone the discussion on inter-CU LTM with PDCP unchanged.