3GPP TSG-RAN WG3 Meeting #125	R3-244056

Maastricht, NL, 19th – 23rd Aug 2024

Agenda Item:	13.2

Source:	ZTE Corporation

Title:	TP to 38.300 for inter-CU LTM

Document for:	Discussions & Approval

- Introduction

This TP is based on the R3-244055 (Discussion on inter-CU LTM).

- TP to 38.300

===============================Start of the change====================================

Figure 2. Signaling procedure for inter-CU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB.

2.	The source gNB determines to initiate inter-gNB LTM configuration.

3.	Source gNB sends a HANDOVER REQUEST message to the candidate gNB(s) for each candidate cell to e.g., request LTM configurations and Early Sync Information.

4.  Admission Control may be performed by the target gNB.

5.	If the candidate gNB accepts the request of LTM configuration, it responds with a HANDOVER REQUEST ACKNOWLEDGE including LTM configuration information and Early Sync Information of LTM candidate cell to the source gNB. The HANDOVER REQUEST ACKNOWLEDGE message is sent for each candidate cell.

6a.	The source gNB may send a UE associated XnAP message (e.g., HANDOVER MODIFICATION REQUEST) to the candidate gNB(s), e.g., for subsequent LTM, to inform the candidate gNB(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB, or to update the existing configuration.

6b.	The candidate gNB responds with a UE associated XnAP message (e.g., HANDOVER MODIFICATION REQUEST ACKNOWLEDGE), to respond with the candidate configuration to the source gNB-CU accordingly (if needed), or to update lower layer configuration (e.g., the updated CSI report configuration).

7.	The source gNB transmits an RRCReconfiguration message to the UE including the LTM candidate configurations.

8.	The UE stores the LTM candidate configurations and transmits an RRCReconfigurationComplete message to the gNB.

8a. If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

9a. Same as that in step 4a in Figure Figure 9.2.3.5.2-1.

9b-1. Same as that in step 4b in Figure Figure 9.2.3.5.2-1.

9b-2.	The candidate gNB forwards the TA value, the associated CFRA resource information and the candidate cell ID to source gNB via an XnAP signaling (e.g., TA INFORMATION TRANSFER).

10.	The UE sends the Lower layer measurement report to the source gNB.

11.	The source gNB decides to execute LTM to a candidate target cell.

12.	The source gNB sends the Cell Switch command to the UE via MAC CE.

13.	The source gNB forwards the target cell ID, the TCI state ID, and the TA values to target gNB via LTM CELL SWITCH NOTIFICATION message.

14. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

15. Same as that in step 8 in Figure 9.2.3.5.2-1. Signalling procedure for LTM. The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 7 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.

15a/15b	The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

16-18	Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network.

19. If there is no LTM candidate cell configured in the source gNB, the target gNB sends an UE CONTEXT RELEASE message to the source gNB.

NOTE 1: Late data forwarding may be initiated as soon as the source gNB receives HANDOVER SUCCESS message from target gNB in step 16.

NOTE 2: The steps 9a-19 can be performed multiple times for the following execution of subsequent LTM.

===============================End of the change====================================