3GPP TSG-RAN WG3 Meeting #125	R3-244058

Maastricht, NL, 19th – 23rd Aug 2024

Title: 	(TP for TR38.769) RAN architecture for Ambient-IoT

Source: 	ZTE Corporation

Agenda item:	16.2

Document for:	Discussion and Decision

- Introduction

In the last RAN3 #124 meeting, we have achieved the following progress, and have also agreed the TP R3-243962.

- Focus on the two system architecture figures in 3549

- Capture the first figure for Topology1 in TR or for both topologies

- Update the big box with dash line in figure2 and check further on Topology2

- Work on the definition of common reader function and AIoT RAN function

- Reuse existing NG interface rather than design a new interface for A-IoT function?

This paper addresses to discuss RAN architecture and protocol stack for Ambient-IOT.

- Discussion

  - Architecture

In the last meeting, the architecture for Topology 1 has been agreed as below.

Figure 1: Agreed Logical System Architecture for Topology 1 AIoT.

However, RAN3 has not agreed the Topology 2 architecture. The reason is that some companies does not think UE/reader is belonged to AIOT RAS but is as UE with AIoT reader capabilities, detail can also be seen in e.g., RP-241270.

As compromised, we provides the following architecture for AIoT topo2, specially, remove the RAS.

Figure 2: Suggested Logical System Architecture for Topology 2 AIoT.

Proposal 1: Agree the logical System Architecture for Topology 2 AIoT as figure 2.

  - Data Transport

There are two candidate options to perform data transport between AIoT CN and AIoT device.

- Option 1: Control plane (i.e., NAS-based data transfer)

- Option 2: User plane

  - For option 2, it needs to establish PDU Session/QoS flow and the related UE/reader shall be moved into RRC connected mode, it is suitable for the data packets are large and transmission duration is long. However, it is more complex than Control plane.

  - For option 1, it does NOT need to

- XX/NG interface: establish dedicated connection between RAN and CN

- Uu interface: establish RRC connection (UE can be in RRC inactive mode)

- Dedicate bearer: establish PDU Session/QoS flow/DRB

  - Compared with option 2, option 1 is simple. For Ambient data transport, its data packets are small and its transmission duration is short, which is very similar to the legacy NB-IoT, so option 1 is preferred.

- Proposal 2: For both topology 1 and topology 2, AIoT data transport between AIoT CN and AIoT device is via control plane (i.e., NAS based), named as AIoT device NAS PDU.

  - AIoT device NAS PDU is only decoded by device and AIoT CN. The detail data content encapsulated in AIoT device NAS PDU is decided by SA2, SA3, which includes e.g., :

- DL “write” data from AIoT CN;

- UL “read” data from AIoT device,

- Security related data

  - Protocol stack

    - Topology 1

  - If a new interface is introduced, considering that the new protocol is used for context management and signalling transmission over new interface for AIoT, the protocol stack of XXAP (name FFS) can use the same protocol stack as NG-C, i.e., including XXAP/SCTP/IP/Date link layer/Physical layer. For the new XXAP protocol, it should be very simple, i.e., with basic interface management procedures and AIoT dedicated new procedures.

  - Based on above discussion on network architecture and data transport, from our view, the following Protocol stack for Topology 1 should be supported:

In the TR 38.769, we have captured the following content.

XX interface: interface between the AIoT RAS and the AIoT CN on which certain AIoT specific functions are performed. Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between AIoT RAS and AIoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAS, or one instance for NG alone

Assumed AIoT data transport is via Control plane, we suggest to introduce the following figure for topology 1.

Figure 3. Suggested Protocol Stack for Topology 1 AIoT

Proposal 3: Adopt Figure 3 as Protocol Stack for Topology 1 AIoT

    - Topology 2

In topology 2, the UE-reader shall support all RRC states and register to the 3GPP network, which is the same as the legacy UE behaviour.

In topology 2, the gNB can be an AIoT enhanced gNB. The gNB for topology 2 can exclude supporting AIoT radio.

This gNB shall connect AIoT CN with legacy NG interface to perform normal UE behaviour, e.g., Paging. Meanwhile, the gNB shall also connect AIoT CN with XX interface to support AIoT feature, e.g., perform AIoT device inventory and command.

So that, it is benefit that gNB connects with AIoT CN node via current NG interface and XX interface.

In case of Topology 2, the UE (reader) shall perform inventory/command procedure. The question is whether gNB shall be known of inventory/command procedure.

Solution 1: RRC based solution.

In this solution 1, gNB receives explicit IEs (e.g., inventory/command) via NGAP/XXAP, then it instructs RRC message to UE (reader) via Uu interface.

In this solution 1, gNB can acquire full information of inventory/command, then it can reserve accurate radio resource for AIoT accordingly.

More, AIoT CN can send the union NGAP/XXAP message to gNB for both topology 1 and topology 2.

Figure 4. Suggested Protocol Stack-RRC for Topology 2 AIoT- RRC based

Solution 2: NAS based solution.

In this solution 2, AIoT CN shall generate NAS PDU including inventory/command (named e.g., Reader NAS), and sends this NAS PDU via NGAP/XXAP. So that, AIoT CN shall instruct two kinds of inventory/command message, i.e., explicit IEs for topology 1 and NAS PDU for topology 2.

  - More, gNB will not encode the NAS PDU and it has no idea of inventory/command procedure. In order for gNB to reserve suitable radio resource for AIoT, gNB shall additionally acquire assistant information explicit IEs via from AIoT CN

- Figure 5. Suggested Protocol Stack for Topology 2 AIoT - NAS based

  - Proposal 4: Adopt Figure 4 as Protocol Stack for Topology 2 AIoT

- Conclusion

After the above analysis, we provide the following observations and proposals.

- Proposal 1: Agree the logical System Architecture for Topology 2 AIoT as figure 2.

- Proposal 2: For both topology 1 and topology 2, AIoT data transport between AIoT CN and AIoT device is via control plane (i.e., NAS based), named as AIoT device NAS PDU.

Proposal 3: Adopt Figure 3 as Protocol Stack for Topology 1 AIoT

Proposal 4: Adopt Figure 4 as Protocol Stack for Topology 2 AIoT

- Text Proposal for TR38.769

================================Start of change===============================

6.3	RAN architecture aspects

Editor’s note 1: Corresponds to the second RAN3 objective in the SID, to identify RAN architecture aspects, including whether support for split architecture is necessary.

6.3.1 General

This chapter attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

Editor’s Note 2: What functionalities are hosted by the 5GS for AIoT is TBD.

This chapter also attempts to identify a functional split between RAN and CN.

Figure 6.3-1 depicts the logical system architecture for AIoT.

It consists of the following architectural elements:

AIoT device: equipment with characteristics outlined e.g. in TS 22.369 [x] and TR 38.848 [2].

Editor’s Note 3: Further details FFS, if any.

AIoT RAS (AIoT Radio Access System): hosts certain functions for AIoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAS and the respective functional split to be decided by RAN2, RAN3 and SA2.

AIoT radio: radio interface between AIoT device and Common reader function.

Editor’s Note 5: Further details on AIoT radio to be discussed by RAN1 and RAN2.

AIoT CN: hosts certain functions for AIoT as of the functional split between RAN and CN

Editor’s Note 6: Further details regarding AIoT functions hosted in the AIoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface: interface between the AIoT RAS and the AIoT CN on which certain AIoT specific functions are performed.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between AIoT RAS and AIoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAS, or one instance for NG alone.

Common reader function: a function that communicates with the AIoT device by means of AIoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

AIoT RAN node function: a function residing in AIoT RAS. It contains e.g. the control of the AIoT radio resources used towards the AIoT device.

Editor’s Note 9: further details are FFS. Note that “control of AIoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

6.3.2 Architecture

Figure 6.3.2-1: Logical System Architecture for AIoT Topology 1.

Editor’s Note 9: whether the logical system architecture above is applied to topology 2 to be discussed in next meeting.

Figure 6.3.2-2: Logical System Architecture for AIoT Topology 2.

6.3.3 Protocol Stack

Device NAS to deliver the UL or DL data between AIoT Device and 5GC, e.g., “read/write” data, security data

Reader NAS to deliver the UL or DL data between UE (reader) and 5GC, e.g., inventory, command

Figure 6.3.3-1. Protocol Stack for AIoT Topology 1

Figure 6.3.3-2. Protocol Stack for AIoT Topology 2 - RRC based

Figure 6.3.3-3. Protocol Stack for AIoT Topology 2 - NAS based

================================End of change===============================