3GPP TSG-RAN WG3 Meeting #125	R3-244059

Maastricht, NL, 19th – 23rd Aug 2024

Title: 	(TP for TR38.769) NG interface impact for Ambient-IoT

Source: 	ZTE Corporation

Agenda item:	16.3

Document for:	Discussion and Decision

- Introduction

In the last RAN3 #124 meeting, we have achieved the following progress and have agreed a TP R3-243963.

Inventory over CN-RAN interface can be used for a single device, or a group of devices, or all devices.

Command over CN-RAN interface can be used for a single device. FFS for command on a group of devices, or all devices.

In the Inventory Request from the AIoT CN towards the AIoT RAS, it is FFS whether the following information may be included:

Inventory Session ID (e.g., to allow multiple Inventory procedure instances to run in parallel): the purpose, scope and handling of this information in AIoT RAS are FFS.

Estimated Number of expected responses from devices, or Minimum number of required responses from devices.

FFS the usage of these information.

Periodicity: to enable periodic inventory at AIoT RAS triggered by a single Inventory Request from the AIoT CN

In Topology 2, it is FFS which entity (e.g., AIoT CN and/or AIoT enabled gNB) performs the UE reader selection.

In topology1, it is FFS whether in response to a single Inventory Request, AIoT RAS provides only a single Inventory Report message or there can be also multiple Inventory Report messages provided.

This paper addresses to discuss NG interface impact for Ambient-IOT.

- Discussion

  - Device context management

Before CN sends command message to AIoT devices, it shall select the suitable reader(s).

CN shall firstly send inventory message to AIoT devices if CN does not store the association between reader and devices.

CN can also send inventory message to AIoT devices if CN wishes to acquire the possibly updated association between reader and devices.

It means CN shall know each device to be inventory/command, is controlled by which reader, i.e., the CN shall establish/store association between each device and reader. This association is included in device context, and stored in the CN, e.g., device context = {device id list, reader id}. The reader id is UE id or gNB/BS/cell/TRP id.

Observation 1: CN initiates Inventory procedure to establish/update the association between reader and devices.

Proposal 1: CN shall establish/update and store AIoT Device context, which includes association between device id list and reader id.

  - AIoT device identity

Whether AIoT device id can be decoded by reader?

Inventory procedure will consume more or less Ambient-Uu radio resource. In case UE/reader shares the same frequency/band with NR-gNB and in case gNB/reader is co-site with NR-gNB, the NR-gNB shall precludes the radio resource when the reader handles inventory/command.  So, the reader shall know the quantity of the device to be inventoried.

AIoT device has ultra-low power consumption. When the device is in charging phase, it even cannot respond inventory request. In order to perform efficient inventory, the reader shall perform delta inventory. This means that only devices have not responded yet shall respond inventory, to decrease the RACH collision at Ambient-Uu interface. So that, the reader shall know the device ids to perform delta inventory.

Observation 2: The reader shall configure suitable radio resources to perform inventory/command procedure.

Proposal 2: The reader shall be able to decode AIoT device id.

  - Inventory

The following content is captured in the latest TR38.769.

6.4.1.1 Inventory Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices. The Inventory Request from the AIoT CN to the AIoT RAS, may include the following: (1) AIoT Device Identification (to find a single device, a group of devices, or all devices) Note 1: The definition of this identification is out of RAN3 scope. Editor’s Note 1: It is FFS whether AIoT RAS needs to interpret/store/process it. (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered) Multiple individual AIoT Device IDs (one ID per device) can be provided to the AIoT CN via a single Inventory Report. Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

RAN2 agreement: Baseline procedure:

Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS

Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS

Step C: The device may perform the data communication with the reader as needed, Details FFS

According to the RAN2 agreement as above, we provide the overall procedure as below.

Figure 1: Overall procedure for AIoT Topology 1

In this procedure figure, the step 3 is the same as Step A to B, defined by RAN2.

The step 1, 2 and step 4 are at CN-RAN interface (e.g., named XX interface). In step 1, CN initiates inventory procedure. In step 4, gNB as reader responds inventory result to CN.

Proposal 3: Adopt Figure 1 as Overall procedure for AIoT Topology 1.

Figure 2: Overall procedure for AIoT Topology 2

For AIoT topology 2, before CN initiates Inventory procedure at XX interface, the UE/reader shall be reachable (e.g., by CN paging if needed).

In this procedure figure, the step 3 is the same as Step A to B, defined by RAN2. The other steps are the same as that in figure 1.

Proposal 4: Adopt Figure 2 as Overall procedure for AIoT Topology 2.

  - Command

From RAN3 point of view, there is no special difference at XX interface, the CN initiated command procedure can also include one or more AIoT devices.

Proposal 5: RAN3 further discusses command procedure based on the Inventory procedure.

- Conclusion

After the above analysis, we provide the following proposals.

Observation 1: CN initiates Inventory procedure to establish/update the association between reader and devices.

Proposal 1: CN shall establish/update and store AIoT Device context, which includes association between device id list and reader id.

Observation 2: The reader shall configure suitable radio resources to perform inventory/command procedure.

Proposal 2: The reader shall be able to decode AIoT device id.

Proposal 3: Adopt Figure 1 as Overall procedure for AIoT Topology 1.

Proposal 4: Adopt Figure 2 as Overall procedure for AIoT Topology 2.

Proposal 5: RAN3 further discusses command procedure based on the Inventory procedure.

- Text Proposal for TR38.769

================================Start of change===============================

6.4	Impacts on CN-RAN interface

Editor’s note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

6.4.1	Information exchanged between AIoT CN and AIoT RAS

6.4.1.1	Inventory

Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices.

The AIoT CN initiates Inventory procedure to establish and update association between device and reader.

Device context management includes device context establishment and modification. The association between device and reader is included in the device context stored in the AIoT CN.

The Inventory Request from the AIoT CN to the AIoT RAS, may include the following:

(1)	AIoT Device Identification (to find a single device, a group of devices, or all devices)

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note 1: AIoT RAS needs to interpret/store/process it.

(2)	Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

Multiple individual AIoT Device IDs (one ID per device) can be provided to the AIoT CN via a single or multiple Inventory Reports.

Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

6.4.1.2	Command

Command can be sent by the AIoT CN for a single or multiple devices.

Editor’s Note 1: it is FFS for command on a group of devices, or all devices.

Editor’s Note 2: it is FFS whether AIoT RAS can remain agnostic of the type of request from the AIoT CN (need to differentiate command and inventory)

6.4.1.3	Overall procedure

Figure 6.4.1.3-1: Overall procedure for AIoT Topology 1

1. When the AIoT CN decides to initiate inventory procedure, it sends Inventory request message to the gNB.

2. The gNB sends Inventory response message to the AIoT CN.

3. The gNB initiates inventory procedure at AIoT radio interface accordingly.

Editor’s Note 1: This step is defined by RAN2.

4. After receiving part or full of inventory result from devices, the gNB sends a single or multiple Inventory reports to the AIoT CN including the received inventory result.

Figure 6.4.1.3-2: Overall procedure for AIoT Topology 2

0. The UE as reader shall be reachable, if needed, by legacy CN paging.

1/2. The step 1/2 are the same at step 1/2 in Figure 6.4.1.3-1.

3. The gNB initiates inventory procedure at Uu interface and AIoT radio interface accordingly.

Editor’s Note 1: This step is defined by RAN2.

4. After receiving part or full of inventory result from UE, the gNB sends a single or multiple Inventory reports to the AIoT CN including the received inventory result.

================================End of change===============================