3GPP TSG-RAN WG3 Meeting #125	R3-244060

Maastricht, NL, 19th – 23rd Aug 2024

Title: 	Discussion on unified procedure for AIoT topology 1 and 2

Source: 	ZTE Corporation

Agenda item:	16.3

Document for:	Discussion and Decision

- Introduction

In the last RAN3 #124 meeting, we have achieved the following progress and have agreed a TP R3-243963.

Inventory over CN-RAN interface can be used for a single device, or a group of devices, or all devices.

Command over CN-RAN interface can be used for a single device. FFS for command on a group of devices, or all devices.

In the Inventory Request from the AIoT CN towards the AIoT RAS, it is FFS whether the following information may be included:

Inventory Session ID (e.g., to allow multiple Inventory procedure instances to run in parallel): the purpose, scope and handling of this information in AIoT RAS are FFS.

Estimated Number of expected responses from devices, or Minimum number of required responses from devices.

FFS the usage of these information.

Periodicity: to enable periodic inventory at AIoT RAS triggered by a single Inventory Request from the AIoT CN

In Topology 2, it is FFS which entity (e.g., AIoT CN and/or AIoT enabled gNB) performs the UE reader selection.

In topology1, it is FFS whether in response to a single Inventory Request, AIoT RAS provides only a single Inventory Report message or there can be also multiple Inventory Report messages provided.

This paper addresses to discuss NG interface impact for Ambient-IOT.

- Discussion

  - Which node to select suitable reader for inventory/command

Top 1 Case

It is obvious that CN selects the suitable gNB/reader.

Top 2 Case

In Topology 2, it is FFS which entity (e.g., AIoT CN and/or AIoT enabled gNB) performs the UE reader selection

Since the UE/reader can be in IDLE mode, the gNB will not store the IDLE-UE context, it is reasonable that CN selects the UE/reader.

Proposal 1: It is the AIoT CN to select suitable reader for both topology 1 and topology 2.

  - Unified Inventory procedure for both topology 1 and 2

Figure 1: Overall unified procedure for AIoT Topology 1 and Topology 2

The AIoT CN has already stored the device context including the association between device and reader.

Association 1: device list: 1-100, reader: gNB2;

Association 2: device list: 101-200, reader: UE1 (controlled by gNB1)

…; Association n: device list: xxxx-yyy, reader: UEn

0. AIoT CN decides to initiate Inventory procedure, device list=1-200

1a/2a. Based on the stored association1, AIoT CN initiates Inventory procedure with gNB2 (reader), device list=1-100

1b/2b. Based on the stored association2, AIoT CN initiates Inventory procedure with UE1 (reader) via gNB1, device list=101-200

3a. gNB2 performs Inventory procedure at AIoT interface, device list=1-100

3b. UE1 (reader) is reachable, gNB2 triggers legacy Paging if needed.

4b. UE1 performs Inventory procedure at AIoT interface, device list=101-200.

5a/5b. gNB1 sends Inventory report(s) to the AIoT CN, including the inventory result.

Proposal 2: AIoT CN can initiate unified Inventory procedure for both AIoT topology 1 and topology 2.

- Conclusion

After the above analysis, we provide the following proposals.

Proposal 1: It is the AIoT CN to select suitable reader for both topology 1 and topology 2.

Proposal 2: AIoT CN can initiate unified Inventory procedure for both AIoT topology 1 and topology 2.