3GPP TSG-RAN WG3 Meeting #R3-125	R3-244063

Maastricht, NL, 19 - 23 August 2024

Agenda item:	 11.3

Source:		Jio

Title:		Discussion on AI/ML based CCO De-Centralized beam prediction

Document for:		Discussion

# 1	Introduction

In this contribution, we would further propose and discuss details of AI/ML supported beam enhancements based CCO.

This is mirrored to the Discussion document R3-XXXXXX

# 2	General Discussion

In this Discussion document, we explore the concept of AI/ML based Coverage and Capacity Optimization (CCO) within the context of 5G RAN Release 19, focusing on the innovative approach of super-resolution beam prediction in the proposed network architecture for both training and inference in gNB CU.

3GPP Release 18 and subsequent Release 19 introduced advanced AI/ML techniques for CCO, which are poised to revolutionize network management and optimization. Beam selection, as part of Beam Management in 5G, is a technique mainly used to optimize the alignment of highly directional transmission and reception beams. It includes processes, based on 3GPP Rel.19 T-Doc enhancements, like beamforming, beam shaping, beam selection, beam switching, beam steering and beam tracking, which aim to maintain high-quality communication links despite challenges like path loss, blockages, and rapid changes in user equipment position and orientation. To compensate for the higher pathloss, the use of mMIMO dense antenna elements in both gNB and UE is used to implement a narrower beam to utilize the high beamforming gain. However in order to efficiently utilize the narrow beams properly, efficient algorithms needs to be implemented to select the beam pairs of Tx and Rx that are best aligned to each other and maintain the best alignment.

As mobile networks become increasingly complex with diverse traffic patterns and user behaviors, traditional optimization methods fall short in addressing dynamic and real-time requirements. This is where AI/ML comes into play, offering a data-driven approach to predict and manage network conditions effectively. The 3GPP specifications outline how AI/ML models should be trained, validated, and deployed for various network functions, including CCO.

As we advance towards more sophisticated 5G deployments and look ahead to future generations of wireless technology, AI/ML-based CCO will become even more integral. The continued evolution of AI and ML will drive innovations in network optimization, enabling more granular and automated control over coverage and capacity. The 5G network landscape is highly dynamic, with rapidly changing traffic loads, varying user demands, and diverse environmental conditions. Traditional optimization methods, which often rely on static rules and manual adjustments, are insufficient for addressing these complexities. AI/ML technologies excel in processing and analyzing vast amounts of data in real-time, enabling them to adapt to changing conditions more effectively. AI/ML algorithms can continuously learn from network performance data and adjust optimization strategies dynamically, ensuring that coverage and capacity are consistently optimized.

AI and ML provide advanced predictive capabilities that are critical for proactive network management. By leveraging historical data and real-time inputs, AI/ML models can forecast traffic patterns, user behavior, and potential network issues with high accuracy. This predictive power allows for anticipatory adjustments in network configuration, such as reallocating resources or modifying coverage parameters before issues arise. For example, predictive analytics can foresee peak usage periods and adjust capacity allocation to prevent congestion, enhancing overall network reliability and user experience. In 5G networks, the need for real-time decision-making is paramount. AI/ML systems can process and analyze data almost instantaneously, enabling real-time adjustments to coverage and capacity. This capability is particularly important in scenarios where network conditions fluctuate rapidly, such as during high-traffic events or in highly populated urban areas. Real-time AI/ML-driven optimizations help maintain optimal network performance by quickly responding to changing conditions, thereby minimizing disruptions and improving service quality.

Capacity management in 5G involves the efficient allocation of network resources to handle varying demands. AI/ML technologies enhance this process by optimizing resource distribution based on real-time traffic data and usage patterns. Machine learning algorithms can identify patterns in data consumption and network load, allowing for more precise and adaptive resource allocation. This ensures that network resources are used efficiently, reducing wastage and improving overall network performance. Ultimately, the goal of Coverage and Capacity Optimization is to enhance the user experience. AI/ML-driven optimizations contribute to this by ensuring that users experience consistent and high-quality connectivity, even in challenging conditions. By optimizing coverage and capacity in real-time, AI/ML technologies help to reduce latency, increase data speeds, and improve overall network reliability, leading to a better user experience.

  - 2.1 AI/ML assisted beam prediction for CCO

Super-resolution beam prediction is an advanced signal processing technique that leverages machine learning and deep learning algorithms to predict and optimize beamforming patterns in the RAN. Beamforming involves directing the radio signal towards a specific user or group of users, thus enhancing signal strength and quality.

Super-resolution beam prediction can help in accurately tracking the position of mobile users, allowing the network to dynamically adjust beams for optimal signal strength and quality. By more accurately determining the directions of interfering signals, super-resolution techniques can help in designing beams that minimize interference. Finally enhanced beam prediction can lead to more efficient use of network resources, including spectrum and energy, by ensuring that beams are accurately aligned with user positions.

The optimization of beam patterns through super-resolution techniques also addresses capacity issues by:

- Reducing Inter-Cell Interference: Precise beamforming reduces the overlap of signals from adjacent cells, decreasing interference and enhancing the quality of the received signal.

- Optimizing Spectrum Utilization: Efficient beam management allows for better use of available spectrum resources, increasing overall network capacity.

- Load Balancing: By dynamically adjusting beams and directing users to less congested cells, the technique helps balance the network load, ensuring consistent throughput.

Proposal 1: In the content of AI/ML based CCO, the use of super-resolution beam prediction would provisionally improve CCO and User Mobility

  - 2.2 AI/ML Training and Inference algorithm model for CCO

In 3GPP TR 37.743 [1] in clause 4.2 and subclause 4.2.1 there is a discussion on AI/ML for CCO. Training AI/ML models for CCO involves using vast amounts of network data, including user mobility patterns, signal strength measurements, traffic loads, and environmental factors. This data is used to train algorithms for pattern recognition and predicting network performance under varying conditions.

3GPP TR 38.843 focuses on enhancing performance through AI/ML models, including potential use cases such as channel state information (CSI) feedback enhancement, beam management, and positioning accuracy improvements.

3GPP TR 28.908 outlines the management capabilities required for AI/ML across different phases: training, validation, testing, deployment, and inference. This includes aspects such as ML entity loading, inference control, and performance evaluation.

During last RAN3 meeting it was agreed that for AI/ML based CCO [1]:

-	AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the NG-RAN node (gNB-CU).

-	AI/ML Model Training and AI/ML Model Inference may be both located in the NG-RAN node (gNB-CU).

Currently, to support AI/ML in NG-RAN, inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results) are required (see Figure 5.1.2.1-1). For AI/ML based CCO, for AI/ML Model Training or AI/ML Model inference, the inference input may be collected from UE or neighbour NG-RAN node (in case of non-split architecture) or neighbour gNB-CU/gNB-DU (in case of split architecture):

- Inputs from UE:

-	UE measurement report (e.g., UE SS-RSRP, SS-RSRQ, SS-SINR measurement, CSI-RSRP etc), including cell level and beam level UE measurements

-	UE location information (e.g., coordinates, serving cell ID)

-	Random Access performance and RLF report from the UE

- Inputs from local node:

-	Measured/Predicted Radio Resource Status

-	Measured/Predicted UE Traffic (e.g., data volume)

- Inputs from neighboring node:

-	Measured/Predicted Radio Resource Status

-	Measured/Predicted UE Traffic (e.g., data volume)

-	Measured/Predicted UE trajectory

Our proposal in this discussion document is to implement both the AI/ML training and the inference, including the super resolution beam prediction, in the gNB CU. The Coverage and Capacity Optimiation (CCO) architecture in Release 19 involves the integration of AI/ML models within the gNB CU. This integration can be divided into two primary components:

- AI/ML Training Pipeline:

  - Data Collection: Aggregation of data from various sources, including user equipment (UE) feedback, environmental context, and historical network performance metrics.

  - Feature Engineering: Preprocessing data to extract relevant features such as channel state information (CSI), user mobility patterns, and environmental conditions.

  - Model Training: Utilizing datasets to train AI/ML models capable of predicting optimal beam configurations. This phase includes supervised, unsupervised, and reinforcement learning techniques, depending on the nature of the problem.

- AI/ML Inference Engine:

  - Real-time Decision Making: Deployment of trained models in the gNB CU for real-time beam prediction and adjustment.

  - Feedback Loop: Continuous monitoring and updating of models based on real-time data to adapt to changing network conditions.

In this proposal the gNB CU in 5G RAN Release 19 serve as the server for AI/ML training. This involves, as stated in [1]:

- Data Collection: gNB CU collects vast amounts of data from the network, including user mobility patterns, signal strength measurements, and traffic statistics.

- Data Preprocessing: The collected data is cleaned, normalized, and prepared for training AI/ML models.

- Model Training: Using the processed data, various AI/ML models are trained to predict and optimize network performance parameters. This includes models for user localization, traffic prediction, and beamforming optimization.

- Model Management: gNB CU also handles the lifecycle of AI/ML models, including versioning, updating, and deployment.

In the training phase, gNB CU collect comprehensive datasets that include:

- Geolocation Data: Information about the positions of UEs and environmental features.

- Network Performance Metrics: Data on throughput, signal strength, and interference levels.

- User Behavior Patterns: Insights into user mobility and usage patterns.

These datasets are used to train deep learning models that predict the optimal beamforming configuration for various scenarios. The models are refined continuously as new data is collected and analyzed.

We further propose that the gNB CU utilizes trained AI/ML models to perform as well real-time inference, involving [1]:

- Model Deployment: The trained models are deployed in gNB CU to be used for real-time decision-making.

- Real-Time Inference: The gNB CU uses these models to infer optimal parameters for network operations, such as beam directions and power levels.

- Feedback Loop: Inferences made by the CU are used to adjust network configurations, and performance metrics to DU and are fed back to CU for model retraining and improvement.

During the inference phase, the gNB CU uses the trained models to:

- Predict User Location and Mobility: Accurately determine the current and future positions of UEs.

- Adjust Beam parameters: Dynamically adjust the direction (beam steering), the shape (SSB beam-width and traffic beamwidth) and beam power to optimize signal delivery.

- Mitigate Interference: Predict and minimize inter-cell interference by adjusting beam patterns and power levels.

Proposal 2: For AI/ML based CCO, both AI/ML training and inference, including the super resolution beam prediction, resides in the gNB CU. The system collects geolocation data, network performance metrics and user behaviour patterns.

  - 2.3 AI/ML assisted high resolution beam prediction for CCO Training and Inference algorithm model

We further propose a generic way to implement in algorithmic flow the AI/ML super beam prediction.

Data Collection

- Inputs: Radio measurement data, traffic data, channel state information (CSI), user equipment (UE) data, network performance metrics.

- Processes: Aggregation of data from various sources such as gNB DU (Distributed Unit), UEs, and network elements.

Data Preprocessing

- Inputs: Raw collected data.

- Processes: Cleaning, normalization, and transformation of data to ensure quality and consistency.

- Outputs: Preprocessed data ready for training or inference.

AI/ML Model Training

- Inputs: Preprocessed historical data, training labels (e.g., beam patterns, traffic load).

- Processes:

  - Model Selection: Choose appropriate AI/ML models (e.g., neural networks, reinforcement learning).

  - Training: Use algorithms to train models on the preprocessed data.

  - Validation: Validate the trained models with separate validation datasets to ensure performance.

- Outputs: Trained AI/ML models.

Super-Resolution Beam Prediction

- Inputs: Real-time or recent data, beamforming parameters, and model inputs.

- Processes:

  - Feature Extraction: Extract relevant features from the data.

  - Inference: Apply the trained model to predict high-resolution beam patterns.

  - Beam Prediction: Generate predicted beamforming directions and patterns using the model.

- Outputs: Predicted high-resolution beam patterns.

Inference and Deployment

- Inputs: Real-time network data, updated models.

- Processes:

  - Model Inference: Use AI/ML models to make real-time predictions or adjustments based on current network conditions.

  - Decision Making: Adjust beamforming strategies, resource allocation, and other network parameters.

- Outputs: Optimized network parameters and beam patterns.

Feedback Loop

- Inputs: Performance metrics, network feedback, user experience data.

- Processes: Monitor the performance of AI/ML predictions, collect feedback, and adjust models or parameters if necessary.

- Outputs: Updated models, improved predictions, and refined strategies.

Proposal 3: For AI/ML assisted high resolution beam prediction in CCO, include the beam prediction in the algorithm flow diagrams for both the training and inference in gNB CU network nodes.

# 3	Conclusion

In this contribution, AI/ML based CCO are discussed. We have the following proposals:

Proposal 1: In the content of AI/ML based CCO, the use of super-resolution beam prediction would provisionally improve CCO and User Mobility

Proposal 2: For AI/ML based CCO, both AI/ML training and inference, including the super resolution beam prediction, resides in the gNB CU. The system collects geolocation data, network performance metrics and user behaviour patterns.

Proposal 3: For AI/ML assisted high resolution beam prediction in CCO, include the beam prediction in the algorithm flow diagrams for both the training and inference in gNB CU network nodes

# References

[1] R3-242231, (TP to TR 38.743) AI/ML enabled CCO, Samsung, Ericsson, Huawei, Telecom Italia, ZTE, Nokia, CATT, Deutsche Telekom, LG Electronics, Telecom Italia.

[2] 3GPP TS 38.300, "NR; Overall description; Stage 2".

[3] 3GPP TR 38.820: Study on AI/ML for NR

[4] 3GPP TR 38.913: Study on scenarios and requirements for next generation access technologies

[5] 3GPP TR 38.816: Study on enhanced network automation

[6] Jio White paper on CCO

- [7] R3-213391, CCO over F1: functional split and signaling between gNB-CU and gNB-DU (NEC)

- [8] R3-213410, (TP for SON BLCR for 38.473) Coverage and Capacity Optimization (Huawei)

- [9] R3-213660, Coverage and Capacity Optimization (Qualcomm Incorporated)

- [10] R3-213687, Coverage aspect of NR CCO (Nokia, Nokia Shanghai Bell)

- [11] R3-213820, Inter-gNB CCO (Ericsson)

- [12] R3-214006, Support of CCO (Samsung)

- [13] R3-214035, Further Discussion on Coverage and Capacity Optimization in NR (ZTE, China Telecom, China Unicom)

- [14] R3-221249 TP for BL CR 38.423 on Coverage and Capacity Optimization

- [15] R3-216189 LS on CCO related assistance information from OAM to NG-RAN  RAN3

- [16] R3-220887 (TP for SON BL CR 38.300) Coverage and Capacity Optimization  ZTE, China Telecom, China Unicom

- [17] R3-220888 (TP for SON BL CR 38.423) Coverage and Capacity Optimization  ZTE, China Telecom, China Unicom

- [18] R3-220889 (TP for SON BL CR 38.473) Coverage and Capacity Optimization  ZTE, China Telecom, China Unicom

- [19] R3-243385 Discussion on AIML based CCO 2024

- [20] R3-243255_CCO AI ML 2023

- [21] R3-243458 - Further discussion on AIML support for CCO

- [22] R3-243467 AI ML CCO

- [23] "IEEE TRANSACTIONS ON NEURAL NETWORKS AND LEARNING SYSTEMS Publication Information," in IEEE Transactions on Neural Networks and Learning Systems, vol. 31, no. 8, pp. C2-C2, Aug. 2020, doi: 10.1109/TNNLS.2020.3009707.