3GPP TSG-RAN WG3 Meeting #125	R3-244067

Maasticht, The Netherlands, EU, 19 – 23 Aug 2024

Agenda item:	10.3.2

Source:	Nokia

Title:	MDT solution for slice support and slice-related mobility enhancements

Document for:	Discussion

# 1	Introduction

At RAN3#124 several scenarios related to mobility problems in network slicing deployments were discussed. It was agreed to work on logged MDT enhancements for slice-based cell reselection as well as to further study successful handovers in case of slice discontinuity.

[Agreement] RAN3 to support immediate MDT measurements collection on specific area per slice(s).

FFS on which immediate MDT measurements can be collected per slice.

FFS on whether to support logged MDT on specific area per slice(s).

[Agreement] RAN3 to work on logged MDT enhancement for slice-based cell reselection.

For successful handover, FFS if solutions are needed to optimize handovers in the case of slicing discontinuity, e.g., target cell not supporting slice.

In this paper, we provide further details on potential enhancements for logged MDT as well as the successful handover scenario where some of the connected slices of the UE experienced discontinuity.

# 2	Discussion

## 2.1	Logged MDT Enhancements for Slice-based Cell Reselection

Rel-17 introduced network slice-based cell reselection, where the UE is provided with one or more slice groups and associated frequency priorities [1], [2] and [3]. Based on network slice(s) the UE is interested in, it performs frequency prioritization based on the selected slice group and tries to camp on the frequency that the network prioritizes for this slice group. However, if these cell re-selection configurations are not correctly set, it could happen that the UE is not able to connect to its required slice in the cell where it is camping based on the slice-specific frequency priorities provided by the network. Therefore, it is important that the network gets some feedback from the UE about how well the slice-based cell reselection configurations are working and if the UE is experiencing some slice connectivity problems or not. In RAN3 #124, it was agreed to enhance logged MDT for slice-based cell reselection. Logged MDT measurements are configured as part of the LoggedMeasurementConfiguration message [4]. Here, we discuss the potential enhancements and measurements that the UE should log to better capture the impact of slice-based cell reselection and ultimately help the network to identify the root cause as well as rectify the erroneous configurations, if needed.

- Logging trigger: there needs to be a new trigger for these slice-based cell reselection measurements as they are specific for cell reselection that use slice-based cell resection configurations

- Slice Group(s): the slice group(s) that were used by the UE to identify the frequency priority configuration should be logged to help the network identify the corresponding slice groups that could be having problematic configurations

- Frequency Priorities: the UE should also log the used frequency priorities along with the slice group(s).

- As both, the mapping of slice(s) to slice group as well as frequency priority associated with slice group can be changed dynamically by the network therefore, it is important to log this information to exactly capture what configurations the UE used.

- Slice Access Problem: if the UE experienced some problems in connecting to the required network slice after using the slice-based cell reselection configuration, this should also be logged and reported to the network. The problems may include, for example

  - Selected cell does not support the required network slice

  - Service request for the required slice was not accepted in the selected cell

  - Service request was delayed, for example, the selected cell/frequency does not support the slice or it was congested

- Measurement Overload: the measurement done for slice-based cell reselection depends on the priority of the slice groups as well as the frequency priorities for those slice groups [4]. It can happen that the UE has to do multiple measurements in multiple iterations over different frequencies. To help UEs minimize this measurement effort the UE should also log measurements like

  - With slice-based cell reselection procedure it can also happen that the UE does not find a suitable cell on the prioritized frequency for the selected slice group. In such cases, an indication should also be included in the logged information

  - Possibly, number of cell measurements or number of frequencies measured

Proposal 1: RAN3 kindly requests RAN2 to specify the above measurement enhancements for logged MDT for slice-based cell reselection.

## 2.2	Successful Handovers with Slice Discontinuity

Some potential mobility problems in network slicing deployments were earlier presented in [1]. Here, we first recap some key points.

Rel-18 network slicing enhancements introduced support of network slice deployments with service areas that are smaller than the Tracking Area (TA) [2]. This Rel-18 feature is important to deploy highly customized and localized network slices without the need for TA re-design. In such deployments, one potential mobility issue at network slice border is, for example, about how to select the neighbour cell for handover (HO). For example, based on UE measurements, the source Cell 1 may have two HO candidates, Cell 2 and Cell 3, but only Cell 3 supports all the slices the UE needs. However, as the source cell does not know in advance the UE trajectory, it has to also prepare Cell 2. This is especially relevant for Conditional HO (CHO) type of mobility as there is a delay between HO preparation and execution. Therefore, Cell 2 cannot be excluded from prepared cells to avoid Radio Link Failure (RLF) and total loss of service even if a HO to Cell 2 would mean loss of service for some of the required slices. Also, currently, in the CHO execution phase, the UE cannot make any prioritization of potential target cells based on the required slices of the UE. In Rel-18 networks, this becomes critical as the slices may only be supported in a small number of cells and above situation of mobility across the slice border could appear more often.

Once the UE successfully completes the HO, the source cell would know which target was selected by the UE and therefore would also know which slices of the UE will continue in the selected target cell and which slices will not continue, as the source cell knows the slice support of all its neighbor cells. However, based on current information, the source cell cannot detect what kind of mobility pattern the UE followed. Also, based on single mobility event, the source cell cannot detect if there was a subsequent HO to another cell, which supported the slices that were dropped during the first HO, for example UE1 and UE2 in Figure 2. For proper root cause analysis, it is crucial to differentiate the cases of mobility pattern of UE1, UE2 and UE3 in Figure 1.

- For UEs with mobility pattern similar to UE1, it might help to adjust the mobility parameters like CIO to prioritize HO from Cell 1 to Cell 3 instead of Cell 2 to ensure service continuity for all the slices of the UE.

- However, for UEs with mobility pattern similar to UE2, it might not be sufficient to change the mobility parameters only, as these UEs are far away from Cell 3 at the point of HO. However, if majority of the UEs that are connected to Slice B follow this pattern, this could also indicate a coverage problem in the Slice B planning.

Figure 1: UE Mobility at Network Slice Border

Observation 1: Service degradation due to handovers to cells that do not support necessary slices can be avoided if the source cell can analyze the mobility pattern of the majority of the UEs associated with a certain network slice.

To correctly analyze the problem, for example, whether the slice discontinuity is due to mobility or coverage problem, it is important that the network can also know at what region of the cell border the UE made the handover. For this, recording the radio measurements of the source and candidate target cells at the time of HO execution by the UE could help differentiate the cases of where the UE made the HO, i.e., was there a second-best cell that could support the missing slice (UE1) or there was no second-best option (UE2 and UE3).

Observation 2: The problem of identifying slice discontinuity handovers that can possibly be avoided concerns CHOs, where UE may autonomously select one of the prepared target cells.

One potential way to record above mentioned measurements is to enhance the Successful HO report (SHR) as the HO was successful from radio perspective. The enhancements could include:

- Adding a new trigger condition to record an SHR i.e. if the UE observes after decoding the HO command at the point of HO execution that one or more of its connected slices will not be supported by the selected target cell, the UE triggers logging of SHR.

- The SHR cause value would also need to be enhanced to let the network know that it was due to non-support of the slice at target cell and service discontinuity.

Once the selected target cell detects that the SHR was generated for network slice service continuity problem, the target cell also forwards the SHR to the original source cell for proper root cause analysis and corrective action, as defined for the SHR handling.

Proposal 2: The SHR mechanism is enhanced to enable recording the SHR in case when some of the used slices are not supported in the target cell and that a slice-related cause value is added.

# 3	Conclusions

In this paper, we address two slice-related problems discussed within the Work Item. First, we propose what enhancements are needed to enhance MDT reporting of slice support. We propose:

Proposal 1: RAN3 kindly requests RAN2 to specify the above measurement enhancements for logged MDT for slice-based cell reselection.

Then, we analyse options to avoid unnecessary handovers to cells not supporting all needed slices:

Observation 1: Service degradation due to handovers to cells that do not support necessary slices can be avoided if the source cell can analyze the mobility pattern of the majority of the UEs associated with a certain network slice.

Observation 2: The problem of identifying slice discontinuity handovers that can possibly be avoided concerns CHOs, where UE may autonomously select one of the prepared target cells.

Based on the above, we propose:

Proposal 2: The SHR mechanism is enhanced to enable recording the SHR in case when some of the used slices are not supported in the target cell and that a slice-related cause value is added.

The above proposals are implemented in the two annexes below: Annex 1 proposes a TP related to mobility enhancements, while Annex 2 proposes an LS to RAN2 to request needed support.

# References

- R3-243146, RAN3 #124

- 3GPP TS 23.501

- 3GPP TS 38.300

- 3GPP TS 38.331

# Annex 1: Text proposal to TS 38.300 for SHR enhancement

The below text is proposed to be added to the TS 38.300.

First change, ommited text not changed

#### 15.5.2.7	Successful HO

One of the functions of Mobility Robustness Optimization is to detect a sub-optimal successful handover event. The aim is to identify underlying conditions during successful ordinary handovers, successful DAPS handovers, or successful Conditional handovers.

For analysis of successful handover, the UE may collect Successful Handover Report (SHR) based on configuration by network, if stored, and makes the SHR available to the network as specified in TS 38.331 [12]. The UE stores the SHR until it is fetched by the network or for 48 hours after the SHR is recorded.

For SHR collected during intra-NR handover, if the target NR node fetches the SHR from the UE and the trigger of SHR is T310/T312 or slice inavailability, it may forward the information to the source NR node, i.e. the node handling the cell reported as source cell in this SHR, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

If the NG-RAN node that fetches the SHR from the UE is neither the source node nor the target node of the handover, it forwards the information to the node(s) which configured the SHR trigger causing the SHR to be generated, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

In case of failure shortly after successful Handover, the same mobility event may generate both a SHR and a RLF report. In this case, the node(s), which configured the SHR trigger causing the SHR, may take the duplication into account e.g. ignore the SHR.

Upon retrieval of an SHR, the receiving node may analyse whether its mobility configuration needs adjustment.

The SHR report can be used to detect one case of Intra-system Too Late Handover, namely when DAPS HO is configured but an RLF is detected in the source cell during a successful DAPS HO.

The SHR report can be collected for intra-NR handover and for handover from NR to E-UTRA.

Remaining text not changed

# Annex 2: Draft LS to RAN2

3GPP TSG-RAN WG3 #125		R3-24xxxx

Maastricht, The Netherlands, EU, 19-23 Aug 2024

Title:	[DRAFT] LS on SON for Network Slicing

Response to:

Release:	Release 19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Nokia [to be RAN3]

To:	RAN2

Cc:

Contact Person:	Krzysztof Kordybach (krzysztof.kordybach@nokia.com)

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:	R3-24xxxx

1. Overall Description:

RAN3 would like to inform RAN2 about decisions made in the context of SON for Network Slicing.

RAN3 agreed to work on enhancements for slice-based cell reselection with logged MDT measurements

RAN3 identified following enhancements in MDT mechanism:

- New logging trigger for slice-based cell reselection measurements;

- Logging slice groups used for identifying reselection priorities;

- Used frequency priorities;

- Logging problems to connect to needed slice;

- Logging measurements in relation to frequency priorities;

RAN3 agreed to work on network slice mobility optimization utilizing Successful HO report

RAN3 identified following enhancements to SHR mechanism:

- Adding a trigger to record SHR in case of successful execution of a HO to a cell that does not support all the needed slices;

- Adding a new SHR cause to enable identification of slice-related reports;

RAN3 has agreed a TP to the BL CR to TS 38.300 related to the SHR enhancement, as attached.

2. Actions:

To RAN2

RAN3 kindly asks RAN2 to enable the needed signalling.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #125bis	14-18 October 2024			Hefei, China

TSG-RAN3 Meeting #126	18-22 November 2024			Orlando, USA