3GPP TSG-RAN WG2 Meeting #125		R3-244068

Maastricht, Netherlands, 19th – 23rd Aug. 2024

Source:	vivo

Title:	Discussion on LS from SA2 on FS_XRM Ph2

Agenda Item:	21.1

Document for:	Discussion and Decision

- Introduction

In RAN3#125 meeting, an LS from SA2 is received with the below questions on XRM_Ph2 [1]:

SA2 has started to evaluate solutions for key issues for the study of XR and Media Services Ph2 (TR 23.700-70). SA2 thanks the LS replies for XRM Ph2 from RAN2, RAN3 and SA4 and SA2 would like to further coordinate on the following aspects: - Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension? - Question2 [for SA4 and RAN2]: There is some discussion about time to next burst. - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6? - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server? - Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

In this contribution, we will discuss the issues in the LS and provide the draft response from RAN3 point of view.

- Discussion

  - Indicating periodicity via in-band signaling

- Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

Periodicity defined in SA2 is evaluated by UPF based on the arrival time for each burst. Currently, it was indicated in CP from UPF through SMF to RAN node. In case the periodicity is changed dynamically, it could be also indicated in CP. Some companies may argue that the UP approach, i.e. from UPF to RAN, is faster than CP approach. But our understanding is: if the change frequency for periodicity is not so high, there is no difference between CP and UP approach; if the change frequency for periodicity is very high, it is not periodic service, which is related to the below question. Thus, we think indicating periodicity via in-band signaling for dynamic changes of the periodicity is not needed from RAN2 point of view.

Proposal 1: RAN3 thinks indicating periodicity via in-band signaling for dynamic changes of the periodicity is not needed.

  - Available data rate exposure for GBR QoS Flow

- Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

In RAN3#124 meeting, RAN3 confirmed that it is feasible to estimate available data rate for GBR QoS flows, which is up to RAN implementation. Regarding the report of available data rate, companies have different views on how to provide it to core network (e.g., via CP or UP) in the reply LS to SA2 in [2]. In our understanding, for GBR QoS flow, whether satisfying the data rate for GBR could be already provided in CP. If the available data rate exposure for GBR QoS Flow is stable enough, CP approach is enough. But if it may change dynamically, UP approach could also be considered a complimentary for CP approach.

Proposal 2: RAN3 thinks available data rate exposure for GBR QoS Flow via user plane could be considered as a complimentary for CP approach.

  - Reply LS to SA2

Based on the above discussion, a draft reply LS is provided in the Annex.

Proposal 3: RAN3 to discuss the draft reply LS in Annex.

- Conclusion

In this contribution, we discuss the issues in the LS from SA2 [1]. Based on the discussion, we have the following proposals:

Proposal 1: RAN3 thinks indicating periodicity via in-band signaling for dynamic changes of the periodicity is not needed.

Proposal 2: RAN3 thinks available data rate exposure for GBR QoS Flow via user plane could be considered as a complimentary for CP approach.

Proposal 3: RAN3 to discuss the draft reply LS in Annex.

- References

- R3-244045/S2-2407351, LS on FS_XRM Ph2, SA2/vivo

- R3-243958, Response LS on FS_XRM, RAN3/Lenovo

- Annex - draft reply LS

Title:	[Draft] reply LS on FS_XRM Ph2

Response to:	R3-244045/S2-2407351

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	vivo [to be RAN3]

To:	SA2

Cc: 	RAN2, SA4

# Contact Person:

# Name:	              Chenli

E-mail Address:	 Chenli5g@vivo.com

# 1. Overall Description:

RAN3 would like to thank SA2 for the LS on FS_XRM Ph2 in [R3-244045/S2-2407351].

RAN3 has discussed the questions in the LS, and concluded the following responses [To be updated based on the discussion conclusion]:

- Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

RAN3 thinks indicating periodicity via in-band signaling for dynamic changes of the periodicity is not needed.

- Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

# RAN3 thinks available data rate exposure for GBR QoS Flow via user plane could be considered as a complimentary for CP approach.

# 2. Actions:

# To SA WG 2:

# RAN3 kindly request SA2 to take the above information into account during the future work, and provide feedback, if any.

# 3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #125bis	 14th Oct. – 18th Oct. 2024		Hefei, China

TSG RAN WG3 Meeting #126		18th Nov. – 22nd Nov. 2024		Orlando, USA