3GPP TSG-RAN WG3 Meeting #125		R3-244070

Maastricht, Netherlands, 19th – 23rd Aug. 2024

Source:	vivo, NTT DOCOMO INC.

Title:	Discussion on signaling design for LP-WUS subgrouping

Agenda Item:	18.2

Document for:	Discussion

- Introduction

In RAN#103 meeting, the updated WID of Rel-19 low-power Wake-up Signal and Receiver (LP-WUS/WUR) for NR in [1] was approved with the below objective involved RAN3:

For IDLE/INACTIVE modes • Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

In previous meeting [2][3], RAN2 has discussed the LP-WUS/WUR, and some progress on LP-WUS subgrouping have been reached as below:

RAN2 agreement: The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

In this contribution, we discuss the corresponding RAN3 impacts on LP-WUS subgrouping, including CN assigned subgrouping and UE_ID based subgrouping.

- Discussion

  - Background

In Rel-17, RAN3 discussed the corresponding design for PEI subgrouping, which also includes both CN assigned subgrouping and UE_ID based subgrouping, and we reached the following key agreements [4][5][6]:

Agree that some subgroup information should be sent over NG and F1 The CN provides subgroup ID included in subgroup information. Include UE paging subgroup information into the NG Paging message for CN paging. Introduce UE subgroup information into NGAP Core Network Assistance Information. Introduce UE paging subgroup information into F1 Paging message for paging a UE in RRC_INACTIVE/RRC_IDLE. Add some clarification on stage 2 behaviour related PEIPS assistance information for RRC INACTIVE for TS38.300. Remove the Editor Note in Xn BL CR. Introduce a new F1 only UE Capability e.g. F1 UE Paging Capability IE, that could be populated with IEs as needed in the future. (the first would be UEID subgroup support). Turn the "WA: NG-RAN node can know the total number of subgroups supported by CN via OAM." into agreement.

It could be observed that for PEI subgrouping, the NG paging message for CN paging, Xn paging message for RAN paging, and F1 paging message for both CN paging and RAN paging, besides the NGAP Core Network Assistance Information for RRC_INACTIVE UE will be enhanced to include the subgrouping information.

Since RAN2 has agreed that PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned subgrouping and UE_ID based subgrouping, a similar interface signaling design could be introduced for LP-WUS subgrouping.

Proposal 1: Using signaling design on NGAP interface, XnAP interface, and F1 interface for PEI subgrouping as the starting point for LP-WUS subgrouping for both CN paging and RAN paging.

  - CN assigned subgrouping

NGAP interface enhancement

CN assigned subgrouping was agreed for LP-WUS subgrouping in RAN2, and RAN2 agreed it is up to CN to assign the LP-WUS subgroup ID to the UE, which relies on NAS signalling between UE and AMF similar to the PEI subgrouping mechanism as below:

RAN2 agreement: RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

According to the signaling design for CN assigned subgrouping for PEI, we can have the following observations on NGAP interface enhancement, which mainly impacts TS 38.413:

- For a UE in RRC_IDLE state, since there is no UE context stored within the NG-RAN node, it doesn’t know whether the UE supports the CN assigned subgrouping or which subgroup ID the UE is monitoring. The AMF needs to inform NG-RAN of the CN assigned subgroup ID, i.e. the PEIPS Assistance Information IE within the NG paging message, then the NG-RAN node could send the PEI with the corresponding CN assigned subgroup ID to the UE.

- For a UE in RRC_INACTIVE state, the NG-RAN node will send the RAN paging to the UE with PEI. Hence, the AMF needs to send the corresponding CN assigned subgrouping information, i.e. the PEIPS Assistance Information IE to the NG-RAN node in Core Network Assistance Information for RRC INACTIVE IE during the initial Context Setup/Modification procedure, or the handover resource Allocation or Path Switch procedure.

Observation 1: For NGAP interface enhancement on CN assigned subgrouping for PEI, the CN assigned subgroup ID, i.e. the PEIPS Assistance Information IE, is included within the NG paging for RRC_IDLE UEs or the Core Network Assistance Information for RRC_INACTIVE UEs.

In our view, the NGAP interface enhancement on CN assigned subgrouping for PEI should also be introduced for LP-WUS similarly. Regarding the content for CN assigned subgrouping information within the NG paging message or Core Network Assistance Information for RRC INACTIVE IE, in our view, it should also be the CN assigned subgroup ID similar to PEI based on the RAN2 agreement that “it is up to CN to assign the LP-WUS subgroup ID to the UE” [3]. Regarding the details on the CN assigned subgroup ID, e.g. whether introducing new IE or reusing the legacy IE for PEI could be further discussed after RAN2 and SA2/CT1 have further progress, and how many bits for this IE if new IE is introduced depends on the subgrouping number discussed in RAN1 and RAN2.

From the above, some enhancements are needed to support the CN assigned grouping in RAN3 on NGAP interface. It is proposed to enhance the following messages and IEs:

Proposal 2: CN provides CN assigned subgroup ID in the NG paging message for CN paging. FFS details on CN assigned subgroup ID, e.g. whether introducing new IE or reusing legacy IE for PEI, and the size of IE.

Proposal 3: Introducing CN assigned subgroup ID into NGAP core Networking assistance information IE for RAN paging. FFS details on CN assigned subgroup ID, e.g. whether introducing new IE or reusing legacy IE for PEI, and the size of IE.

XnAP interface enhancement

For Xn interface enhancement on CN assigned LP-WUS subgrouping, RAN3 should consider whether there is a need to include the CN assigned LP-WUS subgroup ID in Xn paging message, which impacts the TS 38.423.

Similar to the PEI discussion, this issue depends on another issue whether the LP-WUS is only used in the last serving cell. If the LP-WUS could be used in other camped cells except the last serving cell, the CN assigned LP-WUS subgroup ID should be included in the Xn paging message, otherwise, there is no need to include this information in Xn paging message. In our understanding, it could bring more power saving gain if not restrict the use of LP-WUS in the last serving cell, however, the issue should be determined in RAN2 as the same issue for CIOT WUS or PEI are both discussed and determined by RAN2. RAN3 could discuss the Xn enhancement after RAN2 has further progress.

Observation 2: whether anchor gNB provides CN assigned subgroup ID into the Xn paging message for RAN paging should wait for further RAN2 progress.

F1 interface enhancement

Based on the above discussion, the NG-RAN node could acquire the CN assigned LP-WUS subgroup ID for RRC_IDLE UEs via NG paging message or for RRC_INACTIVE UEs via NGAP core Networking assistance information IE. However, only the CU of the NG-RAN node could know the information while the DU of the NG-RAN node couldn’t get it. Considering the LP-WUS signal is generated and transmitted by the DU, the CU needs to inform DU of the CN assigned LP-WUS subgrouping information, i.e. the CN assigned LP-WUS subgroup ID.

Similarly, the details on the CN assigned subgroup ID, e.g. whether introducing new IE or reusing the legacy IE for PEI could be further discussed after RAN2 and SA2/CT1 have further progress, and how many bits for this IE if new IE is introduced depends on the subgrouping number discussed in RAN1 and RAN2. Thus, we propose:

Proposal 4: Introduce CN assigned subgroup ID into F1 paging message for CN paging and RAN paging. FFS details on CN assigned subgroup ID.

Based on the above understanding, we propose to consider the following signalling chart on the information interaction on CN assigned LP-WUS subgrouping between UE, gNB and AMF, which is shown as follows:

Fig. 1: Overall signalling chart to support CN assigned LP-WUS subgrouping in NG-RAN

  - UE_ID based subgrouping

In Rel-17, RAN2 agreed to introduce a UERadioPagingInfo IE in the RRC UECapabilityInformation message in NR on whether the UE supports PEI, and it is the same UE capability for supporting PEI and UE_ID based PEI subgrouping. Since the UE Radio Capability for Paging IE is sent within the NG Paging for RRC_IDLE UEs, and within NGAP core Networking assistance information IE and Xn paging for RRC_INACTIVE UEs, the NG-RAN node could acquire whether the UE supports the UE_ID based PEI subgrouping information.

Observation 3: UE capability on UE_ID based PEI subgrouping is included in the UE Radio Capability for Paging IE in PEI.

For LP-WUS, since RAN2 has not discussed the UE capability on LP-WUS subgrouping information, the detailed signalling design on UE_ID based subgrouping for LP-WUS on RAN3 impacts should wait for more progress in RAN2. But anyway, similar as PEI design, the NG-RAN node should acquire the capability information for UE_ID based subgrouping for LP-WUS for CN paging and RAN paging.

Proposal 5: NG-RAN node should acquire the UE capability information on UE_ID based LP-WUS subgrouping, and wait for RAN2 for the detailed signalling design.

If the NG-RAN node acquires the UE capability information on UE_ID based LP-WUS subgrouping, a similar enhancement as PEI for CN assigned LP-WUS subgrouping on the F1 interface should be considered. Thus, we propose:

Proposal 6: Introducing UE_ID based LP-WUS subgrouping capability related information into F1 paging message for CN paging and RAN paging.

Besides, for UE_ID used in UE_ID based PEI subgrouping mechanism, it is 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192 considering the maximum number of UE_ID based PEI subgrouping is 8, so the UE_ID is extended 8 times.

Similarly, for UE_ID based LP-WUS subgrouping, the UE_ID for the calculation would also be extended. In this case, similarly, the “Extended UE Identity Index Value” IE should be included in NG paging for CN paging and RRC_IDLE UEs, and it should be included in NGAP core Networking assistance information IE and Xn paging for RAN paging and for RRC_INACTIVE UEs. Besides, the F1 paging message should also include the IE to inform the DU. But the detailed UE_ID for LP-WUS should be determined in RAN2, i.e. whether reusing the same IE as PEI or introducing new bits for UE_ID.

Proposal 7: Introducing Extended UE Identity Index Value for UE_ID based LP-WUS subgrouping into NG paging and F1 paging for CN paging message. FFS details, e.g. whether reusing same IE as PEI or introducing new bits for UE_ID.

Proposal 8: Introducing Extended UE Identity Index Value for UE_ID based LP-WUS subgrouping into NGAP core Networking assistance information, Xn paging message, and F1 paging message for RAN paging. FFS details, e.g. whether reusing same IE as PEI or introducing new bits for UE_ID.

  - LP-WUS subgrouping number

According to RAN2 agreement, the maximum number for LP-WUS subgrouping has been agreed to be no less than 8, and the detail number is being discussed in RAN1, and would be determined further. In PEI design, the CN assigned subgrouping for LP-WUS is determined by CN, and then the maximum number of UE_ID based subgrouping is determined by the total subgroup number minus the CN assigned subgroup numbers.

During the discussion for PEI in Rel-17, some companies proposed the NG-RAN node should communicate with AMF to acquire the CN assigned subgrouping number and assign suitable UE_ID based subgrouping. However, most companies opposed this suggestion since the NG-RAN node could acquire the information via OAM and no need to introduce the enhancement between NG-RAN with AMF. We think the similar principle for LP-WUS could be re-used.

Proposal 9: NG-RAN node can acquire the total number of subgroups supported by CN via OAM. Detailed subgrouping number waits for RAN1 and RAN2.

- Conclusions and Proposals

In this contribution, we discussed the RAN3 specification impacts to support paging subgrouping for LP-WUS including the CN assigned subgrouping and UE_ID based subgrouping in NG-RAN. Based on the discussion, we have the following observations and proposals.

Observations:

Observation 1: For NGAP interface enhancement on CN assigned subgrouping for PEI, the CN assigned subgroup ID, i.e. the PEIPS Assistance Information IE, is included within the NG paging for RRC_IDLE UEs or the Core Network Assistance Information for RRC_INACTIVE UEs.

Observation 2: whether anchor gNB provides CN assigned subgroup ID into the Xn paging message for RAN paging should wait for further RAN2 progress.

Observation 3: UE capability on UE_ID based PEI subgrouping is included in the UE Radio Capability for Paging IE in PEI.

Proposals:

Proposal 1: Using signaling design on NGAP interface, XnAP interface, and F1 interface for PEI subgrouping as the starting point for LP-WUS subgrouping for both CN paging and RAN paging.

Proposal 2: CN provides CN assigned subgroup ID in the NG paging message for CN paging. FFS details on CN assigned subgroup ID, e.g. whether introducing new IE or reusing legacy IE for PEI, and the size of IE.

Proposal 3: Introducing CN assigned subgroup ID into NGAP core Networking assistance information IE for RAN paging. FFS details on CN assigned subgroup ID, e.g. whether introducing new IE or reusing legacy IE for PEI, and the size of IE.

Proposal 4: Introduce CN assigned subgroup ID into F1 paging message for CN paging and RAN paging. FFS details on CN assigned subgroup ID.

Proposal 5: NG-RAN node should acquire the UE capability information on UE_ID based LP-WUS subgrouping, and wait for RAN2 for the detailed signalling design.

Proposal 6: Introducing UE_ID based LP-WUS subgrouping capability related information into F1 paging message for CN paging and RAN paging.

Proposal 7: Introducing Extended UE Identity Index Value for UE_ID based LP-WUS subgrouping into NG paging and F1 paging for CN paging message. FFS details, e.g. whether reusing same IE as PEI or introducing new bits for UE_ID.

Proposal 8: Introducing Extended UE Identity Index Value for UE_ID based LP-WUS subgrouping into NGAP core Networking assistance information, Xn paging message, and F1 paging message for RAN paging. FFS details, e.g. whether reusing same IE as PEI or introducing new bits for UE_ID.

Proposal 9: NG-RAN node can acquire the total number of subgroups supported by CN via OAM. Detailed subgrouping number waits for RAN1 and RAN2.

References

RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

R2-2403732, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

R2-2405702, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

R3-221428	CB: # 18_UEPowerSaving - Summary of email discussion

R3-222567	CB: # 20_UEPowerSaving - Summary of email discussion

R3-223961	CB: # UEPowSav_Corrections - Summary of email discussion