3GPP TSG-RAN WG3 Meeting #R3-125	R3-244071

Maastricht, NL, 19 - 23 August 2024

Agenda item:	 11.3

Source:		Jio

Title:		Discussion on beam prediction for AI/ML based CCO

Document for:		Discussion

# 1	Introduction

In this contribution, we would further propose and discuss details of AI/ML based CCO as a mirror submission document to RAN3#125 R3-244036 [23].

Furthermore this document is supported by a provisional patent [24]

# 2	General Discussion

The introduction of advanced AI and ML techniques in 3GPP Releases 18 and 19 marks a significant advancement in Coverage and Capacity Optimization (CCO), revolutionizing network management and optimization. A key focus is the management of Synchronization Signal Block (SSB) beamwidth, crucial for optimizing network coverage and capacity. As mobile networks grow more complex with diverse traffic patterns and evolving user behaviors, traditional optimization methods become inadequate in addressing the dynamic and real-time requirements of modern networks.

AI/ML offers a robust, data-driven approach to predict and manage these conditions effectively. 3GPP's specifications and guidelines play a critical role in ensuring the consistent, interoperable, and secure implementation of AI/ML models across various network components and vendors. These specifications detail the processes for training, validating, and deploying AI/ML models, ensuring that they meet the high standards necessary for effective CCO.

The integration of AI/ML technologies in CCO is a crucial step toward developing self-organizing networks (SON). SON can autonomously manage and optimize network parameters, such as load balancing, interference management, and the dynamic configuration of network resources. This autonomy allows networks to adapt to changing conditions and demands, thereby enhancing overall efficiency and user experience. The use of AI/ML in network management not only addresses current challenges but also sets the stage for future innovations in telecommunications..

The relevant 3GPP documents for AI/ML integration in CCO are:

- Release 18:

  - TS 38.300: NR; Overall description; Stage-2

  - TR 38.913: Study on scenarios and requirements for next generation access technologies

  - TR 38.816: Study on enhanced network automation

- Release 19:

  - TS 38.331: NR; Radio Resource Control (RRC); Protocol specification

  - TR 38.820: Study on AI/ML for NR

SSB beamwidth refers to the angular width of the beam used to transmit synchronization signals. Proper management of SSB beamwidth is essential for ensuring optimal coverage and capacity. Narrower beams can provide higher gain and better coverage in specific directions, while wider beams can cover larger areas with lower gain.

SSB beam steering refers to the elevation angle and/or the azimuth angle.

The trade-off between azimuth beamwidth, azimuth/elevation beam steering and coverage area is crucial for network performance.

  - 2.1 AI/ML Training and Inference algorithm model for CCO

3GPP TR 37.817 in clause 5.1, sub-clause 5.1.1 refers to AI/Ml and the Efficient energy consumption can also be achieved by other means such as reduction of load, coverage modification, or other RAN configuration adjustments. The optimal energy saving decision depends on many factors including the load situation at different RAN nodes, RAN nodes capabilities, KPI/QoS requirements, number of active Ues and UE mobility, cell utilization, etc

Potential improvements with SSB Beamwidth Management are:

- Enhanced Coverage - By dynamically adjusting the SSB beamwidth based on real-time network conditions, AI/ML algorithms can ensure that coverage is maximized in areas with high user density or poor signal quality. This adaptive beamwidth management allows for more efficient utilization of network resources, reducing coverage gaps and enhancing overall user experience.

- Improved Capacity - AI/ML algorithms can analyze traffic patterns and predict areas where capacity enhancements are needed. By narrowing the SSB beamwidth in these areas, the network can provide higher signal strength and better data rates to users, effectively increasing network capacity. This is particularly beneficial in urban environments with high user density.

- Interference Management -Interference is a significant challenge in dense network deployments. By intelligently managing the SSB beamwidth, AI/ML algorithms can minimize interference between neighboring cells. Narrower beams can be used to reduce overlap and interference, while wider beams can be employed in less congested areas to maintain coverage without causing excessive interference.

- Energy Efficiency - Adjusting SSB beamwidth can also contribute to energy efficiency. By focusing energy only where it is needed, the network can reduce unnecessary power consumption. AI/ML algorithms can optimize beamwidth settings to balance coverage and capacity needs with energy efficiency goals.

The management of SSB (Synchronization Signal Block) beam-width, along with azimuth and elevation beam steering, offers various potential improvements for mobile network performance. These improvements leverage AI/ML algorithms to dynamically adapt network parameters based on real-time conditions, thus enhancing coverage, capacity, interference management, and energy efficiency. Here are potential improvements focusing on both beamwidth and beam azimuth/elevation steering:

Potential improvements with SSB Beamwidth Management and beam steering are:

- Enhanced Coverage

- Dynamic Beamwidth Adjustment: By adjusting the beamwidth dynamically, networks can maximize coverage in areas with high user density or poor signal quality. For instance, wider beamwidths can cover larger areas, which is useful in sparsely populated regions.

- Adaptive Beam Steering: Azimuth and elevation steering can be used to direct beams more precisely towards areas with higher demand, ensuring better signal penetration and reducing dead zones. This is especially beneficial in complex terrains or urban environments with high-rise buildings.

2. Improved Capacity

- Targeted Narrow Beams: Narrowing the beamwidth in areas with high traffic demand can increase signal strength and data rates, thereby boosting network capacity. This is crucial in urban hotspots or events with dense user concentrations.

- Optimal Beam Steering Angles: By optimizing the azimuth and elevation angles, the network can focus beams more effectively on clusters of users, enhancing data throughput and reducing signal interference with other users.

3. Interference Management

- Interference Mitigation Through Beamwidth Control: In dense network deployments, narrower beams can be utilized to reduce overlap and interference between adjacent cells. This minimizes cross-cell interference and improves the quality of service.

- Directional Steering to Avoid Interference: By steering beams away from interference-prone areas or towards underutilized spectrum segments, networks can further mitigate interference issues. This is especially useful in areas with dense infrastructure or high-frequency reuse.

4. Energy Efficiency

- Energy Conservation with Focused Beamwidth: By narrowing the beamwidth and targeting specific areas, the network can reduce power wastage, focusing energy only where it is needed. This leads to a more efficient use of power resources.

- Steering for Optimal Power Usage: Adjusting the azimuth and elevation angles can also contribute to energy efficiency. For example, in less congested areas, beams can be steered to broader coverage with lower power settings, while in high-demand areas, more precise, high-power beams can be used.

Following RAN3#125 R3-244036 Discussion on Enhancements for AIML based CCO [23], training AI/ML models for CCO involves using vast amounts of network data, including user mobility patterns, signal strength measurements, traffic loads, and environmental factors. This data is used to train algorithms to recognize patterns and predict network performance under varying conditions.

The training process includes several stages:

- Data Collection and Preprocessing: Collecting data from various network elements and preprocessing it to ensure quality and relevance.

- Feature Selection: Identifying key features that influence network coverage and capacity.

- Model Training: Using supervised or unsupervised learning techniques to train the model. Techniques such as neural networks, decision trees, or support vector machines can be used depending on the complexity of the task.

- Validation and Testing: Validating the model with a separate dataset to ensure accuracy and generalizability. This step includes testing the model under different scenarios to evaluate its performance.

Typically once trained, the AI/ML model is deployed in the network to perform real-time inference. The model continuously analyzes incoming data to make predictions and recommendations for optimizing coverage and capacity.

Key aspects of the inference process include:

- Real-Time Data Processing: Continuously collecting and processing data from network elements to feed into the model.

- Predictive Analysis: Using the trained model to predict potential coverage gaps, congestion hotspots, and other network issues.

- Decision Making: Making real-time decisions on parameter adjustments, such as antenna tilt, power levels, and handover thresholds, to optimize network performance.

- Feedback Loop: Implementing a feedback mechanism to update the model based on the effectiveness of the optimization actions taken.

3GPP TR 38.843 focuses on enhancing performance through AI/ML models, including potential use cases such as channel state information (CSI) feedback enhancement, beam management, and positioning accuracy improvements.

3GPP TR 28.908 outlines the management capabilities required for AI/ML across different phases: training, validation, testing, deployment, and inference. This includes aspects such as ML entity loading, inference control, and performance evaluation.

Our proposal involves utilizing a diverse set of SSB beam shapes with varying beam-width configurations and incorporating SSB power boosting. These parameters are dynamically selected and adjusted by an AI/ML optimization algorithm for each sector. The AI/ML algorithm will dynamically choose the appropriate beam shape and optimal SSB power level for each individual SSB beam within the beam-sweeping set per sector, considering both azimuth and elevation angles. This selection process aims to optimize coverage and Signal-to-Interference-plus-Noise Ratio (SINR) while meeting network performance constraints.

Specifically, the algorithm will assess real-time network conditions to determine the most suitable beam-width and steering angles. By adjusting the azimuth and elevation angles, the beams can be precisely directed to target specific user clusters or areas with high traffic demand. This ensures that coverage is maximized, and signal quality is maintained, even in challenging environments such as urban canyons or high-density event venues.

Additionally, the algorithm will control the power boosting for each SSB beam, fine-tuning the power levels to balance between achieving sufficient coverage and minimizing interference. This dynamic adjustment helps to ensure efficient energy usage and enhances overall network performance. The proposed approach enables a flexible and adaptive network configuration, capable of responding to varying user densities and traffic patterns, thereby optimizing both capacity and user experience.

Proposal 1: For AI/ML based CCO, for the existing SSB beam sweeping functionality on serving node, different SSB beams with specific SSB beam-width configurations along with SSB beam steering azimuth and elevation angles are dynamically selected to optimize coverage and SINR.

The ultimate goal is to minimize the intra-cell and inter-cell beam overlapping, preserve the sector coverage and maximize the expected SINR for different inter-site distance and geography. Hence ultimately the Multi-SSB beam-width beams will improve the DL coverage and throughput without affecting the UE capability.

Proposal 2: For AI/ML based CCO, the proposed algorithm refers to P1 beam management algorithm, hence the user device is unaffected and follows the specific 3GPP and GSMA.

The proposed algorithm, to support AI/ML in NG-RAN, gets inputs only from connected UEs in the serving cell, building a cost function to predict the SSB coverage per individual SSB beam width out of the existing SSB beam sweeping beams.

Proposal 3: For AI/ML based CCO, the inference input may be collected from UE or local node or neighbour node, e.g. including:

- Inputs from connected UEs as following:

- connected UE measurement report (e.g., L1 SS-RSRP, SS-RSRQ, SS-SINR measurement, CQI, etc), including cell level and beam level UE measurements

- connected UE performance (HARQ NACK KPI retransmissions (BLER>10%)),

- connected UE location information (e.g., coordinates, serving cell ID)

- idle mode UE’s Random Access performance (RACH success rate, number of RACH attempts) to convert to SINR from existing graph of number of PRACH attempts vs SINR).

- connected UE RLF report

The future coverage status may be Cell Coverage Modification(s) (i.e.power parameters, SSB boosting etc) or SSB Coverage Modification(s) (i.e. SSB beam width modifications).

Proposal 4: For AI/ML based CCO, the future coverage status may include:

-	Cell Coverage Modification including SSB beam power boosting, for the Cell Coverage State which is used to solve the predicted CCO issue of the certain predicted cell;

-	SSB Coverage Modification including SSB beam width and SSB beam steering Coverage State which is used to solve the predicted CCO issue of the certain predicted SSB.

DL performance is investigated based on SSB power boosting and SSB beam-width shaping for different SSB beam gains on the beam-sweeping procedure, building a proper cost function.

Ultimate goal is to improve (increase) DL SINR and average DL cell throughput

Proposal 5: For AI/ML based CCO, the future coverage status may include the following NG-RAN gNB or gNB-CU/gNB-DU algorithm steps:

Step 1: Initial SSB Beam Sweeping

- Start SSB beam sweeping at the cell level with the default (operator-configurable) SSB beam-width shape. Perform sweeping for all SSB beams within a 5ms cycle.

Step 2: SINR Estimation and Data Collection

- Estimate the Downlink (DL) SINR per SSB sweeping beam using a combined cost function. This includes:

  - Number of Uplink (UL) PRACH counter attempts (build a graph of PRACH attempts vs. SINR) for idle UEs.

  - Connected UEs' UL Channel Quality Indicator (CQI) reports.

  - HARQ KPI retransmissions (Block Error Rate > 10%).

  - L1-SINR and L1-Reference Signal Received Power (RSRP) reporting.

Step 3: Beam Steering and Beam-width Shape Testing

- For each SSB beam-width shape, dynamically adjust the SSB beam's azimuth and elevation angles. Repeat Step 1 and Step 2 for each configuration, ensuring all combinations of beam-width shapes and steering angles are exhaustively tested.

Step 4: Coverage Footprint Estimation

- For each tested configuration, estimate the SSB beam coverage footprint as a percentage of the default SSB beam shape.

Step 5: Selection of Optimal Beam Configuration

- Identify the configuration with the argmax DL UL_SINR performance that also meets a predefined coverage footprint threshold (configurable). This configuration includes the optimal combination of beam-width shape, azimuth/elevation angles, and SSB power level.

Step 6: Fine-tuning with SSB Power Boosting

- For the selected optimal beam-width shape and steering angles, adjust the SSB power boosting parameters to further enhance SINR. Re-estimate the DL SINR based on the refined settings.

Step 7: Final Configuration Application

- Apply the optimal beam configuration (beam-width shape, steering angles, and power boosting) to the network for active use, ensuring maximum coverage and optimal signal quality.

Step 8: Continuous Monitoring and Adaptation

- Continuously monitor the network's performance metrics, such as SINR, coverage, and user experience indicators. Use AI/ML algorithms to dynamically adjust the configurations in response to changing network conditions and traffic patterns.

# 3	Conclusion

In this contribution, AI/ML based CCO are discussed. We have the following proposals:

Proposal 1: For AI/ML based CCO, for the existing SSB beam sweeping functionality on serving node, different SSB beams with specific SSB beam-width configurations along with SSB beam steering azimuth and elevation angles are dynamically selected to optimize coverage and SINR.

Proposal 2: For AI/ML based CCO, the proposed algorithm refers to P1 beam management algorithm, hence the user device is unaffected and follows the specific 3GPP and GSMA.

Proposal 3: For AI/ML based CCO, the inference input may be collected from UE or local node or neighbour node, e.g. including:

- Inputs from connected UEs as following:

- connected UE measurement report (e.g., L1 SS-RSRP, SS-RSRQ, SS-SINR measurement, CQI, etc), including cell level and beam level UE measurements

- connected UE performance (HARQ NACK KPI retransmissions (BLER>10%)),

- connected UE location information (e.g., coordinates, serving cell ID)

- idle mode UE’s Random Access performance (RACH success rate, number of RACH attempts) to convert to SINR from existing graph of number of PRACH attempts vs SINR).

- connected UE RLF report

Proposal 4: For AI/ML based CCO, the future coverage status may include:

-	Cell Coverage Modification including SSB beam power boosting, for the Cell Coverage State which is used to solve the predicted CCO issue of the certain predicted cell;

-	SSB Coverage Modification including SSB beam width and SSB beam steering Coverage State which is used to solve the predicted CCO issue of the certain predicted SSB.

Proposal 5: For AI/ML based CCO, the future coverage status may include the following NG-RAN gNB or gNB-CU/gNB-DU algorithm steps:

Step 1: Initial SSB Beam Sweeping

- Start SSB beam sweeping at the cell level with the default (operator-configurable) SSB beam-width shape. Perform sweeping for all SSB beams within a 5ms cycle.

Step 2: SINR Estimation and Data Collection

- Estimate the Downlink (DL) SINR per SSB sweeping beam using a combined cost function. This includes:

  - Number of Uplink (UL) PRACH counter attempts (build a graph of PRACH attempts vs. SINR) for idle UEs.

  - Connected UEs' UL Channel Quality Indicator (CQI) reports.

  - HARQ KPI retransmissions (Block Error Rate > 10%).

  - L1-SINR and L1-Reference Signal Received Power (RSRP) reporting.

Step 3: Beam Steering and Beam-width Shape Testing

- For each SSB beam-width shape, dynamically adjust the SSB beam's azimuth and elevation angles. Repeat Step 1 and Step 2 for each configuration, ensuring all combinations of beam-width shapes and steering angles are exhaustively tested.

Step 4: Coverage Footprint Estimation

- For each tested configuration, estimate the SSB beam coverage footprint as a percentage of the default SSB beam shape.

Step 5: Selection of Optimal Beam Configuration

- Identify the configuration with the argmax DL UL_SINR performance that also meets a predefined coverage footprint threshold (configurable). This configuration includes the optimal combination of beam-width shape, azimuth/elevation angles, and SSB power level.

Step 6: Fine-tuning with SSB Power Boosting

- For the selected optimal beam-width shape and steering angles, adjust the SSB power boosting parameters to further enhance SINR. Re-estimate the DL SINR based on the refined settings.

Step 7: Final Configuration Application

- Apply the optimal beam configuration (beam-width shape, steering angles, and power boosting) to the network for active use, ensuring maximum coverage and optimal signal quality.

Step 8: Continuous Monitoring and Adaptation

Continuously monitor the network's performance metrics, such as SINR, coverage, and user experience indicators. Use AI/ML algorithms to dynamically adjust the configurations in response to changing network conditions and traffic patterns.

# References

[1] R3-242231, (TP to TR 38.743) AI/ML enabled CCO, Samsung, Ericsson, Huawei, Telecom Italia, ZTE, Nokia, CATT, Deutsche Telekom, LG Electronics, Telecom Italia.

[2] 3GPP TS 38.300, "NR; Overall description; Stage 2".

[3] 3GPP TR 38.820: Study on AI/ML for NR

[4] 3GPP TR 38.913: Study on scenarios and requirements for next generation access technologies

[5] 3GPP TR 38.816: Study on enhanced network automation

[6] Jio White paper on AIML based CCO

- [7] R3-213391, CCO over F1: functional split and signaling between gNB-CU and gNB-DU (NEC)

- [8] R3-213410, (TP for SON BLCR for 38.473) Coverage and Capacity Optimization (Huawei)

- [9] R3-213660, Coverage and Capacity Optimization (Qualcomm Incorporated)

- [10] R3-213687, Coverage aspect of NR CCO (Nokia, Nokia Shanghai Bell)

- [11] R3-213820, Inter-gNB CCO (Ericsson)

- [12] R3-214006, Support of CCO (Samsung)

- [13] R3-214035, Further Discussion on Coverage and Capacity Optimization in NR (ZTE, China Telecom, China Unicom)

- [14] R3-221249 TP for BL CR 38.423 on Coverage and Capacity Optimization

- [15] R3-216189 LS on CCO related assistance information from OAM to NG-RAN  RAN3

- [16] R3-220887 (TP for SON BL CR 38.300) Coverage and Capacity Optimization  ZTE, China Telecom, China Unicom

- [17] R3-220888 (TP for SON BL CR 38.423) Coverage and Capacity Optimization  ZTE, China Telecom, China Unicom

- [18] R3-220889 (TP for SON BL CR 38.473) Coverage and Capacity Optimization  ZTE, China Telecom, China Unicom

- [19] R3-243385 Discussion on AIML based CCO 2024

- [20] R3-243255_CCO AI ML 2023

- [21] R3-243458 - Further discussion on AIML support for CCO

- [22] R3-243467 AI ML CCO

- [23] RAN#125 R3-244036 Discussion on Enhancements for AIML based CCO

- [24] Jio provisional patent Jio_Pxxxx _Multi SSB Configuration for Energy efficiency and System Performance Optimization_August 06 2024