3GPP TSG-RAN3 Meeting #125	R3-244072

Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	11.3

Source: 	Jio

Title: 	(TP for TR 38.743) Further enhancements for AI ML NG-RAN NRDC mobility

Document for:	Discussion/Decision

# Introduction

This paper contains the TP to capture the agreement for the AI/ML NRDC mobility, based on R3-243501 [2], R3-243501, R3-243646 and following the discussion in R3-241148 [3]

It is related to a mirror contribution from Jio discussion documents R3-XXXXXX “Discussion on Centralized beam prediction for AIML based CCO”, and Re-XXXXXX “Discussion on AIML based CCO De-Centralized beam prediction”.

# Text proposal to TR 38.743

------------------------------------------- Start of changes -------------------------------------------

5.1	Mobility optimization for NR-DC

Editor Note: Capture the description and its potential standard impacts.

Following R3-243501, for NR-DC scenario, the similar enhancements in Rel-18 phase can be specified during Rel-19 normative work phase..

5.1.1 Use case description

Following R3-241148 Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only (e.g., no conditional Dual Connectivity procedures are in scope).

It is proposed to add extra inference capability in the SN as well, related with different cause codes as Coverage and Capacity Optimization issues (CCO) and Energy saving.

Following R3-243501, the potential impacts for mobility optimization for NR-DC during normative work phase are:

-	Enhance existing Handover Request message to transfer following predicted information:

-	Predicted PSCell List

-	Predicted PCell List

-	Enhance existing dual connectivity procedure message to transfer following predicted information:

-	Predicted PCell List

-	Enhance existing data collection procedure message to transfer following measured information:

-	Measured PSCell List

-	Measured PCell List

Including the SN inference capability, the potential impacts for mobility optimization for NR-DC during normative work phase are:

- 	Enhance the X2-AP SGNB RELEASE REQUIRED message, introducing as cause code IE the coverage CCO and the energy efficiency reasons

Moreover following R3-243646, Mobility management is the scheme to guarantee the service-continuity during the mobility by minimizing the call drops, RLFs, unnecessary handovers, and ping-pong. For the future high-frequency network, as the coverage of a single node decreases, the frequency for UE to handover between nodes becomes high, especially for high-mobility UE.

With the AI/ML based Mobility optimization for NR-DC, the M-NG RAN node can utilize the input data to perform AI model training and/or AI model inference, and deduce the mobility metrics for future mobility decisions, which can further decrease the handover failures and reduce the user data interruption time.

Moreover with the AI/ML based SN Mobility optimization based on the SN inference capability for NR-DC, the SN gNB RAN node can utilize the input data to perform AI model training and/or AI model inference, and act with a mobility decision for SN release due to CCO or energy efficiency with a Energy Cost prediction function.

The AI/ML based Mobility Optimization for NR-DC should include following use cases:

-	Inter-MN PCell change without SN released.

-	MN-initiated PScell Addition or Change.

-	SN-initiated PScell Change based on NR A3 event

- 	SN-initiated PSCell change based on CCO coverage (i.e. NR A2 event)

- 	SN-initiated PSCell change based on Energy Cost function estimation

- 	SN-initiated SN release based on CCO coverage (i.e. NR A2 event)

- 	SN-initiated SN release based on Energy Cost function estimation

5.1.2 Input of AI/ML-based Network Energy Saving

To predict the optimized network energy saving decisions, NRDC SN gNB may need following information as input data for AI/ML-based network energy saving:

-	Current/Predicted Energy efficiency, including an Energy Cost function with among others the Power Amplifier energy efficiency and the crucial functionality of the Digital Pre-Distortion (DPD) optimization [5]

- Current/Predicted resource status

- Current SSB beam gain, beamwidth and coverage/capacity [4]

- UE mobility/trajectory prediction

- using AI/ML algorithms for the high resolution beam prediction as a proactive method [4]

5.1.3 Output of AI/ML-based Network Energy Saving

AI/ML-based network energy saving model for NRDC SN can generate following information as output:

-	Energy saving strategy, such as:

- recommended cell activation/deactivation,

- power adjustments on SSB beam gain [4]

- power adjustments on SSB beamwidth and beam steering [4],

- act on the high resolution beam prediction as CCO optimization as a reactive approach [4]

- Power adjustments in the RRUs or mMIMO RF chain Pas, including AI/ML assisted Digital Pre-Distortion (DPD) optimization [5]

-	Predicted Energy efficiency,  including the proper Energy cost function

-	Predicted energy state (e.g., active, high, low, inactive)-

5.1.4	Feedback of AI/ML-based Network Energy Saving

To optimize the performance of AI/ML-based network energy saving model for NRDC SN gnB node, following feedback can be considered to be collected from NG-RAN nodes:

-	Energy efficiency of serving cell, based on EC function including CCO with high beam resolution predictions and off-loading proposals.

-	System KPIs (e.g., integrity throughput, delay, RLF of current and neighbouring, retainability, for SN RAN node)

# References

[1] R3-241148 3GPP TR 38.743

[2] R3-243501 for 3GPP TR 38.743

[3] R3-241148

[4] Jio’s White paper on AIML NR-RAN mobility

[5] Jio Provisional Patent P1314