3GPP TSG-RAN3 Meeting #125	R3-244073

Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	11.3

Source: 	Jio

Title: 	(TP for TR 38.744) Enhancements in AI ML NG-RAN mobility

Document for:	Discussion/Decision

# Introduction

This paper contains the TP to capture the agreement for the AI/ML assisted mobility.

# Text proposal to TR 38.744

# 5	Evaluations

## 5.1	Common evaluation methodology, metrics and assumptions

Editor Note: This section intends to capture evaluation metrics, methodology and simulation assumptions common for all use cases

In 5G NR, SS-RSRP measurement, being defined in 3GPP TS 38.215 and 3GPP TS38.133, is performed and reported at Layer 1 (Physical Layer) and Layer 3 (RRC Layer). For example, 5G capable device can provide SS-RSRP measurements at Layer 1 when sending Channel State Information (CSI) and at Layer 3 when sending an RRC: Measurement Report to gNB.

5.1.1 Evaluation Metrics: How 5G Measurement is performed

To generate SS-RSRP measurement results, 5G UE is allowed to measure PBCH-DMRS signal. DMRS and SS signals are transmitted with equal power so results can be averaged. Indeed for ss-rsrp determination, demodulation reference signals (DMRS) for PBCH with an option of CSI-RS (Channel state information reference signals) is used. ss-rsrp is measured by the linear averaging over the power contributions of the resource elements that carry corresponding reference signals SSS and DMRS taking into account of power scaling of reference signals.
The DMRS and SS signals are transmitted with equal power so results can be averaged directly.

RSRP, RSRQ, SINR, and RSSI are quantities measured at L1, not L3. We use the term "L3 measurement" to imply that L3 does filtering on the values and does the final reporting. Filtering is done to remove the effect of fast fading and ignore short-term variations. Though L1 may collect measurements more often, L3 might report them at a larger configured periodicity. Thus, L3 takes a longer-term view of channel conditions.

Measurement Characteristics

- Layer 3 (L3) Measurement:

  - L3 measurements are useful for radio resource management (RRM) decisions which require a long term view of channel conditions, e.g. handover procedures should be triggered after Layer 3 filtering to reduce the risk of ping-pong between serving cells

  - Measurements are filtered at Layer 3 to remove the impact of fast fading and to help reduce short term variations in results

  - L3 measurements can be either ‘beam level’ or ‘cell level’ which can be reported within an RRC message: Measurement Report (MR)

  - Beam level measurements are generated directly from the L1 measurements by applying L3 filtering

  - Cell level measurements are derived from the L1 measurements using the certain rules

  - L3 SS-RSRP  reporting range is defined from -156 dBm to -31 dBm with 1 dB resolution

  - L3 report requires 7 bits payload to represent 128 value mapped to RSRP in dB

- Layer 1 (L1) Measurement:

  - L1 measurements are useful for procedures which must react with minimal delay, e.g. beam management procedures which require the UE to rapidly switch between beams

  - Measurements are filtered at Layer 1 to help remove the impact of noise and to improve measurement accuracy

  - L1 measurements are ‘beam level’

  - L1 SS-RSRP and CSI-RSRP reporting range is defined from -140 to -40dBm with 1dB resolution

  - L1 measurement requires 7 bits payload to represent 128 value mapped to RSRP in dB

## 5.2	RRM measurement prediction

Proposal for further enhancements

L1 Beam Level Measurement: L1 measurements are primarily concerned with beam-level information, which includes signal strength, beam quality, and other parameters. These measurements are critical for beamforming and initial access.

L3 Cell Level Measurement: L3 measurements provide a broader perspective of the cell's performance, including metrics like Reference Signal Received Power (RSRP), Reference Signal Received Quality (RSRQ), and Signal-to-Interference-plus-Noise Ratio (SINR).

AI/ML models, particularly supervised learning techniques, can be trained in OAM using historical L1 and L3 measurement data to predict L3 measurements from L1 data.

AI/ML models, particularly supervised learning techniques, can be trained in gNB CU-CP using historical L1 and L3 measurement data to predict L3 measurements from L1 data.

Key trained models in either OAM or CU-CP may include:

- Regression Models: To directly predict continuous L3 metrics from L1 inputs.

- Classification Models: To generally categorize L3 states based on L1 data patterns.

### 5.2.1	Evaluation methodology, metrics and assumptions

Editor Note: This section intends to capture RRM measurement prediction specific metrics, methodology and assumptions

Proposed 3GPP Rel.19 Framework

Data Collection and Preprocessing

- Data Sources: Collect DU L1 beam level and CU-CP L3 cell level measurements from network elements.

- Preprocessing: Clean and normalize data in either OAM or CU-CP, handle missing values, and extract relevant features.

Model Training and Validation

- Training: Use historical data to train AI/ML models in either OAM or CU-CP, splitting data into training and validation sets.

- Validation: Evaluate models in OAM or CU-CP using metrics (i.e. Mean Squared Error (MSE)) for regression and accuracy for classification.

Integration with RRM

- Real-time Prediction: Deploy trained models to CU-CP to predict L3 measurements in real-time using incoming L1 data.

Decision Making: Use in CU-CP predicted L3 measurements to inform mobility decisions such as handovers and load balancing.

### 5.2.2	Performance result

### Performance Evaluation

- Simulate an NR NG-RAN environment with typical user mobility patterns and traffic scenarios.

- Implement AI/ML models for L3 prediction and integrate with RRM functions.

### Key Performance Indicators (KPIs)

- Prediction Accuracy: Measure the accuracy of L3 predictions against actual measurements.

- Handover Efficiency: Evaluate the impact on handover success rates and latency.

- Network Performance: Assess improvements in overall network throughput and user experience.

## 5.3	Measurement event prediction

### 5.3.1	Evaluation methodology, metrics and assumptions

Editor Note: This section intends to capture measurement event prediction specific metrics, methodology and assumptions

Proposed 3GPP Rel.19 Framework

Data Collection and Preprocessing

- Data Sources: Collect measurement data in DU from the Uu interface, including signal strength, quality metrics, and historical handover events.

- Preprocessing: Clean and normalize data, handle missing values, and extract relevant features in DU.

Model Training and Validation

- Training: Use historical data to train AI/ML models in either OAM or CU-CP, splitting data into training and validation sets.

- Validation: Evaluate models in OAM or CU-CP using metrics like accuracy, precision, recall, and F1 score for classification models.

Integration with Mobility Management

- Real-time Prediction: Deploy trained models to CU-CP to predict Event A3 in real-time using incoming measurement data over the Uu interface.

- Decision Making: Use in CU-CP predicted events to optimize handover decisions and enhance mobility management.

The appropriate metrics would be the event A3

### 5.3.2	Performance result

To evaluate performance a simulation setup is proposed to simulate an NR NG-RAN environment with typical user mobility patterns and traffic scenarios and test AI/ML models for Event A3 prediction integrated with mobility management functions.

The expected KPIs could be:

- Prediction Accuracy: Measure the accuracy of Event A3 predictions.

- Handover Efficiency: Evaluate the impact on handover success rates and latency.

## Network Performance: Assess improvements in overall network throughput and user experience.

## 5.4	Failure event prediction

### 5.4.1	Evaluation methodology, metrics and assumptions

Editor Note: This section intends to capture unintended event prediction specific metrics, methodology and assumptions

To evaluate performance a simulation setup is proposed to simulate an NR NG-RAN environment with typical user mobility patterns and traffic scenarios and test AI/ML models for Event A3 prediction integrated with mobility management functions.

The expected KPIs could be:

- Prediction Accuracy: Measure the accuracy of Event A3 predictions.

- Handover Efficiency: Evaluate the impact on handover success rates and latency.

## Network Performance: Assess improvements in overall network throughput and user experience.

### 5.4.2	Performance result

The expected KPIs could be:

- Prediction Accuracy: Measure the accuracy of RLF and HOF predictions.

- Reduction in Failure Rates: Evaluate the impact on the frequency of RLF and HOF events.

- Network Performance: Assess improvements in overall network throughput, latency, and user experience.

# References

# [1] Jio’s White paper

[2] Jio Patent P1314

[3] TS 37.340: Multi-connectivity overall description: Stage-2. Specifies measurement model for multi-connectivity operation involving E-UTRA and NR.

[4] TS 38.133: Requirements for support of radio resource management. Specifies measurement requirements (including performance requirements), procedures and UE measurement capabilities.

[5] TS 38.215: Physical layer measurements. Specifies measurement quantities.

[6] TS 38.300: NR and NG-RAN overall description: Stage-2. Specifies measurement model.

[7] TS 38.304: User Equipment (UE) procedures in idle mode and in RRC Inactive state. Specifies cell reselection measurement rules and relaxed measurements.

[8] TS 38.314: Layer 2 measurements. Specifies mostly network requirements but section 4.3 is for UE.

[9] TS 38.331: Radio Resource Control (RRC): protocol specification. Specifies UE reporting of measurements.

[10] TS 38.533: User Equipment (UE) conformance specification: Radio Resource Management (RRM).