3GPP TSG-RAN WG3 Meeting #125	R3-244084

# Masstricht, NL, 19th – 23th August, 2024

Agenda Item:	11.2

Source:	ZTE Corporation

Title:		(TP to TR38.743) AI/ML assisted Network Slicing

Document for:	Other

1	Introduction

In the previous meeting, AI/ML network slicing was thoroughly discussed. Compared to other cases, the basic content of network slicing has already been included in TR 38.743[1]. Below are the agreements and unresolved issues for AI/ML-based network slicing:

In Rel-19, predicted information from CN to NG-RAN is not considered.

Agree to improve the granularity of UE performance feedback in Rel-19 for slicing.

The following points are still under discussion (FFS):

FFS whether to include measured/predicted UE throughput per slice as input from local node.

FFS whether to introduce delivery of slice level UE throughput prediction/measurement during handover procedure.

FFS on SLA requirements fulfilment information exchange.

In this contribution, we further present our understanding and analysis of these unresolved issues and provide the corresponding TP [1].

2	Discussion

2.1 Measured/Predicted UE traffic

In an end-to-end network slicing architecture, incorporating UE traffic information enables more proactive and efficient resource allocation strategies, ultimately enhancing service quality. NG-RAN nodes equipped with real-time monitoring capabilities can dynamically adjust resource allocation in response to changing UE traffic patterns. This dynamic resource management ensures that resources are efficiently distributed across slices to meet fluctuating demand while adhering to QoS requirements. Moreover, allocating resources in advance based on predicted UE traffic enables operators to address potential congestion points and ensure sufficient capacity to maintain service quality levels stipulated in SLAs.

- Current/predicted UE traffic (data volume) can be considered as one of assistance information used for AI/ML network slicing.

In the last meeting, it was proposed by some companies that measured/predicted UE throughput can be considered as a kind of input information from local node, and be delivered to the target node via handover procedure.

UE throughput: UE throughput refers to the amount of data that can be transmitted or received by UE per unit of time, typically measured in bits per second (bps) or bytes per second (Bps). UE Throughput directly reflects the UE's data transfer speed under current network conditions, showing the device's actual performance in terms of network connection quality, signal strength, and bandwidth. In the case of AI/ML NG-RAN, average UE throughput is a beneficial metric for evaluating the performance of a handover-ed UE from the source node to the target node. It reflects real-time performance, which is essential for optimizing the AI/ML model.

- UE throughput is more suitable for reflecting the real-time performance of a UE. Measured UE throughput can be used to evaluate the mobility decision made by the AI/ML function after a handover.

- UE throughput, as one of UE performance metrics, is better to evaluate the real-time performance after handover procedure.

UE traffic (Data volume): Data volume refers to the total amount of data transmitted or processed by the UE over a specific period, usually measured in MB, GB, etc. Data volume reflects usage behaviour and data consumption rather than real-time performance. For example, the total data consumed over a month/day/hour indicates user services but does not directly indicate real-time network performance. Also, in the case of AI/ML NG-RAN, predicted information is used to optimize subsequent mobility optimization scenarios for the target node. By obtaining the data volume of the UE for a future period, the target node can better formulate appropriate mobility decisions and implement effective traffic management policies to balance loads and prevent network overloads during peak times.

- Data volume is more beneficial for long-term mobility optimization and resource allocation. Predicted data volume per UE can be used by target node to helps in mobility planning and long-term sustainability.

- Introduce delivery of predicted UE traffic (data volume) during handover procedure.

Also, following the agreement during Rel-18 phase, it was agreed that UE traffic metric takes the data volume for a UE as the starting point.

UE traffic metric takes the data volume for a UE as the starting point.

Current UE performance includes average throughputs in the uplink and downlink, which provide average information. Now, the newly introduced UE traffic information consists of data volume, which represents instantaneous values. The following definition has been captured in the current specification as an example, which reflects the data volume.

| >Usage Count UL | M |  | INTEGER (0..264-1) | The unit is: octets. |
| --- | --- | --- | --- | --- |
| >Usage Count DL | M |  | INTEGER (0..264-1) | The unit is: octets. |

- Usage Count UL/DL can represent the data volume for a UE.

In the case of split architecture, gNB-CU-CP is responsible for model inference, so measured UE traffic (data volume) should be transferred from gNB-CU-UP to gNB-CU-CP to generate the measured UE traffic (data volume) via AI/ML model.

- In the case of split architecture, measured UE traffic (data volume) is transferred from from gNB-CU-UP to gNB-CU-CP upon request.

2.2 Fine granularity of UE performance

Currently, it was agreed that UE level performance is derived as the average value of per DRB level measurements of the UE. According to TS38.314 and TS28.552, UE throughput, UE packet delay, and UE packet loss can be measured per UE and per DRB per UE. A more fine-grained UE performance can reflect more accurate information.

It was agreed that improve the granularity of UE performance feedback in Rel-19 for slicing. Therefore, it is proposed to consider UE performance per DRB as a starting point.

- UE performance per DRB per UE level can be considered as a starting point.

2.3 Potential standard impacts:

Following standard impacts is listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

- Enhanced existing procedure to collect predicted information between NG-RAN nodes:

  - Predicted Resource Status per slice level between neighbouring NG-RAN nodes and source NG-RAN node

  - Predicted Slice Available Capacity between neighbouring NG-RAN nodes and source NG-RAN node

  - Predicted UE traffic forward to the target NG-RAN node via Handover Request message

- Enhanced existing procedure to collect feedback information between NG-RAN nodes:

  - Measured UE traffic after UEs handoverd from target NG-RAN node to source NG-RAN node

F1 interface:

- New signaling procedure to collect feedback information over F1 interface:

- Measured UE performance feedback from DU to CU

E1 interface:

- New signaling procedure to collect feedback information over E1 interface:

- Measured UE performance feedback from CU-UP to CU-CP

- Measured UE traffic feedback from CU-UP to CU-CP

- Agree to capture above potential impacts into TR.

The corresponding TP is provided in [1].

3 Conclusion

Following is our observations and proposals:

- Current/predicted UE traffic (data volume) can be considered as one of assistance information used for AI/ML network slicing.

- UE throughput is more suitable for reflecting the real-time performance of a UE. Measured UE throughput can be used to evaluate the mobility decision made by the AI/ML function after a handover.

- UE throughput, as one of UE performance metrics, is better to evaluate the real-time performance after handover procedure.

- Data volume is more beneficial for long-term mobility optimization and resource allocation. Predicted data volume per UE can be used by target node to helps in mobility planning and long-term sustainability.

- Introduce delivery of predicted UE traffic (data volume) during handover procedure.

- Usage Count UL/DL can represent the data volume for a UE.

- In the case of split architecture, measured UE traffic (data volume) is transferred from from gNB-CU-UP to gNB-CU-CP upon request.

- UE performance per DRB per UE level can be considered as a starting point.

- Agree to capture above required data for network slicing in TR.

- Agree to capture above potential impacts into TR.

Annex TP to 38.743

4	Use cases and Solutions

4.1	AI/ML based Network Slicing

4.1.1	Use case description

Editor Note: Capture the description of use case

Support of network slicing in NG-RAN is defined in TS 38.300 [x].

The NG-RAN plays a key role in taking mobility, load balancing and Radio Resources Management decisions for the purpose of meeting target requirements derived from the SLA of each supported network slice.

AI/ML function can analyze metrics related to network and UE level performance related to perform optimal resource management and mobility decisions for network slicing to meet the requirements.

4.1.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based network slicing:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

4.1.2.4	Input data of AI/ML based Network Slicing:

To predict the optimized network slicing decisions, a gNB may need the following information as input data for AI/ML-based network slicing:

From local node:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

-	Legacy predicted UE trajectory

-	Current/Predicted UE traffic

From neighbouring gNBs:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

4.1.2.5	Output data of AI/ML based Network Slicing:

AI/ML-based network slicing model in a gNB can generate the following information as output:

-	Predicted radio resource status per slice

-	Predicted slice available capacity

-	Resource management decisions for resources within RRM policies (used by gNB internally)-	Slice aware mobility decisions (used by gNB internally)

4.1.2.6	Feedback of AI/ML based Network Slicing:

To optimize the performance of AI/ML-based network slicing model, the following feedback can be considered to be collected from gNBs:

-	Measured Radio resource status per slice

-	Measured Slice available capacity

-	Legacy UE performance feedback for those UEs handed over from the source gNB

- -	Finer granularity UE performance feedback per DRB per UE for those UEs handed over from the source gNB.

- Measured UE traffic after UE handed over from the source gNB

4.1.2.7	Potential standard impacts:

Following standard impacts is listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

-	Enhanced existing procedure to collect predicted information between gNBs:

■	Predicted radio resource status per slice

■	Predicted slice available capacity

■	Predicted UE traffic forward to the target NG-RAN node via Handover Request message

- Enhanced existing procedure to collect feedback information between NG-RAN nodes:

■	Measured UE traffic after UEs handoverd from target NG-RAN node to source NG-RAN node

F1 interface:

- New signaling procedure to collect feedback information over F1 interface:

- Measured UE performance from DU to CU

E1 interface:

- New signaling procedure to collect feedback information over E1 interface:

- Measured UE performance from CU-UP to CU-CP

- Measured UE traffic from CU-UP to CU-CP