# 3GPP TSG-RAN WG3 Meeting #125	R3-244085

# Maastricht, NL, 19 - 23 Aug 2024

# Agenda Item:	11.3

# Source:	ZTE Corporation

# Title:	(TP to TR 38.743) AI/ML assisted CCO

# Document for:	Discussions & Approval

1	Introduction

Last meeting, following is the agreement for AI/ML assisted CCO:

For split architecture, the standardization of AI/ML based future CCO state inference in the gNB-DU is not supported in R19, how to derive future CCO state is up to implementation in the gNB-DU, e.g., legacy or AI/ML.

For non-split architecture, future CCO state can be derived via legacy mechanism or AI/ML techniques which is up to implementation.

In this contribution, we further provide our understanding on the left issues and the solutions on AI/ML assisted Coverage and Capacity Optimization. And the corresponding TP is provided in annex.

2	Discussion

2.1 Left issues

The following FFS is left after the end of previous meeting:

Time information to be further discussed in next meeting.

More specifically, the time info we discussed in previous meeting is used to indicate the when will the gNB-CU/gNB predicted future coverage status happen. From our point of view, such timing info combined with the predicted future coverage status information is useful for the receiver gNB/gNB-CU.  By using the predicted future coverage status with the time information, the receiver side can response more promptly to the predicted coverage changing and reduce the latency. Moreover, understanding the timing of predicted covrage status changes can help optimize resource allocation at receiver side, to maximize the network preformance and to reduce the operational costs.

According to the reporting process for AI/ML functions, the requesting node may include the requested prediction time point in the request message. The receiving node then uses this configuration to predict the information for that time point and returns it to the requesting side. However, for the CCO case, the predicted information is reported directly to the neighboring nodes. Generally, when using AI to predict time series information, the predicted information will also include the prediction time, similar to how the UE trajectory's residence time is included in the handover procedure. Therefore, it is reasonable to carry the timing information with the future CCO configuration.

- gNB-CU or gNB may send the future coverage status together with the timing info about when such future coverage status will be executed to neighbour gNB.

2.2 Alternatives for AI/ML assisted CCO procedure

The following alternatives may further be discussed for AI assisted CCO procedure.

2.2.1 Alternative 1

In this alternative, NG-RAN node 1 may predict CCO issue and/or derive future coverage status. Then NG-RAN node 1 may send the data collection request with its predicted/derived data to NG-RAN node 2. With the further input data from NG-RAN node 2, NG-RAN node 1 may re-predict the future coverage status.

Figure 2.2.1-1 Call flow for alternative 1

Step 1:  Based on the AI/ML assistance, NG-RAN node 1 predicts an CCO issue.

[Optional] Step1a: NG-RAN node 1 may derive its future coverage status.

Step 2: The NG-RAN node 1 sends the AI Data Collection Request message to the NG-RAN node 2, to ask the NG-RAN 2 to collect and report the information needed for AI-assisted CCO. The message may contain the following information:

- Current coverage status for NG-RAN node 1;

- [Optional]Derived coverage modification of NG-RAN node 1.

Step 3: The NG-RAN node 2 sends the AI Data Collection Response message to the NG-RAN node 1.

Step 4: The NG-RAN node 2 collects the information for AI-assisted CCO, such as the current and/or predicted CCO configuration status.

Step 5: The NG-RAN node 2 sends the AI Data Collection Update message to the NG-RAN node 1, including the collected data including current and/or derived coverage status info of NG-RAN node 2 for AI-assisted CCO.

Step 6: The NG-RAN node 1 predicts the coverage configuration status of its own, based on the information collected and received as input.

Step 7: The NG-RAN node 1 sends the predicted coverage configuration status with time info to the NG-RAN node 2, using NG-RAN Node Configuration Update message.

2.2.2 Alternative 2

Compared with the first alternative, this is more straightforward. NG-RAN node 1 predicts the CCO (e.g. CCO issue, future coverage status) info and forwards the predicted data to NG-RAN node 2.

Figure 2.2.2-1 Call flow for alternative 2

Step 1:  Based on the AI/ML assistance, NG-RAN node 1 predicts an CCO issue.

Step 2: The NG-RAN node 1 derive the coverage configuration status of its own, based on the collected information.

Step 3: The NG-RAN node 1 sends the future coverage configuration status and the time information which presents the effective time to the NG-RAN node 2, using NG-RAN Node Configuration Update message.

- It is proposed for RAN3 to capture both alternatives for AI assisted CCO in TR:

Alt 1: NG-RAN node 1 may send the predicted CCO configuration to NG-RAN node 2 and adjust its predicted CCO configuration based on feedback from NG-RAN node 2.

Alt 2: NG-RAN node 1 is fully responsible for prediction and only sends the predicted CCO configuration together with related time information to NG-RAN node 2.

The following part discusses the information required for AI function.

Firstly, for the input data, with the aim to perform CCO related model inference in NG-RAN node 1, the current coverage configuration status of its neighbor nodes shall be provided, per cell or per SSB. With the assumption that the NG-RAN node 2 is also deployed with AI model, i.e., the NG-RAN node 2 is also able to perform model inference for CCO, then the input of AI model inference at NG-RAN node 1 could also include the predicted coverage configuration status of the NG-RAN node 2. Besides, similar as what has been discussed in Rel-18, the current UE traffic as well predicted UE traffic of the NG-RAN node 1 could also be considered as the input of AI-assisted CCO.

- Current/Predicted Coverage Configuration Status, current/predicted UE traffic can be transferred between NG-RAN nodes to assist with AI/ML-assisted CCO.

When it comes to the output data of the AI-assisted CCO model inference at the NG-RAN node 1, the predicted coverage configuration status of NG-RAN node 1 shall no doubt be counted as the main output, which reflects the AI inference result on how the coverage of the current node could be modified, based on all the information collected from itself and its neighbor nodes. Aside from the predicted coverage configuration of NG-RAN node 1 itself, the AI model inference could also generate the coverage modification solution for NG-RAN node 2, for the purpose of creating the optimized coverage and capacity for the whole area. Also, the predicted UE traffic can also be a part of AI inference output, given that the current UE traffic is contained as an input.

Finally, regarding the feedback information from the neighbor node, the actual coverage configuration status is worth considered to measure the performance of the AI model. The NG-RAN node 2 would perform coverage modification after it receives the predicted coverage status of NG-RAN node 1 and/or the recommended coverage status for NG-RAN node 2 in the NG-RAN node configuration message. How the NG-RAN node 2 actually makes the decision to optimize the coverage and capacity would assist with the optimization of the AI model, to make its prediction more precise and close to the actually outcome of the network. Similar as other use cases discussed in Rel-18, the UE performance feedback after handover and UE traffic would be useful to be considered as feedback for the AI/ML model inference.

A summary of the required data is provided in section 2.4.1 of this paper.

2.3 Split Architecture

The following part discusses the interaction between CU and DU in the two different cases where AI model inference is performed by the CU and  DU.

For the scenario of CU-DU split, the procedures between CU and DU are depicted as below:

Figure 2.3-1. gNB-CU performs model inference

The figure is drafted based on the assumption that only the gNB-CU is capable of performing AI function, and the AI models have already been deployed respectively.

- Step 1: The gNB-CU sends the Data Collection Request message to the gNB-DU, to ask the gNB-DU to collect and report the information needed for AI-assisted CCO.

- Step 2: The gNB-DU sends the Data Collection Response message to the gNB-CU.

- Step 3: The gNB-DU collects the information for AI-assisted CCO, including the current coverage configuration status, which could be cell level and/or SSB level.

- Step 4: The gNB-DU sends the Data Collection Update message to the gNB-CU, including the collected data for AI-assisted CCO.

- Step 5: The gNB-CU predicts the coverage configuration status of its own, based on the information collected as input.

Following Rel-18 principles, CU is responsible for model inference in the case of split architecture. Therefore, in this solution, CU needs to collect the past/current coverage status from DU, and perform model inference to predict coverage status by its deployed model.

- For split architecture, gNB-DU may send the current coverage configuration status to gNB-CU upon request.

The figure shown below illustrate the gNB-CU may predict the future coverage status and send to gNB-DU as suggestion. gNB-DU may use the received info as reference. The final decision on the coverage status shall be decided by gNB-DU.

Figure 2.3-2. The gNB-CU provides future coverage status suggestion to gNB-DU

- Step 1: The gNB-CU peforms AI inference and generate the future coverage configuration status and the predicted CCO issue, based on the collected information.

- Step 2: The gNB-CU sends the future coverage configuration status to gNB-DU.

- Step 3: The gNB-DU replys.

- Step 4:The gNB-DU may consider the received future coverage status information as reference. gNB-DU modify the coverage statues of cells and SSBs.

- Step 5: The gNB-CU replies.

- For split architecture, the gNB-CU may send the future coverage configuration status suggestion to gNB-DU as reference.

2.4 Potential impacts of network interfaces

2.4.1 Required Data

Required data:

Input for AI/ML assisted CCO:

From local node:

- Current CCO configuration status

- Measured/Predicted Resource status

- Current/predicted UE traffic

- Predicted CCO issue(s)

From the UE:

- UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, RLF report etc), including cell level and beam level UE measurements

From neighbouring NG-RAN nodes:

-	Current/Predicted coverage configuration status (including cell level, beam level and relevant time info)

Output for AI/ML assisted CCO:

  - Predicted Coverage Configuration(including cell level, beam level and relevant time info)

  - Predicted UE traffic

Feedback for AI/ML assisted CCO:

  - Actual Coverage Configuration after modification

  - UE performance feedback for those UEs handed over from the source NG-RAN node

  - Measured UE traffic

- Agree to capture above required data for CCO in TR.

2.4.2 Potential standard impacts

Based on the discussion above, the potential impacts on network interfaces can be concluded as:

XnAP:

  - Predicted coverage configuration status between NG-RAN nodes (including cell level, SSB level and relevant time info)

  - Predicted UE traffic over handover request

  - UE performance feedback after UE handover from source node to target node

  - Measured UE traffic after UE handover from source node to target node

F1AP:

  - UE performance feedback after handover

  - Predicted CCO issues from gNB-CU to gNB-DU

E1AP:

  - Measured UE performance feedback from CU-UP to CU-CP

  - Measured UE traffic feedback from CU-UP to CU-CP

- Agree to capture above potential impacts into TR.

The corresponding TP is provided in annex.

3	Conclusion

Proposal 1 gNB-CU or gNB may send the future coverage status together with the timing info about when such future coverage status will be executed to neighbour gNB.

Proposal 2 It is proposed for RAN3 to capture both alternatives for AI assisted CCO in TR:

Alt 1: NG-RAN node 1 may send the predicted CCO configuration to NG-RAN node 2 and adjust its predicted CCO configuration based on feedback from NG-RAN node 2.

Alt 2: NG-RAN node 1 is fully responsible for prediction and only sends the predicted CCO configuration together with related time information to NG-RAN node 2.

Proposal 3 Current/Predicted Coverage Configuration Status, current/predicted UE traffic can be transferred between NG-RAN nodes to assist with AI/ML-assisted CCO.

Proposal 4 For split architecture, gNB-DU may send the current coverage configuration status to gNB-CU upon request.

Proposal 5 For split architecture, the gNB-CU may send the future coverage configuration status suggestion to gNB-DU as reference.

Proposal 6 Agree to capture above required data for CCO in TR.

Proposal 7 Agree to capture above potential impacts into TR.

Annex TP to TR 38.743

## 4.2	AI/ML based Coverage and Capacity Optimization

### 4.2.1	Use case description

Editor Note: Capture the description of use case

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. An NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included.

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

### 4.2.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.2.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based CCO:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

For non-split architecture, further CCO state can be derived via legacy mechanism or AI/MLtechniques which is up to implementation.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

For split architecture, the standardization of AI/ML based future CCO state inference in the gNB-DU is not supported in R19, how to derive future CCO state is up to implementation in the gNB-DU, e.g., legacy or AI/ML.

4.2.2.2	Input of AI/ML-assisted CCO

To predict the optimized coverage and capacity optimization decisions, NG-RAN may need following information as input data for AI/ML-based coverage and capacity optimization:

Input for AI/ML assisted CCO:

From local node:

-	Current CCO configuration status

-	Measured/Predicted Resource status

-	Current/Predicted UE traffic

-	Predicted CCO issue(s)

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, RLF report etc), including cell level and beam level UE measurements

From neighbouring NG-RAN nodes:

-	Current/Predicted coverage configuration status (including cell level, beam level and/or time info)

If existing UE measurements are needed by a gNB for AI/ML-based coverage and capacity optimization, RAN3 shall reuse the existing framework (including MDT and RRM measurements).

4.2.2.3	Output of AI/ML-assisted CCO

AI/ML-based coverage and capacity optimization model can generate following information as output:

-	Predicted Coverage Configuration (including cell level, beam level and/or relevant time info)

-	Recommended Coverage Configuration Status

-	Predicted UE traffic

4.2.2.4	Feedback of AI/ML-assisted CCO

To optimize the performance of AI/ML-based coverage and capacity optimization model, following feedback can be considered to be collected from NG-RAN nodes:

-	Actual Coverage Configuration after modification

-	UE performance feedback for those UEs handed over from the source NG-RAN node

-	Measured UE traffic

4.2.2.5	Standard Impact

Details should be discussed during the normative phase.

# XnAP:

  - Predicted coverage configuration status between NG-RAN nodes (including cell level, SSB level and/or relevant time info)

  - Predicted UE traffic over handover request

  - UE performance feedback after UE handover from source node to target node

  - Measured UE traffic after UE handover from source node to target node

# F1AP:

  - UE performance feedback after handover

  - Predicted CCO issues from gNB-CU to gNB-DU

  - Future coverage configuration status suggestion from gNB-CU to gNB-DU

# E1AP:

  - Measured UE performance feedback from CU-UP to CU-CP

  - Measured UE traffic feedback from CU-UP to CU-CP