3GPP TSG-RAN WG3 Meeting #125	R3-244087

# Masstricht, NL, 19th – 23rd August, 2024

Agenda Item:	11.4

Source:	ZTE Corporation

Title:	(TP for TR38.743) Mobility Optimization for NR-DC

Document for:	Discussions & Approval

- Introduction

During the previous meeting, mobility optimization for NR-DC case was agreed to be resolved during Rel-19:

Agree to take as baseline for the Mobility Optimization for NR-DC that the use case is studied assuming inference at the MN. The main use case is limited to dual connectivity only (e.g. no conditional procedures are in scope).

Enhance the Dual Connectivity procedure (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance.

In this contribution, we further provide our analysis and understanding on the mobility optimization for NR-DC, and corresponding TP to TR 38.743.

- Discussion

As discussed before, it was agreed that MN is in charge of model inference, and dual connectivity procedure (e.g., MN-initiated SN addition, MN-initiated SN change) is enhanced to trigger the collection of measured UE performance, as similar mechanism as UE performance collection via Handover procedure.

Here, we would like to emphasize that besides UE performance collection in the NR-DC scenario, predicted cell information as assistance information is also beneficial for the SN to choose and configure SCG cells. In the legacy procedure, the MN could provide the assistance information (e.g., the latest measurements) to the SN to choose and configure SCG cells.

After reviewing TS38.423, and without considering the conditional procedures, the MN does not provide detailed SCG cell information to the SN. This means that the SN itself decides the PSCell based on the measurements provided by the MN. Therefore, in the case of the AI/ML function in the NR-DC scenario, it is straightforward that the MN could also provide AI/ML-related information to the SN.

- MN could provide the assistance information to SN to choose and configure SCG cells in legacy way.

- MN could also provide the predicted cell information via AI/ML function as assistance information for SN to choose and configure the SCG cell(s).

Within a standalone architecture, predicted UE trajectory involves a predicted cell list of information that the UE will be connected to. Thus, for NR-DC scenarios, the MN could also predict PCell list information and PSCell list information based on historical UE location, UE measurement reports, etc.:

- For predicted PCell list information, it can be used internally by the MN itself or transferred to the target MN via a Handover Request message for subsequent mobility optimization.

- For predicted PSCell list information, it can be transferred via an SN Addition Request message/SN Modification Request message from the MN to the SN to help choose and configure SCG cell(s).

In addition to the predicted information, measured information (measured PSCell list and measured PCell list) as feedback should also be collected and reported for model performance evaluation.

- Predicted PCells information and predicted PSCell information can be considered as assistance information for mobility optimization in NR-DC scenarios.

- Measured PCell information and Measured PSCell information can be considered as feedback for performance evaluation.

Following is the scenario for mobility optimization in NR-DC scenario to support AI/ML function:

Scenario 1: MN-initiated SN Addition/Change

Figure 1. MN-initiated SN Addition/Change procedure in the case of AI/ML NG-RAN

Scenario 2: MN-initiated SN Modification

Figure 2. MN-initiated SN Modification procedure in the case of AI/ML NG-RAN

Scenario 3: Inter-Maste Node handover with/without SN change

Figure 3. Inter-Maste Node handover with/without SN change in the case of AI/ML NG-RAN

Therefore, following is the potential impacts for mobility optimization for NR-DC during normative work phase:

-	Enhance existing Handover Request message to transfer following predicted information:

-	Predicted PSCell List

-	Predicted PCell List

-	Enhance existing dual connectivity procedure message to transfer following predicted information:

-	Predicted PCell List

-	Enhance existing data collection procedure message to transfer following measured information:

-	Measured PSCell List

-	Measured PCell List

-	 Enhance existing dual connectivity procedure message to trigger the collection of measured information.

Details can be further discussed during normative work.

- Agree to capture above standard impacts for mobility optimization for NR-DC in TR.

3	Conclusion

- MN could provide the assistance information to SN to choose and configure SCG cells in legacy way.

- MN could also provide the predicted cell information via AI/ML function as assistance information for SN to choose and configure the SCG cell(s).

- Predicted PCells information and predicted PSCell information can be considered as assistance information for mobility optimization in NR-DC scenarios.

- Measured PCell information and Measured PSCell information can be considered as feedback for performance evaluation.

- Agree to capture above standard impacts for mobility optimization for NR-DC in TR.

4	TP to 38.743

5	Rel-18 Leftovers and solutions

Editor Note: Such topics are listed here for further selection/down selection for normative work.

5.1	Mobility optimization for NR-DC

Editor Note: Capture the description and its potential standard impacts.

5.1.1	Use case description

Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only (e.g., no conditional Dual Connectivity procedures are in scope). For NR-DC scenario, the similar enhancements in Rel-18 phase can be specified during Rel-19 normative work phase.

5.1.1	Potential Standard impacts

Following is the scenario for mobility optimization in NR-DC scenario to support AI/ML function:

Scenario 1: MN-initiated SN Addition/Change

Figure 1. MN-initiated SN Addition/Change procedure in the case of AI/ML NG-RAN

Scenario 2: MN-initiated SN Modification

Figure 2. MN-initiated SN Modification procedure in the case of AI/ML NG-RAN

Scenario 3: Inter-Maste Node handover with/without SN change

Figure 3. Inter-Maste Node handover with/without SN change in the case of AI/ML NG-RAN

Following is the potential impacts for mobility optimization for NR-DC during normative work phase:

-	Enhance existing Handover Request message to transfer following predicted information:

-	Predicted PSCell List

-	Predicted PCell List

-	Enhance existing dual connectivity procedure message to transfer following predicted information:

-	Predicted PCell List

-	Enhance existing data collection procedure message to transfer following measured information:

-	Measured PSCell List

-	Measured PCell List

-	 Enhance existing dual connectivity procedure message to trigger the collection of measured information.

Details can be further discussed during normative work.