3GPP TSG-RAN WG3 Meeting #125	R3-244088

# Maastricht, NL, 19th – 23rd August, 2024

Agenda Item:	11.4

Source:	ZTE Corporation

Title:	(TP for TR38.743) Support of AI/ML in split architecture

Document for:	Discussions & Approval

- Introduction

Support of AI/ML in split architecture was agreed to be resolved during Rel-19:

Split architecture support for Rel-18 use cases based on the conclusions from Rel-18 WI

In this contribution, we further provide our analysis and understanding support of AI/ML in split architecture and corresponding TP to TR 38.743.

- Discussion

During the previous meetings in Rel18, it was agreed that UE traffic metric takes the data volume for a UE as the starting point.

UE traffic metric takes the data volume for a UE as the starting point.

Considering the gNB-CU-UP knowing the UE traffic related information, and CU-CP has the ability to perform the AI/ML function according to the TR37.817. One way to support generating predicted UE traffic is that CU-CP needs to collect the current/historical UE traffic over E1 interface.

However, in the current E1 spec,  the existing DATA USAGE REPORT message can not satisfy the requirement of data collection for AI/ML function, because the existing procedure is a class 2 procedure without the corresponding class 1 procedure. There is no request message for requesting entity to use to trigger the data collection, and set the configuration, e.g., periodicity.

The following definition has been captured in the current specification as an example, which reflects the data volume.

| >Usage Count UL | M |  | INTEGER (0..264-1) | The unit is: octets. |
| --- | --- | --- | --- | --- |
| >Usage Count DL | M |  | INTEGER (0..264-1) | The unit is: octets. |

- Usage Count UL/DL can represent the data volume for a UE.

In the case of split architecture, gNB-CU-CP is responsible for model inference, so measured UE traffic (data volume) should be transferred from gNB-CU-UP to gNB-CU-CP to generate the measured UE traffic (data volume) via AI/ML model.

- In the case of split architecture, measured UE traffic (data volume) is transferred from from gNB-CU-UP to gNB-CU-CP upon request.

- Using the new class 1& class2 procedure over E1 to support transferring UE traffic from gNB-CU-UP to gNB-CU-CP.

3	Conclusion

- Usage Count UL/DL can represent the data volume for a UE.

- In the case of split architecture, measured UE traffic (data volume) is transferred from from gNB-CU-UP to gNB-CU-CP upon request.

- Using the new class 1& class2 procedure over E1 to support transferring UE traffic from gNB-CU-UP to gNB-CU-CP.

- TP to 38.743

5.2.1	Use case description

The split architecture should be enhanced to support the Rel-18 use cases, e.g, Load Balancing, Energy Saving, and Mobility Optimization.

In case of CU-DU architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP);

-	AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP).

5.2.2	Potential Standard impacts

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

E1 interface:

-	Measured UE performance from gNB-CU-UP to gNB-CU-CP.

-	Measured UE traffic (data volume) from gNB-CU-UP to gNB-CU-CP

F1 interface:

-	Measured Energy Cost from gNB-DU to gNB-CU.