3GPP TSG-RAN WG3 Meeting #125	R3-244089ss

Maastricht, NL, from – to Month YEAR

Agenda Item:	11.4

Source:	ZTE Corporation

Title:	Discussion on Finer granularity of Energy Cost

Document for:	Other

- Introduction

In the last meeting, companies had a heated discussion on energy saving enhancements. Finally, the topic was limited to a further discussion on the finer granularity of energy costs.

Only discuss on finer granularity of energy cost in Aug meeting and make the conclusion.

In this contribution, we further discuss on the finer granularity of energy cost and giver our understanding of this feature.

2	Discussion

In Rel-18, we agreed to transmit node-level energy costs between gNBs. This is because the high principle of energy consumption should take SA5 as a basis. Therefore, for transmission between base stations, we defined a mapping rule relationship to establish new metrics for node-level energy costs. However, in case of AI/ML assisted network energy saving, using node-level energy cost cannot accurately reflect the effectiveness of energy-saving strategies. This is because node-level energy cost is not only affected by cell shutdowns but may also be influenced by other factors.

- Node-level energy cost as feedback does not clearly reflect the performance of energy-saving strategies.

Given this, some companies have proposed finer granularity of energy cost (e.g., cell-level EC). Energy consumption in network infrastructure is intrinsically linked to hardware utilization. In theory, if a single cell is associated with one RRU and one BBU, it is straightforward to determine energy consumption. However, in practical scenarios, multiple cells typically share RRUs or BBUs, leading to shared hardware resources across various cells. This shared nature complicates the accurate determination of energy consumption on a per cell.

The primary challenge lies in the shared hardware resources. Multiple cells sharing an RRU or BBU means that a simple per cell energy consumption model is unrealistic. When we attempt to disaggregate this, RRUs, which are more closely tied to cell load, could theoretically be weighted according to cell load. Further complicating matters is the non-linear relationship between cell load and RRU load.

Conversely, BBUs, less influenced by cell load, could be averaged out. Nonetheless, these approaches are merely statistical simplifications and yield only rough estimates of cell-level energy consumption.

- Cell-level EC metrics could not be precisely measured in case that multiple cells share RRUs or BBUs, which is only rough estimation.

Moreover, it was agreed that energy efficiency metrics should use SA5’s definition as a baseline. As we know, SA5 has not defined cell-level energy cost. If companies wish to introduce cell-level energy cost, it would be better to first coordinate with SA5 to determine its feasibility.

- It is not feasible to measure accurate cell-level EC metric in case that multiple cells share the same hardware resources.

Additionally, the diverse configurations of complex hardware, such as base stations with varying numbers of antennas (e.g., some cells using 16 antennas while others use 64), introduce further discrepancies. Although some of hardware components are shared across all cells, determining precise energy consumption for different hardware configuration is challenging. Simplified statistical approaches to grouping cells may provide some estimates, but these remain coarse and lack the precision required for meaningful insights. Comparing these estimates with node-level EC reveals their roughness and inability to achieve finer granularity or more accurate results.

- Simplified estimates do not provide the detailed accuracy needed for effective energy efficiency and could mislead.

- If the cell-level EC metric is calculated by statistical approach by gNB implementation, it is not suggested to introduce the finer granularity of EC.

3	Conclusion

- Node-level energy cost as feedback does not clearly reflect the performance of energy-saving strategies.

- Cell-level EC metrics could not be precisely measured in case that multiple cells share RRUs or BBUs, which is only rough estimation.

- It is not feasible to measure accurate cell-level EC metric in case that multiple cells share the same hardware resources.

- Simplified estimates do not provide the detailed accuracy needed for effective energy efficiency and could mislead.

- If the finer granularity of EC metric is calculated by statistical approach by gNB implementation, it is not suggested to introduce the cell-level energy cost.