3GPP TSG-RAN WG3 Meeting #125	R3-244094

Maastricht, Netherlands, Aug 19th – 23rd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0415 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Uu Relay RLC channel configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, China Telecom, Samsung, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In RAN2 spec TS 38.300, it specifies that Uu Relay RLC channel for Remote UE’s SRB0 relaying is configured after Relay UE’s RRC connection establishment procedure and sending SUI to gNB. However, in RAN3 specs TS 38.401 and TS 38.473, it specifies that it is possible to configure Uu Relay RLC channel for Remote UE’s SRB0/1 relaying during Relay UE’s F1 UE context setup procedure and before sending SUI to gNB. There is gap between RAN2 and RAN3 specs on the understanding of the timing for Uu Relay RLC channel setup for SRB0/1 relaying. Since there is no relay related indication during relay UE RRC connection setup, the gNB-CU does not know the UE is a relay UE or whether the UE is capable of L2 U2N relay UE operation, it is not reasonable to configure the Uu Relay RLC channel during F1 UE context setup procedure. In addition, it is not reasonable for gNB to configure Uu Relay RLC channel before getting knowledge of the capability of the UE (i.e. receiving relayUE-Operation-L2-r17 in UECapabilityInformation from UE). What’s more, the Uu Relay RLC channel will only be used after bearer mapping is configured, and bearer mapping can be configured after relay UE sending SUI including remote UE’s L2 ID to gNB. So there is no benefits to configure the Uu Relay RLC channel in advance. Based on the discussion above, it is suggested that Uu Relay RLC channel is not configured during F1 UE context setup procedure. - Since there is a “NOTE 1” in step 15 in Figure 8.19.1-1 to clarify the SRB1 relaying Uu Relay RLC channel (step 15) may be performed earlier via steps 5~8, the “SRB1” can be removed from steps 5/7/8. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 8.19.1, remove “gNB may configure the U2N Relay UE with Uu Relay RLC channel(s) for relaying of U2N Remote UE’s SRB0/1” in step 3. - Remove the “SRB1” from steps 5/7/8. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because it only impacts the L2 U2N relay operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is gap between RAN2 and RAN3 specs on the understanding of the SRB0/1 relaying Uu Relay RLC channel configuration. SRB0/1 relaying Uu Relay RLC channel may be configured during F1 UE context setup procedure, which is not reasonable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.19.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Start of Change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

- 8.19.1	Remote UE initial access

The signalling flow for Remote UE Initial access is shown in Figure 8.19.1-1.

Figure 8.19.1-1: Remote UE Initial Access procedure

1.	The U2N Remote UE and the U2N Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure.

2.	The U2N Remote UE sends an RRCSetupRequest message to the U2N Relay UE via PC5 Relay RLC Channel.

3.	The U2N Relay UE withholds the received RRC message and sends the SidelinkUEInformationNR message to the gNB-DU. Before that, if the U2N Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should trigger the RRC establishment/resume procedure to enter RRC_CONNECTED state upon reception of the RRC message.

4.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N Relay UE by encapsulating the SidelinkUEInformationNR message to gNB-CU, and gNB-CU allocates the local ID of U2N Remote UE.

5.	The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message of the U2N Relay UE to gNB-DU. Such message may request the establishment of Uu Relay RLC channel(s) for the transmission of U2N Remote UE’s SRB0.

6.	The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message of the U2N Relay UE to gNB-CU.

7.	The gNB-CU sends the DL RRC MESSAGE TRANSFER message of the U2N Relay UE to gNB-DU by encapsulating the RRCReconfiguration message, which contains the local ID allocated to the U2N Remote UE. The RRCReconfiguration message shall also contain the Uu Relay RLC channel(s) configuration if not configured and bearer mapping for relaying of U2N Remote UE’s SRB0.

8.	The gNB-DU sends the RRCReconfiguration message to the U2N Relay UE to configure the local ID of the U2N Remote UE, the Uu Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0.

9.	The U2N Relay UE sends the RRCReconfigurationComplete message to gNB-DU.

10.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N Relay UE by encapsulating the RRCReconfigurationComplete message to gNB-CU.

11.	After receiving the local ID of the U2N Remote UE and the Uu Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0, the U2N Relay UE sends the RRCSetupRequest message of the U2N Remote UE to gNB-DU. The local ID of the U2N Remote UE and RB ID for SRB0 are conveyed in the SRAP header.

12.	The gNB-DU allocates a C-RNTI and a gNB-DU UE F1AP ID for the U2N Remote UE and sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCSetupRequest message of the U2N Remote UE. In addition, the local ID of the U2N Remote UE , the gNB-DU UE F1AP ID of the U2N Relay UE and the sidelink configuration container for at least the PC5 Relay RLC channel configuration for relaying of U2N Remote UE’s SRB1 are included in the INITIAL UL RRC MESSAGE TRANSFER message.

13.	The gNB-CU allocates a gNB-CU UE F1AP ID for the U2N Remote UE and generates a RRCSetup message towards the U2N Remote UE. The RRC message is encapsulated in the DL RRC MESSAGE TRANSFER message, and includes the configurations of PC5 Relay RLC channel and bearer mapping at least for the transmission of U2N Remote UE’s SRB1.

14.	The gNB-DU sends the RRCSetup message to the U2N Remote UE via the U2N Relay UE.

15.	The gNB-CU configures the U2N Relay UE with PC5 Relay RLC channel, Uu Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the U2N Relay UE establishes a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB1 over PC5 and establishes a Uu Relay RLC channel for relaying of U2N Remote UE’s SRB1 towards gNB-DU if not configured yet.

NOTE 1: This step may be performed earlier, e.g., via steps 5~8.

16.	The U2N Remote UE sends the RRCSetupComplete message to the gNB-DU via the U2N Relay UE.

17.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

18.	Upon receiving the RRCSetupComplete message of U2N Remote UE, the gNB-CU sends the INITIAL UE MESSAGE message to the AMF.

19.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

20.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the U2N Remote UE context in the gNB-DU. Such message may request the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs, and may also encapsulate the SecurityModeCommand message.

21.	The gNB-DU sends the SecurityModeCommand message to the U2N Remote UE via U2N Relay UE.

22.	The gNB-DU sends the UE CONTEXT SETUP RESPONSE message of the U2N Remote UE to the gNB-CU, which contains the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs.

23.	The U2N Remote UE responds with the SecurityModeComplete message.

24.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

25.	The gNB-CU generates the RRCReconfiguration message for U2N Remote UE and encapsulates it in the DL RRC MESSAGE TRANSFER message. The RRCReconfiguration message contains the configuration of PC5 Relay RLC channels and bearer mapping for the transmission of U2N Remote UE’s SRB2 and DRBs.

26.	The gNB-DU sends RRCReconfiguration message to the U2N Remote UE via the U2N Relay UE.

27.	The U2N Remote UE sends RRCReconfigurationComplete message to the gNB-DU via the U2N Relay UE.

28.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

29.	The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

30.	The gNB-CU configures additional Uu Relay RLC channels between the gNB-DU and the U2N Relay UE, and additional PC5 Relay RLC channels for the U2N Relay UE for relaying of U2N Remote UE’s DRBs and SRBs. Also, such step may configure the bearer mapping between U2N Remote UE’s DRB/SRB and PC5/Uu Relay RLC channel at the U2N Relay UE.

NOTE 2: This step may be performed earlier.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<End of Change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>