3GPP TSG-RAN WG3 Meeting #125	R3-244102

Maastricht, NL, 19 - 23 Aug, 2024

Title:	(TP for TR 38.769) RAN architecture aspects

Agenda Item:	16.2

Source:	Huawei

Document for:	other

# 1	Introduction

In [1], about the RAN architecture, the Logical System Architecture for AIoT was captured, in this contribution:

- for topology 1, we first discuss the Deployment scenario for Topology 1 and the related protocol stack

- for topology 2, we discuss the deployment scenario and the potential solutions of Topology2

- try to solve some Editor’s Notes and FFSs in the endorsed pCR

The corresponding TP for TR 38.769 is also provided in section 6.

# 2	Discussion for Topology 1

## 2.1	Deployment scenario for Topology 1

In [1], the Logical System Architecture for AIoT was captured as below:

Figure 6.3-1: Logical System Architecture for AIoT.

Note that it was proposed to capture a Deployment scenario for topology 2 based on the Logical System Architecture in [2], following the same logic, it should be helpful to add the Deployment scenario for topology1 based on the logical system architecture, and it could be illustrated as below:

Figure 1 Deployment scenario for topology 1 based on the Logical System Architecture in Figure 6.3-1.

Figure 1 depicts a deployment scenario for topology 1, based on Figture 6.3-1, where AIoT RAS functions are deployed within an AIoT RAN node.

The entity definition of AIoT RAN node could be:

AIoT RAN node: a RAN node hosts the Common reader function and the AIoT RAN node function.

Proposal 1-1: Adopt the Deployment scenario for topology 1 based on the endorsed Logical System Architecture.

## 2.2	Protocol Stack for Topology 1

Based on the previous discussion, there is a common understanding to support the data transport over XXAP, and it is regarded by majority companies as control plane transmission.

In [1], XX interface is defined as the interface between the AIoT RAS and the AIoT CN on which certain AIoT specific functions are performed. And it is FFS whether this interface represents a new logical interface or is equal to NG.

As discussed in last meeting, in case the AIoT RAN node connects with AMF, the NG interface has to be used, even in case the AIoT RAN node directly connects with a new AIoT CN node, it is also possible to reuse NG interface via introducing a new set of procedures over NGAP. Comparing with introducing a brand-new interface (and create lots of new specificaitons), there are more advantages to use NG interface between AIoT RAN node and AIoT CN, to be more precise, it should be a NG-C interface.

Proposal 1-2: Assume that the XX interface in Topology 1 is a NG-C interface.

  - The Figure 2 shows the Protocol stack for Topology 1:

- Figure 2. Protocol Stack for Topology 1

Note that whether NAS is applicable to AIoT devices is out of RAN3 scope, “Upper layer” is used in the Figure 2 to represents APP/NAS or APP layer.

Proposal 1-3: Capture the Protocol Stack for Topology 1 into the TR.

# 3	Discussion for Topology 2

## 2.1	Deployment scenario for Topology 2

As proposed in [2], based on the endorsed Logical System Architecture, the deployment scenario for topology 2 was captured as follows:

Figure 6.4-2: Deployment scenario for topology 2 based on the Logical System Architecture in Figure 6.4-1.

Several companies have concern abou the above Figure, after further consideration, we would like to illustrate Topology 2 as follows:

Figure 1 Deployment scenario for topology 2

Figure 1 illustrates that the Comman reader function is located at UE reader, and the AIoT RAN node function is located at the AIoT-enabled gNB, it represents how to support topology 2 under the endorsed Logical System Architecture, it is preferred to also capture it into the TR.

Proposal 2-1: Adopt the Deployment scenario for topology 2 based on the endorsed Logical System Architecture.

## 2.2	Solutions for Topology 2

Based on the submitted contributions in previous RAN3 meetings and the latest version of SA2 TR [3], there are multiple options to support topology 2, which can be classified as follows:

Solution1: UE Reader’s RRC based solution

In this solution, upon receiving AIOT service related messages from AIoT CN via XXAP, the gNB forwards the corresponding information towards the UE Reader via NR Uu RRC, and upon receiving AIOT service related messages from UE Reader via NR Uu RRC, the gNB further transmits the response/reports towards AIoT CN via XXAP.

Figure 1: UE Reader’s RRC based solution of Topology 2

Solution 2: UE Reader’s NAS based solution

In this solution, the AIoT service related messages between the AIoT CN and the UE Reader are carried via UE Reader’s DL/UL NAS packets, the gNB handles the UE Reader’s NAS packets as legacy, i.e., using DL NAS Transport and UL NAS Transport over NGAP.

Figure 2: UE Reader’s NAS based solution of Topology 2

Solution 3: UE Reader’s PDU Session based solution

In this solution, the AIoT service related messages between the AIoT CN and the UE Reader are carried via UE Reader’s PDU Session, the gNB handles the UE Reader’s user plane data as legacy, i.e., over NG-U GTP-U tunnels.

Figure 3: UE Reader’s PDU Session based solution of Topology 2

Proposal 2-2: Capture these three solutions of Topology 2 into the TR.

## 2.3	UE Reader Authorization for Topology 2

Regardless of the solutions mentioned in section 2.2, the UE Reader Authorization is an important aspect to be considered for Topology 2. The UE should be authorized to perform AIoT service, i.e., only if the UE is allowed to act as a UE reader, the UE can communicate with AIoT devices. To support this, the CN needs to perform the authorization of whether the UE can act as a UE Reader, and informs the NG-RAN node the authorization status.

- Proposal 2-3: The CN needs to perform the authorization of whether the UE can act as a UE Reader, and informs the gNB about the authorization status.

# 2.3	Editor’s Notes and FFSs

In section 6.3 of [1], there are several Editor’s Notes and FFSs for Topology 1, based on the discussion, some of them can be solved.

AIoT device: equipment with characteristics outlined e.g. in TS 22.369 [x] and TR 38.848 [2].

Editor’s Note 3: Further details FFS, if any.

The Editor’s Note 3 has nothing to do with RAN3, and other specification have already been referred to for the definition of AIoT device, therefore it should be removed.

AIoT radio: radio interface between AIoT device and Common reader function.

Editor’s Note 5: Further details on AIoT radio to be discussed by RAN1 and RAN2.

Common reader function: a function that communicates with the AIoT device by means of AIoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

The Editor’s Note 5 and 8 also have nothing to do with RAN3, and it is obvious that the AIoT radio are in the scope of RAN1 and RAN2, these two Editor’s notes should also be removed.

Based on the discussion in section 3, it could be understood that the logical system architecture is also applicable to Topology 2, therefore, the Editor’s Note 9 should be removed as well.

Proposal 3: Remove the Editor’s Note 3, 5, 8 and 9 in section 6.3.

# 4	Proposals

In this contribution, we further discussed the RAN Architecture aspects about Topology 1, and get the following proposals:

For Topology 1:

Proposal 1-1: Adopt the Deployment scenario for topology 1 based on the endorsed Logical System Architecture.

Proposal 1-2: Assume that the XX interface in Topology 1 is a NG-C interface.

Proposal 1-3: Capture the Protocol Stack for Topology 1 into the TR.

For Topology 2:

Proposal 2-1: Adopt the Deployment scenario for topology 2 based on the endorsed Logical System Architecture.

Proposal 2-2: Capture these three solutions of Topology 2 into the TR.

- Proposal 2-3: The CN needs to perform the authorization of whether the UE can act as a UE Reader, and informs the gNB about the authorization status.

Editor’s Notes in baseline pCR:

Proposal 3: Remove the Editor’s Note 3, 5, 8 and 9 in section 6.3.

It is also proposed to capture the corresponding TP provided in section 5 into the TR 38.769.

# 5	Reference

- R3-243972 (BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR, Huawei, CMCC

- R3-243807 [TP for TR 38.769] CB:#AIoT1_Architecture, Ericsson (Moderator), Huawei

# 6	Text Proposal for TR 38.769

------------Start of the Change-------------

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AIoT	Ambient IoT

DO-A	Device-originated autonomous

DO-DTT	Device-originated by device-terminated trigger

DT	Device-terminated

FR	Frequency Range

IoT	Internet of Things

LPWA	Low-power, wide-area

LTE-MTC	Long Term Evolution – Machine Type Communication

NB-IoT	Narrowband IoT

RFID	Radio frequency identification

SFO	Sampling frequency offset

------------Start of the Next Change-------------

# 6	Solutions for ambient IoT

## 6.1	Physical layer

## 6.2	Protocol stack and signalling procedures

Editor’s note: Corresponds to the RAN2 objective in the SID.

## 6.3	RAN architecture aspects

Editor’s note 1: Corresponds to the second RAN3 objective in the SID, to identify RAN architecture aspects, including whether support for split architecture is necessary.

This chapter attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

Editor’s Note 2: What functionalities are hosted by the 5GS for AIoT is TBD.

This chapter also attempts to identify a functional split between RAN and CN.

Figure 6.3-1 depicts the logical system architecture for AIoT.

It consists of the following architectural elements:

AIoT device: equipment with characteristics outlined e.g. in TS 22.369 [x] and TR 38.848 [2].

AIoT RAS (AIoT Radio Access System): hosts certain functions for AIoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAS and the respective functional split to be decided by RAN2, RAN3 and SA2.

AIoT radio: radio interface between AIoT device and Common reader function.

AIoT CN: hosts certain functions for AIoT as of the functional split between RAN and CN

Editor’s Note 6: Further details regarding AIoT functions hosted in the AIoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface: interface between the AIoT RAS and the AIoT CN on which certain AIoT specific functions are performed. In topology 1, the XX interface is a NG-C interface.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAS, or one instance for NG alone.

Common reader function: a function that communicates with the AIoT device by means of AIoT radio.

AIoT RAN node function: a function residing in AIoT RAS. It contains e.g. the control of the AIoT radio resources used towards the AIoT device.

Note that “control of AIoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

Figure 6.3-1: Logical System Architecture for AIoT.

### 6.3.1 Support of Topology 1

Figure 6.3.1-1 depicts a deployment scenario for topology 1, based on Figture 6.3-1, where AIoT RAS functions are deployed within an AIoT RAN node.

AIoT RAN node: a RAN node hosts the Common reader function and the AIoT RAN node function.

Figure 6.3.1-1 Deployment scenario for topology 1 based on the Logical System Architecture in Figure 6.3-1.

  - Figure 6.3.1-2 shows the Protocol stack for Topology 1:

- Figure 6.3.1-2. Protocol Stack for Topology 1

The XX control plane interface (XX-C) is defined between the AIoT RAN node and the AIoT CN.

The control plane protocol stack of the XX interface is supported by XXAP over SCTP/IP/L2/L1.

### 6.3.2 Support of Topology 2

Figure 6.3.2-1 depicts a deployment scenario for topology 2, based on Figture 6.3-1, AIoT RAS functions are deployed within an AIoT-enabled gNB and an AIoT enabled UE, i.e., the Command reader function located at AIoT-enabled UE, and the AIoT RAN node function located at the AIoT-enabled gNB.

The definitions of the 2 additional entities are given below:

AIoT-enabled gNB: a gNB able to communicate with the AIoT enabled UE.

UE Reader: a UE able to communicate with the AIoT Device via the Common reader function

Figure 6.3.2-1 Deployment scenario for topology 2 based on the Logical System Architecture in Figure 6.3-1.

#### 6.3.2.1 Solutions for Topology 2

This section There are multiple potential solutions to support Topology 2.

In all Topology 2 solutions, the CN needs to perform the authorization of whether the UE can act as a UE Reader, and informs the gNB about the authorization status.

Solution1: UE Reader’s RRC based solution

In this solution, upon receiving AIOT service related messages from AIoT CN via XXAP, the gNB forwards the corresponding information towards the UE Reader via NR Uu RRC, and upon receiving AIOT service related messages from UE Reader via NR Uu RRC, the gNB further transmits the response/reports towards AIoT CN via XXAP.

Figure 6.3.2.1-1: UE Reader’s RRC based solution of Topology 2

Solution 2: UE Reader’s NAS based solution

In this solution, the AIoT service related messages between the AIoT CN and the UE Reader are carried via UE Reader’s DL/UL NAS packets, the gNB handles the UE Reader’s NAS packets as legacy, i.e., using DL NAS Transport and UL NAS Transport over NGAP.

Figure 6.3.2.1-2: UE Reader’s NAS based solution of Topology 2

Solution 3: UE Reader’s PDU Session based solution

In this solution, the AIoT service related messages between the AIoT CN and the UE Reader are carried via UE Reader’s PDU Session, the gNB handles the UE Reader’s user plane data as legacy, i.e., over NG-U GTP-U tunnels.

Figure 6.3.2.1-3: UE Reader’s PDU Session based solution of Topology 2

------------End of the Changes-------------