3GPP TSG-RAN WG3 Meeting #125	R3-244103

Maastricht, NL, 19 - 23 Aug, 2024

Title:	(TP for TR 38.769) Ambient IoT device locating

Agenda Item:	16.4

Source:	Huawei

Document for:	other

# 1	Introduction

In last RAN3 meeting, the use cases (purposes) on Ambient IoT device locating were captured [1], with two editor’s notes to list FFSs.

In this contribution, we provide our further analyses on Ambient IoT device locating, try to solve the FFSs, and provides the corresponding TP to the TR38.769 in section 5.

# 2	Discussion

## 2.1	How to report Reader ID?

Locating an Ambient IoT device at “reader ID granularity” was agreed in last meeting, but signalling details on how to provide the location information of the AIoT device to the AIoT CN needs further study.

Editor’s Note 2: Signalling details on how to provide the location information of the AIoT device to the AIoT CN needs further study.

In NGAP specification, in almost all UE associated NGAP messages sent from RAN node to AMF, like INITIAL UE MESSAGE message, UPLINK NAS TRANSPORT message, the User Location Information IE is also included to provide user location report.

Similarly, in Topology 1, the location report can be also sent from RAN node to CN via XXAP in the message sent from the RAN node to the CN during Inventory/Command procedures, e.g., together with the received device ID(s)/UL data. Considering the signlling call flow to support pure inventory, and the basic solution of command after inventory, it is straight forward to include the location information within the Inventory Report message, to be reported together with device ID received from the device.

Proposal 1: In topology 1, the AIoT RAN node includes the Ambient IoT device location information (RAN Reader ID) within the Inventory Report message.

In topology 2, in case of UE Reader’s RRC based solution, similar with topology 1, the gNB also needs to include the Ambient IoT device location information (Reader ID) within the Inventory Report message.

In Topology 2, in case of UE Reader’s NAS based solution, and UE Reader’s PDU session based solution, there is no need for the gNB to provide any location information to the AIoT CN, as the AIoT procedures terminated between the UE Reader and the AIoT CN directly, CN knows which UE reader the Ambient IoT device accessing from, and CN is able to get the UE reader’s location by existing mechanisms.

Proposal 2: In topology 2, in case of UE Reader’s RRC based solution, the gNB includes the Ambient IoT device location information (UE Reader ID) within the Inventory Report message.

Proposal 3: In topology 2, in case of UE Reader’s NAS based solution and UE Reader’s PDU session based solution, the AIoT CN knows which UE reader the device comes from by default.

## 2.2	How to know the Reader location?

Another Editor’s note in the pCR includes two FFSs, the first one is how to know the “reader” location.

Editor’s Note 1: How to know the “reader” location is FFS. Whether to use more than one “readers” within one AIoT RAS for location purposes is FFS.

In Topology 1, an AIoT RAN node can be regarded as multiple RAN Readers, considering that the RAN reader does not move, there is no need to update the reader location to the AIoT CN. Therefore, the AIoT CN can get the RAN reader location based on e.g. OAM configuration, or AIoT XX Setup procedure.

Proposal 4: In topology 1, the AIoT CN can get the RAN reader’s location based on e.g. OAM configuration.

In Topology 2, as we mentioned above, the AIoT CN is able to get the UE reader’s location by existing mechanisms, there is no need to introduce new method to enable this in this study.

Proposal 5: In topology 2, the AIoT CN is able to get the UE reader’s location by existing mechanisms.

There is also another FFS about whether to use more than one “readers” within one AIoT RAS for location purposes is FFS. From our view there is no need to discuss this in the Study, as we need to figure our the basic solutions first.

# 3	Proposals

In this contribution, we discuss Ambient IoT device locating, and get the following proposals:

For Topology 1:

Proposal 1: In topology 1, the AIoT RAN node includes the Ambient IoT device location information (RAN Reader ID) within the Inventory Report message.

Proposal 4: In topology 1, the AIoT CN can get the RAN reader’s location based on e.g. OAM configuration.

For Topology 2:

Proposal 2: In topology 2, in case of UE Reader’s RRC based solution, the gNB includes the Ambient IoT device location information (UE Reader ID) within the Inventory Report message.

Proposal 3: In topology 2, in case of UE Reader’s NAS based solution and UE Reader’s PDU session based solution, the AIoT CN knows which UE reader the device comes from by default.

Proposal 5: In topology 2, the AIoT CN is able to get the UE reader’s location by existing mechanisms.

It is also proposed to capture the corresponding TP provided in section 5 into the TR 38.769.

# 4	Reference

- R3-243972 (BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR, Huawei, CMCC

# 5	Text Proposal for TR 38.769

------------Start of the Change-------------

## 6.8	Locating ambient IoT devices

Editor’s note 1: Proximity determination may be in a 6.8.x sub-clause, or another arrangement, depending on how the study proceeds.

Editor’s note 2: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network.

### 6.8.1	General

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by AIoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

In Topology 1:

The AIoT CN gets the RAN reader’s location based on e.g. OAM configuration.

The AIoT RAN node includes the Ambient IoT device location information (Reader ID) within the Inventory Report message.

In Topology 2:

The AIoT CN gets the UE reader’s location by existing mechanisms.

In case of UE Reader’s RRC based solution, the gNB includes the Ambient IoT device location information (UE Reader ID) within the Inventory Report message.

In case of UE Reader’s NAS based solution and UE Reader’s PDU session based solution, the AIoT CN knows which UE reader the device comes from by default.

-----------End of the Change-------------