3GPP TSG-RAN WG3 Meeting #125	R3-244104

Maastricht, NL, 19 -23 Aug, 2024

Title:	(TP for TR 38.769) Signalling and Procedures for Topology1

Agenda Item:	16.3

Source:	Huawei

Document for:	other

# 1	Introduction

In RAN3#124 meeting, regarding to Inventory, RAN3 agreed and captured the following into pCR to TR38.769:

Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices. The Inventory Request from the AIoT CN to the AIoT RAS, may include the following: (1) AIoT Device Identification (to find a single device, a group of devices, or all devices) Note 1: The definition of this identification is out of RAN3 scope. Editor’s Note 1: It is FFS whether AIoT RAS needs to interpret/store/process it. (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered) Multiple individual AIoT Device IDs (one ID per device) can be provided to the AIoT CN via a single Inventory Report. Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

And the following FFSs were list as to be continued:

In the Inventory Request from the AIoT CN towards the AIoT RAS, it is FFS whether the following information may be included:

Inventory Session ID (e.g., to allow multiple Inventory procedure instances to run in parallel): the purpose, scope and handling of this information in AIoT RAS are FFS.

Estimated Number of expected responses from devices, or Minimum number of required responses from devices.

FFS the usage of these information.

Periodicity: to enable periodic inventory at AIoT RAS triggered by a single Inventory Request from the AIoT CN

In Topology 2, it is FFS which entity (e.g., AIoT CN and/or AIoT enabled gNB) performs the UE reader selection.

In topology1, it is FFS whether in response to a single Inventory Request, AIoT RAS provides only a single Inventory Report message or there can be also multiple Inventory Report messages provided.

Regarding to Command, RAN3 agreed and captured the following into pCR to TR38.769:

Command can be sent by the AIoT CN for a single device. Editor’s Note: it is FFS for command on a group of devices, or all devices. Editor note: it is FFS whether AIoT RAS can remain agnostic of the type of request from the AIoT CN (need to differentiate command and inventory)

And the following FFSs were list as to be continued:

FFS for command on a group of devices, or all devices.

In this paper, we futher discuss the support of AIoT signalling and procedures for Topology1, and provide the corresponding TP.

# 2	Discussion

## 2.1	Information carried in Inventory Request from AIoT CN

Last meeting, there were two items captured in the pCR, i.e. AIoT Device Identification, and Scope of inventory request.

There is an Editor’s note about the AIoT Device Identification:

Editor’s Note 1: It is FFS whether AIoT RAS needs to interpret/store/process it.

Based on currently discussion in SA2, RAN2 and RAN3, there is no need for the AIoT RAS to interpret/process it, but there is a need for the AIoT RAS to store it, in order to allow the paging repetition over the radio.

Proposal 1: There is no need for the AIoT RAS to interpret/process the AIoT Device Identification, but the AIoT RAS needs to store it in order to allow the paging repetition over the AIoT radio.

Besides of these, in our view, the following information are also needed/beneficial to be included in the Inventory Request:

- Inventory Session ID, to distinguish different requests from AIoT CN(s).

- Assistance Information

  - Estimated Number of devices, e.g., to enable CFRA, to assists resource allocation.

Proposal 2: The Inventory Request may also include the 	Inventory Session ID and Assistance Information (e.g., the Estimated Number of devices).

## 2.2 Inventory in Topology 1

Inventory Report

It was agreed last meeting that multiple individual AIoT Device IDs (one ID per device) can be provided to the AIoT CN via a single Inventory Report. But it is still FFS whether AIoT RAS provides only a single Inventory Report message or there can be also multiple Inventory Report messages provided.

One Inventory service may involve multiple devices, and the number of devices maybe large, if the AIoT RAS has to report the devices IDs from all involved devices in a single inventory response message, the time duration to get the full report will be relatively long. Therefore, we would like to allow a more flexible report method, i.e., using a class 2 procedure, to allow the AIoT RAS to report the received devices via either a single Inventory Report message or via multiple Inventory Report messages.

Proposal 3: Introduce a Class 2 Inventory Report procedure, to allow the AIoT RAS to report the received device IDs via either a single Inventory Report message or via multiple Inventory Report messages, in response to a single Inventory Request.

Inventory Request

When the AIoT RAS is performing an inventory service, the air interface resource is occupied for a period of time. If the AIoT RAS receives another inventory request from CN, the AIoT RAS may not perform or delay the new request. If no feedback to the newly received inventory request, the CN is not aware of whether the request is received or not. Thus, to support a more reliable Inventory Request procedure, it is better for RAN3 to introduce class 1 Inventory Request procedure to allow AIoT RAS providing response message before the actual inventory report.

Proposal 4: Introduce a class 1 Inventory Request procedure with request/response/failure messages.

Inventory Complete

When there is no further device performing random access and no further UL Data (Device ID) to be reported, the AIoT RAS should send Inventory Complete Request message to the AIoT CN. Or the AIoT CN may also trigger Inventory Complete procedure by sending Inventory Complete Request message to the AIoT RAS, when the AIoT CN would like to terminate the ongoing Inventory procedure. After the Inventory Complete procedure, any stored context of the completed inventory service at the AIoT RAS should be released.

Proposal 5:  Introduce an Inventory Complete procedure to indicate the compeletion of the inventory service.

Based on above proposals, the overall call flow about Inventory in topology 1 can be illustrated as follows:

Figure 1. Overall Inventory call flow in Topology 1

Proposal 6: Capture the overall inventory call flow in Topology 1 into the TR.

## 2.3 Command in Topology 1

  - Command procedure

  - In the RAN2#126 meeting, there are some agreements about how to enable command operations, as follows:

Agreements 1 As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS 2 As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

- Based on above RAN2 agreements, we can see that the “inventory and command” case was agreed as the baseline. In this baseline, network first performs “paging” over AIoT radio, the device sends its device ID to the network, and then the network sends command data to the device.

- In our view, the network should send DL data to the single device after the security checking (authentication/validation) at the AIoT CN, therefore, the following command procedure should be adopted as baseline.

Figure 2. Command transmission after Inventory procedure in Topology 1

- To support this baseline solution “command after inventory”, from RAN3 perspective, similar to NGAP DL NAS Transport and UL NAS Transport procedures, we need to introduce DL Command Transport procedure and UL Command Transport procedure to transmit the DL Command and UL Command Response.

  - Proposal 7: Introduce XXAP DL Command Transport procedure and UL Command Transport procedure to transmit the DL Command and UL Command response over XX interface.

  - Proposal 8. Support Command Transmission after Inventory procedure as the basic solution for command operation.

- On the other hand, as a potential optimization method for fast read command, CN provides Device ID and Read Command within the Inventory Request. The following fast Command (e.g., fast read command) procedure may be considered:

Figure 2. Fast read command during Inventory procedure in Topology 1

In Figure 2, the AIoT CN transmits the DL command in step 1, and then the reader first page the device and then transmit DL command towards the device. Note that the steps over AIoT radio maybe further optimized, i.e., read command transmitted in step 3, skip step 5, and combine step 4 and 6, but its feasibility is up to SA3, SA2 and RAN2 decision.

Proposal 9. The fast read command during Inventory procedure is subject to SA3/SA2/RAN2 discussion.

- 2.4 Context management over CN-RAN interface

  - Device association establish

- As shown in the Figure 2, the AIoT RAS can allocate RAN Device XXAP ID for the device and provide it to the AIoT CN in the Inventory Report, then the AIoT CN allocates the CN Device XXAP ID to establish the XXAP device association. The XXAP device association could be used for e.g., parallel transactions for multiple devices.

- Proposal 10. To support command, it is needed to establish the RAN/CN Device XXAP ID to establish XXAP device association.

  - Session context establishment

To allow parallel transactions for multiple inventory procedure instances, the AIoT CN(s) may need to allocate Inventory Session ID for the Inventory procedure instance (one ID per Inventory procedure instance) and provide it to the AIoT RAS in Inventory Request, then the AIoT RAS can report AIoT Device ID(s) together with corresponding Inventory Session ID in Inventory Report to establish session association.

- Proposal 11. For Inventory service, session context is established at the AIoT RAS during Inventory Request procedure, the session context includes at least the session ID.

# 3	Conclusion

In this contribution, we discussed the potential impacts on signaling and procedures over CN-RAN interface for Ambient IoT to support Inventory, and got the following proposals:

- Proposal 1: There is no need for the AIoT RAS to interpret/process the AIoT Device Identification, but the AIoT RAS needs to store it in order to allow the paging repetition over the AIoT radio.

- Proposal 2: The Inventory Request may also include the 	Inventory Session ID and Assistance Information (e.g., the Estimated Number of devices).

- Proposal 3: Introduce a Class 2 Inventory Report procedure, to allow the AIoT RAS to report the received device IDs via either a single Inventory Report message or via multiple Inventory Report messages, in response to a single Inventory Request.

- Proposal 4: Introduce a class 1 Inventory Request procedure with request/response/failure messages.

- Proposal 5:  Introduce an Inventory Complete procedure to indicate the compeletion of the inventory service.

- Proposal 6: Capture the overall inventory call flow in Topology 1 into the TR.

- Proposal 7: Introduce XXAP DL Command Transport procedure and UL Command Transport procedure to transmit the DL Command and UL Command response over XX interface.

- Proposal 8. Support Command Transmission after Inventory procedure as the basic solution for command operation.

- Proposal 9. The fast read command during Inventory procedure is subject to SA3/SA2/RAN2 discussion.

- Proposal 10. To support command, it is needed to establish the RAN/CN Device XXAP ID to establish XXAP device association.

- Proposal 11. For Inventory service, session context is established at the AIoT RAS during Inventory Request procedure, the session context includes at least the session ID.

The corresponding TP for TR 38.769 is provided in section 5.

# 4. Reference

- RP-234058, "New SID: Study on solutions for Ambient IoT (Internet of Things) in NR", RAN#102

- 3GPP TR 23.700-13 V0.2.0, Study on Architecture support of Ambient power-enabled Internet of Things

- 3GPP TR 38.848 V0.2.0, Study on Ambient IoT (Internet of Things) in RAN

- R3-5G240378.1 RAN architecture aspects for Topo2

# 5. TP for TR 38.769 (based on R3-243972)

----------------Start of the First Change----------------

## 6.4	Impacts on CN-RAN interface

Editor’s note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

### 6.4.1	Information exchanged between AIoT CN and AIoT RAS

#### 6.4.1.1	Inventory

Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the AIoT CN to the AIoT RAS, may include the following:

(1)	AIoT Device Identification (to find a single device, a group of devices, or all devices)

Note 1:	The definition of this identification is out of RAN3 scope.

Note 2: There is no need for the AIoT RAS to interpret/process the AIoT Device Identification received in Inventory Request from AIoT CN, but the AIoT RAS may need to store it to enable paging repetition over the AIoT radio.

(2)	Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

(3) Inventory Session ID (to distinguish different requests from AIoT CN(s).)

(4) Assistance Information:

-	Estimated Number of devices (e.g. to enable CFRA)

Multiple individual AIoT Device IDs (one ID per device) can be provided to the AIoT CN via a single Inventory Report.

#### 6.4.1.2	Command

Command can be sent by the AIoT CN for a single device.

Editor’s Note 1: it is FFS for command on a group of devices, or all devices.

Editor’s Note 2: it is FFS whether AIoT RAS can remain agnostic of the type of request from the AIoT CN (need to differentiate command and inventory)

### 6.4.2	Overall procedures for Topology 1

#### 6.4.2.1	Inventory

The XXAP Inventory Request procedure is triggered by the AIoT-aware CN node, to request the AIoT RAN node to reach one or more AIoT devices. The request may also include the AIoT Device Identification, the Scope of inventory request, the Inventory Session ID and the Assistance Information (e.g., the Estimated Number of devices, etc).

The XXAP Inventory Report procedure is triggered by the AIoT RAN node, to report the received Device ID(s) from the AIoT Device(s) to the AIoT-aware CN node.

The overall inventory procedure involves one Inventory Request procedure, one or multiple Inventory Report procedure(s), and one Inventory Complete procedure, as illustrated in Figure 6.4.2.1-1.

Figure 6.4.2.1-1: Inventory call flow in Topology 1

1.	AIoT-aware CN node triggers Inventory Request with AIoT Device Identifier towards the relavent AIoT RAN node.

2.	The AIoT RAN node sends Inventory Response to the CN.

3.	The AIoT RAN node triggers Paging over radio.

NOTE: step 2 and step 3 may happen in parallel.

4.	The AIoT device1 performs random access and provides UL Data (Device ID1) to the AIoT RAN node.

5.	The AIoT RAN node sends the Inventory Report towards the CN with the UL Data (Device ID1) received in step4.

6.	The AIoT device2 performs random access and provides UL Data (Device ID2) to the AIoT RAN node.

7.	The AIoT device3 performs random access and provides UL Data (Device ID3) to the AIoT RAN node.

8.	The AIoT RAN node sends the Inventory Report towards the AIoT-aware CN node with the UL Data (Device ID2, 3) received in step 6 and 7.

9.	The Inventory Complete procedure is triggered to complete the Inventory service.

#### 6.4.2.2	Command

The Command operation enables the network to send instruction to an Ambient IoT Device.

Figure 6.4.2.2-1 illustrates overall callflow for command operation, i.e., Command Transmission is performed after Inventory procedure.

Figure 6.4.2.2-1. Command transmission after Inventory procedure for Topology 1

1.	AIoT-aware CN node triggers Inventory Request with AIoT Device ID towards the relavent AIoT RAN node, and indicates that there will be command transmission for the device.

2.	The AIoT RAN node sends Inventory Response to the CN.

3.	The AIoT RAN node triggers Paging over radio.

NOTE: step 2 and step 3 may happen in parallel.

4.	The AIoT device1 performs random access and provides UL Data (Device ID1) to the AIoT RAN node.

5.	The AIoT RAN node sends the Inventory Report towards the CN with the UL Data (Device ID) received in step4. Based on the command indication received in step1, the AIoT RAN node allocates RAN Device XXAP ID for the device, and include it in the Inventory Report.

6.	The AIoT-aware CN node triggers DL Command Transport towards the AIoT RAN node. The AIoT-aware CN node allocates CN Device XXAP ID for the device, and provide the RAN/CN Device XXAP ID pair and the DL Data towards the AIoT RAN node, a command type may also be provided.

7.	The AIoT RAN node transmits the DL Data towards the AIoT Device.

8.	The AIoT Device transmits the UL Data towards the AIoT RAN node.

9.	The AIoT RAN node triggers UL Command Transport towards the AIoT-aware CN node, including the RAN/CN Device XXAP ID pair and the UL Data.

----------------End of the Changes----------------