3GPP TSG-RAN WG3 Meeting #125	R3-244105

Maastricht, NL, 19 - 23 Aug, 2024

Title:	(TP for TR 38.769) Signalling and Procedures for Topology2

Agenda Item:	16.3

Source:	Huawei

Document for:	other

# 1	Introduction

In RAN3#124 meeting, regarding to Inventory, RAN3 agreed and captured the following into pCR to TR38.769:

Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices. The Inventory Request from the AIoT CN to the AIoT RAS, may include the following: (1) AIoT Device Identification (to find a single device, a group of devices, or all devices) Note 1: The definition of this identification is out of RAN3 scope. Editor’s Note 1: It is FFS whether AIoT RAS needs to interpret/store/process it. (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered) Multiple individual AIoT Device IDs (one ID per device) can be provided to the AIoT CN via a single Inventory Report. Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

And the following FFSs were list as to be continued:

In the Inventory Request from the AIoT CN towards the AIoT RAS, it is FFS whether the following information may be included:

Inventory Session ID (e.g., to allow multiple Inventory procedure instances to run in parallel): the purpose, scope and handling of this information in AIoT RAS are FFS.

Estimated Number of expected responses from devices, or Minimum number of required responses from devices.

FFS the usage of these information.

Periodicity: to enable periodic inventory at AIoT RAS triggered by a single Inventory Request from the AIoT CN

In Topology 2, it is FFS which entity (e.g., AIoT CN and/or AIoT enabled gNB) performs the UE reader selection.

In topology1, it is FFS whether in response to a single Inventory Request, AIoT RAS provides only a single Inventory Report message or there can be also multiple Inventory Report messages provided.

Regarding to Command, RAN3 agreed and captured the following into pCR to TR38.769:

Command can be sent by the AIoT CN for a single device. Editor’s Note: it is FFS for command on a group of devices, or all devices. Editor note: it is FFS whether AIoT RAS can remain agnostic of the type of request from the AIoT CN (need to differentiate command and inventory)

And the following FFSs were list as to be continued:

FFS for command on a group of devices, or all devices.

In this paper, we futher discuss the support of AIoT signalling and procedures for Topology2, and provide the corresponding TP.

# 2	Discussion

## 2.1 Inventory in Topology 2

In case of Topology 2, regarding to the solutions for AIoT service related message transmission, there are the following three different solutions can be found in SA2 TR 23.700-13 and [our RAN3 arch discussion paper in R3-5G240378.1]:

  - Solution 1: UE Reader’s RRC based solution

  - Solution 2: UE Reader’s NAS based solution

  - Solution 3: UE Reader’s PDU session based solution

- In Solution 1, NR Uu RRC Transfer is used to forward the AIoT service related messages between the gNB and the UE Reader. The signallings over AIoT radio interface and XX interface are almost the same as Topology 1, the delta part for Topology 2 is that there is one more step added i.e., NR Uu RRC message between gNB and UE Reader. Therefore, the inventory call flow can be illustrated as:

Figure 1: Inventory call flow with UE Reader’s RRC based solution in Topology 2

- In Solutions 2 and 3, since AIoT service related message between the AIoT CN and the UE Reader are carried via UE Reader’s NAS/PDU Session, which are transparently forwarded by the gNB, therefore, the inventory call flow can be illustrated as:

Figure 2: Inventory call flow with UE Reader’s NAS/PDU Session based solutions in Topology 2

- Proposal 1: Capture the overall inventory call flows in Topology 2 into the TR.

## 2.2 Command in Topology 2

According to agreements in RAN2#126 meeting, “command after inventory” has been supported as be baseline solution to enable command operation. To support the baseline Command procedure UE Reader’s RRC based solution (i.e., Solution 1) in Topology 2, the signalling over AIoT radio interface and XX interface are almost the same as the ones in the case of Topology 1. Therefore, the baseline command call flow can be illustrated as:

- Figure 3. Command call flow with UE Reader’s RRC based solution in Topology 2

On the other hand, for the baseline Command procedure with UE Reader’s NAS/PDU Session based solutions (i.e., Solution 2 and 3) in Topology 2, since AIoT service related message between the AIoT CN and the UE Reader are transparently forwarded by the gNB, therefore, the baseline command call flow can be illustrated as:

- Figure 4. Command call flow with UE Reader’s NAS/PDU Session based solutions in Topology 2

- Proposal 2: Capture the Baseline Command call flows in Topology 2 into the TR.

## 2.3	UE reader selection

In last RAN3 meeting, the following FFS were listed:

In Topology 2, it is FFS which entity (e.g., AIoT CN and/or AIoT enabled gNB) performs the UE reader selection.

In UE Reader’s NAS/PDU session based solution 2 and solution 3, considering that the AIoT procedures are terminated between the UE Reader and the AIoT CN, the AIoT Inventory/Command procedures are transprate to the gNB, the UE Reader selection has to be performed by the AIoT CN.

- Proposal 3: In Topology 2 UE Reader’s NAS/PDU session based solutions, the UE reader selection is performed by AIoT CN.

In case of UE Reader’s RRC based solution, as the gNB is aware of the ongoing AIoT services, the UE Reader selection could be done by either the gNB or the CN or both.

- Proposal 4: In Topology 2 UE Reader’s RRC based solutions, the UE reader selection may be performed by AIoT CN or the gNB or both.

# 3	Conclusion

In this contribution, we discussed the potential impacts on signaling and procedures over CN-RAN interface for Ambient IoT to support Inventory, and got the following proposals:

Proposal 1: Capture the overall inventory call flows in Topology 2 into the TR.

Proposal 2: Capture the Baseline Command call flows in Topology 2 into the TR.

- Proposal 3: In Topology 2 UE Reader’s NAS/PDU session based solutions, the UE reader selection is performed by AIoT CN.

- Proposal 4: In Topology 2 UE Reader’s RRC based solutions, the UE reader selection may be performed by AIoT CN or the gNB or both.

The corresponding TP for TR 38.769 is provided in section 5.

# 4. Reference

- RP-234058, "New SID: Study on solutions for Ambient IoT (Internet of Things) in NR", RAN#102

- 3GPP TR 23.700-13 V0.2.0, Study on Architecture support of Ambient power-enabled Internet of Things

- 3GPP TR 38.848 V0.2.0, Study on Ambient IoT (Internet of Things) in RAN

- R3-5G240378.1 RAN architecture aspects for Topo2

# 5. TP for TR 38.769 (based on R3-243972)

----------------Start of the First Change----------------

## 6.4	Impacts on CN-RAN interface

Editor’s note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

### 6.4.1	Information exchanged between AIoT CN and AIoT RAS

#### 6.4.1.1	Inventory

Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the AIoT CN to the AIoT RAS, may include the following:

(1)	AIoT Device Identification (to find a single device, a group of devices, or all devices)

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note 1: It is FFS whether AIoT RAS needs to interpret/store/process it.

(2)	Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

Multiple individual AIoT Device IDs (one ID per device) can be provided to the AIoT CN via a single Inventory Report.

Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

#### 6.4.1.2	Command

Command can be sent by the AIoT CN for a single device.

Editor’s Note 1: it is FFS for command on a group of devices, or all devices.

Editor’s Note 2: it is FFS whether AIoT RAS can remain agnostic of the type of request from the AIoT CN (need to differentiate command and inventory)

### 6.4.3	Overall procedures for Topology 2

#### 6.4.3.1	Procedures for UE Reader’s RRC based solution

##### 6.4.3.1.1	Inventory

Comparing with the Inventory call flow in Topology 1, in this solution, the AIoT related information is further exchanged between the UE Reader and the gNB via NR Uu RRC, as illustrated in Figure 6.4.3.1.1-1.

Figure 6.4.3.1.1-1: Inventory call flow with UE Reader’s RRC based solution in Topology 2

1.	AIoT CN node triggers Inventory Request with AIoT Device Identifier towards the relavent gNB.

2.	The gNB sends Inventory Response to the AIoT CN.

3a.  The gNB sents Inventory information received in step1 to UE Reader over NR Uu.

NOTE: step 2 and step 3a may happen in parallel.

3b.	The UE Reader triggers Paging over radio.

4a.	The AIoT device1 performs random access and provides UL Data (Device ID1) to the UE Reader.

4b. The UE Reader sends the UL Data (Device ID1) received in step4a to the gNB.

5.	The gNB sends the Inventory Report towards the AIoT CN with the UL Data (Device ID1) received in step4b.

6a.	The AIoT device2 performs random access and provides UL Data (Device ID2) to the UE Reader.

7a.	The AIoT device3 performs random access and provides UL Data (Device ID3) to the UE Reader.

6b.	The UE Reader sends the UL Data (Device ID2) received in step6a to the gNB.

7b.	The UE Reader sends the UL Data (Device ID3) received in step7a to the gNB.

8.  The gNB sends the Inventory Report towards the AIoT CN node with the UL Data (Device ID2, 3) received in step 6b and 7b.

9.	The Inventory Complete procedure is triggered to complete the inventory service.

##### 6.4.3.1.2	Command

Comparing with the Command call flow in Topology 1, in this solution, the AIoT related information is further exchanged between the UE Reader and the gNB via NR Uu RRC, as illustrated in Figure 6.4.3.1.2-1.

- Figure 6.4.3.1.2-1. Command call flow with UE Reader’s RRC based solution in Topology 2

1.	AIoT CN triggers Inventory Request with AIoT Device ID towards the relavent gNB, and indicates that there will be command transmission for the device.

2.	The gNB sends Inventory Response to the AIoT CN.

3a.  The gNB sents Inventory information received in step1 to UE Reader over NR Uu.

NOTE: step 2 and step 3a may happen in parallel.

3b.	The UE Reader triggers Paging over radio.

4a.	The AIoT device1 performs random access and provides UL Data (Device ID1) to the UE Reader.

4b. The UE Reader sends the UL Data (Device ID1) received in step4a to the gNB.

5.	The gNB sends the Inventory Report towards the CN with the UL Data (Device ID) received in step4b. Based on the command indication received in step1, the gNB allocates RAN Device XXAP ID for the device, and include it in the Inventory Report.

6.	After Security checking successfully, the AIoT CN triggers DL Command Transport towards the gNB. The AIoT CN allocates CN Device XXAP ID for the device, and provide the RAN/CN Device XXAP ID pair and the DL Data towards the gNB.

7a. The gNB sends the DL Data towards the UE Reader.

7b.	The UE Readertransmits the DL Data towards the AIoT Device.

8a.	The AIoT Device transmits the UL Data towards the UE Reader.

8b.	The UE Reader sends UL Data to the gNB.

9.  The gNB triggers UL Command Transport towards the AIoT CN node, including the RAN/CN Device XXAP ID pair and the UL Data.

#### 6.4.3.2	Procedures for UE Reader’s NAS/PDU session based solution

##### 6.4.3.2.1	Inventory

Figure 6.4.3.2.1-1 illustrates the Inventory call flow with UE Reader’s NAS/PDU session based solutions in Topology 2, in these solutions, the AIoT service related singaling and data transmission are carried via UE Reader’s NAS PDU, or via UE Reader’s PDU session data packets, the interactions between the UE Reader and AIoT CN is transparently forwarded by the gNB.

Figure 6.4.3.2.1-1: Inventory call flow with UE Reader’s NAS/PDU Session based solutions in Topology 2

1.	AIoT CN triggers Inventory Request with AIoT Device ID towards the UE Reader via UE Reader’s NAS/PDU session data transmission.

2.  UE Reader sends Inventory Response towards the AIoT CN via via UE Reader’s NAS/PDU session data transmission.

3.	The UE Reader triggers Paging over radio.

- NOTE: step 2 and step 3 may happen in parallel.

4.	The AIoT device performs random access and provides UL Data (Device ID) to the UE Reader.

5. The UE Reader sends the UL Data (Device ID) received in step4 to the AIoT CN via UE Reader’s NAS/PDU session data transmission.

##### 6.4.3.2.2	Command

Figure 6.4.3.2.2-1 illustrates the Command call flow with UE Reader’s NAS/PDU session based solutions in Topology 2, in these solutions, the AIoT service related singaling and data transmission are carried via UE Reader’s NAS PDU, or via UE Reader’s PDU session data packets, the interactions between the UE Reader and AIoT CN is transparently forwarded by the gNB.

- Figure 6.4.3.2.2-1. Command call flow with UE Reader’s NAS/PDU Session based solutions in Topology 2

- 1.	AIoT CN triggers Inventory Request with AIoT Device ID towards the UE Reader, via UE Reader’s NAS/PDU session data transmission.

- 2.	The UE Reader sends Inventory Response to the AIoT CN, via UE Reader’s NAS/PDU session data transmission.

- 3.	The UE Reader triggers Paging over radio.

- NOTE: step 2 and step 3 may happen in parallel.

- 4.	The AIoT device1 performs random access and provides UL Data (Device ID) to the UE Reader.

- 5.	The UE Reader sends the Inventory Report towards the AIoT CN with the UL Data (Device ID) received in step4, via UE Reader’s NAS/PDU session data transmission.

- 6.	After Security checking successfully, the AIoT CN transmits DL Command towards the UE Reader, via UE Reader’s NAS/PDU session data transmission

- 7.	The UE Reader transmits the DL Data towards the AIoT Device.

- 8.	The AIoT Device transmits the UL Data towards the UE Reader.

- 9.  The UE Reader transmits Command Response towards the AIoT CN node, via UE Reader’s NAS/PDU session data transmission.

----------------End of the Changes----------------