3GPP TSG-RAN WG3 Meeting #125	R3-244106

Maastricht, NL, 19 - 23 Aug, 2024

Title: 	(TP for TS 38.473) Discussion on on-demand SSB SCell operation

Source: 	Huawei

Agenda item:	17.2

Document Type:	 Other

# 1. Introduction

In the RAN#102 meeting, a new Work Item for network energy saving enhancement has been approved in [1] including the following objective on on-demand SSB SCell operation:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we provide our considerations on on-demand SSB SCell operation, based on RAN1 progress.

# 2. Discussion

## 2.1 Scenarios and related RAN1 progress

Previous RAN1 meetings made many agreements on the on-demand SSB SCell operation. The relevant agreements to higher layers are given as follows.

Agreement at RAN1-116bis meeting For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - One signal - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Agreement at RAN1-116 meeting Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

It can be observed that RAN1 agreed four use cases for on-demand SSB SCell operation (Scenario #2 and Scenario #2A combined with case #1 and case #2), i.e., before SCell activation and when SCell is activated. The figure below provides these cases.

Figure 1: The illustration of potential cases in [R2-2405122]

- Scenario #2:

  - For Case #1, no always-on SSB can be transmitted. When the network decides to activate the SCell, it can first trigger the on-demand SSB for L3 measurement reporting, AGC setting, DL synchronisation etc, then sends the MAC CE SCell Activation command.

  - For Case #2, the SSB with long periodicity is transmitted. When the network decides to activate the SCell, it can first trigger the on-demand SSB with short periodicity for fast L1/L3 measurement reporting, AGC setting etc, then sends the MAC CE SCell Activation command.

- Scenario #2A:

  - For Case #1, no always-on SSB can be transmitted. When the network decides to activate the SCell, it can trigger the on-demand SSB command for L3 measurement reporting, AGC setting, DL synchronisation etc, and sends the MAC CE SCell Activation command simultaneously.

  - For Case #2, the SSB with long periodicity is transmitted. When the network decides to activate the SCell, it can trigger the on-demand SSB command with low periodicity for fast L1/L3 measurement, AGC setting, and send the MAC CE SCell Activation command simultaneously.

It can also be observed that RAN1 will continue to discuss more uses cases, e.g., scenario 3A and 3B. RAN3 can monitor the progress and continue to work on this.

- Observation 1: RAN1 agreed two cases (No always-on SSB on the cell and Always-on SSB) and two scenarios for on-demand SSB, before SCell activation and when SCell is activated.

## 2.2 Potential RAN3 impacts

### 2.2.1 Case 2 - Always-on SSB is periodically transmitted on the cell

For Case #2, the always-on SSB is periodically transmitted on the SCell. From RAN3 perspective, this cell can be considered working at normal state. For example, this SCell can be already activated for one UE, while is deactivated for another UE.

Then over Xn interface, this cell can be treated by neighbouring NG-RAN nodes as normal served cell, i.e., without deactivation indication. Hence there is no need to activate such cell. Similarly, over F1 interface, this cell can be treated by the CU as activated and “in service” cell. Regarding the transition from the long SSB periodicity and the short periodicity, this can be discussed in future meetings for the objective 3 – SSB in time domain adaptation. There is no need to discuss it within this objective.

For case #2 (Always-on SSB is periodically transmitted on the cell) on-demand SSB, RAN3 to assume that no Xn or F1 specification impact is needed.

### 2.2.2 Case 1 - Not always-on SSB on the cell

For case#1, the SSB is not always broadcast. As observed in [1] above, the on-demand SSB SCell operation has three possible triggering methods as follows:

- UE uplink wake-up-signal using an existing signal/channel,

- cell on/off indication via backhaul,

- Scell activation/deactivation signaling

In current XnAP specification, the cell activation procedure has been supported so that an NG-RAN node can request a neighbouring NG-RAN node to switch on the whole cell. For this case, it is clear that the existing the cell activation message can be reused to trigger the on-demand SSB over Xn, without any specification impact.

For case #1 (Not always-on SSB on the cell) on-demand SSB, RAN3 to assume no further Xn specification impact.

For the CU/DU split case, the general on-demand SSB procedure over F1 is exemplified in Figure 2 below.

Figure 2: The general on-demand SSB procedure

First, in the F1 interface manage message, the CU could provide some allow indication (i.e., Cells allowed to be in on-demand SSB state), based on which the DU can decide whether/when to broadcast SSB transmission. For example, when the cell load is low, the DU could enter into on-demand SCell state and stop SSB broadcast. On the contrary, it can leave the on-demand SSB Cell state and return to the normal Cell state. This is aligned with the R18 inter-node beam activation, where the CU could provide cells allowed to be deactivated list.

Second, when the UE accesses the CU and is configured with the SCell, the CU needs to indicate the on-demand SSB transmission of this SCell to the UE. As previous RAN1 meetings, the RRC and MAC CE based signaling to indicate on-demand SSB transmission was supported.

Agreement@RAN1#117 - For a cell supporting on-demand SSB Scell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide Scell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. FFS: Scenarios where the above signalings are applicable

It can be observed that the RRC can be used for the SCell configuration and MAC CE may be used to dynamically change the transmission of on-demand SSB.

- For RRC based signaling to indicate the on-demand SSB transmission, one candidate solution is that the DU decides the SSB transmission per SCell and indicate to the CU and finally to the UE. RAN3 can discuss whether this can be explicitly indicated via the F1AP, or in the RRC container which is to be decided by RAN2.

- For MAC CE based signalling, there is no RAN3 impact foreseen.

Last when the DU decides to stop the SSB broadcast, it can notify the CU via the legacy procedure, i.e., setting the Service State as Out-Of-Service in the GNB-DU configuration update message.

The CU could decide allowed cells in on-demand SSB state and indicate to the DU.

For RRC based signalling to indicate the on-demand SSB transmission, RAN3 to consider that the DU decides the SSB transmission state per SCell and indicates to the CU. RAN3 can discuss whether this can be explicitly indicated, or in the RRC container which is to be decided by RAN2

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- For case #2 (Always-on SSB is periodically transmitted on the cell) on-demand SSB, RAN3 to assume that no Xn or F1 specification impact is needed.

- For case #1 (Not always-on SSB on the cell) on-demand SSB, RAN3 to assume no further Xn specification impact.

The CU could decide allowed cells in on-demand SSB state and indicate to the DU.

For RRC based signalling to indicate the on-demand SSB transmission, RAN3 to consider that the DU decides the SSB transmission state per SCell and indicates to the CU. RAN3 can discuss whether this can be explicitly indicated, or in the RRC container which is to be decided by RAN2

The TP to capture Proposal 3 is provided in the Annex.

# 4. Reference

- RP-241650 Revised WID: Enhancements of network energy savings for NR, Ericsson

- Chair notes RAN1#117 eom4.doc

- TS 38.423 V18.0.0

- TS 38.473 V18.0.0

# 5. TP for TS 38.473

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If the SSBs within the cell to be Activated List IE is included in the Cells to be Activated List Item IE within the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, only activate those SSB beams indicated by the SSB Index IE.

If at least one requested SSB beam in the SSBs within the cell to be Activated List IE is activated, the gNB-DU includes the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message. The gNB-CU shall consider that the SSB beams indicated by the SSBs activated List IE as activated.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the Cells Allowed to On-Demand SSB List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to initiate on-demand SSB for the indicated cells for network energy saving purpose.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-CU TNL Association To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Information | M |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | M |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| gNB-CU TNL Association To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Transport Layer Address gNB-DU | O |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-DU. | YES | reject |
| gNB-CU TNL Association To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| Cells to be barred List |  | 0..1 |  | List of cells to be barred. | YES | ignore |
| >Cells to be barred List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Barred | M |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| >>IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) | Corresponds to information provided in the iab-Support contained in the PLMN-IdentityInfo IE or contained in the NPN-IdentityInfo IE as defined in TS 38.331 [8]. The codepoint value “barred” indicates that the iab-Support is not sent in SIB1, and the codepoint value “not-barred” indicates that the iab-Support is sent in SIB1. | - |  |
| >>Mobile IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) | Corresponds to information provided in the mobileIAB-Support contained in the PLMN-IdentityInfo IE or contained in the NPN-IdentityInfo IE as defined in TS 38.331 [8]. The codepoint value “barred” indicates that the mobileIAB-Support is not sent in SIB1, and the codepoint value “not-barred” indicates that the mobileIAB-Support is sent in SIB1. | - |  |
| Protected E-UTRA Resources List |  | 0..1 |  | List of Protected E-UTRA Resources. | YES | reject |
| >Protected E-UTRA Resources List Item |  | 1.. <maxCellineNB> |  |  | EACH | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | - |  |
| >>E-UTRA Cells List |  | 1 |  | List of applicable E-UTRA cells. | - |  |
| >>>E-UTRA Cells List Item |  | 1 .. <maxCellineNB> |  |  | - |  |
| >>>>EUTRA Cell ID | M |  | BIT STRING (SIZE(28)) | Indicates the E-UTRAN Cell Identifier IE contained in the ECGI as defined in subclause 9.2.14 in TS 36.423 [9]. | - |  |
| >>>>Served E-UTRA Cell Information | M |  | 9.3.1.64 |  | - |  |
| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| CCO Assistance Information | O |  | 9.3.1.211 | Indicates CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cells Allowed to On-Demand SSB List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be On-Demand SSB List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |