3GPP TSG-RAN WG3 Meeting #125	R3-244107

Maastricht, NL, 19 - 23 Aug, 2024

Title: 	Discussion on on-demand SIB1 for UEs in idle or inactive mode

Source: 	Huawei

Agenda item:	17.3

Document Type:	Discussion

# 1. Introduction

The WID agreed in [RP-241650] includes the following objective for on-demand SIB1 for UEs in idle/inactive mode.

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105

The previous RAN3-123#bis meeting made the following agreements [1][2].

Agreement @ RAN3-123bis meeting: Scenario1: UE obtains the UL WUS configuration from Cell A, and transmits UL WUS to NES Cell. - UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed Agreement @ RAN3-123bis meeting: Focus on case2 in RAN3 first.

And RAN1-117 meeting agreed that case 2 and case3 are prioritized two use cases.

Agreement For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3 - Case 2 (Option 1+B+X) is feasible from RAN1 perspective. - Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

In this contribution, we provide further considerations on on-demand SIB1 operation from RAN3 perspective. It is important for RAN3 to make conclusion at this meeting, and move to normative work afterwards.

# Discussion

The general procedure for the on-demand SIB1 operation is given in Figure 1.

- Case 2: the UE obtains the UL WUS configuration from Cell A, and transmits UL WUS to NES Cell, and receives on-demand SIB1 from NES Cell.

- Case 3: the UE obtains the UL WUS configuration from Cell A, and transmits UL WUS to Cell A, and receives on-demand SIB1 of NES cell from Cell A.

Figure 1 A general procedure for the on-demand SIB1 operation

## 2.1 Case 2

- The Figure 2 below provides the exemplified procedure of the on-demand SIB1 for case 2.

- Figure 2. The basic on-demand SIB1 procedure of case 2

### 2.2.1 The WUS configuration

It was agreed in previous RAN3 meeting that the UL WUS configuration is signalled from the NES cell to the Cell A over Xn interface. And it was discussed which node could decide the WUS configuration in split architecture.

Which node decides the WUS configuration and On-demand SIB1 mode? DU or CU or OAM?

Currently both RAN1 and RAN2 have agreed the UL WUS configurations.

3. UL WUS configuration includes at least RACH configuration.

Since the WUS configuration includes at least the RACH configuration per NES cell, which is lower-layer parameter, it makes sense that the WUS configuration should be determined by the DU. Then before entering on-demand SIB1 mode, or when the UL WUS configuration is changed, the DU of the NES node signals its UL WUS configuration information per NES cell to the CU of the NES node, then to the CU of the Cell A. The CU of the Cell A indicates the UL WUS configuration per NES Cell to its DU for broadcast.

The WUS configuration is determined by the DU.

Before entering on-demand SIB1 mode, or when the UL WUS configuration is changed, the NES node DU signals its UL WUS configuration information per NES cell to the NES node CU, then to the CU holding the Cell A, then to its DU for broadcast.

### 2.2.2 The WUS configuration broadcast indication

In addition to discussion above, it should also discuss the case when the NES cell begins or stops the SIB1 broadcast. This was also discussed at the previous meetings.

NES-cell gNB decides on-demand SIB1 operation activation and deactivation?

- Except the WUS configuration, the NES Cell could send the WUS configuration broadcast request to the Cell A. After the DU holding the Cell A broadcasts the WUS configuration, it could send the WUS configuration broadcast indication message to its CU, and to the NES node CU via the Xn interface, finally to the NES node DU to stop broadcasting its SIB1.

The NES node DU can stop broadcast its SIB1 after knowing that the Cell A has broadcast the WUS configuration of NES Cell.

Meanwhile, when the NES cell receives the UL WUS from the UE and begins SIB1 broadcast, it should notify Cell A, so as for the Cell A to stop the UL WUS configuration broadcast. On the contrary, when the NES cell decides to stop the SIB1 broadcast, it can notify the Cell A to begin the UL WUS configuration broadcast. The WUS configuration broadcast indication could indicate whether Cell A to begin/stop broadcast NES Cell’s WUS configuration.

The NES cell can indicate the WUS configuration broadcast indication to the Cell A over the Xn interface, e.g., to stop/begin the UL WUS configuration broadcast.

### 2.2.3 Which node (NES node CU vs. NES node DU) decide to broadcast/not broadcast SIB1

- The next question is which node to determine the SIB1 broadcast status of NES Cell over F1 interface. Hence there are two options.

- The CU decides

- The DU decides with CU assistance.

- In legacy on-demand other SI mechanism, the DU itself determines the broadcast status of other SIBs. Hence it makes sense the DU can decide the SIB1 broadcast. Considering that the CU has the general system level information, e.g., the neighbour cells deployment etc, it is beneficial that that CU could provide some assistance information before DU makes final decision. We consider that the CU could provide a cell list to indicate allowed cells to enable the on-demand SIB1 operation.

The CU of the NES Cell can provide the allowed cell list to the DU, and the CU can decide whether/when the DU can decide the SIB1 broadcast status per NES cell.

### 2.2.4 SI Conclusion for case 2

- There are also many other details, e.g., when the Cell A decides to stop the UL WUS broadcast, it should notify the NES cell. At this study phase, RAN3 can conclude that the case 2 can be considered in the normative phase, while leaving the details to the work item phase.

RAN3 to conclude that case 2 can be considered in the normative phase.

## 2.2 Case 3

This case was also prioritized in RAN1, where the cell A can broadcast the UL WUS configuration, and broadcast on-demand SIB1 of the NES cell. This case has significant energy saving gain for the NES cell.

For this case, since the UE may acquire the NES cell SIB1 from the Cell A directly, then it requires the NES cell to signal its SIB1 contents to the Cell A, and send updates when necessary. Whether to signal other SI contents over network interfaces is pending to other working groups.

For case 3, before entering on-demand SIB1 mode in the NES Cell or when the SIB1 contents s changed, the NES cell signals its SIB1 contents to the Cell A.

- To conclude this case, RAN3 can conclude that the case 3 can be considered in the normative phase if other WGs has the same conclusion, while leaving the details in the work item phase.

RAN3 to conclude that case 3 can be considered in the normative phase, if confirmed by other WGs.

# 3. Conclusion

Based on the discussion in this paper, the following observations and proposals are made.

For Case 2:

- The WUS configuration is determined by the DU.

Before entering on-demand SIB1 mode, or when the UL WUS configuration is changed, the NES node DU signals its UL WUS configuration information per NES cell to the NES node CU, then to the CU holding the Cell A, then to its DU for broadcast.

The NES node DU can stop broadcast its SIB1 after knowing that the Cell A has broadcast the WUS configuration of NES Cell.

The NES cell can indicate the WUS configuration broadcast indication to the Cell A over the Xn interface, e.g., to stop/begin the UL WUS configuration broadcast.

The CU of the NES Cell can provide the allowed cell list to the DU, and the CU can decide whether/when the DU can decide the SIB1 broadcast status per NES cell.

RAN3 to conclude that case 2 can be considered in the normative phase.

- For Case 3:

For case 3, before entering on-demand SIB1 mode in the NES Cell or when the SIB1 contents s changed, the NES cell signals its SIB1 contents to the Cell A.

RAN3 to conclude that case 3 can be considered in the normative phase, if confirmed by other WGs.

# 4. Reference

- RAN3_123bis_agenda_20240419_EOM.doc

- RAN3_124_agenda_20240524_EOM2.doc