3GPP TSG RAN WG3 Meeting #125																R3-244121

Maastricht, NL, August 19th – August 23rd, 2024

Agenda item:		12.2

Title:						Discussion on Network Selection for WAB

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:		Discussion

# Introduction

In the revised version of the SID on “Study on additional topological enhancements for NR” [1], the objectives for WAB are as follows:

The objectives of the Wireless Access Backhaul (WAB) study are as follows: - Study the support of WAB including [RAN3, RAN2]: - Study the architecture and protocol stack of supporting a gNB with MT function providing PDU session backhaul. - Study impact of WAB mobility within an existing RAN (e.g., inter-gNB neighbour relations). - Identify necessary inter-gNB- and gNB-to-CN signalling to address the support of WAB. - Study signalling enhancements on resource multiplexing for WAB. NOTE 1: No impact on the UE. NOTE 2: Coordination with other WGs (e.g. SA2) when needed. The WAB study does not preclude any backhaul scenario (e.g. NTN or TN).

The justification for this SI references another study about vehicular mounted relays; see marked text in yellow:

Wireless Access Backhaul (WAB): The legacy building blocks for 5G RAN topologies should be enhanced to provide a broader range of use cases, such as: - 5G access for UEs onboard aircrafts, cruise ships, helicopters, and vehicles in remote areas with limited sky visibility via an onboard gNB. - Backhauling of NG and Xn via TN and NTN, including support of NTN <-> TN handover for backhaul. - Support for onboard/on-site MEC and local services. - Support for backhauling without RAN-sharing or roaming agreements between access PLMN(s) and backhaul PLMN(s). - Backhauling for local gNB deployed in public safety or disaster recovery scenarios. It is assumed that Wireless Access Backhaul (WAB) is aligned with VMR use cases and with the SA2-endorsed SID on architectural enhancements for Rel-19 VMR. It is expected that single-hop backhauling is sufficient for Wireless Access Backhaul (WAB) and that there is no impact to UEs at this late stage of 5G deployment.

The SID on Architecture Enhancements for Vehicle Mounted Relays (VMR) [2] mentions “architecture options to achieve the functionality of vehicle-mounted relays” [2] other than with IAB to allow local traffic, thus a termination of traffic within the VMR or WAB node.

A local termination of traffic would allow the VMR to seamlessly alter routing for higher-layer applications between Uu and relay paths. Service continuity in this case is achieved by both direct and indirect paths and (L3) routing within the VMR. Local termination also enabled the use of mobile edge computing in the VMR or WAB node.

This document discusses the handling of coverage scenarios for the indirect path.

# Network Selection for WAB

Given the justification of the SID [1] we consider a WAB node to be mobile, e.g. a VMR or an airborne relay. While the backhaul scenario could be NTN or TN and does not necessarily need to be operated by the same MNO, handover scenarios and service continuity need to be further considered.

RAN3 to confirm that a WAB node is considered to be mobile (potentially moving).

Nevertheless, there is no technical reason that a WAB node cannot be stationary.

A WAB node that is connected via a roaming (foreign) network does not automatically return to its own home network when it becomes available again, since the roaming network does not provide the required measurement information for the home network to trigger a re-connection. A WAB node will therefore stay in the roaming network until RLF is detected and a new cell search is triggered. A handover between two different PLMNs is currently not feasible due to the lack of inter-PLMN measurements.

Inter-PLMN measurements for handover between different PLMNs is currently not supported.

This is a typical case in cell edge scenarios near country borders, especially in Europe, where different operators are providing overlapping services.

The same scenario can be true for WAB nodes that are connected to an NTN network of one operator while moving back to an in-coverage area with TN coverage, either E-UTRA or NR, of another operator. The WAB node – having a valid and active connection to a roaming partner network – will keep this connection until it detects an RLF.

A WAB node that is connected to an NTN roaming network will stay in the roaming network even if a better alternative is available, e.g. a terrestrial network.

Given the handover procedures from TS 38.331 [4], the handover between NR and another RAT requires at least a MeasurementReport or an MCGFailureInformation message from the UE. Due to the possibly large footprint of an NTN cell, measurement reports cannot easily be configured for TN measurements in an NTN connected UE.

While TR 38.321 [3] already drafts enhancements for mobility scenarios in NTN/TN architectures, the assumption of a single operator network might not be true for this case and needs further consideration.

RAN3 to check whether current mobility features are sufficient to enable service continuity between NTN and TN or TN and NTN for WAB nodes and their connected UEs.

# Conclusions

In this contribution, we discussed some aspects of WAB network selection and issues originating from roaming scenarios and made the following observations and proposals:

Observation 1:	Inter-PLMN measurements for handover between different PLMNs is currently not supported.

Observation 2:	A WAB node that is connected to an NTN roaming network will stay in the roaming network even if a better alternative is available, e.g. a terrestrial network.

Proposal 1:	RAN3 to confirm that a WAB node is considered to be mobile (potentially moving).

Proposal 2:	RAN3 to check whether current mobility features are sufficient to enable service continuity between NTN and TN or TN and NTN for WAB nodes and their connected UEs.

# References

- RP-241264, “Revised SID on Study on additional topological enhancements for NR”, NTT DOCOMO/AT&T, 3GPP TSG RAN Meeting #104, Shanghai, China, June 2024.

- SP-231799, “New SID on Architecture Enhancements for Vehicle Mounted Relays (VMR) Phase 2, vivo (Moderator) TSG SA Meeting #SP-102, Edinburgh, Scotland, 2023-12.

- TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN)”, V16.2.0, 2023-03.

- TS 38.331, “Radio Resource Control (RRC) protocol specification”, V18.2.0, 2024-06.