3GPP TSG-RAN WG3 Meeting #125 	R3-244122

# Maastricht, The Netherlands, EU, 19. - 23.08.2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0417 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 correction for reporting of requested DL LBT information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Samsung, Huawei, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current stage 2 does not describes the possibility for a gNB-DU to report DL LBT Failure information in case of “too early handover” or “handover to wrong cell” due to RLF shortly after successful handover. There is no reference to TS 38.300 where the scenarios where the reporting of DL LBT Failures information for MRO analysis are described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UE CONTEXT RELEASE COMMAND can be used to trigger the reporting of DL LBT failure information from gNB-DU to gNB-CU. Reference added to MRO analysis in TS 38.300. Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated because the change affects only one procedure in backward-compatible manner. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 2 is not complete in describing how a gNB-DU can report DL LBT information to the gNB-CU. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.473 CR 1442 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change, ommited text not changed

#### 8.14.1.3	Reporting of DL LBT failure information from gNB-DU to gNB-CU

For MRO analysis, a gNB-CU may request to a gNB-DU to report DL LBT failure information related to mobility failures for a UE, as defined in TS 38.300 [2], via the UE CONTEXT SETUP REQUEST message, or the UE CONTEXT MODIFICATION REQUEST message, or the UE CONTEXT RELEASE COMMAND. The gNB-DU may report the information to the gNB-CU via the DU-CU ACCESS AND MOBILITY INDICATION message.

Remaining text not changed