3GPP TSG-RAN WG3 Meeting #125	R3-244123

Maastricht, NL, August 19-23, 2024

Agenda Item:	14.2

Source:	THALES

Title:	Way forward for MBS Broadcast intended service area discussion

Document for:	Decision

# 1	Introduction

The approved work item justifications and objectives for NR NTN Evolution in Rel-19 [1] emphasizes:

3) MBS feature provides an important add-value for NR NTN system, leveraging the large coverage of the NTN compared to TN. Terrestrial MBS features are equally available for NR NTN in the 5G specifications, but for some cases the intended service area is expected to be smaller than the coverage of a Uu cell, some enhancements need to be done to notify the service area of a Broadcast service.

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In this paper, we propose to wait for RAN2 conclusions before discussion the necessary signaling between CN and NG-RAN in RAN3.

# 2	Discussion

Geographical Area

The Working Assumption taken at the RAN3#124 is that “the mapped cell ID can be used to define MBS broadcast”. The mapped Cell IDs are built by the gNB and are transparent to the UE. The UE only knows the Physical Cell ID that could be different (and all the time different in the case of Earth-Moving Cell). In the implementation, mapped Cell ID are expected to be used to have a fixed geographical grid on Earth, for tracking area for instance. The mapping between Mapped cell ID and geographical areas are configured in the RAN and Core Network.

If we go for mapped cell ID to indicate the intended service area, it means the geographical areas indicated by the MBS broadcast service provider will be converted into a list of mapped cell ID at the 5GC and distributed to the concerned gNB/satellites. The gNB will convert the mapped cell ID (since the UE does not have the information of the mapped cell ID it is camping on) into geographical information (associated to a MBS session ID) to be broadcasted into concerned cells. One problem here is that many mapped Cell ID could be related to a single MBS session ID if the cell is larger than a physical cell (with an impact on SI message TBS).

On another solution uses a dedicated geographical area that is transmitted from the MBS broadcast service provider to the RAN. Maybe with a conversion stage to a proper IE and a filtering to avoid transmission of the information to gNB that does not serve the geographical area. The gNB is then responsible to transmit the intended geographical service area to the concerned physical cells. The UE that wants to access to a MBS broadcast gets the System Information (defined by the RAN2) and determines to which services it is intending to access according to its location and the geographical area associated to a MBS session ID. RAN will not define the means for the network to prevent the reception of the content of the service outside of the intended service area.

RAN3 waits for RAN2 conclusions on geographical area formats to model service area to discuss the necessary signaling between the CN and the NG-RAN node

# 3	Conclusion

In the previous sections we made the following proposal:

<Geographical Area>

- RAN3 waits for RAN2 conclusions on geographical area formats to model service area to discuss the necessary signaling between the CN and the NG-RAN node

# References

RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN1 Vice-Chair, RAN#104, 2024-06, Shanghai, CN.