3GPP TSG-RAN WG3 Meeting #125	R3-244129

# Maastricht, NL, 19-23 August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.412 | CR | 0012 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Use of Establishment Cause to determine DSCP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add that Establishment Cause may be used to determine the Diffserv Code Point marking for a signalling connection to support stage 2 requirements for WLAN in TS 23.502 and agreed in SP-230481, where: - “The Establishment cause provides the reason for requesting a signalling connection with 5GC and the N3IWF may use the Establishment cause to determine the DSCP value on N2.” - “The AMF may use the Establishment cause to determine the Message Priority header and then the DSCP value for subsequent signalling as per TS 29.500 [17].” It is covered in stage 3 in TS 24.502 in the downstream direction for QoS signalling but not in the upstream, which is why it is needed in 38.412. The text in TS 24.502 clause 4.4.2.2 is as follows: “In order to support QoS differentiation in the case of access to PLMN services via a WLAN, the N3IWF and TNGF behaviour is as specified in clause 4.4.2.3, with one or more QoS profiles requiring an IPsec child SA which can be associated with a downlink DSCP value determined by taking into account, according to operator policy, the establishment cause, the 5QI, the Priority Level (if explicitly signalled) and optionally, the ARP priority level.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following change is added in clause 6: “The Establishment Cause may be used to determine the Diffserv Code Point marking towards the AMF.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPS UEs' packets will not receive priority treatment on the uplink and may be delayed or dropped during periods of overload or congestion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

# 6	IP layer

The 5GC and NG-RAN shall support IPv6 (IETF RFC 8200 [5]) and/or IPv4 (IETF RFC 791 [6]).

The IP layer of NG-C only supports point-to-point transmission for delivering NG AP message.

The 5GC and NG-RAN shall support the Diffserv Code Point marking as described in IETF RFC 2474 [7]. The Establishment Cause may be used to determine the DiffServ Code Point marking towards the AMF.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>