3GPP TSG-RAN WG3 Meeting #125	R3-244138

Maastricht, NL, 19- 23 August 2024

Title: 	(TP for TR 38.743) TP on General Corrections for CCO and Slicing

Source: 	InterDigital

Agenda item:	11.2

Document Type:	Discussion and decision

# TP to TR38.743

This TP mirrors puts forth, general correction for the network slicing and CCO sections.

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

# 4	Use cases and Solutions

## 4.1	AI/ML based Network Slicing

### 4.1.1	Use case description

Support of network slicing in NG-RAN is defined in TS 38.300 [x].

The NG-RAN plays a key role in taking mobility, load balancing and Radio Resources Management decisions for the purpose of meeting target requirements derived from the SLA of each supported network slice.

The AI/ML function can analyse metrics related to network and UE level performance related to perform optimal resource management and mobility decisions for network slicing to meet the requirements.

### 4.1.2	Solutions and standard impacts

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based network slicing:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.4	Input data of AI/ML based Network Slicing:

To predict the optimized network slicing decisions, a gNB may need the following information as input data for AI/ML-based network slicing:

From local node:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

-	Legacy predicted UE trajectory

From neighbouring gNBs:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

#### 4.1.2.5	Output data of AI/ML based Network Slicing:

The AI/ML-based network slicing model in a gNB can generate the following information as output:

- Predicted radio resource status per slice

- Predicted slice available capacity

- Resource management decisions for resources within RRM policies (used by gNB internally)

- Slice aware mobility decisions (used by gNB internally)

#### 4.1.2.6	Feedback of AI/ML based Network Slicing:

To optimize the performance of AI/ML-based network slicing model, the following feedback can be considered to be collected from gNBs:

- Measured Radio resource status per slice

- Measured Slice available capacity

- Legacy UE performance feedback for those UEs handed over from the source gNB

- Finer granularity UE performance feedback for those UEs handed over from the source gNB. FFS on the exact level of finer granularity.

#### 4.1.2.7	Potential standard impacts:

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

- Enhanced existing procedure to collect predicted information between gNBs:

  - Predicted radio resource status per slice

- Predicted slice available capacity

## 4.2	AI/ML based Coverage and Capacity Optimization

### 4.2.1	Use case description

The objective of the NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. A NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included.

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

### 4.2.2	Solutions and standard impacts

#### 4.2.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based CCO:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>