3GPP TSG-RAN WG3 Meeting #125	R3-244139

Maastricht, Netherlands 19 - 23 August 2024

Agenda Item:	16.2

Source:	InterDigital

Title:	[TP for TR 38.769] RAN Architecture for AIoT

Document for:	Discussion and Agreement

# 1	Introduction

At RAN3#123bis there was a discussion about RAN architecture for AIoT those were:

RAN3 considers both Topologies at the same time looking whether commonalities are applicable.

[Topo1] AIoT RAN node: 
Corresponds to the basestation in Figure 4.2.1.1-1 in TR 38.848; 
A RAN node providing AIoT radio, and connecting with an AIoT-aware CN node via the XX interface. Details of the AIoT-aware CN node is subject to SA2.

[Topo2] UE Reader: 
A UE (corresponds to the intermediate node in Figure 4.2.1.2-1 in TR 38.848);
Providing AIoT radio, and connecting with a gNB (may be an AIoT enhanced gNB, corresponds to the basestation in Figure 4.2.1.2-1 in TR 38.848) via NR Uu interface.

For Topology 1, RAN3 starts with AIoT RAN node being aggregated

At RAN3#124 there was a discussion about RAN architecture for AIoT, the comeback on the discussion and the subsequent agreement are:

# AIoT1_Architecture

- Focus on how to capture the system architecture in TR

- Discuss the definition of common reader function and AIoT RAN function

- the possibility to send LS to other WGs on security?

(moderator – E///)

pCR in R3-243807

- In 6.4: Figure 6.4-1 depicts the logical system architecture for AIoT.

- Figure 6.4-1: Logical System Architecture for AIoT

- Editor’s NOTE: Whether the Logical System Architecture above is applied to topology 2 to be discussed in next meeting.

- Remove the part after figure1

Rev in R3-243962  Agreed unseen

# 2	Discussion

During the initial discussion of the diagrams there was pushback from a number of companies when a diagram was proposed that included the Reader function in the AIoT RAN, which resulted in it being renamed AIoT RAS, this is a clear distinction, that allows for both topologies to be represented in the same diagram which will streamline things. The conclusion that we are only dealing aggregated gNBs in this release means that any interface between the reader and AIOT RAN node function is not subject to standardization in RAN3.

Also, since from our agreements Topology 2 relies on an interface outside of RAN3 (Uu) it is clear that both topologies are represented in the current diagram

# 3	Proposal

Based on the discussion above, the following observations and the proposal are made:

Proposal 1: Approve a TP to remove the note added last meeting so that the diagram applies to both topology 1 and topology 2, TP is below.

# 4	TP for 38.769

## 6.3	RAN architecture aspects

Editor’s note 1: Corresponds to the second RAN3 objective in the SID, to identify RAN architecture aspects, including whether support for split architecture is necessary.

This chapter attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

Editor’s Note 2: What functionalities are hosted by the 5GS for AIoT is TBD.

This chapter also attempts to identify a functional split between RAN and CN.

Figure 6.3-1 depicts the logical system architecture for AIoT.

It consists of the following architectural elements:

AIoT device: equipment with characteristics outlined e.g. in TS 22.369 [x] and TR 38.848 [2].

Editor’s Note 3: Further details FFS, if any.

AIoT RAS (AIoT Radio Access System): hosts certain functions for AIoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAS and the respective functional split to be decided by RAN2, RAN3 and SA2.

AIoT radio: radio interface between AIoT device and Common reader function.

Editor’s Note 5: Further details on AIoT radio to be discussed by RAN1 and RAN2.

AIoT CN: hosts certain functions for AIoT as of the functional split between RAN and CN

Editor’s Note 6: Further details regarding AIoT functions hosted in the AIoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface: interface between the AIoT RAS and the AIoT CN on which certain AIoT specific functions are performed.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between AIoT RAS and AIoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAS, or one instance for NG alone.

Common reader function: a function that communicates with the AIoT device by means of AIoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

AIoT RAN node function: a function residing in AIoT RAS. It contains e.g. the control of the AIoT radio resources used towards the AIoT device.

Editor’s Note 9: further details are FFS. Note that “control of AIoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

Figure 6.3-1: Logical System Architecture for AIoT.