3GPP TSG-RAN WG3 Meeting #125                                                              R3-244141

Maastricht, The Netherlands, May 19 – 23, 2024

Agenda item: 		12.2

Source: 		Qualcomm Incorporated

Title: 	(TP to TR 38.799) Remaining issues for WAB

Document for:		Other

# 1	Introduction

In the last meeting, the following agreements were achieved [1]:

BH-RAN-node: This is an NG-RAN node serving the WAB-MT. BH-gNB: The gNB serving the WAB-MT. BH-AMF: The AMF serving the WAB-MT. BH-5GC: The 5GC serving the WAB-MT. BH-UPF: The UPF serving the WAB-MT for backhauling. UE´s 5GC: The 5GC connected to the WAB-gNB and serving the UEs. UE´s AMF: The AMF connected to the WAB-gNB and serving the UEs. UE´s UPF: The UPF connected to the WAB-gNB and serving the UEs. Whether a BH RAN node can be unaware of WAB needs to be further studied. RAN3 understands that authorization of the WAB-MT is different from the WAB-gNB service authorization/configuration/activation by e.g. OAM/SeGW. WAB-MT authorization provides the WAB MT with the right to support BH PDU sessions. In case MT authorization is based on slice, signaling enhancements to the AS layer for the support of WAB-MT authorization are not needed. SA2 to decide whether MT authorization based on slicing is supported. Authorization of WAB-gNB provides service authorization, i.e., the right to serve UEs. It is RAN3 understanding that WAB-gNB’s authorization status can change. The UEs served by a WAB-gNB whose authorization status changes from “authorized” to “not authorized” can either be handed over to other RAN nodes or released. The NG connection(s) of a WAB-gNB whose authorization status changes from “authorized” to “not authorized” can be removed. It needs to be further studied whether they can be suspended. Any Stage 3 decisions are to be taken during normative work. Support the following scenarios for intra-PLMN WAB-node mobility: - UE´s AMF/UPF remains unchanged as WAB-gNB moves inside a PLMN. - UE´s AMF/UPF changes as WAB-gNB moves inside a PLMN. - BH UPF/AMF remains unchanged as WAB-MT moves inside a PLMN. - BH UPF/AMF changes as WAB-MT moves inside a PLMN. Legacy mobility procedures can be supported for WAB-MT. RAN3 assumes that roaming scenarios for WAB-MT are supported. Establishment of Xn Connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported and can follow legacy procedures. Addition of new information in the legacy procedures is not precluded If needed, the WAB-gNB may power up one or more new cells with new configuration parameters related to its current location and handover UEs between the old and new cell served by the WAB-gNB. FFS if this procedure is needed for AMF relocation. Discuss whether in case this procedure is used for AMF relocation, new cells have to belong to different logical gNBs or can belong to the same gNB.

RAN3 further endorsed current draft TR 38.799 in R3-243171 and agreed on multiple TP to this TR 38.799 in R3-243959, R3-243860, R3-243863, R3-243960, and R3-243862.

This contribution discusses remaining aspects of this study, which have been raised in the TR (Editor Notes) and/or the prior meeting(s).

# 2	Discussion

## 2.1	WAB-unawareness on BH RAN

In the last meeting, RAN3 discussed whether the BH RAN can remain unaware of WAB-backhauling. If such WAB-unawareness could be supported, WAB-nodes could be deployed without need for upgrade of the RAN that serves for backhauling. Such WAB-unawareness could remain optional, i.e., it does not preclude WAB-related optimizations that could be supported via BH RAN upgrade. During the discussion, some companies felt that it was too early to determine whether the BH RAN could remain WAB-unaware.

Moving forward, it is not necessary to explicitly decide whether WAB-unawareness is supported by the BH RAN. Instead, RAN3 can decide, which enhancements are needed for WAB, and whether these enhancements have mandatory impact on the BH RAN.

Proposal 1: No need to explicitly discuss, whether the BH RAN can remain WAB-unaware during the study.

## 2.2	Impact of AMF change

In the last meeting, RAN3 agreed that in case of a change of the WAB-gNB’s AMF is necessary, the WAB-node can create a second logical WAB-gNB, which establishes NG to the new AMF. The initial logical WAB-gNB can then handover all RRC-connected UEs to the second logical WAB-gNB.

The TP to TR 38.799 in R3-243861 includes the following Editor’s Notes related to this topic:

Editor’s NOTE: FFS whether a new logical WAB-gNB needs to be established for the NG connections with the new AMF(s). Editor’s NOTE: FFS whether the following steps are needed to support the AMF change for UE.

In case the UE’s AMF has to be changed, the following aspects need to be considered:

- AMF change for RRC-connected UEs: The WAB-gNB needs to initiate the NG-handover procedure to move the UE to the new AMF. According to TS 23.502 clause 4.9.1.3, the S-AMF will only select a T-AMF if the target gNB-ID is different than the source gNB-ID. This implies that two logical WAB-gNBs with different gNB-IDs need to be supported by the WAB-node during this procedure.

- AMF change for RRC-idle/incative UEs: AMF re-allocation can occur when the UE performs a Registration Update. The Registration Update can occur at the new AMF via the new logical WAB-gNB. However, to ensure paging support at all times, the Registration Update needs to occur before the old logical WAB-gNB removes NG from the AMF. The Registration Update should therefore be triggered at the time of cell reselection, which implies that the cell reselection becomes the trigger of a Registration Update. To trigger such a Mobility Registration Update, the new WAB-gNB should broadcast a different TAC in system information, which is not contained in the UE’s current RA.

Observation 2-1: In case the UE’s AMF needs to change, NG handover will trigger AMF reallocation only if the target gNB is different from the source gNB.

Observation 2-2: For idle/inactive UEs, AMF-relocation needs to be performed before removal of the initial gNB’s NG to ensure paging support for the UE.

Observation 2-3: The new logical gNB’s cell needs to broadcast a different TAC than the initial logical gNB’s cell so that the UE’s cell reselection triggers a Mobility Registration Update with AMF-relocation.

Proposal 2-1: For WAB-gNB mobility with change of UE’s AMF, the WAB-node needs to create a second logical gNB with a different gNB-ID and perform NG-handover of all connected UEs.

Proposal 2-2: For WAB-gNB mobility with change of UE’s AMF, the new logical gNB’s cell to broadcast a different TAC than the initial logical gNB’s cell.

## 2.3	Avoiding WAB-MT connecting to WAB-gNB

SA2 sent an LS to RAN3 on questions regarding FS_VMR_Ph2 (R3-244019 / S2-2407345). This LS includes the following question:

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

This topic is being discussed in a separate contribution. In this discussion, the following observations were made:

Observation 3-1: SA2’s CN-based solutions to avoid WAB-MT from connecting to WAB-gNB do not address all scenarios of issue 1a and 1b, and they require enhancements to NGAP and AMF.

Observation 3-2: The following RAN-based solutions can be considered to address SA2’s issues:

Solution 1: The WAB-gNB uses dedicated frequencies and/or PCI partitions, which are avoided by the WAB-MT for cell-reselection and measurement reports.

Solution 2: WAB-gNB-cells broadcast a “WAB” indicator in SIB1, and the WAB-MT avoids (re)selection or reporting of cells broadcasting this indicator.

Solution 3: WAB-MT includes a “WAB” indicator in UE capabilities, and the network rejects an access attempt or a handover request to a WAB-gNB based on this indicator.

Observation 3-3: These RAN-based solutions can handle all scenarios of issues 1a and 1b raised by SA2.

Observation 3-4: Solution 1 by itself can handle all scenarios of issues 1a and 1b, and this solution has no stage-3 impact.

The following proposals can be derived from these observations:

Proposal 3-1: The TR to capture the above observations.

Proposal 3-2: Further details on prevention of multi-hop WAB can be handled in the normative phase.

## 2.4	PCI collision avoidance

During Rel-18, the avoidance of PCI collisions was discussed for mobile IAB. For WAB, the same issues of PCI collisions may arise as for mobile IAB. Therefore, the same solutions should be applied.

Proposal 4: For WAB, PCI collision avoidance can use the same mechanisms as defined for mobile IAB.

## 2.5	TAC management for WAB-gNB cells

During Rel-18, RAN3 discussed the tracking area management for mobile IAB-DU cells. RAN3 decided that the configuration of the TAC broadcasted by the mobile IAB-DU cell can be left up to implementation.

For WAB-gNB cells, the same issues apply. Therefore, the same solution should be applied.

The TP to TR 38.799 in R3-243861 further includes the following Editor’s Note:

Editor’s NOTE: Whether the TAC can be changed without the change of UE’s AMF needs to be confirmed.

For mobile IAB-DU cells, TAC change can be conducted based on implementation. Further, the TAC is owned by the cell and not the AMF. In case the TAC of a cell changes, legacy procedures can be used to inform the cell’s AMF about the new TAC supported by the gNB. Therefore, TAC change can occur without AMF change.

Proposal 5-1: For WAB-gNB cells, TAC configuration to follow the same procedures as defined for mobile IAB, i.e. it is based on implementation.

Proposal 5-3: The TAC of a WAB-gNB cell can change without change of the UE’s AMF.

## 2.6	QoS support for WAB

QoS support for WAB has been sufficiently well discussed by SA2 and captured in TS 23.700-6. From RAN3 perspective, no additional aspects arise. TR 38.799 to include a reference to TS 23.700-6 on QoS support for WAB.

Observation 6-1: QoS support for WAB has been sufficiently well discussed by SA2 and captured in TS 23.700-6. From RAN3 perspective, no additional aspects arise.

Proposal 6: TR 38.799 to include a reference to TS 23.700-06 on QoS support for WAB.

## 2.7	ULI for UEs connected to WAB-gNB

SA2 sent an LS to RAN3 on questions regarding FS_VMR_Ph2 (R3-244019 / S2-2407345). This LS includes the following question:

SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

This topic is being discussed in a separate contribution. From this discussion, the following observations and proposals can be derived:

Observation 7-1: From RAN3’s perspective, all three options identified by SA2 for the additional ULI to be included with the UE’s ULI are feasible.

Observation 7-2: The enhancements for all three options identified by SA2 only have stage-3 impact on RAN3 specification, the impact is rather comparable and rather small.

Proposal 7: From RAN3’s perspective, all three options to include the WAB-MT’s location information in the UE’s ULI are feasible, and that RAN3 prefers to leave further down-selection to SA2.

## 2.8	Suspension of NG

The TP to TR 38.799 in R3-243860 and R3-243861 further include the following Editor’s Note:

Editor’s NOTE: FFS whether NG connections can be suspended.

The specific procedure for the removal of NG connections can be left up to normative phase.

Proposal 8: Since the removal of NG connections can be left up to normative phase, the Editor’s Note “FFS whether NG connections can be suspended” to be deleted.

## 2.9	Others

Proposal 9-1: Capture the above proposals in TR 38.799 and remove all remaining Editor’s notes.

Proposal 9-2: Agree the TP to TR 38.799 in the Annex.

# 3	Conclusion

This contribution discusses remaining aspects of the study on WAB. The following observations and proposals have been made:

On WAB-unawareness on BH RAN

Proposal 1: No need to explicitly discuss whether the BH RAN can remain WAB-unaware during the study.

On impact of AMF change

Observation 2-1: In case the UE’s AMF needs to change, NG handover will trigger AMF reallocation only if the target gNB is different from the source gNB.

Observation 2-2: For idle/inactive UEs, AMF-relocation needs to be performed before removal of the initial gNB’s NG to ensure paging support for the UE.

Observation 2-3: The new logical gNB’s cell needs to broadcast a different TAC than the initial logical gNB’s cell so that the UE’s cell reselection triggers a Mobility Registration Update with AMF-relocation.

Proposal 2-1: For WAB-gNB mobility with change of UE’s AMF, the WAB-node needs to create a second logical gNB with a different gNB-ID and perform NG-handover of all connected UEs.

Proposal 2-2: For WAB-gNB mobility with change of UE’s AMF, the new logical gNB’s cell to broadcast a different TAC than the initial logical gNB’s cell.

On avoiding WAB-MT connecting to WAB-gNB

Observation 3-1: SA2’s CN-based solutions to avoid WAB-MT from connecting to WAB-gNB do not address all scenarios of issue 1a and 1b, and they require enhancements to NGAP and AMF.

Observation 3-2: The following RAN-based solutions can be considered to address SA2’s issues:

Solution 1: The WAB-gNB uses dedicated frequencies and/or PCI partitions, which are avoided by the WAB-MT for cell-reselection and measurement reports.

Solution 2: WAB-gNB-cells broadcast a “WAB” indicator in SIB1, and the WAB-MT avoids (re)selection or reporting of cells broadcasting this indicator.

Solution 3: WAB-MT includes a “WAB” indicator in UE capabilities, and the network rejects an access attempt or a handover request to a WAB-gNB based on this indicator.

Observation 3-3: These RAN-based solutions can handle all scenarios of issues 1a and 1b raised by SA2.

Observation 3-4: Solution 1 by itself can handle all scenarios of issues 1a and 1b, and this solution has no stage-3 impact.

Proposal 3-1: The TR to capture the above observations.

Proposal 3-2: Further details on prevention of multi-hop WAB can be handled in the normative phase.

On PCI-collision avoidance

Proposal 4: For WAB, PCI collision avoidance can use the same mechanisms as defined for mobile IAB.

On TAC management of WAB-gNB cells

Proposal 5-1: For WAB-gNB cells, TAC configuration to follow the same procedures as defined for mobile IAB, i.e. it is based on implementation.

Proposal 5-3: The TAC of a WAB-gNB cell can change without change of the UE’s AMF.

On QoS support for WAB

Observation 6-1: QoS support for WAB has been sufficiently well discussed by SA2 and captured in TS 23.700-6. From RAN3 perspective, no additional aspects arise.

Proposal 6: TR 38.799 to include a reference to TS 23.700-06 on QoS support for WAB.

On ULI for UEs connected to WAB-gNB

Observation 7-1: From RAN3’s perspective, all three options identified by SA2 for the additional ULI to be included with the UE’s ULI are feasible.

Observation 7-2: The enhancements for all three options identified by SA2 only have stage-3 impact on RAN3 specification, the impact is rather comparable and rather small.

Proposal 7: From RAN3’s perspective, all three options to include the WAB-MT’s location information in the UE’s ULI are feasible, and that RAN3 prefers to leave further down-selection to SA2.

On suspension of NG

Proposal 8: Since the removal of NG connections can be left up to normative phase, the Editor’s Note “FFS whether NG connections can be suspended” to be deleted.

Other issues

Proposal 9-1: Capture the above proposals in TR 38.799 and remove all remaining editor’s notes.

Proposal 9-2: Agree the TP to TR 38.799 in the Annex.

# Reference

[1]  Chair notes, TSG RAN WG3 meeting #124, Fukuoka, Japan, May 2024

# Annex

TP to TR 38.799 on Study for Additional Topological Enhancements.

Start of Change

## 4.3	Operational aspects

### 4.3.1	WAB-node Integration procedure

Figure 4.3.1-1 WAB-node integration procedure

Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling.

Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases:

Phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is configured by the OAM (e.g., with the information of AMF(s) to serve the UE) and service-authorized by the SeGW or by the OAM.

Phase 2-2: NG connection setup. The WAB-gNB establishes NG connection(s) toward the AMF(s). This step may follow legacy procedures. After the NG is set up, the WAB-gNB can start serving UE(s).

Phase 2-3: Xn connection setup. If needed, the WAB-gNB may establish Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s).

### 4.3.2	WAB authorization

WAB authorization includes the authorization of the WAB-MT and the service authorization of the WAB-gNB. The authorization of the WAB-MT is different from the service authorization/configuration/activation of the WAB-gNB.

Authorization of the WAB-MT provides the WAB-MT with the right to support backhauling the co-located WAB-gNB’s traffic via BH PDU session(s).

Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs. The service authorization of the WAB-gNB is performed by e.g., OAM/SeGW using legacy procedures.

The WAB-gNB’s service authorization status may change. In case the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”, the UEs served by the WAB-gNB can either be handed over to other RAN nodes or they can be released, after which the NG and Xn connection(s) of the WAB-gNB can be removed.

### 4.3.3 Configuration of WAB-node

Certain configurations of the WAB-node may need to be updated as the node moves, e.g.:

- The parameters that enable the WAB-gNB to select and connect to the AMF(s) to serve the UE(s).

- The parameters that enable the WAB-gNB to connect to, and communicate with, the OAM system.

- The configuration parameters that the WAB-gNB should broadcast, e.g., the TAC(s), the cell ID(s), the RANAC(s).

A WAB-node may be provisioned with the parameters pertinent to different potential locations of the WAB-node.

Alternatively, the OAM can provision configuration parameters to the WAB-node based on the location of the node. In that case the continuity of OAM connectivity needs to be ensured as the WAB-node moves.

#### 4.3.3.1 IP address configuration for WAB-gNB

A WAB-MT obtains IP address(es) for the PDU sessions in the same manner as a legacy UE.

The WAB-gNB can use the IP address(es) of the WAB-MT for the PDU sessions that backhaul the NG, Xn and OAM traffic. The WAB-gNB supports security protection of NG and Xn via IPsec, as defined by TS 33.501.

In case the WAB-gNB uses the IPsec tunnel mode to protect the OAM, NG and/or Xn traffic, the allocation of the inner tunnel IP address(es) is outside of 3GPP scope.

It is possible to transport OAM, NG or Xn traffic over other types of tunnel protocols on top of the WAB-MT’s PDU session(s), e.g., such as L2TP. In this case, the WAB-gNB uses different IP address(es) from WAB-MT. Since the support of these tunnel protocols is not defined for NG and/or Xn, such tunnel protocols are out-of-scope of this study.

#### 4.3.3.2	TAC/RANAC (re-)configuration for WAB-gNB’s cell

The TAC/RANAC of WAB-gNB’s cell is configured by the OAM, and it can be reconfigured by the OAM during the mobility of WAB-node. The TAC/RANAC of the WAB-gNB’s cell may be the same as, or different than, the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcast by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location.

### 4.3.4	Mobility handling

The following scenarios for WAB-node mobility are supported:

- The UE´s AMF/UPF remains unchanged as the WAB-gNB moves.

- The UE´s AMF/UPF changes as the WAB-gNB moves.

- The BH UPF/AMF remains unchanged as the WAB-MT moves inside a PLMN.

- The BH UPF/AMF changes as the WAB-MT moves inside a PLMN.

RAN3 assumes that the WAB-gNB does not change PLMN while the WAB-node moves across the network.

#### 4.3.4.1	WAB-MT mobility

The WAB-MT reuses legacy mobility handover procedures for the UE as it moves through the BH-RAN.

##### 4.3.4.1.1 The BH-UPF/BH-AMF remains unchanged as the WAB-MT moves inside a PLMN.

In case the WAB-MT’s PSA UPF does not change during these mobility procedures, the IP addresses allocated for the WAB-MT’s PDU sessions do not change. Therefore, the NG and Xn connections of the WAB-gNB carried over the BH PDU session(s) remain unaffected.

##### 4.3.4.1.2 BH-UPF/BH-AMF changes as the WAB-MT moves inside a PLMN.

When the WAB-MT’s PSA UPF need to be changed due to WAB-MT’s mobility, new BH PDU session(s) need to be established reusing existing mechanisms defined in TS 23.501/TS 23.502. In this case, the (outer) IP addresses used by the WAB-gNB for the transport of NG and Xn connections of the WAB-gNB will change.

##### 4.3.4.1.3 Roaming of the WAB-MT

During WAB-node movement, the WAB-MT may connect to a PLMN different than its HPLMN.

#### 4.3.4.2	WAB-gNB mobility

During WAB-node movement, establishment, and removal of the WAB-gNB’s NG and/or Xn connections may be needed.

Establishment of Xn connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported, and it can follow legacy procedures.

##### 4.3.4.2.1 WAB-gNB mobility without change of UE’s AMF(s)

During WAB-node movement, radio configuration parameters of the WAB-gNB may be changed (e.g., cell ID, PCI and/or TAC) without the change of the UE’s AMF(s). This change of radio configuration parameters may require the UE handling by means of, e.g., intra-gNB handover and/or Mobility Registration Update as defined in TS 23.502.

##### 4.3.4.2.2 WAB-gNB mobility with change of UE’s AMF(s)

Due to WAB-node movement, the change of UE’s AMF(s) may be needed, based on, e.g., WAB-node’s current location and/or additional criteria. The NG connection handling and WAB-gNB configuration update may affect the served UEs.

- The WAB-node may obtain the configuration parameters needed to establish the connection to the UE’s new AMF(s).

- The WAB-node establishes a second logical WAB-gNB, which initiates establishment ofNG connection(s) towards the one or more new AMF(s).

- The second logical WAB-gNB may activate one or more new cells, with new cell configuration parameters related to the WAB-gNB’s current location. The new cells may broadcast the radio parameters configured for the new AMF(s), e.g., TAC, etc. The TAC broadcast by the cell(s) of the second logical WAB-gNB should be different from the TAC broadcast by the initial logical WAB-gNB. The cell(s) on the initial logical WAB-gNB remain(s) active.

- The UEs are handled as follows:

- A UE in RRC_CONNECTED state is handed over from the cell served by the initial logical gNB to a cell served by the second logical WAB-gNB via NG-based handover, as defined in TS 23.502. During this handover, the CN performs AMF reallocation for the UE.

- When all UEs in RRC_CONNECTED state have been handed over, the cell(s) of the initial logical WAB-gNB are removed from service. A UE in RRC_IDLE or RRC_INACTIVE state camping on the old cell(s) reselects a cell from the second logical WAB-gNB.  Due to the change of TAC broadcasted by the second logical WAB-gNB cell(s), the UE initiates a Mobility Registration Update procedure as defined in TS 23.502, in which the UE’s AMF is changed.

- The initial logical WAB-gNB removes the NG connection(s) to its AMF(s).

### 4.3.5 Resource multiplexing

In scenarios where WAB-node’s access link and backhaul link mutually interfere, resource coordination may be needed to facilitate the resource multiplexing of the WAB-node’s access links and backhaul link. For this purpose, the resource coordination mechanism introduced for IAB can be considered as the starting point. For resource coordination between the access link and backhaul link, the BH-gNB may need to discover co-location of the WAB-MT and the WAB-gNB.

### 4.3.X Avoiding multi-hop WAB backhauling

During WAB-MT network integration, it should be avoided that the WAB-MT connects to another WAB-gNB. During WAB-node mobility, it should be avoided that a WAB-MT is handed over to the collocated WAB-gNB or another WAB-gNB. The following RAN-based options can be considered:

Option 1: Implementation-based solution, e.g., the WAB-gNB uses dedicated frequencies and/or PCI partitions, which are avoided by the WAB-MT for cell-reselection and measurement reports.

Option 2: WAB-gNB-cells broadcast a “WAB” indicator in SIB1, and the WAB-MT avoids (re)selection or reporting of cells broadcasting this indicator.

Option 3: WAB-MT includes a “WAB” indicator in UE capabilities, and the network rejects a handover request to a WAB-gNB based on this indicator.

On Option 1: This option aligns well with typical WAB use cases and deployment scenario. In out-of-band scenarios, the WAB-MT will not connect to WAB-gNBs. In case WAB is operating in-band with the BH network, PCI portioning may have to be performed for other reasons, e.g., to avoid PCI collisions, as it is already defined for mobile IAB by TS 38.401.

On Option 2: This option requires specification enhancements, but it does not have impact on the BH RAN. This option can be easily applied during WAB-MT integration since the decoding of SIB1 is a prerequisite for initial access. Option 2 has the disadvantage that decoding of SIB1 is necessary for every cell to be included in a measurement report.

On Option 3: This option only applies to prevent the above mobility scenario. It cannot be used to prevent initial access by a WAB-MT to a WAB-gNB. This option requires specification enhancements, but it does not have any impact on the BH RAN.

TS 23.700-06 proposes an AMF-based solution to prevent WAB-MT access to a WAB-gNB. It also proposes a solution to prevent NG-based handover of a WAB-MT to a WAB-gNB. Both solutions have specification impact, but they do not have impact on the BH RAN. These solutions cannot prevent Xn handover of WAB-MT to a WAB-gNB.

Among the above options (and solutions), only Option 1 has no stage-3 impact.

### 4.3.X+1	PCI collision avoidance

WAB can use the same PCI collision avoidance mechanisms as defined for mobile IAB.

### 4.3.X+2 	ULI for UE connected to WAB-gNB

When the WAB-gNB moves across the backhaul network, the UE’s CN has to retain appropriate information on the UE’s location. For that purpose, TS 23.700-06 clause 8.5 provides three options to include information of the WAB-MT into the UE’s ULI. Any of these three options are feasible from RAN3 perspective. The down-selection among these options can occur during normative phase, and it should be left to SA2.

### 4.3.X+3 	QoS support on WAB backhaul

TS 23.700-06 clause 6.3 discusses solutions to the QoS support on the WAB backhaul. These solutions can be applied without RAN impact.

End of Change