3GPP TSG-RAN WG3 Meeting #125                                                              R3-244142

Maastricht, The Netherlands, May 19 – 23, 2024

Agenda item: 		12.2

Source: 		Qualcomm Incorporated

Title: 	(TP to TR 38.799) Conclusion of WAB study

Document for:		Other

# 1	Introduction

RAN3#125 is the final meeting for the SI FS_WAB_5GFemto_NR [1]. This contribution discusses the conclusion of the study on WAB and the recommendations for normative phase.

# 2	Discussion

The study has discussed all aspects of WAB defined in the SID [1]. Based on the study, RAN3 can conclude that all issues identified for WAB can be resolved confirming feasibility of the WAB functionality from RAN3 perspective.

Proposal 1: Based on the study, RAN3 can confirm the feasibility of WAB functionality from RAN3 perspective.

The study further identified various stage-2 and a potentially a few stage-3 issues that require specification. It can therefore be concluded that from RAN3 perspective, a normative phase for WAB should be pursued.

Proposal 2: RAN3 concludes that a normative phase for WAB should be pursued to capture various aspects identified in the study.

The normative work should be based on the terminology and requirements captured in TR 38.799. This aspect of the normative work should be captured in the recommendation for the normative phase.

Proposal 3: The normative work for WAB should be based on the terminology and requirements captured in TR 38.799.

The recommendation for the normative phase should include the following architectural aspects identified in the study phase and captured in TR 38.799, including:

- Backhauling of the WAB-gNB NG and Xn interface is conducted over the WAB-MT’s PDU session.

- The WAB-gNB can have Xn interfaces with the WAB-MT’s serving gNB and with other surrounding gNBs.

- The interface between WAB-MT and WAB-gNB as well as interface between WAB-gNB-CU and WAB-gNB-DU are out-of-scope for the normative phase.

Proposal 4: The normative work should consider the following architectural aspects for WAB according to TR 38.799, including:

- Backhauling of the WAB-gNB interface(s) is conducted over the WAB-MT’s PDU session, where the Xn interfaces can be established with the WAB-MT’s serving gNB and with other surrounding gNBs.

- The interface between WAB-MT and WAB-gNB as well as interface between WAB-gNB-CU and WAB-gNB-DU are out-of-scope for the normative phase.

For the configuration of the WAB-gNB, the normative work should consider the configuration aspects identified in the study phase:

- The configuration of the WAB-gNB follows legacy procedures used for configuration of gNBs.

- The configuration of the WAB-UE follows legacy procedures used for configuration of UEs.

Proposal 5: The recommendation for normative phase so include that WAB-gNB configuration can use legacy procedures defined for gNBs and the WAB-MT configuration can use legacy procedures defined for UEs.

The recommendation for the normative phase should capture the authorization aspects for WAB based on the study phase:

- The WAB-MT can be authorized for the support of backhauling, where the WAB-MT authorization procedure is decided by SA2.

- The WAB-gNB can be authorized to provide service to UEs. WAB-gNB service authorization uses legacy procedures.

- In case the authorization of the WAB-MT and/or WAB-gNB changes to “non-authorized”, the WAB-gNB attempts to hand over UEs and it then removes NG and Xn interface(s). The procedures for removal of NG and/or Xn to be discussed during normative phase.

Proposal 6: The normative phase to establish authorization procedures for WAB, where:

- The WAB-MT can be authorized to support backhauling using a procedure to be defined by SA2, and the WAB-gNB can be authorized to provide serve to UEs using legacy procedures.

- In case the authorization of the WAB-MT and/or WAB-gNB changes to “non-authorized”, the WAB-gNB attempts to hand over all UEs and then removes NG and Xn interfaces, where the procedures for interface removal are to be discussed during normative phase.

The recommendation for normative phase should also include the network integration procedure for the WAB-node based on the study, where the WAB-node network integration includes the integration of the WAB-MT followed by the establishment of the WAB-gNB’s NG (and potentially Xn) interfaces.

Proposal 7: The normative phase to define a network integration procedure for the WAB-node where WAB-node network integration includes the integration of the WAB-MT followed by the establishment of the WAB-gNB’s NG (and potentially Xn) interfaces.

The recommendation for normative phase should explicitly allow the UE connecting to the WAB-gNB to conduct all legacy UE mobility procedures and the WAB-MT connecting to the BH RAN to support all legacy UE mobility procedures.

Proposal 8: The WAB procedures defined in normative phase should allow the UE connecting to the WAB-gNB to conduct all legacy UE mobility procedures and the WAB-MT connected to the BH RAN to support all legacy UE mobility procedures.

The recommendation for normative phase should further include the following aspects related to WAB-node mobility captured in TR 38.799:

- In case the IP address(es) on the backhaul PDU session(s) change, e.g., due to WAB-node mobility, the NG interface can be migrated to the new IP addresses using legacy procedures.

- In case the WAB-gNB’s AMF needs to change, the WAB-node may create a new logical WAB-gNB connected to the new AMF, and it then initiates NG handover for all UEs from the initial to the new logical WAB-gNB.

Proposal 9: The procedures to be defined for WAB-node mobility to allow change of the IP address(es) used on the backhaul PDU sessions as well as change of the UE’s AMF to change. In case of AMF change, the WAB-node to establish a second logical WAB-gNB and conduct NG handover of all UEs as defined in TR 38.799.

The recommendation for normative phase may include enhancements to the UE’s ULI to include information related to the WAB-MT’s location. These enhancements to be discussed in coordination with SA2.

Proposal 10: The normative phase to include enhancements to the UE’s ULI to include information related to the WAB-MT’s location. The enhancements to be discussed in coordination with SA2.

The recommendation for normative phase to include procedures on stage 2 that need to be supported for WAB, but that can be supported via implementation, such as:

- PCI collision avoidance.

- Reconfiguration of TAC on WAB-gNB.

- Avoidance of multi-hop WAB.

- Radio-resource coordination between access and backhaul links.

Proposal 11: The normative phase to capture procedures on stage-2 that can be supported via implementation:

- PCI collision avoidance.

- Reconfiguration of TAC on WAB-gNB.

- Avoidance of multi-hop WAB.

- Radio-resource coordination between access and backhaul links.

The recommendation for normative phase may include aspects that may further discussions, time permitting, and which are related to:

- Need for enhancements to prevent multi-hop WAB.

- Need for enhancements to support radio-resource coordination between in-band operation of access and backhaul links.

Proposal 12: The normative phase to include, time permitting, discussions on enhancements related to the prevention of multi-hop WAB and the support of radio-resource coordination between in-band operation of access and backhaul links.

The Annex provides a TP to TR38.799 for the conclusion on study on WAB.

Proposal 13: RAN3 to agree to the TP to TR38.799 in the Annex.

# 3	Conclusion

This contribution discussed the conclusion of the study on WAB and the recommendations for normative phase. The following observations and proposals have been made:

Proposal 1: Based on the study, RAN3 can confirm the feasibility of WAB functionality from RAN3 perspective.

Proposal 2: RAN3 concludes that a normative phase for WAB should be pursued to capture various aspects identified in the study.

Proposal 3: The normative work for WAB should be based on the terminology and requirements captured in TR 38.799.

Proposal 4: The normative work should consider the following architectural aspects for WAB according to TR 38.799, including:

- Backhauling of the WAB-gNB interface(s) is conducted over the WAB-MT’s PDU session, where the Xn interfaces can be established with the WAB-MT’s serving gNB and with other surrounding gNBs.

- The interface between WAB-MT and WAB-gNB as well as interface between WAB-gNB-CU and WAB-gNB-DU are out-of-scope for the normative phase.

Proposal 5: The recommendation for normative phase so include that WAB-gNB configuration can use legacy procedures defined for gNBs and the WAB-MT configuration can use legacy procedures defined for UEs.

Proposal 6: The normative phase to establish authorization procedures for WAB, where:

- The WAB-MT can be authorized to support backhauling using a procedure to be defined by SA2, and the WAB-gNB can be authorized to provide serve to UEs using legacy procedures.

- In case the authorization of the WAB-MT and/or WAB-gNB changes to “non-authorized”, the WAB-gNB attempts to hand over all UEs and then removes NG and Xn interfaces, where the procedures for interface removal are to be discussed during normative phase.

Proposal 7: The normative phase to define a network integration procedure for the WAB-node where WAB-node network integration includes the integration of the WAB-MT followed by the establishment of the WAB-gNB’s NG (and potentially Xn) interfaces.

Proposal 8: The WAB procedures defined in normative phase should allow the UE connecting to the WAB-gNB to conduct all legacy UE mobility procedures and the WAB-MT connected to the BH RAN to support all legacy UE mobility procedures.

Proposal 9: The procedures to be defined for WAB-node mobility to allow change of the IP address(es) used on the backhaul PDU sessions as well as change of the UE’s AMF to change. In case of AMF change, the WAB-node to establish a second logical WAB-gNB and conduct NG handover of all UEs as defined in TR 38.799.

Proposal 10: The normative phase to include enhancements to the UE’s ULI to include information related to the WAB-MT’s location. The enhancements to be discussed in coordination with SA2.

Proposal 11: The normative phase to capture procedures on stage-2 that can be supported via implementation:

- PCI collision avoidance.

- Reconfiguration of TAC on WAB-gNB.

- Avoidance of multi-hop WAB.

- Radio-resource coordination between access and backhaul links.

Proposal 12: The normative phase to include, time permitting, discussions on enhancements related to the prevention of multi-hop WAB and the support of radio-resource coordination between in-band operation of access and backhaul links.

Proposal 13: RAN3 to agree to the TP to TR38.799 in the Annex.

# Reference

[1]  RP-240319, Revised SID on Study on additional topological enhancements for NR, TSG RAN Meeting #103, Maastricht, Netherlands, March 2024

# Annex

TP to TR 38.799 on Study for Additional Topological Enhancements.

Start of Change

# X	Conclusion of Study

### X.1	Conclusion on Study of WAB

The study has discussed all aspects of WAB defined in the SID [1]. Based on the study, RAN3 can conclude that all issues identified for WAB can be resolved confirming feasibility of the WAB functionality from RAN3 perspective.

The study further identified various aspects that require specification. It can therefore be concluded that from RAN3 perspective, a normative phase for WAB should be pursued.

The following is recommended for the normative phase of WAB:

- The normative work should apply the same terminology and requirements captured in TR 38.799.

- The WAB architectural should follow support the following aspects:

  - Backhauling of the WAB-gNB NG and Xn interface is conducted over the WAB-MT’s PDU session.

  - The WAB-gNB can have Xn interfaces with the WAB-MT’s serving gNB and with other surrounding gNBs.

  - The interface between WAB-MT and WAB-gNB as well as interface between WAB-gNB-CU and WAB-gNB-DU are out-of-scope for the normative phase.

- The configuration of the WAB-node should follow these procedures:

  - The configuration of the WAB-gNB follows legacy procedures used for configuration of gNBs.

  - The configuration of the WAB-UE follows legacy procedures used for configuration of UEs.

- Authorization of the WAB node should include the following procedures:

  - The WAB-MT can be authorized for the support of backhauling, where the WAB-MT authorization procedure is decided by SA2.

  - The WAB-gNB can be authorized to provide service to UEs. WAB-gNB service authorization uses legacy procedures.

  - In case the authorization of the WAB-MT and/or WAB-gNB changes to “non-authorized”, the WAB-gNB attempts to hand over UEs and it then removes NG and Xn interface(s). The procedures for removal of NG and/or Xn to be discussed during normative phase.

- The WAB node integration into the network should includes the integration of the WAB-MT followed by the establishment of the WAB-gNB’s NG (and potentially Xn) interfaces.

- The WAB-gNB can support all legacy mobility procedures for UEs. The WAB-MT can support all legacy mobility procedures. The following mobility-related aspects are supported by WAB:

  - In case the IP address(es) on the backhaul PDU session(s) change, e.g., due to WAB-node mobility, the NG interface can be migrated to the new IP addresses using legacy procedures.

  - In case the WAB-gNB’s AMF needs to change, the WAB-node may create a new logical WAB-gNB connected to the new AMF, and it then initiates NG handover for all UEs from the initial to the new logical WAB-gNB.

- The WAB-gNB includes information on the WAB-MT’s location with the UE’s ULI. The format of this information remains to be discussed during the normative phase in coordination with SA2.

- The following WAB functionalities can be supported via implementation and should be captured on stage-2:

  - PCI collision avoidance.

  - Reconfiguration of TAC on WAB-gNB.

  - Avoidance of multi-hop WAB.

  - Radio-resource coordination between access and backhaul links.

- The WAB-gNB cannot serve a WAB-MT. The normative phase should include stage-2 procedures how this can be avoided via implementation. Time-permitting, further enhancements can be discussed.

- Access and backhaul links can be out-of-band. In-band operation of access and backhaul link can only be supported in case the backhaul link uses a TN. Whether in-band operation is supported and whether enhancements are needed for such support remains to be discussed during the normative phase, time permitting.

End of Change