3GPP TSG-RAN WG3 Meeting #125                                                              R3-244144

Maastricht, The Netherlands, August 19 – 23, 2024

Agenda item: 		12.3

Source: 		Qualcomm Incorporated

Title: 	Support for local services with UPF at NR Femto

Document for:		Discussion

# 1	Introduction

This contribution discusses the following objective related to NR Femto defined in the SID for Additional Topological Enhancements [1]:

- Clarify the access to local services from the 5G Femto via collocated local UPF and identify issues, if any [RAN3].

# 2	Discussion

## 2.1 Requirements for support of local UPF at NR Femto

To clarify how local services can be accommodated at the NR Femto via local UPF, it is imperative to define the underlying requirements.

One requirement certainly is the availability of an architectural solution that provides access to local services via a UPF collocated with the NR Femto,

Another aspect is whether this architecture solution can scale to a large number of Femtos with local UPF, whether enhancements are needed to support such scalability, and whether independent scaling and/or aggregation of C-plane and U-plane interfaces is needed for the local UPF. This discussion should follow analogue to that conducted for NG and Xn interfaces before.

The architecture should further be scrutinized whether robustness to frequent turn on/off can be provided for the local UPF(s).

Proposal 1: The following requirements to be met for the support of access to local services at the NR Femto:

(1) Availability of an architecture solution for access to local services via a local UPF at the NR Femto,

(2) Scalability of this architecture solution to a large quantity of NR Femtos with collocated UPF,

(3) Robustness of the local UPF to frequent turn on/off.

## 2.2 Architectural solution for local services with UPF at NR Femto

TS 23.548 defines 5G System Enhancements for Edge Computing. These specifications should be applicable at the NR Femto. Figure 1 shows 5GC Connectivity Models for Edge Computing based on TS 23.548 clause 4.3. Based on this figure, the UE may have a dedicated PDU session to the local DN via the local UPF at the Femto (Distributed Anchor Point). Alternatively, it may have a local breakout point at the Femto, which splits the PDU session into flows that use the local UPF at the Femto vs. a remote UPF at the 5GC (Session Breakout). The breakout point uses a separate UPF for which supports the UL Classifier/Bridging Point. It should be expected that the local UPF at the NR Femto can support both scenarios.

Figure 1: 5GC Connectivity Models for Edge Computing based on TS 23.548 clause 4.3

TS 23.501 clause 5.6.4 defines the single PDU session with breakout session scenario in more detail. Figure 2 shows the architecture for this solution. The figure indicates the NFs that need to be collocated with the NR-Femto.

Figure 2: User plane Architecture for the Uplink Classifier based on TS 23.501 clause 5.6.4

Proposal 2: RAN3 to confirm that a PDU Session with a single distributed anchor point at the at the NR Femto as well as a PDU Session with session breakout point at the NR Femto can be supported in compliance with 23.548 clause 5.6.4.

## 2.3 Scalability of architecture solution to large number of Femtos with local UPF

It is expected that NR Femtos are subscriber-deployed devices that are deployed in large quantity. It can be expected that the local UPF(s) used for local data sessions is (are) integrated with the NR Femto. Consequently, the same issues discussed for the NG interface of the Femto gNB (NG architecture options 1, 2, and 3 in TR 38.799) or F1 interface of Femto gNB-DU (NG architecture option 4 in TR 38.799) should also be considered for the interfaces exposed by the local UPF to the CN.

The local UPF exposes the N4 C-plane interface to the CN. It further exposes NG-U, N6 interfaces and potentially N9 U-plane interfaces.

Among those, only the U-plane interface of the UPF carrying the UL Classifier needs to be aggregated across a large quantity Femtos, and therefore, only the scalability of this N9 interface needs to be considered.

For the scalability of the N4 and N9 interfaces, the following issues come to mind:

- Scalable solution to discovery and selection of the local UPF by the SMF (N4 interface only).

- Scalable aggregation of a large number of N4 and N9 interfaces

Observation 3-1: To support a scalable solution for local UPFs, the following aspects need to be considered:

- Scalable solution to discovery and selection of the local UPF by the SMF (N4 interface only).

- Scalable aggregation of large number of N4 and N9 interfaces

### 2.3.1 Scalable solution to discovery and selection of local UPF by SMF

To enjoy local services via local UPF, the SMF of a UE connected to an NR Femto needs to be able to find the UPF(s) that are co-located with this UE’s NR Femto. The SMF further needs to be aware that this UPF cannot be used for UEs connected to other NR Femtos or to regular gNBs. To make such discovery/selection procedures scalable, they need to be provided via an automated mechanism that can furnish the SMF with the necessary information across a large number of NR Femtos with local UPFs.

Observation 3-2: For scalable discovery/selection of the local UPF, an automated mechanisms needs to be in place that allows the SMF to identify the local UPF that is collocated with the NR Femto the UE is connected to.

Such a mechanism can be provided via NRF. The local UPF registers with the NRF and includes information, which is specific to the collocated NR Femto, and which is also provided by the UE connected to this Femto. In this manner, the NRF can map the local UPF to the UE connected to the UPF’s collocated NR Femto.

Following TS 23.501 clause 6.3.3.3, the UE location can serve for this purpose. UPF selection via NRF can occur based on UE location. The UE location is provided to the SMF via the UE’s AMF and it can also be included in the local UPF’s registration with the NRF. The UE location includes the NR Femto’s cell ID and TAC, which are unique to the NR Femto. In this manner, the NRF can unambiguously map the UE’s location to the UPF that is collocated with the UE’s NR Femto cell.

Proposal 3-1: RAN3 to confirm that scalable discovery/selection of a local UPF collocated with an NR Femto can be facilitated via the NRF, where the UE’s location, i.e., the NR Femto’s cell ID and TAI, is used to select the UPF.

### 2.3.2  Aggregation of large number of N4 and N9 interfaces

RAN3 performed an extensive discussion on architecture options for the aggregation of a large number of NG interfaces and Xn interfaces. For the support of local UPFs, it needs to be clarified if the same aspects need to be considered for the N4 interface, and if needed, an equivalent discussion on N4 aggregation options should be conducted.

For the N9 interface exposed by the local UPF to the global UPF, the same aggregation aspects can be expected to apply as for the NG-U interface of NR Femtos without local UPF. In analogy to the discussion for NG-C and NG-U, separate scalability of C-plane and U-plane may have to be considered for N4 and N9.

Proposal 3-2: RAN3 to discuss whether aspects related to scalable aggregation of NG-C and NG-U interfaces of the NR Femto should also be considered for the N4 and N9 interfaces of the local UPF.

## 2.4  Impact of frequent turn on/off of N4 interface for local UPF

For the NG-C interface, frequent turn on/off was considered a criterion in the evaluation and comparison of NG architecture options 1 to 3. RAN3 needs to discuss whether any issues related to frequent turn on/off of the NG-C interface will also apply to the N4 interface.

Proposal 4: RAN3 to discuss whether any issues related to frequent turn on/off of the NG-C interface will also apply to the N4 interface.

# 3	Conclusion

This contribution discussed aspects related to local services use a local UPF that is collocated with the NR Femto. The following observations and proposals have been made:

Requirements

Proposal 1: The following requirements to be met for the support of access to local services at the NR Femto:

(1) Availability of an architecture solution for access to local services via a local UPF at the NR Femto,

(2) Scalability of this architecture solution to a large quantity of NR Femtos with collocated UPF,

(3) Robustness of the local UPF to frequent turn on/off.

Architecture solution

Proposal 2: RAN3 to confirm that a PDU Session with a single distributed anchor point at the at the NR Femto as well as a PDU Session with session breakout point at the NR Femto can be supported in compliance with 23.548 clause 5.6.4.

Scalability of architecture solution

Observation 3-1: To support a scalable solution for local UPFs, the following aspects need to be considered:

- Scalable solution to discovery and selection of the local UPF by the SMF (N4 interface only).

- Scalable aggregation of large number of N4 and N9 interfaces

Observation 3-2: For scalable discovery/selection of the local UPF, an automated mechanisms needs to be in place that allows the SMF to identify the local UPF that is collocated with the NR Femto the UE is connected to.

Proposal 3-1: RAN3 to confirm that scalable discovery/selection of a local UPF collocated with an NR Femto can be facilitated via the NRF, where the UE’s location, i.e., the NR Femto’s cell ID and TAI, is used to select the UPF.

Proposal 3-2: RAN3 to discuss whether aspects related to scalable aggregation of NG-C and NG-U interfaces of the NR Femto should also be considered for the N4 and N9 interfaces of the local UPF.

Impact of frequent turn on/off

Proposal 4: RAN3 to discuss whether any issues related to frequent turn on/off of the NG-C interface will also apply to the N4 interface.

# Reference

[1]  RP-240319, Revised SID on Study on additional topological enhancements for NR, TSG RAN Meeting #103, Maastricht, Netherlands, March 2024