3GPP TSG-RAN WG3 Meeting #125                                                              R3-244145

Maastricht, The Netherlands, August 19 – 23, 2024

Agenda item: 		12.3

Source: 		Qualcomm Incorporated

Title: 	Conclusion of study on NR Femto

Document for:		Discussion

# 1	Introduction

RAN3#125 is the final meeting for the SI FS_WAB_5GFemto_NR [1]. This contribution discusses the conclusion of the study on NR Femto and potential recommendations for normative phase.

# 2	Discussion

## 2.1  RAN architecture for deployment of NR Femtos

The study on NR Femto focused on scalable aggregation of NG and Xn for integration of a large quantity of NR Femto cells. For NG integration, four architecture solutions were considered and compared. For Xn integration, two architecture solutions were considered and compared. The architectures and comparisons have been captured in TR 38.799.

For the architecture solutions considered, the last meetings indicated that consensus was evolving among companies:

- For NG aggregation, only architecture options 1 and 2 should be considered. There was not a lot of support for options 3 and 4.

  - Architecture option 1 represents the legacy Rel-15 architecture, and it therefore does not require any normative work.

  - Architecture option 2 introduces the Femto GW to NR. The functionality of this GW would essentially be the same as that defined for the 4G Femto GW. Accordingly, the normative phase would mainly perform copy-and-paste from 4G to 5G specification.

- For Xn aggregation, only architecture option A should be considered. There was not a lot of support for architecture option B. One reason is that the specification of the X2 Femto GW, which is equivalent to Option B, was quite time consuming. Also, companies seemed to agree that Xn aggregation did not seem to be as necessary as NG aggregation.

  - Architecture option A represents the legacy Rel-15 architecture, and it therefore does not require any normative work.

Proposal 1-1: Based on the architecture study so far, only NG aggregation via Femto GW needs to be considered for normative phase.

The decision on whether a normative phase for the definition of a Femto GW should be pursued depends on operator support.

Proposal 1-2: The decision on whether a normative phase for the definition of a Femto GW should be pursued depends on operator support.

## 2.2  Access control mechanisms for NR Femtos

The study included discussion of access control. RAN3 reached a few agreements on this topic. These agreements did not indicate the need for enhancements. The outcome of this discussion has not yet been captured in TR 38.799.

Proposal 2: Based on the study, the NR Femto can reuse legacy access control mechanisms. There is no need for normative work related to access control mechanisms for NR Femtos.

## 2.3  Access to local services for NR Femtos

The support of local traffic via UPF at the NR Femto represents one of the objectives in the SID, but this objective has not yet been discussed. Whether enhancements are needed to support access to local services via UPF at NR Femto needs to be decided in this outstanding discussion.

Proposal 3: The study on access to local services via UPF at NR Femto still needs to be conducted. Whether this results in normative work depends on the outcome of this study.

## 2.4  Other issues

Normative work for the support of NR Femto should preclude impact on the UE.

Proposal 4: Normative work for the support of NR Femto should preclude impact on the UE.

# 3	Conclusion

This contribution discussed the conclusion of the study on NR Femto. The following proposals have been made:

Proposal 1-1: Based on the architecture study so far, only NG aggregation via Femto GW needs to be considered for normative phase.

Proposal 1-2: The decision on whether a normative phase for the definition of a Femto GW should be pursued depends on operator support.

Proposal 2: Based on the study, the NR Femto can reuse legacy access control mechanisms. There is no need for normative work related to access control mechanisms for NR Femtos.

Proposal 3: The study on access to local services via UPF at NR Femto still needs to be conducted. Whether this results in normative work depends on the outcome of this study.

Proposal 4: Normative work for the support of NR Femto should preclude impact on the UE.

# Reference

[1]  RP-240319, Revised SID on Study on additional topological enhancements for NR, TSG RAN Meeting #103, Maastricht, Netherlands, March 2024